IEEE 802.11

Wireless Access Method and Physical Specification

title:	Framework Update for 2 Mbit/s DSSS PHY

date:	May 5, 1993

author:	Jan Boer

NCR Wireless Communications and Networking Division

Zadelstede 1-10

3431 JZ Nieuwegein-Holland

Telephone: (+31) 3402-76483

Fax: (+31) 3402-39125

E-mail: jan.boer@utrecht.ncr.com

INTRODUCTION

This contribution is an update of the framework presented in the appendix of document IEEE P802.11-93/37 (Proposal for 2 Mbit/s DSSS PHY). Some  changes are  based on the discussions held in the March IEEE 802.11 meeting. Further I have the parameters (items) regrouped.

Where different parameters are applicable for Frequency Hopping and Direct Sequence I split the parameter and gave those parameter numbers a suffix DS or FH. In this way it is possible to merge the two frameworks in the course of the proces.

SPECIFICATION

|  | Parameter | Proposed Spec Direct sequence | Comments/Notes |
| --- | --- | --- | --- |

| 1 | Frequency range | 2.4 to 2.4835 GHz (USA) 2.5-2.5 GHz (ETSI) 2.471-2.497 (Japan) | Also the 915 MHz ISM band is usable for DS |
| --- | --- | --- | --- |
| 2 DS | Number of parallel channels | 3 | One channel in Japan More channels is under study |
| 2 FH | Minimum number of hopping channels/set |  |  |
| 3 DS | Spreading sequence | 11 chips | Barker sequence (1,-1,1,1,-1,1,1,1,-1,-1,-1) |
| 3 FH | No. of hops per second |  |  |
| 4 | Data rate | 2 Mbps |  |
| 5 | Fall back data rate(s) | 1 Mbps |  |
| 6 | Symbol rate | 1 MBaud |  |
| 7 | Modulation | DQPSK |  |
| 8 | Channel bandwidth (allocated) | max 83.5 MHz | FCC part 15.247 (for reference, will be deleted) |
| 9 | Occupied channel bandwidth (spectrum shape) | -30 dBc @ Df=11Mhz -55 dBc @ Df=22Mhz | For specification of the transmitter spectrum mask |
| 10 | Spurious emissions out of band | per FCC part 15.247, 15.205,15.209 in USA per ETSI RES 02-09 in Europe |  |
| 11 | Transmitted power level | Max 1000 mW (USA), Max 100 mW (Europe, Japan) | Regulatory maxima |
| 12 | Minimum power level | 10 mW , 1 mW (battery operated equipment only) | required for conformance testing |
| 13 | Power control | required above 100 mW 3-7 levels (2-3 bits) no absolute accuracy monotonicly decreasing |  |
| 14 | Adjacent channel rejection | 50-55dB | Defines receiver filter characteristics |
| 15 | Tx/Rx Frequency stability | £ 50 ppm |  |
| 16 | Frequency deviation |  | Required for interoperability |
| 17 | Rx center frequency acceptance range |  | for interoperability purposes |
| 18DS | Clock accuracy | £ 50 ppm |  |
| 18 FH | Phy supplied Clock jitter |  |  |
| 19 | Preamble length | 146 symbols (146µs) | DS preamble used for: antenna selection clock recovery phase compensation delimiter detection |
| 20 | Scrambler polynomial | 1+x-4+x-7 |  |
| 21 | Clock recovery | Rx should follow Tx clock |  |
| 22 | Carrier response time | £ 30 µs | Time to switch from receive mode (carrier sense mode) to start transmission plus the time to detect energy (by receiver) |
| 23 | Switching time Tx to Rx | faster then gap between messages | DS: In a CSMA/CA like protocol a Inter Frame Space is defined. Rx function (carrier sensing) must be on before new frame may arrive. |
| 24 | Switching time Rx to Tx | see 22 | time from full sensitivity reception to fukll power tranmitter availablity |
| 25 | BER at specified Eb/No | 10e-6 @ Eb/No = 17 dB | Includes modem implementation margin (4dB) At thermal noise -174 dBm/Hz and man made noise 18 dB above thermal noise: receiver sensitivity is better than -76dBm at this BER. |
| 26 | Channel availability | 99.5% | probability of outage, defined for within coverage area and normal indoor channels |
| 27 | SIR for frame in frame reception | 8dB |  |
| 28 | Antenna port impedance (if exposed) | 50 ohms | for interoperability at antenna port |
| 29 | VSWR | Devices shall stand 0£VSWR£¥ with no damage Operational VSWR £2.5 | within 2.4-2.5 band for conformance testing |
| 30 | Interface lines to convergence layer (when exposed) | RX data TX data RX/TX clock RTS/CTS CS (carrier sense) MDA (MAC Data Available) Control line Status line Ctl/Sta clock | Timing and levels TBD |
| 31 | Data Line/clock input/output jitter | TBD | Includes static and dynamic jitter (see 802.3 definition) dependent on MAC requirement. |

| 32 | PHY-MAC Net management info/control variables | a. Signal quality b. Loop back c. No of channels available d. Channel in use(sequence) e. No of diversity channels f. Which diversity in use g. Alternae diversity in use h. Lock i. No of Rx signal strength levels j. Rx signal strength levels k. Rx signal strength l. PHY type m. Channel quality n. Channel available o. No of Tx power levels p. Read Tx output level q. Set Tx output level r. Hop(?) s. Transmit t. Receive u. Sleep mode v. Wake up w. Standby mode (low power) x. Data rate - indication - command y. Jabber control function z. Jabber indication aa. PHY initialize bb. PHY specific tally collection cc. Adaptive power control dd. NID | The interface has to be defined by MAC and PHY group. See also document 92-127 point 41 |
| --- | --- | --- | --- |
| 33 | Safety requirements | compliance with applicable |  |
| 34 | DTE/DCE interface | TBD |  |
| 35 | ACK protocol support | TBD |  |