WMAC Protocol

Comparison against MAC Criteria

Doc: IEEE P802.11-93/96

By : Wim Diepstraten, NCR

Supported Services:

*	Asynchronous Service:

-	Full 802 LLC service with unicast/multicast capability.

-	Broadcast/Multicast reliability depend on medium load.

-	LLC-1 service only mode without Broadcast available for extreme low Power Devices.

*	Isochronous Service:

-	Available as an option, and can mix with Async only implementations.

-	Support mixed Voice/Data applications.

-	Higher speed or Video support depends on PHY speed.

-	BSS overlap dependent on PHY channel isolation.

- Supported configurations:

- *	Infrastructure based network (Async Service)

- -	Default all traffic via AP but Direct Station-to-Station possible.

- -	Single NID per ESA, and unique APID per BSA.

- -	Multiple BSS's can overlap using same channel.

- -	Can overlap with an Ad-Hoc Network on same channel.

- -	Distribution system can be any 802 LAN, also 802.11.

- *	Services provided by AP+Distribution System.

- -	AP contains forward function to other BSS and or a wired Network. This is an expanded "Bridge" function.

- -	(Re-)Association Service provided.

- -	"Initial" and "Previous APID" parameters exchanged to support Re-Association across Routers.

- -	Station to initiate Re-Association process based on "Link Quality".

- -	Synchronization and Power Management provisions included in AP.

- Supported Configurations: (cont'd)

- *	Wireless AP configuration possible.

- -	Single AP without Distribution System connection to expand range.

- -	Allows Wireless AP interconnections (Wireless DS).

- *	Ad-Hoc Networks:

- -	All traffic Direct Station-to-Station.

- -	Assumption that all stations can see each other.

- -	Use separate NID and fixed APID.

- -	Can overlap with Infrastructure Network sharing the same channel.

- -	A Station can be associated with Infrastructure and Ad-Hoc Network at the same time (on a non-Hopping PHY).

- -	Synchronization and Power Management provisions included.

- -	Stations "Listen for Timing Coordinator", and will assume this function when nothing is found.

- Wireless Architecture:

- EMBED MSDraw   \* mergeformatµ §

- *	Allows single and multiple channel overlapping BSA's.

- *	Station can be part of Ad-Hoc and Infrastructure NW.

- Other Configurations:

- EMBED MSDraw   \* mergeformatµ §

- *	Multiple BSS Wireless only networks possible when using the same "Channel".

- Supported Configurations: (cont'd)

- *	Infrastructure Network (Isochronous Service)

- -	Can operate with mixed Async/Isochronous devices.

- -	Isochronous traffic in multiple overlapping BSA's require sufficient channel isolation (limited recovery possible).

- -	Reserved but unused Isochronous Bandwidth is available for Asynchronous traffic without control overhead.

- -	Need separate "Time Bounded" Distribution System to meet time constraints.

- -	Support variable bitrate (Video, ATM backbone).

- -	"Wireless segment" delay depending on PHY speed <=25 msec.

- -	Random service sequence provision allows for time in Station to "Scan for a better AP" during each Frame interval, when "Link Quality" is deteriorating.

- -	Same provision allows for Power Management.

- Performance Characteristics:

- *	Basic CSMA/CA+Ack Access Protocol

- -	Short access delay ideal for Bursty traffic.

- -	Efficient stable throughput at High loads (Ref doc P802.11-92/51)

- -	Shares medium automatically, without additional control overhead.

- -	Ack allows MAC level recovery from lost packets.

- *	Overlapping BSA's

- -	Medium is shared when the same channel is used.

- -	Sharing medium while multiple FH channels collide.

- -	Tx-Power Control provision allows improved spectrum re-use.

- -	Multiple BSA's can not overlap the same channel for Time Bounded Services.

- *	Performance over Distribution System.

- -	Depend on DS access delay,  speed and number of segments.

- -	Performance may depend on Power Save Mode used.

- Performance Example (Asynchr):

- EMBED MSGraph \s  \* mergeformatµ §

- Performance Example (Isochr):

- *	Assumption:	32Kbps ADPCM Voice

- 15 Byte MAC + Wavelan PHY Overhead

- *	Example: @ 2 Mbps Modem speed:

- Voice only:					28 FDX (using talkspurt)

- Voice/Data   576 Byte:		23 FDX (using talkspurt)

- Voice/Data 1500 Byte:		18 FDX (using talkspurt)

- Voice/Data 1500 Byte:		  9  FDX (100% load)

- Async Data throughput:     >  75 KByte/sec (assuming max Isoc load)

- Async Data throughput:     >130 KByte/sec (assuming 9 FDX Talkspurt channels)

- Async Data throughput:     >200 KByte/sec (no Isoc connection active)

- Power Management

- *	Basic Provisions (Infrastructure):

- -	Synchronization provisions within one BSS.

- -	Power Management is application independent.

- -	Stations can operate in different Power Save modes (PM mode indication in Frame Header, and maintained in the AP).

- -	AP provides temporary buffering while Stations sleep.

- -	AP frequently announces buffering status at predictable times.

- -	Stations can dynamically switch PM modes to operate at desired Power/Throughput operating point.

- *	Services provided:

- -	Full LLC service with variable "Initial Response" delays depending on PM mode and parameterization.

- -	LLC-1 service only for Extreme Low Power devices.

- -	Very diverse applications can share the same infrastructure.

- Power Management (cont'd)

- *	Ad-Hoc Network

- -	All stations may choose to operate in Power Save mode.

- -	One station to assume the "Timing Coordination" function using a tbd algorithm, to allow synchronization of the Network.

- -	Transmitters send short traffic announcement frames in a time window where all stations are awake.

- -	Stations receiving a traffic announcement stay awake to receive the data.

- -	Stations can dynamically change PM mode to allow high speed data exchange.

- Power Management (cont'd)

- *	Performance:

- -	Very low Power operation possible during "Idle mode", which is the dominant factor for low power operation.

- -	Increased delay for the initial frame of a session.

- -	Dynamic PM mode switching allows high speed operation for subsequent frames.

- -	Another "Idle mode" Power consumption component that needs to be considered will depend on the re-association frequency and the power consumed during "Scan for a better AP" activity.

- -	A WMAC can be configured to operate at different Speed/Power trade-off points, by controlling the PM mode and timing parameters.

- *	Re-Association speed / Power consumption.

- -	Speed will depend on PHY type and "Scan for better AP" algorithm.

- -	Stations can switch to PSP mode during scan to prevent frame loss.

- Idle Power Consumption example:

- *	Assumptions:	600/150 mA Tx/Rx Power consumption

- -	20/100 msec TIM/DTIM interval

- -	50 ppm Xtal accuracy  (The worst case is shown)

- -	2 Mbps channel (with 64/1088 Byte frame size 60/40% duty cycle)

- Sleep interval

- Avg. Power in mA			100 msec	1 sec	30 sec

- CAM mode:						150	150	150

- PSNP mode synchronized:		  3		  --		  --

- PSNP mode (no synchr):		12		  --		  --

- PSP    mode synchronized:		  3		  .3	 	.016

- PSP    mode (no synchr):		 12		1.2		.041

- Multiple PHY support:

- *	WMAC can be configured for different speeds.

- -	IFS and Backoff protocol parameters are PHY speed dependent (Symbol rate).

- -	WMAC does support mixed bitrates (on same symbol rate).

- *	WMAC can support ISM band PHY's.

- -	Includes provisions for FHopper control.

- -	Basic Synchronization for FHopper already present.

- *	WMAC support for 1.9 GHz PCS band.

- -	LBT and Backoff is very similar to Etiquette, and compliance is achieved by parameter set.

- Multiple PHY support (cont'd):

- *	WMAC support  for others (Wide-Band and IR).

- -	IR:					No limitations known.

- -	ETSI 5.2 GHz:	Is proposed as MAC candidate.

- *	Basic PHY requirements to support WMAC.

- -	Fast Carrier Detect function.

- -	Fast Rx-Tx turnaround times improve efficiency.

- *	MAC provisions to support multiple PHY's.

- -	Synchronization provisions provides FH timing.

- -	Conditional State Machines allow dedicated functions.

- -	Parameters can be PHY dependent.

- -	Intelligence is in the MAC.

- MAC Access Function:

- -	Same Distributed Access Coordination Function for Infrastructure and Ad-Hoc.

- -	No limit on number of stations.

- -	Access mechanism is fair, but close by stations have capture advantage during access collisions.

- -	Good High load stability (use exponential Backoff).

- -	Throughput efficiency similar to CSMA/CD, but depend on Rx-Tx turnaround times and CS delays.

- -	Ack provisions allow MAC level recovery from ISM and co-channel interference sources.

- -	Graceful medium sharing when Networks (and Hops) overlap.

- MAC Access Function:

- -	Async/Time Bounded mix by high priority level.

- -	Reserved but unused Isochronous bandwidth can be fully used by Asynchronous traffic.

- -	Only overhead for mixed operation is one extra slot access delay after a defer to allow Isochronous priority.

- -	Gradual Asynchronous throughput decrease when Time Bounded traffic is active.

- -	Isochronous robustness can be improved when needed by MAC level recovery.

- -	Asynchronous standard can be defined first without the need to fully specify the Time Bounded standard.

- Other MAC Features:

- -	Security on MSDU payload only.

- -	Encryption only on Wireless segment, so no existing network impact.

- -	Allows adoption of basic 802.10 SDE mechanisms.

- -	Element structure in Frame Format allows flexibility for future and proprietary extensions in a transparent way.

- -	Fragmentation provisions are being included.

- -	WMAC supports mixed bitrate, assuming bitrate detection function in the PHY. (allows migration).