The importance of Short Rx-Tx

Turnaround times.

By: Wim Diepstraten

# Turnaround time issue's:

*	Current specifications in the PHY.

-	2 Mbps 	DS							  10 usec

-	1 Mbps	FH							100 usec

-	1.6 Mbps 	FH proposal (Proxim)	200-300 usec

*	Importance addressed by Pablo Brenner (93/109)

-	Determines gap between Data-Ack.

-	Effect of turnaround times on overhead.

-	Applicable to RTS/CTS and Ack schemes.

-	We  are stucked with this in future (Migration).

-	Technology progress can not improve this.

-	Worse for higher speed (more Inefficient)

# Effect on migration:

## *	Technology advantages can not be utilized.

## µ §

# Other effects:

## *	Turnaround time determines CSMA/CA backoff slot time.

## Slot = "CS detect" + Switch + Tx-on + Medium delay

*	Slot time determines "Collision Window"

Longer slot results in higher Collision Probability.

*	Slot time determines Backoff Window.

Longer slot result in large Throughput effect.

Reduces efficiency and increases response times.

# Throughput effect example:

µ §µ §

*	Approx. 28 % Throughput reduction on mixed packet size.

# Slot time effect on throughput:

µ §

*	Higher speed standard extensions will suffer more.

# Background / Tradeoffs:

*	Reason for long turnaround time

-	Relates to Radio architecture.

-	Applicable for Homodyne Tx/ Hetrodyne Rx architecture using single VCO.

-	VCO need to switch every time Tx turns on and off.

-	Requires relative long settling time.

*	What are the tradeoffs?

-	Homodyne pulling problem (especially with integrated antenna), requires isolation investments.

-	Hetrodyne Transmitter with non-switching VCO reduces pulling problem.

-	No switching time so instant Tx rampup.

# Turnaround Time conclusions:

*	Rx-Tx switching time is most important parameter.

*	Tx-Rx switching time determines minimal IFS.

*	Very important for migration.

*	We should focus on performance.

*	Therefore need to stress Rx-Tx turnaround time as much as possible, even when limiting Radio Architectures.

*	Then Tx-Rx will likely become the dominant parameter.

*	Longer preamble is to be preferred over longer turnaround times (no effect on slot time).

# Essential PHY parameters:

*	Carrier Detect response time

Antenna slotting effects

*	Tx-Rx turnaround time

*	Rx-Tx turnaround time

*	Preamble length

# CS response time Parameter:

*	It should not contain Tx-switching time.

*	Will determine the antenna slotting time.

*	Antenna Slotting effects on CS response time.

µ §

*	Advantagous to synchronize slotting on "ED"

# PHY parameter definitions:

*	CS response time:

-	Should not include switching time.

*	Tx-Rx turnaround time.

-	Should not include preamble time

*	Rx-Tx turnaround time.

-	Should not include preamble time

*	PHYpreamble length:

-	This parameter should be added.

Issue:	How to specify slotting parameters.