IEEE P802.11

_________________________________________________________________

EXIRLAN - a multichannel, high speed, medium range IR-Local Area Network

Date:	November 2, 1993

Author:	Peter Blomeyer

AndroMeDa GmbH

Benzstr. 30

82178 Puchheim-Bhf

Phone:xx49-89-8901480

Fax: xx49-89-806040

__________________________________________________________________________________

The basic idea

EXIRLAN is the Physical Layer of an EXpandable InfraRed Local Area Network.

It covers the following requirements of a future oriented IR-PHY-layer:

*	it should be able to coexist with other IR communication devices in the same

room

*	to avoid a limitation of the possible number of terminals, segmentation of the

total room into subareas with one network server per subareas is required.

*	transmission speed must compromise between achievable data rate and

available bandwidth

*	to allow medium range transmission distances (i.e. 30..80 meters) at

acceptable cost and energy consumption, low-cost /high efficiency

IR-emitters have to be used.

Implementation

EXIRLAN uses narrow-band technique with 8 different carriers. The 8 communication channels occupy the high end of the available bandwidth; in the demodulator, the modulated signal is mixed down to 0 MHz and demodulated in the baseband.

The low end of the total bandwidth is reserved for lowest cost, non carriered baseband transmission at reduced data rate; as in this band also most IR-devices can communicate that do not follow the EXIRLAN-standard, this band is refered to as "Coexistence Band".

To achieve maximum possible transmission speed for the narrow band channels, QPSK-modulation is being used; similar techniques such as FQPSK and derivatives are considered if suited IC's are available.

For the Coexistence Band, non-carriered pulse phase modulation (PPM) is being used.

Target specification

Number of bands:			8 narrow band channels, 1 baseband channel

Narrow-band channels:

*	Centre frequencies (SW selectable)			24.....10 MHz

*	Bandwidth/channel						2 MHz

*	Data transmission speed					1Mbit/s

*	Modulation 							QPSK

(change to FQPSK under consideration)

Coexistence Channel:

*	Frequency range						0,6.....5 MHz

*	Data transmission speed				19,2 or 115,2 Kbit/s(selectable)

*	Modulation							PPM

*	Pulse width							200 ns

General

IR emitting diodes

Cut-off frequency							30 MHz

Price/diode at medium quantities					0,40 US$

Communication distance						30..80 meters

HW-implementation: 	Mix of standard, semicustom and full custom IC's

Status of development

*	Definition - and experimental- phase

*	Undependently on the IEEE activities a consortium of semiconductor

munufacturers and equipment manufacturers is being founded

(additional members welcome).

*	Fully functional HW available in Q2'94