IEEE 802.11

Wireless Access Method and Physical Specification

Draft Proposal for a Direct Sequence Spread Spectrum PHY standard

edited by Jan Boer, NCR

This document is a revised verson of document IEEE P802.11-93/83r2 for the direct sequence proposed specification only. It incorporates the changes agreed upon in the West Palm Beach November meeting. The FH specific parameters are omitted in this template, the numbering still refers to doc 83r2.

|  |  | Parameter | Proposed Spec Direct sequence | Comments/Notes | Status |
| --- | --- | --- | --- | --- | --- |

| 1 | T&R | Frequency range | 2.4 to 2.4835 GHz (USA) 2.4-2.5 GHz (Europe) 2.471-2.497 GHz (Japan) | Other frequency bands will follow | closed |
| --- | --- | --- | --- | --- | --- |
| 2b | T&R | Number of channels | 3+2 | See appendix for the definition of the channels | closed |
| 3b | T&R | Spreading sequence | 11 chips Barker sequence (1,-1,1,1,-1,1,1,1,-1,-1,-1) |  | closed |
| 3c | T&R | Data scrambler | polynomial 1+x-4+x-7 | ALL bits transmitted by the PHY shall be scrambled. See appendix for a description of the scrambler | open |
| 4a | Tx | Transmitted power levels [mW] | Max 1000/100 | US/Europe,Japan | closed |
| 4b | Tx | Minimum transmitted power level | 10 mW | Required for conformance testing | closed |
| 5 | Tx | Transmitted power control | up to a maximum of 4 levels required above 100mW |  | closed |
| 6 | Tx | Max. Radiated EIRP | Per FCC part 15.247 in US Per ETSI res 02-09 in Europe Per TBD in Japan | total radiated power including antenna gain, as defined by regulatory agencies in each country. For reference only. | closed |
| 8 | Rx | Receiver minimum input level sensitivity | -80 dBm @ 10-5 BER | At 2 Mb/s, 11 chip code rate at the antenna connector | closed |
| 9 | Rx | Receiver maximum input level | - 4 dBm |  | closed |
| 10b | Rx | Adjacent channel rejection | 40 dB at 10-5 measured by the following method: input an inchannel receive signal level 2dB above the level specified in item 8; set a adjacent channel signal modulated in the same fashion to a level 42dB above the level specified in item 8. The BER shall be better than 10-5. | adjacent channel is defined per group (see 2b) | open |
| 12 | Tx | Occupied channel bandwidth (spectrum shape) | -30 dBc @ Df=±11Mhz -50 dBc @ Df=±22Mhz linear slope between 30 dBc and 50 dBc | Defines transmitted spectrum mask. | closed |
| 13a | Tx | Transmitter Center Frequency tolerance | ±25 ppm Type I: 0 to 40 oC Type II: -30 to 70 oC |  | closed |
| 14 | T&R | Modulation | DBPSK at 1 Mb/s DQPSK and/or OQPSK at 2 Mb/s | see appendix for DBPSK and DQPSK specifications | closed |
| 14a | T&R | OQPSK specification |  |  | open |
| 15 | T&R | Channel data rate | 1 Mb/s and 2 Mb/s |  | closed |
| 17a | Tx | Chip clock frequency accuracy | £ 50 ppm |  | closed |
| 18 | Tx | Preamble length | 160 bits sent at 1 Mb/s |  | closed |
| 18a | Tx | Preamble content | A: synchronization field: 128 bits of 'ones' B: unique word: 16 bits (Octal 2717) C: 802.11 signal bits: 11000000 1Mb/s BBPSK 00110000 2Mb/s DQPSK 00001100 2Mb/s OQPSK D:Service bits: 8 bits used at vendors discretion all 'zeros' for 802.11 |  | closed |
| 18b | Tx | End Delimiter | End of frame shall be detected by presence of both: -end of message word(EOM) -loss of correlation EOM: 16 bits (octal value 171640) EOM immediately follows the last frame of MAC data (MPDU) End-of-frame indication shall be inserted during the 4th byte from end of MPDU. Tx shall be disabled immediately following transmission of EOM. |  | closed |
| 18c | T&R | Rate change | All symbols following the PHY preamble shall be at the rate indicated in 802.11 signal bits. |  | closed |
| 20 | Rx | Energy detect time | £ 20 µs | Maximum response time from energy incident upon receiver antenna to receiver signal level crossing the threshold (20a) | closed |
| 20a | Rx | Energy detection threshold | TBD |  | open |
| 22 | T&R | Out of band spurious emissions | Per FCC part 15.247,15.205 and 15.209 in USA per ETSI RES 02-09 in Europe | For reference only | open |
| 23 | T&R | Switching time Tx to Rx | 10 µs |  | open |
| 24 | T&R | Switching time Rx to Tx | 10 µs |  | open |
| 26 | Rx | Frame to Frame reception SIR | 8 dB |  | open |
| 27 | Rx | BER at specified Eb/No | 10-5 @ Eb/No = 16 dB | Includes 4dB modem implementation margin | open |
| 28 | T&R | Channel availability | 99.5% | Could also be specified as probability of outage; with no interference. From the PAR. | open |
| 29 |  | Data Line/clock input/output jitter |  | Includes static and dynamic jitter (see 802.3 definition) Dependent on MAC requirements. | open |
| 30 | T&R | Antenna port impedance (if exposed) | 50 ohms | For interoperability at antenna port | open |
| 31 | T&R | VSWR | Devices shall stand 0£VSWR£¥ with no damage Operational VSWR £TBD | For conformance testing | open |
| 32 |  | Interface lines to convergence layer (when exposed) |  |  | open |
| 33 |  | PHY-MAC Net management info/control variables |  |  | open |
| 34 |  | Other PHY-MAC Net management info/control variables |  |  | open |
| 35 |  | Safety Requirements |  |  | open |
| 36 |  | DTE/DCE Iterface |  |  | open |

Appendix

This appendix gives additional descriptions of the items in the template.

# ITEM 2B Channelization

5 different channels are supported  divided in two groups.

The centre frequencies of these channels are:

group 1:

channelnumber 1a: 2412 KHz

channelnumber 1b: 2442 KHz

channelnumber 1c: 2472 KHz

group 2:

channelnumber 2a: 2427 KHz

channelnumber 2b: 2457 KHz

A group 1 channel  can be used simultaneously with any other group 1 channel with no channel interference in adjacent cells.  A group 2 channel can be used simultaneously with the other group 2 channel.

Group 1 and group 2 channels may be used simultaneously in a cellular frequency plan where interference effects have been factored into a cell plan.

ITEM 3C  Data Scrambler

Data Scrambler implementation :

µ §

Figure 1 : Scrambler implementation; polynomial 1+x-4+x-7

# ITEM 14 Modulation parameters for DQPSK and DBPSK

| dibit pattern (d0,d1) (d0, first in time) | Phase jump |
| --- | --- |
| 0, 0 | 0 |
| 0 ,1 | p/2 |
| 1 ,1 | p |
| 1 ,0 | -p/2 |

Table 1. Mapping for DQPSK

Data dibit to differential phase mapping

| bit | Phase jump |
| --- | --- |
| 0 | 0 |
| 1 | p |

Table 2. Mapping for DBPSK

Bit to differential phase mapping