IEEE 802.11

Wireless Access Method and Physical Specification

Draft Proposal for a Direct Sequence Spread Spectrum PHY standard

edited by Jan Boer, AT&T

This document is a revised version of document IEEE P802.11-232 for the direct sequence proposed specification. It incorporates the changes agreed upon in the San Jose January 1994 meeting.

|  |  | Parameter | Proposed Spec Direct sequence | Comments/Notes | Status |
| --- | --- | --- | --- | --- | --- |

| 1 | T&R | Frequency range | 2.4 to 2.4835 GHz (USA) 2.4-2.5 GHz (Europe) 2.471-2.497 GHz (Japan) | Other frequency bands will follow | closed |
| --- | --- | --- | --- | --- | --- |
| 2b | T&R | Number of channels | 3+2 | See appendix for the definition of the channels | closed |
| 3b | T&R | Spreading sequence | 11 chips Barker sequence (1,-1,1,1,-1,1,1,1,-1,-1,-1) |  | closed |
| 3c | T&R | Data scrambler | polynomial 1+x-4+x-7 | ALL bits transmitted by the PHY shall be scrambled. See appendix for a description of the scrambler | closed |
| 4a | Tx | Transmitted power levels [mW] | Max 1000/100 | US/Europe,Japan | closed |
| 4b | Tx | Minimum transmitted power level | 10 mW | Required for conformance testing | closed |
| 5 | Tx | Transmitted power control | up to a maximum of 4 levels required above 100mW |  | closed |
| 6 | Tx | Max. Radiated EIRP | Per FCC part 15.247 in US Per ETSI res 02-09 in Europe Per TBD in Japan | total radiated power including antenna gain, as defined by regulatory agencies in each country. For reference only. | closed |
| 8 | Rx | Receiver minimum input level sensitivity | -80 dBm @ 10-5 BER | At 2 Mb/s, 11 chip code rate at the antenna connector | closed |
| 9 | Rx | Receiver maximum input level | - 4 dBm |  | closed |
| 10b | Rx | Adjacent channel rejection | 37 dB at 10-5 measured by the following method: input an inchannel receive signal level 2dB above the level specified in item 8; set a adjacent channel signal modulated in the same fashion to a level 39dB above the level specified in item 8. The BER shall be better than 10-5. | adjacent channel is defined per group (see 2b) | closed |
| 12 | Tx | Channel spectrum mask | -30 dBc @ Df=±11Mhz -50 dBc @ Df=±22Mhz linear slope between 30 dBc and 50 dBc | includes all transmit spectral products | closed |
| 13a | Tx | Transmitter Centre Frequency tolerance | ±25 ppm Type I: 0 to 40 oC Type II: -30 to 70 oC |  | closed |
| 14 | T&R | Modulation | DBPSK at 1 Mb/s DQPSK and/or DOQPSK at 2 Mb/s | see appendix for specifications | closed |
| 14a | T&R | DOQPSK offset | 0.5 chip delay in Q transmit path |  | closed |
| 15 | T&R | Channel data rate | 1 Mb/s and 2 Mb/s |  | closed |
| 17a | Tx | Chip clock frequency accuracy | £ 50 ppm |  | closed |
| 18 | Tx | Preamble length | 160 bits sent at 1 Mb/s |  | closed |
| 18a | Tx | Preamble content | A: synchronization field: 128 bits of 'ones' B: unique word: 16 bits (Octal 2717) C: 802.11 signal bits: 11000000 1Mb/s DBPSK 00110000 2Mb/s DQPSK 00001100 2Mb/s DOQPSK D:Service bits: 8 bits used at vendors discretion all 'zeros' for 802.11 |  | closed |
| 18b | Tx | End Delimiter | End of frame shall be detected by presence of both: -end of message word(EOM) -then loss of correlation Loss of correlation shall be detected within 16 bits from EOM EOM: 16 bits (octal value 171640) EOM immediately follows the last frame of MAC data (MPDU) End-of-frame indication shall be inserted during the 4th byte from end of MPDU. Tx shall be disabled immediately following transmission of EOM. |  | closed |
| 18c | T&R | Rate change | All symbols following the PHY preamble shall be at the rate indicated in 802.11 signal bits. |  | closed |
| 20 | Rx | Energy detect time | £ 20 µs | Maximum response time from energy incident upon receiver antenna to receiver signal level crossing the threshold (20a) | closed |
| 20a | Rx | Energy detection threshold | TBD |  | open |
| 22 | T&R | Out of band spurious emissions | Per FCC part 15.247,15.205 and 15.209 in USA per ETSI RES 02-09 in Europe |  | closed |
| 22a | Rx | In band spurious emissions on receive | -60 dBm |  | open |
| 23 | Tx | Transmit power down | £ 5 µs | Time from transition of exposed Rx/Tx control line till -70 dBm power output measured at the antenna | closed |
| 23a | Tx | Transmit power ramp down | - 40 dB ramp down in no less than 3 µs |  | closed |
| 23b | T&R | Transmit to Receive turnaround time | £ 25 µs | Measured from transition of exposed Rx/Tx control line | closed |
| 24 | T&R | Switching time Rx to Tx | £10 µs | Time from transition of exposed Rx/Tx control line till 90% full power measured at antenna | closed |
| 24a | Tx | Tx power on ramp | 10% to 90% Tx power on ramp shall be greater than 5 µs |  | closed |
| 26 | Rx | Frame capture ratio | 10 dB | Given a continuous interferer at -50 dBm, an error rate of 10-5 shall be maintained from a desired packetized source of -40 dBm. Both signals shall be of DS modulation. | closed |
| 27 | Rx | BER at specified Eb/No | 10-5 @ Eb/No = 17 dB |  | closed |
| 28 | T&R | Channel availability | 99.5% | Could also be specified as probability of outage; with no interference. From the PAR. | deleted |
| 29 |  | Data Line/clock input/output jitter |  | MAC/PHY interface issue | deleted |
| 30 | T&R | Antenna port impedance (if exposed) | 50 ohms |  | closed |
| 31 | T&R | VSWR | Devices shall withstand no damage and remain unconditional stable over the range 0£VSWR£¥. |  | closed |
| 32 |  | Interface lines to convergence layer (when exposed) |  |  | open |
| 33 |  | PHY-MAC Net management info/control variables |  |  | open |
| 34 |  | Other PHY-MAC Net management info/control variables |  |  | open |
| 35 |  | Safety Requirements |  |  | open |
| 36 |  | DTE/DCE Interface |  |  | open |

Appendix

This appendix gives additional descriptions of the items in the template.

# ITEM 2B Channelization

5 different channels are supported  divided in two groups.

The centre frequencies of these channels are:

group 1:

channelnumber 1a: 2412 KHz

channelnumber 1b: 2442 KHz

channelnumber 1c: 2472 KHz

group 2:

channelnumber 2a: 2427 KHz

channelnumber 2b: 2457 KHz

A group 1 channel  can be used simultaneously with any other group 1 channel with no channel interference in adjacent cells.  A group 2 channel can be used simultaneously with the other group 2 channel.

Group 1 and group 2 channels may be used simultaneously in a cellular frequency plan where interference effects have been factored into a cell plan.

ITEM 3C  Data Scrambler

Data Scrambler implementation :

µ §

Figure 1 : Scrambler implementation; polynomial 1+x-4+x-7

# ITEM 14 Modulation parameters for DQPSK,  DOQPSK and DBPSK

| dibit pattern (d0,d1) (d0, first in time) | Phase jump |
| --- | --- |
| 0, 0 | 0 |
| 0 ,1 | p/2 |
| 1 ,1 | p |
| 1 ,0 | -p/2 |

Table 1. Mapping for DQPSK and DOQPSK

Data dibit to differential phase mapping

| bit | Phase jump |
| --- | --- |
| 0 | 0 |
| 1 | p |

Table 2. Mapping for DBPSK

Bit to differential phase mapping