# DFWMAC

# Distributed Foundation		Wireless Access Protocol

## Basic access method and Contention Free 	Service support.

## for

## IEEE 802.11

By:	Phil Belanger  		Xircom

Greg Ennis			Symbol Technologies

Wim Diepstraten   	NCR

# Wireless Network Architecture

µ §

## *	Infrastructure mode can overlap with Ad-Hoc.

## *	Support single and multi-channel PHY's.

# Supported Services:

*	Asynchronous Data Service:

-	Short response time with high instantaneous Throughput, suitable for Bursty traffic.

-	No BSS isolation required

*	Contention Free Service (Optional):

-	Time Bounded Service dimensioned for Voice.

.	Connection oriented reserves bandwidth.

.	Allows mixed Voice/Data.

.	Allows variable bitrates (frame sizes)

-	Asynchronous Contention Free Service

.	Supports the bursty nature.

-	Requires sufficient BSS isolation, (TDMA type access protocols have similar requirements).

# Basic Access Protocol:

## *	Use Distributed Access Protocol for efficient medium sharing.

## *	Robust for interference.

-	CSMA/CA + Ack for unicast frames.

With MAC level recovery

-	CSMA/CA for Broadcast frames.

## *	Optional RTS / CTS to provide a "Virtual CS" function to protect against Hidden Nodes.

## *	Supports Ad-Hoc operation seamlessly, so does not require any infrastructure.

# CSMA/CA explained:

µ §

## *	Reduce collision probability where mostly needed.

## *	Efficient Backoff algorithm stable at high loads.

-	Exponential Backoff for retransmissions.

## *	Implement different priority levels.

(to allow immediate Ack and PCF coexistence)

# CSMA/CA+Ack Access Protocol

µ §

*	With optional RTS / CTS provisions.

*	Defer on NAV and "Carrier Sense", also for the Contention Free service.

*	RTS / CTS use is controlled by a NoRTS parameter per station. Used for long inbound frames.

# Contention Free Service Model:

µ §

*	Contention Free Service uses Point Coordination Function (PCF) on a DCF Foundation.

*	Async Data, Voice or mixed implementations possible.

*	Contention Free capability does not burden the Async service implementation.

# Point Coordination Function:

## µ §

*	SuperFrame to allow mixed "Contention" and "Contention Free" operation.

*	"Contention Free" period under PCF control.

*	Both PCF and DCF Defer for each other causing SuperFrame Stretching.

*	Tx-Blackout to prevent DCF with PCF collision.

# PCF Protocol:

µ §

*	CF-Burst by Polling bit in CF-Down frame.

*	Immediate response by Station on a Poll.

*	Stations to maintain NAV to protect Up-traffic.

*	Variable length or no response possible.

*	Last frame from AP to reset the NAV in stations.

*	"Ack Previous Frame" bit in Header.

# Station-to-Station PCF Protocol:

## µ §

*	Poll from AP can start a "Station-to-Station" transfer with individual Ack.

*	NAV update based on maximum frame duration.

*	Works with both Time Bounded and Async frames.

# Contention Free Service Types:

## µ §

## *	Time Bounded Service

-	Highest priority based on reservation.

## *	Async Contention Free  Service

-	Dynamic Polling scheme to maintain Bursty 	characteristics.

*	Contention Free limit based on max. Async frame size.

# Time Bounded Service:

## µ §

*	Connection oriented repetitive traffic with low delay variance.

*	Reservation for "Time Bounded Frame Period" and maximum frame size per TBFP per connection.

*	Uses different Frame format based on "ConnID".

*	"Connection Setup" uses Async service.

# Time Bounded Characteristics:

## *	Built on Asynchronous Access method (DCF).

## *	Uses CSMA/CA + (Ack) with highest priority.

-	With limited recovery (QoS parameter)

## *	Dimensioned to support mixed Voice/Data.

## *	Video support possible at higher  PHY rates.

*	Time Bounded Framing Period is PHY speed dependent, but also depend on max. Async frame size.

SFP= 20-30 msec for 1-2 Mbps, 5-10 msec at 10 Mbps.

## *	Unused reserved Time Bounded Bandwidth can be used for Asynchronous traffic.

# Time Bounded Characteristics:

## *	Can support different PHY speeds (1-20 Mbps). Which will allow different TBFP's.

## *	Support variable frame size on a per SuperFrame basis, without control overhead.

-	Can take full advantage of "Talk Spurt" characteristics of Voice.

-	Allows flexible congestion control.

## *	Support direct Station-to-Station.

## *	Includes provisions for Power Consumption Management.

## *	Includes basic re-association provisions.

# Async Contention Free service:

## µ §

*	Stations "Declare" ACF capability to the AP during setup.

*	AP maintains Dynamic Polling list for ACF-Burst

-	Request or traffic activity detection in Contention period can cause "Add to Poll-list"

-	n*SFP Inactivity  can cause "Delete from Poll-list".

*	Limit on max number of Polls per SFP.

# ACF Characteristics:

µ §

*	ACF can provide stable throughput during High Loads.

*	Is optimized for efficiency without reservation.

*	Can increase response times when Load is low.

*	Uses same Async frame format as in Contention Period.

*	Dynamic Polling algorithm can be AP proprietary.

*	Could also provide large population fixed polling service.

# Scan and Power Management:

## µ §

## *	Time Synchronization assumed by regular Beacon or TimeStamp embedded in TBS Frame Header.

## *	Random Polling Sequence allows equal average active time per station.

## *	CF-Burst stations are awake from beginning of SuperFrame until they are serviced.

# Frame Formats:

## µ §

# DFWMAC Frames:

µ §

# Performance Example (TBS):

## *	Assumption:	32Kbps ADPCM Voice

12 Byte MAC + Wavelan PHY Overhead

## *	Example: @ SFP of:		 20 msec			30 msec

Modem speed  		  @ 2 Mbps	@1 Mbps	@2 Mbps	@1 Mbps

Voice only:				16 FDX   	10 FDX 	20 FDX		11 FDX

Voice/Data   576 Byte:	14 FDX    	  7 FDX		18 FDX		 9  FDX

Voice/Data 1500 Byte:	11 FDX   	  3 FDX		15 FDX		 6  FDX

The following is an estimate of the maximum Async throughput @ 2Mbps

Async Data throughput:     >  75 KByte/sec (assuming max TBS load)

Async Data throughput:     >130 KByte/sec (assuming 11 FDX Talkspurt channels)

Async Data throughput:     >200 KByte/sec (no TBS connection active)

# PCF limitations:

## *	PCF's can not overlap on the same channel.

(Same restrictions as in any TDMA system)

## *	Application is therefore limited to:

-	Multi channel environment with sufficient BSS isolation.

-	In single BSS of single channel ESS.

## *	Only supported in Infrastructure Networks.

## *	Capability needed in both Station and AP.

## Conclusion:	Therefore the PCF should be an option.

# Conclusion:

*	A PCF on a CSMA/CA + Ack based DCF is an efficient  access method which maintains the good medium sharing characteristics.

-	Hidden node vulnerability decreased through NAV.

*	A very flexible Contention Free capability has been demonstrated, built on top of CSMA/CA.

-	Support both Time Bounded and Async services.

*	Advantage over TDMA:

-	Frame size flexibility - allows variable bitrates.

-	Optimum capacity sharing between Asynchronous and Time Bounded services.

-	Dynamic allocation possible without reservation.

*	Like TDMA it only works when there is no overlap.

# Service required from the PHY

µ §

*	Fast CS function with various CS sensitivity parameters.

*	Fast Rx-to-Tx Turnaround time.

*	Both parameters affect performance efficiency.

# Mixed bitrate aspects:

## *	A MAC needs to support multiple bitrates:

-	To support Fallback rates suitable for flexible robustness improvement.

-	To support migration to higher speeds utilizing the same band of the scarce Spectrum resource.

-	To allow easy reuse of the infrastructure when higher speeds develop.

## *	The MAC has a longer life-time in that it should support multiple (future) PHY's.

## *	As higher speeds develop in the same band, then the products will likely support multiple (PHY) standards for backward compatibility.

# What is needed to support Fallback:

## *	Primary consept/assumptions:Mixed rate in BSS

-	PHY is capable to receive the Nominal rate and a number of fallback rates.

-	PHY can dynamically detect incoming bitrate, and will deliver the proper clock + data to the MAC.

-	Short frames error probability on nominal rate may be similar to long frame error probability at the fallback speed.

## *	Mixed Rate (per station in BSS) consequence:

-	Broadcast traffic  at lowest common rate.

-	Nominal rate "Control" traffic (short frames) may be acceptable.

# What is needed for speed migration:

## *	Primary assumptions:

## -	Not all PHY's can receive all rates.

-	PHY is capable to receive a Nominal rate and a number of fallback rates automatically.

-	Higher speed PHY's can support lower speed standards (multi-standard implementation).

## *	Mixed Rate consequence:

-	Broadcast traffic  at lowest common rate.

-	"Control" traffic (short frames) transmission at "Highest common capable rate" may be acceptable.

# DFWMAC supports mixed bitrates:

## *	Frame structure Consequences :

-	NAV processing in duration instead of bits.

-	Broadcasts (also Beacons) at lowest common rate.

-	RTS/CTS at "Highest common capable rate".

-	Contention Free "Up frame" at station speed.

-	Contention Free "Down frame" at:

.	Lowest common speed.

.	or separate small "CF-RTS" frame to distribute Ack and NAV control bits.

## *	AP and station to maintain operating speed status on a per destination (MAC-to-MAC) basis.

## *	PHY to report received frame bitrate to the MAC.

# Contention Free burst impact:

## µ §

*	Short CF-RTS frame at "Highest common capable rate"

*	Short CF-Down frames and Polls at the "Highest common capable rate"

*	Function:	Distribute NAV, and return Ack.

# Range is Interference Limited:

µ §

*	Defer threshold should be twice as sensitive as Data.

# Hop Stretching:

## µ §

## *	Hop interval stretching is natural mechanism when SuperFrame stretching is applied.

## *	Reduced complexity and higher efficiency, especially for short Hop dwell times.

# Directed Data/Ack MPDU:

## µ §

## *	Defer access based on CS and NAV state.

## *	Direct access when medium is sensed free.

# Access Backoff procedure:

## µ §

## *	Backoff timer elapses only when medium is Free.

## *	So higher relative priority for deferring stations.

## *	Exponential Contention window increase at every retransmission.