IEEE 802.11

Wireless Access Method and Physical Layer Specifications

Title:		AN UNINTELLIGENT RADIO INTERFACE

Presented by :

W. B. Cooper

National Semiconductor

Santa Clara, CA

408-721-5815 (V)

408-721-4115 (Fax)

Burch@Berlioz.nsc.com (Email)

Abstract

This paper provides an example of a simple service interface between a wireless LAN radio PHY  and the digital hardware and associated software which implements the controller for the PHY.   This proposed interface is between the medium dependent layer and the convergence layer.   The implementation is simple in the sense that it implies no intelligence on the radio side of the PHY layer.   All synchronization and timing functions are placed in the controller on the MAC side of  service interface.  The system is assumed to be frequency hopping, but it is believed  that the proposed structure would serve for other physical layers as well.  The purpose of this paper is to illustrate what is required for direct control of the radio and baseband functions as contrasted to a intelligent radio interface.

Conclusion

An exposed interface between the medium dependent layer and convergence layers is better suited for a low cost radio than the present DTE/DCE interface presently planned between the convergence and medium independent layers.

1.0 Introduction

The type of command and control structure used between the controller and PHY layers as implications on the complexity and performance of the PHY.  The difference between an interface that uses a command set for control, and simple a unintelligent interface is the addition of a controller or state machine for interpretation and translation of the command set into device dependent operations.   A simple command structure where PHY implementation independent primitives are passed back and forth over the interface allows:

·	A small number of interconnect lines.

·	A command set for control of the PHY.

·	Command set that is implementation independent.

The following block diagram gives an example of an intelligent interface:

µ §

There are,  however, disadvantages in using architecture which uses a highly structured command interface between the controller and PHY.

·	More complexity in the radio.

·	Higher power consumption.

·	More digital noise sources in the radio.

·	Difficulty in  synchronizing MAC and PHY functions.

The following block diagram represents an example of what a radio interface would look like with no imbedded control logic.  The microprocessor and memory are no longer required.  The interface is still serial in order to reduce the number of interconnects.  The result is a simpler radio with no intelligence and lower power consumption.

µ §

2.0 Intelligent Interface

A structured intelligent interface for a frequency hopping radio might have a command set that looks something like this:

Command			Operation

Reset				Initializes controller

Sleep Mode Enable			Places radio in low power mode

Load Hop Table			Load hopping pattern

Set Channel Immediate		Go to a particular channel

Set Output Power			Set output power of the transmitter

Select Antenna			Manual diversity control of antenna

Read RSSI			Read the signal strength of the last received message

Loop Back			Loop back digital data

Enter Diagnostic Mode		Stop hopping

Leave Diagnostic Mode		Start hopping

There are many other possible commands that would be applicable, but the idea is to have command set which is interpreted by a controller to remove any implementation dependencies in the PHY.

An interface connector  for such a implementation might have a very few number of connections:

| SIGNAL | PIN | DIRECTION | DESCRIPTION |
| --- | --- | --- | --- |
| tx_clk | 1 | radio to Controller | Transmit Clock |
| gnd | 2 |  | Ground |
| tx_data | 3 | Controller to radio | Transmit Data |
| vcc | 4 |  | Supply |
| rx_clk | 5 | radio to Controller | Receive Clock |
| rx_data | 6 | radio to Controller | Receive Data |
| cmd_in | 7 | radio to Controller | Serial data In to Controller |
| cmd_clk | 8 | Controller to radio | Serial Wire clock |
| cmd_out | 9 | Controller to radio | Serial Wire data Out of Controller |

Table 1:	Connector Definition Intelligent PHY

The attractiveness of this structure is that it can remove the synchronization and control issues from the MAC side of the interface  and allows for an implementation independent design  around which a standards specification can be written.  The disadvantage of course is that the commands over the interface must be interpreted and executed in the PHY.  An alternative implementation is to provide for direct control of the PHY.

3.0 A Simple PHY with Direct Control of PHY  Functions

If the interface between the PHY and Controller to be buried within the design and no exposed service point is to be provided to the user then the control of the PHY need not be as structured.   The controller functions that provide for operation of the MAC can also control the PHY.    The  result can be an overall reduction in the complexity of the design and power consumption.

In addition, if the Controller does not have direct control over PHY then it is difficult to provide synchronization and control functions to efficiently provide for sleep mode operation.  For example, if the Access Point Controller could provide a list of addressed  mobiles during the header then a longer period in a low power mode could be achieved by the mobile during inactive periods.

If a mobile did not here its address during the first header then it could go into a low power mode until the start of the next hop.

The interface to an unintelligent PHY might require a few more connections to bring out to the interface some of the critical control functions.  Table II is an example of what an  interface connector to an unintelligent PHY might look like.

4.0 Connector Definition

| SIGNAL | PIN | DIRECTION | DESCRIPTION |
| --- | --- | --- | --- |
| tx_clk | 1 | radio to Controller | Transmit Clock |
| gnd | 2 |  | Ground |
| tx_data | 3 | Controller to radio | Transmit Data |
| vcc | 4 |  | Supply |
| rx_clk | 5 | radio to Controller | Receive Clock |
| rx_data | 6 | radio to Controller | Receive Data |
| mw_in | 7 | radio to Controller | Serial data In to Controller |
| mw_clk | 8 | Controller to radio | Serial Wire clock |
| mw_out | 9 | Controller to radio | Serial Wire data Out of Controller |
| rts | 10 | Controller to radio | Request To Send |
| cd | 11 | radio to Controller | Carrier Detect |
| rx_start | 12 | Controller to radio | Receiver Start |
| spare | 13 |  |  |
| sel/norm | 14 | Controller to radio | Select / Normal |
| fault | 15 | radio to Controller | Radio summary fault |

Table 2:	Connector Definition Simple PHY

5.0 Transmit Interface and Timing

The transmit data over the interface is serial in order to reduce the number of interconnects.  The data is synchronous with transmit clock.  A request to send line is used to activate the transmitter.

Signal		Direction	Description

tx_clk		r>m		Transmit Clock.  This clock is provided by the radio to clock data 					in.  The nominal clock rate is 1 MHz  It will begin toggling 100uS 					after RTS is raised.

tx_data		m>r		Transmit Data.  This is the data to be transmitted.  The data is 			sampled by the radio, on the falling edge of the tx_clk.  A preamble 			of  10... (50 bits) will be inserted prior to any data including 			opening flag,  etc. This 50 bit preamble aids the receive radio in 			acquiring the received signal.  The preamble will precede every new 		transmission.

rts		m>r		Request to Send.  This is a signal generated by the Controller to 			request that the radio switch to transmit mode.  The radio will delay 		100uS  before turning on transmit amps and clocking transmit 			data.  The delay is required for the rf synthesizer to settle.                        		When RTS is low the radio will switch to the receive mode.

µ §

6.0 Receive Interface and Timing

The receive side of the radio interface is also serial.  The receive clock is extracted from the data by the baseband processor.  The radio is in the receive mode whenever RTS is inactive.   Receive start  resets the acquisition sequence and places the antennas in the diversity selection mode.  Carrier detect is a baseband derived signal which indicates that a valid preamble has been detected.

Signal		Direction	Description

rx_clk	r>m	Receive Clock.  The radio will generate this clock only when it detects carrier.  The clock will become active after CD goes high.

rx_data	r>m	Receive Data.  This is the data received by the radio.  The data should be sampled on the falling edge of the rx_clk signal.   It is up to the Controller to decide whether the data is valid.

rts	m>r	Request to Send.  This is a signal generated by the controller to request the radio to switch to the receive mode (rts low).  The synthesizer receive frequency can be preloaded anytime prior to applying the latch signal.

cd 	r>m	Carrier Detect.  This signal indicates that the radio is receiving RF energy.  CD could be triggered by the radio in response to noise or interference.  It is up to the controller to decide this and restart the receiver by toggling the rx_start signal.

rx_start	m>r	Receiver Start.  In response to this signal the receiver  reset Internal slicing and DC offset registers and begin searching for CD again.

µ §

7.0 Serial Interface

The Serial Interface allows the Controller to address one of the following devices:

o the Baseband Processor

o the Synthesizer chip

o the Configuration Data Registers

o other serial devices

Serial Control Signals

Signal		Direction	Description

sel/norm         	m>r		Select/Normal. This is a signal supplied by the MAC to access the 					 serial Select Register.

sc           	m>r		Serial  Clock. This clock is supplied by the MAC to clock in/out 					the Serial interface serial data.

sdi            	m>r		Serial Data In. The serial data input for the  Serial Interface.

sdo		r>m		Serial Data Out. The serial data output for the Serial 						Interface.

µ §

7.2 PHY Data Registers

The Data registers of the PHY are used to control various PHY functions and to read information gathered by the PHY.

The data word for the PHY consists first of three (3) bits of Address to determine which data is to be accessed. This is followed by one bit to indicate read or write and the remaining bits are to/from the data register.

µ §

7.2.3 PHY  Data Register Description

Status/Fault Register.

Jabber Timer Fault.  A jabber timer fault indicates the radio has been transmitting for too long a period of time. The maximum allowed transmit time is 400 ms.  When this condition is detected the radio will automatically switch off the transmitter and set the Jabber Timer Fault.

Synthesizer Unlock.   This fault indicates the radio is not generating the correct channel frequency.

Transmit Amplifier Fault.   This fault indicates the radio output power amplifier has failed.  The radio will not be able to transmit when  this fault occurs.

Control Register 1.

RF Power Control.  These 3 bits control the radio Transmit output power level.  The values listed in 6.2.2 are in dB below the peak power level (dBc), i.e. 0 dBc represents maximum output power.   At power-up these three bits will be 000.  The MAC should set these bits to 011 (for maximum output power).

Antenna Diversity Control.  This bit selects one of two antenna patterns for antenna diversity.  Due to the multipath environment signal reception on one pattern may be significantly better than the other pattern.

Active/Standby Control.   This bit controls the power consumption mode for  the 2240 (IF Discriminator), 2216B (LNA/Mixer) and the 2320 (Synthesizer).  For normal operation this bit should be high (Active).  At power up this bit will be low so it will be necessary for the controller to set this bit high.  The controller can set this bit low to put the radio in a sleep mode.

Control Register 2.

Powerdown  When this bit is set high the receive functions are in a powerdown mode.  At powerup this bit is low (normal power mode). The Controller can set this bit high to put the radio in a sleep mode.

/Baseband Reset.   This is the reset control (active low) for the 2410 (Baseband  Processor).  At powerup this bit will be low, so it will be necessary for the Controller to set this bit high.

/RSSI_en.  The RSSI input from the 2240 is sampled every 11.1 us (200/sys clk for the 2410).  Each sample is compared against the present peak  value, and if larger, it replaces the peak value.  The peak value is reset on every falling edge of the  /RSSI_en signal.   /RSSI_en should be low 5 us before the RSSI measurement is taken to allow for ADC settling.

Loopback Control.  The loopback mode is a test mode for loopback of baseband signal.  In this mode (active high) RTS must be high to activate this mode.

7.0 THE SYNTHESIZER CHIP

The Synthesizer  on the RF/IF Board is used to generate the RF frequencies for the Radio.  There are 82 discrete Transmit frequencies and 82 discrete Receive frequencies used.  That is, the synthesizer is programmed for different frequencies in the transmit and receive modes.  The synthesizer generates these frequencies based on data words received through its serial interface.

The channel frequencies are generated through the following two synthesizer control words:

o the Programmable Reference Divider (R Counter) and Prescalar Select (S Latch)

o the Programmable Divider (N Counter)

8.0 THE BASEBAND PROCESSOR CHIP.

The Baseband Processor on the Baseband Board provides 3 major functions for the Radio.  These functions are:

o generate the transmit waveform for the RF signal

o control of the baseband DC correction

o provides access to a digitized version of the RSSI.

Read RSSI Peak Hold Value.

The Received Signal Strength Indicator (RSSI) is a 6-bit value that is log linearly proportional to the input signal level to the IF Demod.