IEEE 802.11

Wireless Access Method and Physical Specification

title:	Establishment of DSSS PHY parameters

date:	September 14, 1993

authors:	Jan Boer

NCR Wireless Communications and Networking Division

Zadelstede 1-10

3431 JZ Nieuwegein-Holland

Telephone: (+31) 3402-76483

Fax: (+31) 3402-39125

E-mail: jan.boer@utrecht.ncr.com

Paul Struhsaker

Telxon Corporation

3300 West Market Street

Akron Ohio 44333

Telephone: 216-867-3700 x3331

Fax: 216-869-2255

E-mail: pstru@telxon.com

# Introduction

Since the March meeting a lot of time was spent in the DS meetings in order to set the parameters in the DS template of document 93/83r1.  There was a lot of consensus on important parameters, such as spreading code and modulation.

The basis for the template parameters was set by the NCR proposal: doc 93/37.  In document 93/67 Telxon gave comments and came up with some alternatives.

In order to speed up the process of standardization Telxon and NCR had extensive E-mail and telephone contacts in order to come up with proposals for the standard which they regard to be acceptable for the majority of the parties involved in Direct Sequence.

This document gives proposals for the different parameters of the DS template as discussed between NCR and Telxon. Numbers between brackets refer to the template parameter numbers.

# modulation parameters

Channel data rate (17)  is 2 Mbit/s. Modulation DQPSK. The mapping is as in table 1.

| dibit pattern (d0,d1) (d0, first in time) | Phase jump |
| --- | --- |
| 0, 0 | 0 |
| 0 ,1 | p/2 |
| 1 ,1 | p |
| 1 ,0 | -p/2 |

Table 1. Data dibit to differential phase mapping

The fall back data rate (18) is 1 Mbit/s. Modulation  BPSK. The mapping is as in table 2.

| bit | Phase jump |
| --- | --- |
| 0 | 0 |
| 1 | p |

Table 2.  Phase mapping for fall back rate

For the spreading sequence the 11 chips Barker sequence is used:

(1,-1,1,1,-1,1,1,1,-1,-1,-1)

Note: No other data rates or spreading sequences will be defined in the DS standard .

# Channelization

5 different channels are supported (2b), divided in two groups.

The centre frequencies of these channels are:

group 1:

channelnumber 1a: 2412 KHz

channelnumber 1b: 2442 KHz

channelnumber 1c: 2472 KHz

group 2:

channelnumber 2a: 2427 KHz

channelnumber 2b: 2457 KHz

A group 1 channel  can be used simultaneously with any other group 1 channel with no channel interference in adjacent cells.  A group 2 channel can be used simultaneously with the other group 2 channel.

Group 1 and group 2 channels may be used simultaneously in a cellular frequency plan where interference effects have been factored into a cell plan.

The adjacent channel rejection(12b) (between type 1 channels or between type 2 channels) will be 50dB.

The requirements for the transmitted spectrum mask (14) are:

-30 dBc @ Df=±11Mhz -50 dBc @ Df=±22Mhz

Above channelization scheme will be applied in Europe and the US. For Japan a separate centre frequency has to be selected.

# transmit power

The transmitted power level is maximum 1000 mW for the US and 100 mW for Europe and Japan (4a), the minimum transmitted power level (4b) will be 10 mW.

Transmitted Power control is required above 100mW (4 discrete levels ).

# lo accuracy

(15a)Good design for phase measurements dictates that they be made after despreading the signal (lower phase noise by > 10dB). Thus, the phase error is measured at the end of each symbol resulting in a phase observation rate equal to the symbol rate. For a BPSK  training sequence (see chapter preamble) the phase offset caused by a LO difference between the transmitter and the receiver cannot exceed +/- 90 degrees of the desired symbol point. This is for the individual LO +/- 45 degrees. The desired +/-ppm is thus +/- 1/8 the symbol rate (absolute worst case). This results in the following worst case limits based on symbol rate and a middle of the ISM band frequency of 2450MHz:		1M Symbol rate   => +/- 125Khz  = +/-51 ppm

Good engineering practice requires that we back off for the worst case. 50% is a reasonable number:

The worst case DS LO accuracy is defined as +/- 25 ppm at 2450MHz. The 802.11 specification will provide 2 environment temperature ranges of  operation. These will be designated as follows:	- Type 1  Office Environment 0 to 40 degree C	- Type 2  Industrial  -30 to 70 degree CAll 802.11 radios shall operate at a minimum of Type 1.

# preamble

The 802.11 DS PHY preamble must provide the user the capability to perform diversity antenna selection, achieve code lock, achieve carrier lock and establish the start of MAC frame data. The capability to negotiate between the currently proposed QPSK full rate and BPSK fall back rate on a packet by packet must also be incorporated in the preamble. Further, the preamble should provide for future growth of 802.11 capabilities (higher symbol/data rates, longer code length, etc.) as well as allow support for vendor specific capabilities.

Based on these factors, the following preamble is defined for the 802.11 DS packet (21):

<123 scrambled all 1's><16 bit delimiter><additional 8 bit PHY signalling field(s) (PSF's)>  |< Mac data>

The preamble shall be transmitted using 1 Mbps BPSK as described in the modulation parameter section.

Change of bit rate to 2 Mbps DQPSK will be designated within the PSF field. The bit rate change will start with the first symbol in the MAC data frame.

All data (preamble and MAC data) will be scrambled as described in doc 93/37 section 8.1, using a feed through randomizer with the polynomial:

1+x-4+x-7

No initialization is the scrambler will be required for either the transmit or receive functions.

A 16 bit delimiter shall be provided to identify the position of the PHY signalling field(s) and associated start of the MAC data frame. The actual 16 bit sequence is to be determined. The following criteria for the delimiter selection have been established:

The delimiter shall be chosen to maximize the hamming distance from the following 16 bit data patterns which would be out of the scrambler during demodulation.

- All 1's (desired descrambler preamble sequence)

- All 0's (demodulator stuck at 0 - feeds all 0's into the scrambler)

- 127 16 bit sequences generated by scrambling an all 1's sequence (demodulator 	 		stuck at 1 - all 1's feed into et scrambler)

PHY signalling bits are provided to support current and future 802.11 DS PHY features as well as provide for proprietary (vendor specific) capabilities. The PSF bits are defined  first in time to last in time as follows:

E0,S1,S2,S3,S4,V1,V2,V3

E0: extension field, E0=1 signals presence of an additional PSF,  E0=0 notifies last PSF.

S1 to S4: 802.11 signalling bits. The following are currently defined in S1,S2,S3,S4 order:

- 1000 DQPSK at 2 Mbps

- 0100 BPSK at 1 Mbps

- remaining patterns reserved for future use.

V1 to V3: vendor bits, These bits signal the use of proprietary vendor features.

As previously stated, the E0 extension bit signals the presence of an additional PSF. The extension field in each new PSF can be used to signal an additional PSF. All PSF will have the same format described above. The signalling extension capability provides maximum flexibility for future 802.11 DS standard growth.

(A maximum number of PSF's must be defined to inhibit chaining errors).

A PHY packet termination capability (end-delimiter) must be defined but has not currently been determined.

The following techniques have been determined to be unacceptable and can be immediate discarded:

- HDLC/SDLC or other bit insertion access protocol

- A fixed End Of Message Word(s) which would require "word substitution" to be 	unique and provide some hamming distance for random data transmission (e.g. 	BYSINC)

- A fixed packet length which for a long length would be wasteful for short message traffic or a short length which would be wasteful for long message traffic such as file transfers.

NCR proposes to determine the end of a frame like in  802.3.  The Phy can signal the end of frame to the MAC by monitoring 'end of energy' or 'end of modulation' after the correlator. The drop of several dB's within a few (e.g. 8) symbols in the output level of the correlator can only be caused by a signal drop and thus is easy detectable. The accuracy requirement is deduced from the FCS processing in the MAC. In 802.3 the FCS boundary is octet based, which means that the 'end of frame' must be detected within 8 symbols after the last FCS byte.

The advantage of the above described approach is that the end delimiting function is a clear PHY function; there is no mixture of PHY and MAC functionality.

Telxon proposes a deterministic method to establish packet termination:

- Add a 16 bit packet length field following the signalling bits. These bits would form a 16 bit word which indicates the MAC packet length in bytes. The 16 bit length field would be included in the CRC check sum calculation.

-A maximum packet length (less than full 16 bits) would be specified to set a worst case maximum boundary in case of errors in the length field. Errors which result in packet lengths over the maximum length would automatically terminate the reception transaction. Errors which result in valid packet lengths will be flagged by the CRC check sum. Telxon proposes a maximum packet length of 2048 bytes + MAC overhead (TBD).

- Additionally, extended loss of correlation could be used to terminate packet reception prior to reaching the indicated packet length. This is not a formal requirement. Additional packet length error handling capabilities are left to individual developers/vendors to implement at their own discretion.

# BIT error rate

The bit error rate will be specified (30) at 10 exp-5 @ Eb/No =16 dB. The receiver sensitivity will be better than -77 dBm at this BER.

Calculation of the bit error rate is as follows:

In the textbooks it can be found for a DQPSK signal a BER of 10 exp-8 for a SNR of 18 dB. In the curves is a factor 10 in error rate .9dB in SNR.

Taking into account 4 dB implementation margin we have 10exp-8 @ SNR 22 dB.

This translates to a BER of 10exp-5 @ SNR 19 dB. Due to two bits per symbol the difference between SNR and Eb/No is 3 dB which results in:

BER 10exp-5 @ Eb/No = 16 dB

Receiver sensitivity is based on the required SNR of 19 dB and the effective unspreaded bandwidth of 1 MHz. For an environment with man made noise of 18 dB above thermal noise, which is -174dBm/Hz the man made noise power in 1 MHz is -96dBm. This results in a minimum receive level of -77dBm.

# Tx/rx and rx/tx switch times

The RX to TX and TX to RX worst case time specification is to be determined (26 and 27).

For reasons of CSMA/CA protocol efficiency, the RX to TX switch time should be set as low as possible.

# Signal detection

A 'listen before talk' educate should be used in the ISM band. A spread spectrum educate must be established with the FH PHY group for collision avoidance.