# MAC State Machine

Proposed Changes

By  Pablo Brenner

Bob O'Hara

# What can be done better?

## Current Situation

The transaction for CTS and ACK timeout, are triggered by strict timeouts, during these "waiting states" all information is discarded.

## Potential Problems

1. A "slow" station CTS may be discarded if CTS timeout too short.

2. A new RTS (from other station) may be discarded if timeout too long.

# Proposed Solution

Use a longer timeout (more flexible to slow implementations),and

let any new frame trigger a "timeout" transition (based on the fat that if we have received other frame we can assume that we are not receiving the CTS or the ACK)

## Changes Description

New Flag: Rx_flag (indicates we've received a frame, even if not addressed to us)

On Rx State Machine:

Any Transition from R1 sets the Rx_flag

On Control State Machine:

·	Transition C12 clears Rx_flag, and Frame Type Flags

·	Transition C34 clears Rx_flag, and Frame Type Flags

·	Add Transition C27 when Rx_flag is set

·	Add Transition C47 when Rx_flag is set

·	Enlarge CTS_timeout and ACK_timeout

·	C70 does not Reset Frame Type Flags