IEEE P802.11

Wireless Access Methods and Physical Layer Specifications

_________________________________________________________________

Interferences between existing IR-Techniques and Present/Future Requirements

to Avoid Them

Date:	March , 1994

Author:	Peter Blomeyer

AndroMeDa GmbH

Benzstr. 30

82178 Puchheim-Bhf

Phone:xx49-89-8901480

Fax: xx49-89-806040

__________________________________________________________________________________

ieee10.doc

Interferences between existing IR-techniques and present/future requirements to avoid them

1. Avoidance of interferences between present products

In present IR-communication devices, no provision is made to avoid influence at devices of other manufacturers.

For my best knowledge, the first concept that explicitely allows the coexistence of different systems, is EXIRLAN.

As the amount of IR-communication applications grow rapidely, DKE/CENELEC/IEC have evaluated the opportunities to avoid those interferences [1]. The idea is, to label each IR emitting device showing certain characteristics, and thus to allow the user to take precautions from the beginning to avoid interferences.

The result of this proposal will be shown and discussed in this paper.

2. Classification Criteria

The IEC-proposal chooses 4 criteria to separate IR-transmission.

These criteria are:

1. IR-wave length

2. Kind of modulation

3. Duration of transmission

4. Power charasteristics

It suggests the following:

a) every IR-transmitter will be marked with a label defining the 4 criteria listed above.

b) a user can compose a mix of different applications not interferencing

with each other, selecting the right products according the 4 criteria they are marked with.

THIS ASSUMPTION IS MEANINGSLESS DUE TO TECHNOLOGICAL FACTS; ECCEPT CRITERION 2, IF ELECTRICAL CHANNEL ASSIGNMENT IS STANDARDIZED.

The problem is, that those criteria for separation of IR-applications in 1 room are badly needed, but again, that criterion 1,3, and 4 do not meet the technical requirements of the users, though manufacters and organisations such as IRDA claim they did. Also criterion 2 does only help, if IR-data transmission will be standardized (if possible worldwide) with fixed modulation schemes and mandatory electrical channalization as proposed in the EXIRLAN-concept.

3. Discussion of the Classification Criteria

The following page shows Table 3 of [1]. Here are some comments to these criteria:

3.1. IR wave length

There are no optical separation techniques available today basing on the distinction of wavelengths at a cost, which makes any sense for the addressed applications, and there is nothing underway for at least the next 10 years to change this situation.

If this changes, say, after 15 years, this will give an additional flexibility to IR-communication, but it is not an item standardization can rely on. In addition, the 4 wavelength classes (as proposed) are not enough to give the required flexibility.

3.2. Modulation

The only way to separate IR-applications is by assigning electrical bands similar to the radio environment.

It is not sufficient to classify kind of modulation. Instead, electrical bands of the IR-modulation must be limited and standardized.

3.3. Duration of Transmission

A true time multiplex or time slot assignement between non-synchornized applications is impractical.

Of course (again, as an analogue to the transmission power of the radio world), a minimum transmission duration could be specified, that will not be considered by the standard. In practice, it will apperar as occasional noise, which in the most case just slows down the other system.

3.4. Power Characteristics

This criterion is a very dangerous one. It suggests, that it is possible, to separate IR-systems

a) by the output power of the transmitter and

b) by the beam angle of the transmitter.

Specifically the IRDA-association uses those arguments, i.e. they would not interfere in practice with other systems, as their operating distance is only 1 or 3 meters.

THAT IS AN ABSOLUTELY INVALID STATEMENT, because the operating range is a function not only of the transmitting power, but also of the sensitivity of a receiver.

Primitive and cheap receivers, like in the IRDA-concept or in consumer electronic products, will have a very low sensitivity and, therefore, need high power outputs from their associated transmitters.

Such a transmitter will interfere consequently with another (professional) application device over

a) a much larger distance than specified for its own operation

b) a much larger angle due to a) and reflections("bouncing").

4. Conclusion

The following page shows Table 4 of [1]. We strongly support to issue a committing rule to label all IR-transmitters (also lamps !!!) according to this document.

But, in addition, a standard has to be implemented, which guarantees IR-communication applications in the same room via electrical channalization - again,  EXIRLAN is such an approach, which tries to compromise with all existing industrial options. We also suggest to IEEE802.11, to take into account these activities by IEC and, if necessary, to find a common concept soon.

5. References

[1]	IEC document 84 (sec) 340 Draft IECxxx: Transmission using Infra-red radiation,

Part 1: General, Nov 1993