IEEE P802.11

Wireless Access Methods and Physical Layer Specifications

__________________________________________________________________________________________________

Specification and Template of the IEEE802.11 Infrared PHY

EXIRLAN / FQPSK-based Modulation and Baseband, Draft 1

Date:			March , 1994

Author:			Peter Blomeyer

Editor IEEE802.11-IR Standardization

Benzstr. 30

82178 Puchheim-Bhf

Phone:xx49-89-8901480

Fax: xx49-89-806040

____________________________________________________________________________________________________________________

ieee9.doc

| Topic | Val | Parameter | Specification | Comments |
| --- | --- | --- | --- | --- |
| 1 | G | Number of carriered channels | 8 |  |
| 2 | N | Frequency range | 6...30 MHz |  |
| 3 | N | Bandwidth per carriered channel | 3 MHz |  |
| 4 | N1 | Modulation | FQPSK |  |
| 5 | N2 | Modulation | multistate FQPSK (4*4)\(8*8) | linearized diodes required |
| 6 | N1 | Data rate/channel | 1, 1.4, 2.8, 4.2 Mb/s | selectable (no hopping!) |
| 7 | N2 | Data rate/channel | 1, 1.4, 2.8, 4.2, 5.6, 8.4, 10, 12.6 Mb/s | selectable (no hopping!), 2nd compatible generation |
| 8 | N | Eb/ No. for BER = 10 exp-5 | 10...23 dB |  |
| 9 | N | Spectral Suppression (PSD) | -20 dB | at edge of band |
| 10 | N | IR-wave length | 880 nm |  |
| Topic | Val | Parameter | Specification | Comments |
| 11 | B1 | Modulation | PPM |  |
| 12 | B2 | Modulation | 16PPM |  |
| 13 | B | IR-wave length | 880 nm |  |
| 14 | B1 | Data rate | 19.2, 115.2 kb/s | with PPM |
| 15 | B2 | Data rate | 1 Mb/s | with 16PPM |
| 16 | B | Eb/No. for BER = 10 exp-5 | normalized to bit rate bandwidth |  |
| 17 | B | receiver bandwidth | max. 0.5...5.5 MHz |  |
| 18 | B | transmitted spectrum | < 5.5 MHz | at 20 dB |

Legend for Val (=validity):			G	General

N	Narrowband part

B	Baseband part

1	1st generation

2	compatible 2nd generation