Title:	2 Mbps  FREQUENCY HOPPING PHY

PROPOSAL

Date:	March 7,  1994

Author:	Juan Grau

P P roxim

295  N. Bernardo Av.

Mt. View,  CA  94043

Telephone: (415) 960-1630

# Introduction

This contribution  is submitted to complete the definition of a 2 Mbps Frequency Hopping PHY introduced to the 802.11  committee  on Jan '94.  The data presented in this document in support of the 2 Mbps FH PHY is based on measurements made on existing technology available today.  The document's main objective is to build on the specifications presented previously and provide adequate justification for the choices made.  It addresses issues previously raised such as interoperability with the baseline PHY,  Clear Channel Assessment (CCA),  throughput,  implementation complexity and gear switching.

# Objectives of a 2 Mbps FH PHY

The specification of the proposed 2Mbps FH PHY was based on achieving a set of objectives consistent with the 802.11 PAR,  the approved 1 Mbps FH PHY,  and developments on the DSSS PHY and in the MAC sub-committees.  These objectives are summarized below.

1.-  Provide a higher speed FH PHY option to nodes within a BSA population served by a 1 Mbps FH PHY.

2.-  Make the high speed capability accessible at minimal additional cost/complexity.

3.-  Be interoperable with a 1 Mbps base mode PHY without burdening the design of the base mode with additional complexity.

4.-  Remove obstacles to the implementation of efficient gear switching algorithms.

5.-  Be consistent and competitive with multiple rates options available in the DSSS PHY and in the process of being addressed by the MAC.

This contribution is targeted to show that these objectives can be met using 4GFSK modulation, removing obstacles to the acceptance of 4GFSK as the FH high speed mode standard.  This proposal complies in all aspects with the requirements set forth in  "Modulation Selection Criteria for Higher Data Rate FH" document originally drafted in the September '93 meeting.

# proposal

We propose  to use  4GFSK modulation to achieve  2 Mbps channel rate in the available  1 MHz bandwidth.  A number of advantages can be found in using 4GFSK modulation as a high speed mode to the 1 Mbps, GFSK  base mode.  These advantages were discussed in a previous contribution  and are summarized below.

•	Both receiver and transmitter architectures (including modulator/demodulator)  can be the same for the high speed mode as for the base mode,  simplifying implementation.

•	Coherent or non-coherent types of demodulation can be used.

•	Symbol rate is the same for the high rate and the base mode.   Filters do not have to change.

•	Clock recovery is easier since the symbol rate does not change.  The same preamble can be used for high speed packets as for the normal packets, simplifying the gear switch algorithms in the MAC.

•	Modulation based Carrier Sense (for CCA) can be implemented if desired.  Since symbol rate does not change,  symbol timing information can be used.

•   2 Mbps functionality can be implemented as an enhancement to the radio's back end,  leaving the front end (through the modulator/demodulator) unchanged .  Class C amplification and hard limiting in the demodulation can be used.

## System Parameters

The following table summarizes the more significant modulation parameters of the 4GFSK approach.  The complete specification framework can be found in Appendix A.

Bit Rate = 2 Mbps Symbol Rate = 1 Mbps Modulation = 4GFSK 4GFSK BW20dB = < 1 MHz Using conservative interpretation _F = ± 225 KHz p-p => h = .150 (= 2/3*_F//Fs) BT = .5 Eb/No= 22.5 dB Non-coherent demodulation with .

Eb/No  used is based on a non-coherent demodulator model with the modulation parameters specified above.  This number has been confirmed in a practical implementation.  Coherent demodulation techniques would yield better results at the expense of increased system complexity.

The following figures depict real measurements made on a functional system with the modulation characteristics specified above. The modulation filter employed in these measurements is a 6th order Bessel filter,  equally distributed between the transmitter and the receiver.  If all the filtering were allocated to the transmitter,  the modulation bandwidth would be slightly narrower than what is shown.  We used this filter because it gives performance that is very close to a Gaussian filter and is relatively easy  to implement. A pseudo random binary sequence of length 29-1 is used for all measurements.

Figures 1 and 2 depict the power spectral density (psd) to demonstrate compliance with the 1 MHz 20 dB bandwidth FCC requirement. Figure 3 shows the corresponding eye diagram at the output of the receiver including all of the channel filtering  (note that  a digital oscilloscope with infinite persistence  was used for the measurement, making the eye opening look a bit worse than it really is).  Figure 4 depicts  adjacent channel interference (ACI) values.  This is accomplished by measuring the output spectrum on a 1MHz resolution bandwidth.  The ACI value can be read directly off the chart at the desired separation.  A practical receiver bandwidth will be < 1 MHz (but > 500 KHz).  The resulting ACI for such a receiver will be a bit better than the chart indicates.

A similar set of measurements for a GFSK system is included in Appendix B for reference. The modulation parameters used are those defined in Motorola's  proposal.

FIGURE 1.-

4GFSK power spectral density meeting the required 1 MHz 20 dB bandwidth.

FIGURE 2.-

4GFSK psd in adjacent and alternate channels.

FIGURE 3.-

4GFSK EYE diagram.

FIGURE 4.-

4GFSK Adjacent Channel Interference

## Block Diagram

One of the advantages of using 4GFSK as the high speed mode to the GFSK base mode is that the two transceiver's block diagrams can be very similar.  Figure 5 shows a generic block diagram for either a 4GFSK or a GFSK transceiver.   The Back-End Processor is the only element that distinguishes the two systems and only a few components of the Back-End Processor need change.

Because the symbol rate is always 1 Mbaud,  there is no need to change IF, Pre-Mod or Post Detection filter bandwidths, loop bandwidths or any other time constants in the system.  Most of the block diagrams available for a GFSK designs can be used with the proposed 4GFSK modulation.  Techniques such as coherent and non-coherent demodulation, heterodyne or homodyne transmitter and receiver architectures,  linear and non-linear amplification are all available alternatives and usable with both modulation schemes.

Note:  It should be noted that the 4GFSK system does require better linearity in the FSK modulator/demodulator than would be necessary for a GFSK only system .

µ §

BLOCK DIAGRAM

Figure 5

The Back-End Processor  provides the interface from the analog portion of the transceiver to the digital section and on into the PHY Adaptation layer. Back-End Processor functionality includes threshold detection (slicing),  clock and data recovery,  carrier sense (modulation based),  encoding,  scrambling, preamble generation and detection etc.   All of these functions are required in either  GFSK or  4GFSK  implementations.  In some cases the GFSK implementation will be simpler than the 4GFSK one.

Figure 6 expands on a block diagram for a possible 4GFSK Back-End Processor design.   The shaded boxes depict those functional blocks that differ between a GFSK and a 4GFSK system.  As evident from this figure,  there is a lot of commonality between the two systems. The following is a brief description of how each of the blocks differ.

BACK END PROCESSOR

Figure 6

Selected elements of the Back end processor are described in more detail in the following paragraphs.

### Slicer.-

The slicer is the block responsible for making the state decision at the decision instant.  The simplest slicer is a straight 4-level slicer.  It must discern between four possible states which requires setting up three threshold levels.   The thresholds are derived from knowledge of the average received frequency  (Fc) and the deviation (h) between states.

4-LEVEL SLICER

Figure 9

### Clock Recovery.-

A variety of standard clock recovery circuits will work equally well for both GFSK and 4GFSK systems due to the fact that the symbol rates are the same.  The probability of a transition occurring at a symbol boundary is materially the same in both cases and circuits that look for transitions and lock onto them are viable solutions.

### Preamble Detection/Generation.-

Several proposals have been made on preambles,.  Most of them include a period of time for antenna diversity selection,  a pre-determined pattern suitable for quick clock recovery and CCA, a unique  framing word  followed by a PHY header and a PHY CRC.

PHY Header/Preamble

FIGURE 10

The preambles are always sent in GFSK which 4GFSK systems are required to detect.  In order to know whether a packet is a low or high rate packet, the PHY header should include information indicating so.   This information is used to activate the gear switching mechanism.  The 4GFSK receiver maintains clock synchronization (bit and byte) through the gear switching process and therefore packet synchronization is also maintained.  Note that since the symbol timing is the same for GFSK and 4GFSK, maintaining synchronization during the gear switching process is trivial and essential to keeping the integrity of the packet framing.  The switch from GFSK to 4-GFSK, if requested,  occurs at the boundary between the end of the PSF and the beginning of the MPDU.

Proposals using symbol rates that are not  multiples of the GFSK rate would have difficulty maintain synchronization during the gear switching process .  The gear switching process would be further complicated by the need to switch filter bandwidths, compromising the bit, byte and packet synchronization achieved during the preamble.

The DSSS PHY group is addressing the very same issues in a fashion similar to what is described above.  Consistency between the two approaches is desirable.

### Clear Channel Assessment (CCA).-

Several proposals have been made regarding CCA algorithms.  They require two basic capabilities:  an Energy Detect mechanism and a Carrier Sense  (modulation based) mechanism. A third capability that has been considered is a Preamble Detect mechanism.  The previous section dealt with the preamble detection and showed that since all packets would send preamble in GFSK, there wouldn't be any compatibility problems between a GFSK system or a 4GFSK one. Additionally,  there is clearly no difference between how the GFSK and the 4GFSK system would provide the energy detection capabilities. Thus, the only required CCA element that needs to be addressed in this contribution is modulation based Carrier Sense.

CCA proposals have placed the following requirements in the Carrier Sense capability:

1.-  	Carrier Sense must be modulation based and not merely energy based.

2.- 	Carrier Sense most be available throughout the duration of a packet.  If a station comes on line in the middle of an ongoing transmission from another station,  the CS function needs to be able to detect such an occurrence of whether the ongoing packet is a GFSK or a 4GFSK packet.

3.-  	Carrier Sense must be reciprocal for GFSK and 4GFSK systems.  A 4GFSK station must be able to yield to a friendly GFSK transmission and vice-versa.

This contribution proposes the following  simple and  effective modulation based carrier sense mechanism meeting the above requirements.   The phase error signal from the clock recovery circuitry is fed into the Carrier Sense block.  This phase error is the difference between the phase of the recovered clock and the modulation clock  (this signal is typically found in clock recovery circuits).  The Carrier Sense block averages the absolute value of the phase error.

FIGURE 11

If the average error between the phase of the modulation clock and the recovered clock  is less than a pre-determined threshold,  it can be concluded that a signal with the "appropriate" modulation was present (phase error for "other" types of signals  will be uniformly distributed between 0 and 2_).  The time constant for the averaging function should be approximately 30 symbol times.  Shorter time constant would result in false triggers while longer time constants would increase the CS response time unnecessarily.  Since both GFSK and 4GFSK have a 1 Mbaud symbol rate,  this approach to  Carrier Sensing would work equally well for both type of signals.

### Signal Conditioner.-

The signal conditioner prepares the signal for the modulator.  Some of the functions required in this block are filtering and level control.  As has been mentioned before,  the filtering requirements proposed are the same as for the GFSK system: gaussian filter with a BT = .5.  The filter can be implemented in digital or analog form.

## Other System Considerations

### •  Coverage.-

It was stated as one of  the  objectives of this proposal to  provide a higher speed FH PHY option to nodes within a BSA population served by the base  1 Mbps mode.  Figure 12 shows a typical BSA.  Multipath and other propagation impairments will cause the coverage area to be irregular.  For any kind of BSA or ESA planning, a guaranteed coverage area will have to be specified which ensures a predetermined level of  services are available to the population within the specified area.  The resulting guaranteed coverage area will be a circle (or a hexagon) which includes a fading margin as shown.

If the system were being designed to operate exclusively in 4GFSK,  then the fading margins required to establish the limits for the minimum service area would cause the reduced Eb/No performance of the 4GFSK system to result in a reduced service area. However, this is not the case.  The 4GFSK system will always be able to revert back to GFSK performance and therefore only the GFSK performance will determine the size of the BSA.  Since the 4GFSK capability is made available on a "best effort" basis,  it will improve the performance of all stations  within the 4GFSK region.  Since packets transmit faster,  they will also indirectly increase the overall system throughput capability for all units operating in either GFSK or 4GFSK modes.

N. Chayat from Lannair reports in his "2 Mbit/sec  M-FSK Higher-Speed PHY Proposal" paper that the packet loss probability in a flat fading Rayleigh channel is inversely related to Eb/No without diversity and inversely related to the square root of Eb/No using two level diversity. The BSA is originally setup to provide the desired 99.5% channel availability  to the GFSK stations.   Assuming an increase on required Eb/No of 5 dB for the 4GFSK system  (given proposed 18 to 19 dB for GFSK and 22.5dB for 4GFSK)  would yield an increase in  the outage probability by a factor of 10 using 2 level diversity and a factor of slightly over 3 with no diversity.  The resulting channel availability numbers would be 95% for  the diversity case and 98.5% with no diversity.  In either case,  a vast majority of stations within the GFSK BSA will have access to the higher speed channel.

µ §

FIGURE 12

In contrast to proposals whose objective is to increase the BSA's guaranteed coverage area (by improving its performance in terms of Eb/No) this proposal increases the throughput available to the majority of stations within the "guaranteed" GFSK coverage area.

### •  Gear Switching.-

Gear switching algorithms will be defined by the MAC using primitives provided by the PHY to control and detect the rate of transmissions of packets.  The MAC group is already chartered to address the DSSS PHY gear switching capability and this proposal shouldn't require any special considerations to achieve the same goal.   The earlier section on  Preamble Detection/Generation discussed  the advantages of using  the same symbol rate for the high speed mode as for the baseline mode.  Avoiding filter bandwidth switching and maintaining bit, byte and frame synchronization  during the gear switching process significantly  simplify the implementation of the gear switching mechanisms.

It should be noted that gear switching algorithms are being used successfully today in commercially available frequency hopping PHYs.  Following are some alternatives in use and to be considered:

a)  Transmit all control and broadcast packets in the low rate mode.

b)  Include a bit in the MAC header indicating whether the station is capable of transmitting at the high rate.

c)  When initiating communication with a new station,  transmit first packet in GFSK.  The first response from the destination station will indicate whether the station is capable of the higher speed capability.

d)  Enter station's destination address in a table and send future packets in 4GFSK.

e)  If an ACK error is received  in a 4GFSK transmission, retry in 4GFSK.  If this results in a second ACK error,  make note in table and attempt  further retries in the GFSK mode.  This table entry should be aged relatively quickly.  The aging could occur over a few seconds or alternatively over a fixed number of packets or possibly both.  Choosing to wait a fixed number  packets before attempting to retransmit in 4GFSK will minimize the overhead due to possible 4GFSK errors.  If the aging number chosen is 100,  the few stations outside the 4GFSK range but within the GFSK range will waste 2 4GFSK transmissions every 100 packets.  The overhead is low particularly in light of the fact that  there will be only a few stations in this situation within one BSA.

f)  Connection oriented services negotiate the speed during the connection setup.

There are different optimizations available for the different services available from the MAC (Time bounded,  contention free-asynchronous,  contention based RTS-CTS protected and non-protected etc.) and for different types of traffic.  The algorithm described above is fairly simple, provides good performance and is intended as an example of a practical implementation being used today.

It should be noted that the MAC is required to support both low rate and high rate transmissions.  This is not saying, however,  that the MAC is also required to implement the algorithms that determine when transmissions should occur at the high rate.  Depending on the algorithms developed,  this may or may not be the case  (e.g.  a MAC management  or LLC management function may be appropriate entities to implement this functionality)

### •  Throughput.-

The question of what improvement in throughput could be expected by a doubling of the channel data rate is highly dependent on the choice of protocol.  The problem can be framed in a general context by considering the following two cases.  An assumption is made that a certain amount of overhead is required per transmission (overhead can be control packets, wait periods, back-offs etc.)  It is also assumed that the overhead rate and duration are the same for the low rate and the high rate modes.

1)  A protocol assigns a fixed amount of time to a station to complete a transmission (e.g.TDMA).  A given amount of protocol overhead is necessary for each allocated time slot.  Doubling the channel data rate would consequently allow twice as many bits to be transmitted per unit of overhead, increasing the effective throughput by 100%.

|  | Overhead | Data = 500 bits |  | in GFSK |
| --- | --- | --- | --- | --- |
|  | Overhead | Data = 1000 bits |  | in 4GFSK |

2)  A protocol  assigns a maximum number of bits to be transmitted by a station per unit of overhead.  For example,  contention overhead results in a 500 byte packet being transmitted.  The packet is then transmitted in either low rate or high rate modes.  The fixed overhead will then use a higher proportion of the channel for the high rate mode than for the low rate mode.  The throughput improvement  is described by  1/(1-p/x )  where x is the ratio of the high rate to the base rate (2 in our case) and p is the efficiency of the protocol in the base mode.  Assuming that the protocol achieves a 70% efficiency with 500 byte packets, then doubling the rate result in a throughput improvement of  approximately 55 %.

|  | Overhead=30% | Data=500 B = 70 % |  |  | in GFSK |
| --- | --- | --- | --- | --- | --- |
|  | Overhead=45 % | Data= 500 B = 55 % | Overhead | Data … | in 4GFSK |

Protocols under consideration will have some of both and can therefore expect to achieve throughput improvements somewhere between 50% and 100%.

### •  Complexity.-

It should be fairly obvious by now that the 4GFSK  high rate capability can be achieved with only a minor increase in complexity over the GFSK system.  Just like in GFSK,  various options are available for implementation,  including coherent or non-coherent  demodulation,  heterodyne or homodyne architectures,  non-linear amplification and all digital or analog/digital back end processor implementations.

To put the level of effort required to implement this capability in context,  the functionality described in the Back-End processor block diagram  (including dual mode -GFSK/4GFSK- operation) has been implemented in an ASIC with less than 4000 gates running on an 8 MHz clock. This implementation is consistent with low cost, low power robust operation.   This implementation is all digital (needs a 6 bit A/D), requires no adjustments and is packaged in a small 44 pin TSOP package.

### •  Impact on the GFSK system.-

Allowing the 2 Mbps FH PHY to share the frequency hopping PHY standard with the 1 Mbps GFSK PHY has the following impact:

•	Preamble:  Since the preamble will be in GFSK,  there is little impact other than the inclusion of some bits to indicate and allow gear switching.

•	Clear Channel Assessment:  Symbol timing based carrier sense can be used for clear channel assessment.  Since symbol timing is the same for both the GFSK and 4GFSK systems,  there will not be any significant impact on the design of the base mode functionality.

• 	Gear Switching:   All information necessary for network synchronization and control is transmitted in GFSK  (only payload is transmitted in 4GFSK).  The GFSK only system does not get involved or need to support  the gear switching algorithm other than observing CCA rules.

Consistent with the objectives of this proposal,  there is minimal impact on performance,  complexity or other operation of the baseline GFSK system.

# Summary

To close this discussion,  a summary of the advantages and objections to the 2 Mbps 4GFSK system is presented in the following paragraphs.

Advantages.-

•	Both receiver and transmitter architectures (including modulator/demodulator)  can be the same for the high speed mode as for the base mode,  simplifying implementation.

•	Most architectural options available for the GFSK design can be used for the 4GFSK implementation:  Coherent, non-coherent, heterodyne, homodyne,  linear and non-linear amplification,  digital or analog signal processing etc.  Manufacturers can make the design tradeoffs that best suit their target markets.

•	Symbol rate is the same for the high rate and the base mode.   Filters do not have to change. This simplifies gear switching significantly.

•	Clock recovery is easier since symbol rate does not change.  The same preamble can be used for high speed packets as for the normal packets also simplifying the gear switch algorithms in the MAC.

•	Modulation based Carrier Sense can be easily implemented.  Since symbol rate does not change,  symbol timing information can be used in Carrier Sense.

•   2 Mbps functionality can be implemented as an enhancement to the radio's back end,  leaving the front end (through the modulator/demodulator) unchanged.  Vendors with pure GFSK designs have an easy, backwards compatible upgrade path to the higher speed mode.

Objections.-

•	Decreased Eb/No performance.-  The decrease only cuts into the fading margin.  Since the high speed mode is additional capability to the base mode,  coverage is not compromised.  Moreover,  most of the population within a guaranteed coverage area will have access to the high speed functionality.

• 	Increased Complexity.-  Desired functionality can be achieved with minimal increase in complexity.  All of the required functionality can be implemented in a low cost, low power consumption digital ASIC.

•	Burden to the 1 Mbps GFSK PHY.-  Implementing Carrier Sensing based on symbol timing will eliminate any burden to the GFSK systems.

# conclusions

It has been shown that 4GFSK modulation can be used to provide a frequency hopping PHY with a higher speed capability without a significant increase in complexity and more importantly,  without burdening designs not providing the high speed capability.

In as much as products with higher speed capability will be commercially offered in the future,  it is desirable to have a FH PHY standard that reflects the current state of the art.   Speed is a key factor in enabling the use of wireless technology in a wide variety of applications.

What has been offered here is a 2 Mbps FH capability with an architecture and modulation scheme complying with all the criteria requirements established for the high speed FH PHY standard and fully compatible with the proposed 1 Mbps GFSK standard.

* The author would like to thank Jeff Sutton for his invaluable help in generating the data backing up the foundation of this proposal.

# Appendix A:  Specification Framework

| 1. | Tx & Rx | Frequency Range | 2.4 to 2.4835 GHz U.S.A 2.471 to 2.497 GHz Japan 2.4 to 2.485 GHz Europe | Same as GFSK |
| --- | --- | --- | --- | --- |
| 2a. | Tx & Rx | Minimum number of channels / set | 75 in U.S.A 20 in Europe 10 in Japan | Same as GFSK |
| 3. | Tx & Rx | Minimum number of hops per sec. | 2.5 | Same as GFSK |
| 3b. | Tx | Hopping sequences | TBD | Same as GFSK |
| 4a. | Tx | Transmitted power levels (mW) | a. max. 1000, 100, 10 mW b. 250, 50, 10 mW (Optional) | Same as GFSK |
| 4b | Tx | Minimum Trans. power level | 10 mW 1 mW for battery operated equip | Same as GFSK |
| 5. | Tx | Optional Transmitted power control | Four discrete power levels as in 4a. | Same as GFSK |
| 6. | Tx | Max. Radiated EIRP | per FCC part 15.247 in US per ETSI res 02-09 in Europe per RCR STD-33 in Japan | Same as GFSK |
| 7. | Rx | Receiver minimum input level sensitivity | -75 dBm @ 10-5 in 4GFSK -80 dBm @ 10-5 in GFSK | Non-coherent demod with margins. |
| 8. | Rx | Receiver Maximum input level | - 20 dBm | Same as GFSK |
| 9. | Rx | Alternate channel interference tolerance | -45 dB | Same as GFSK |
| 10. | Tx& RX | Channel bandwidth @20 dB | ±500 KHz | Same as GFSK |
| 11. | Tx | Occupied channel bandwidth (spectrum shape) | 20 dBc @ _F = ± 0.5 MHz 45 dBc @ _F = ± 2.0 MHz 60 dBc @ _F = ± 3.0 MHz | Same as GFSK |
| 11a. | Tx | Modulation Mask | TBD | Same as GFSK |
| 12a. | Tx | Transmitter Center frequency tolerance | ±25 ppm | Same as GFSK |
| 12b. | Rx | Receiver center frequency acceptance range | ±25 ppm | Same as GFSK |
| 13. | Tx & Rx | Modulation | 4GFSK Diff. Encoded _F = ± 225 KHz BT=.5 | 4 level GFSK h = 2/3 _F where _F = Peak Deviation |
| 14. | Tx & Rx | Channel Data Rate | 2 Mbps |  |
| 15. | Tx & Rx | Fallback data rate | a. 1 Mbps (GFSK BT=.5) |  |
| 16. |  | Phy supplied Clock Jitter | < .0625 µsec (rms) | Same as GFSK |
| 17. |  | Bit Clock Accuracy (baseband) | TBD | Same as GFSK |
| 18. | Tx & Rx | Preamble length | 114 symbols | Same as GFSK + 8 sym. |
| 19. | Tx & Rx | Clock Recovery | withstands patterns up to (7) continuous 1's or 0's with no degradation in output SNR and BER. Scrambling polynomial: 1+x-4+x-7 | Same as GFSK |
| 20. | Rx | Carrier detect response time | TBD | Same as GFSK |
| 21. | Tx & Rx | Spurious emissions in the frequency band | -64 dBc (@ _F_ º4 MHz from Fc) | Same as GFSK |
| 22. | Tx & Rx | Spurious emissions out of band | per FCC part 15.247, 15.205 and 15.209 in USA per ETSI RES 02-09 in Europe. | Same as GFSK |
| 23. | Tx& Rx | Switching time TX to Rx | 100 µsec | Same as GFSK |
| 24. | Tx & Rx | Switching time RX to TX | TBD | Same as GFSK |
| 25. | Tx & Rx | Channel switching time (hop settling time) | 300 µsec | Same as GFSK |
| 26. | Rx | BER at specified Eb/No (pending channel model) | BER _10-5 @ Eb/No = 22.5 dB | -non-coherent demod. -Includes modem implementation margin |
| 27. | Tx & Rx | Channel availability | 99.5 % | Same as GFSK |
| 28. |  | Data Line / Clock input / output jitter | TBD | Same as GFSK |
| 29. | Tx & Rx | Antenna port impedance (if exposed) | 50 ohms | Same as GFSK |
| 30. | Tx & Rx | VSWR | Device shall stand 0_VSWR__ with no damage - Operational VSWR = TBD | Same as GFSK |
| 31. |  | Interface lines to Convergence layer (when exposed) | Rx Data Tx Data Rx/Tx clock Data valid Control line Status line Ctl/Status clock RSSI | Same as GFSK |
| 32. |  | PHY-MAC Net Management info/control variables | TBD by PHY group | Same as GFSK |
| 33. |  | Safety Requirements | Compliance with applicable Safety Agencies requirements [TBD] | Same as GFSK |
| 34. |  | DTE/DCE Interface | TBD | Same as GFSK |
| 35. |  | Higher Data Rate Negotiation | preamble same as GFSK | Same as GFSK |

# Appendix B:  GFSK MEASUREMENTS

The following table summarizes the more significant modulation parameters of the 4GFSK approach.  Complete specifications framework can be found in Appendix A.

Bit Rate =  	2 Mbps

Symbol Rate =	1 Mbps

Modulation =	GFSK*

BW20dB =  	< 1 MHz	Using conservative interpretation

_F =	± .17 KHz p-p	=>  h = .34

BT =	.5

*  The channel filtering used is a 6 pole Gaussian filter (3 poles in the receiver and 3 poles in the transmitter). A pseudo random binary sequence of length 29-1 is used for all measurements.

FIGURE B.1.-

GFSK power spectral density.

FIGURE B.2.-

GFSK psd in adjacent and alternate channels.

FIGURE B.3.-

GFSK EYE diagram.

FIGURE B.4.-

GFSK Adjacent Channel Rejection

Adjacent Channel Rejection plot is a measurement of the transmitter's psd in a 1 MHz resolution bandwidth.  As was the case with the 4GFSK measurement,  the 1 MHz resolution BW is slightly higher than would be required by an optimum receiver and makes the numbers look a bit worse (by one or two dB) than could be achieved,  although the results are probably more in line with what a practical system would yield.

.- J. Grau,  "High  Speed Frequency Hopping PHY Proposal,"  Doc. IEEE P802.11/94-8,  January, 1994.

.-  N. Sileberman,  "Modulation Selection Criteria for Higher Data Rate FH",  Doc. IEEE P802.11-93/164,  September, 1993.

.- J. Grau,  et.al. …

.-  S. Haykin, "Communications Systems"  John Wiley & Sons, Inc.,  New York,  1978.

.- Jim McDonald,  "Discussion of Modulation Parameters for the 2.4 GHz Frequency Hop Physical Layer",  Doc.  IEEE  P802.11-93/102,  July,  1993.

.-  Jim McDonald,  "Preamble Proposal for the 2.4 GHz Frequency Hop Standard with revisions", Doc. IEEE P802.11-93/209r.

.-  E. Geiger,  "Wireless Access Methods and Physical Layer Specifications",  Doc.  IEEE p802.11/94/018,  January,  1994.