Propose Standard:

Physical Layer Specifications

for

2.4GHz Frequency Hopping Spread Spectrum

Wireless LAN

This is an unapproved draft of a portion of a proposed IEEE Standard, subject to change.  Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities.  Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for the purposes of developing a national position.  Other entities seeking permission to reproduce this document for standardization or other activities, or to reproduce portions of this document for these or other uses, must contact the IEEE Standards Department for the appropriate license.  Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department

Copyright and Permissions

445 Hoes Lane, P.O. Box 1331

Piscataway, NJ  08855-1331, USA

Copyright (c) 1993 IEEE.  All rights reserved

Editors:      	Ed Geiger

Apple Computer

One Infinite Loop

Cupertino,  CA   95014

edg@apple.com

Dean Kawaguchi

Symbol Technologies Inc.

1101 South Winchester Blvd.

San Jose, CA 95128

deank@symbol.com

Keith Furuya

Xircom

kfuruya@xircom.com

Tim Blaney

Xircom

tblaney@xircom.com

Table of Contents

9. Introduction	1

9.1  Introduction to FHSS Phyiscal Layer.	1

9.2.1  Introduction.	1

9.2.2  PHY_DATA.request Parameter List.	1

9.2.3  PHY_DATA.indicate Parameter List	2

9.3. FHSS Physical Layer Convergence Procedure	3

9.3.1  Introduction.	3

9.3.2  Physical Layer Convergence Procedure Frame Format.	3

Figure 9-1.  PLCP Frame Format	3

9.3.2.1  PLCP Preamble.	3

9.3.2.1.1  Sync.	3

9.3.2.1.2  Start Frame Delimiter.	3

9.3.2.2  PLCP Header.	3

9.3.2.2.1  PLCP Signaling Field.	4

Table 9.1.  PLCP Signaling Field Bit Descriptions	4

9.3.2.2.2  PLCP_PDU Length Word.	4

9.3.2.2.3  Header Error Check Field.	4

9.3.2.3  PLCP_PDU Data Whitener.	5

Figure 9-2  Frame Synchronous Scrambler/Descrambler.	5

Figure 9-3  PLCP_PDU Data Whitener Format.	5

9.3.3  PLCP Transmit Procedure.	5

9.3.3.1  Transmit State Machine.	6

Figure 9-4  Transmit State Machine	7

Figure 9-4a  Data Whitener Encoding Procedure	8

9.3.3.2  Transmit State Timing (OPEN).	8

Figure 9-5  Transmit State Timing	9

9.3.4  Carrier Sense Procedure (OPEN).	9

9.3.4.1  Carrier Sense State Machine (OPEN).	9

Figure 9-6  Carrier Sense State Machine	10

9.3.4.2  Carrier Sense With Diversity State Timing (OPEN).	10

Figure 9-7  Carrier Sense State Timing	10

9.3.5  PLCP Receive Procedure.	10

9.3.5.1  Receive State Machine (OPEN).	10

Figure 9-8  Receive State Machine	12

Figure 9-8a  Data Whitener Decoding Procedure	13

9.3.5.2  Receive State Timing (OPEN).	13

Figure 9-9  Receive Timing	14

9.4 PLME_SAP Layer Management.	15

9.4.1  PLME_SAP Sublayer Management Primitives	15

Table 9.2. MPHY_SAP Sublayer Management Primitives	15

9.4.2  PLME_SAP Management Service Primitive Parameters.	15

Table 9.3.  PHY_SAP Service Primitive Parameters	15

9.4.3  PLME_SAP Detailed Service Specifications.	15

9.4.3.1  PLME_RXRESET.request	15

9.4.3.2  PLME_RXRESET.confirm	16

9.4.3.3  PLME_TXRESET.request	16

9.4.3.4  PLME_TXRESET.confirm	16

9.4.3.5  PLME_SETCHNL.request	17

9.4.3.4  PLME_SETCHNL.confirm	17

9.5 FHSS Physical Medium Dependent Sublayer 1.0M Bit	18

9.5.1 Scope and Field of Application.	18

Figure 9-10  PMD layer Reference Model	18

9.5.2  Overview of Services.	18

9.5.3  Overview of Interactions.	18

9.5.4  Basic Service and Options.	19

9.5.4.1  PMD_SAP Peer-to-Peer Service Primitives.	19

Table 9.4. PMD_SAP Peer-to-Peer Service Primitives	19

9.5.4.2  PMD_SAP Sublayer-to-Sublayer Service Primitives.	19

Table 9.4. PMD_SAP Sublayer-to-Sublayer Service Primitives	19

9.5.4.3  PMD_SAP Service Primitives Parameters.	20

Table 9.5.  List of Parameters for PMD Primitives.	20

9.5.5  PMD_SAP Detailed Service Specification.	20

9.5.5.1  PMD_DATA.request	20

9.5.5.2  PMD_DATA.indicate	21

9.5.5.3  PMD_TXRX.request	21

9.5.5.4  PMD_PARAMP.request	21

9.5.5.5  PMD_ANTSEL.request	22

9.5.5.6  PMD_TXPWRLVL.request	22

Table 9.6.  Transmit Power Levels	22

9.5.5.7  PMD_FREQ.request	23

9.5.5.8  PMD_RSSI.indicate	23

9.5.6 MPMD_SAP Sublayer Management Primitives.	23

Table 9.7.  MPMD Layer Management Primitives	23

9.5.6.1 MPMD_SAP Management Service Primitives Parameters.	24

Table 9.8.  List of Parameters for PMD	24

9.5.6.1  MPMD_PWRMGNT.request	24

9.5.6.2  MPMD_SYNLOCK.indicate	24

9.5.7  1M Bit PMD Operating Specifications General.	25

9.5.7.1  Operating Frequency Range.	25

Table 9.9.  Operating Frequency Range.	25

9.5.7.2  Number of Operating Channels.	25

Table 9.10.  Number of Operating Channels	25

9.5.7.3  Operating Channel Center Frequency.	26

Table 9.11  USA and Europe Requirements	26

Table 9.12.  Japan Requirements	27

9.5.7.4  Occupied Channel Bandwidth.	27

Figure 9-11.  Occupied Channel Bandwidth.	27

9.5.7.5  Hop Rate.	28

9.5.7.6  Hop Sequences.	28

9.5.7.7  Spurious In-Band Emissions.	28

Figure 9-12.  In-Band Spurious Response	29

9.5.7.8  Spurious Out-of-Band Emissions.	29

9.5.7.9  Modulation.	29

Figure 9-13.  Transmit Modulation Mask	30

9.5.7.10  Channel Data Rate.	30

9.5.7.11  Channel Switching/Settling Time (OPEN)	30

9.5.7.12  Transmit to Receive Switch Time (OPEN)	30

9.5.7.13  Receive to Transmit Switching Time (OPEN)	30

9.5.7.14  Operating Channel Availability (Closed per PAR)	30

9.5.7.15  Antenna Port Impedance/VSWR  (OPEN).	30

9.5.8  PMD Transmit Specifications.	30

9.5.8.1  Transmit Power Levels.	31

Table 9.13.  Transmit Power Limits	31

9.5.8.2  Transmit Power Level Control.	31

9.5.8.3  Transmit Spectrum Shape.	31

Figure 9-14.  Transmit Spectrum Mask	32

9.5.8.4  Transmit Center Frequency Tolerance.	32

Table 9.14.  Transmit Center Frequency Tolerance	32

9.5.9  PMD Receiver Specifications.	33

9.5.9.1  Spurious Free Dynamic Range.	33

9.5.9.2  Selectivity.	33

9.5.9.3  Channel BER.	33

9.5.9.4  Receive Center Frequency Tolerance.	33

Table 9.15.  Operating temperature Range	33

9.5.9.5  Carrier Detect Response Time.	33

9.5.9.6  Clock Recovery Time	34

9.5.9.7  Receiver Data Jitter Tolerance(OPEN)	34

Figure 9-15  Data Jitter Tolerance	34

9.5.9.8  Preamble Definition	35

9.5.9.8.1 Ramp Up Period (Closed per A19)	35

Figure 9-16  Ramp Up	36

9.5.9.8.2 Ramp Down Period (Closed per A19)	36

Figure 9-17  Ramp Down	37

9.5.9.9 Recieve Data DC Offset (Open)	38

9.5.9.10 Received Data Maximum Run Length (DC Balance)  (Open)	38

9.5.9.11 Receiver Sensitivity   (CLOSED per July 1994)	38

9.5.9.12 Intermodulation (CLOSED per July 1994)	38

9.5.9.13  Desensitization (CLOSED per July 1994)	38

9.6 FHSS Physical Medium Dependent Sublayer 2.0M Bit	39

9.6.1  Introduction.	39

9.6.2  Modulation.	39

Figure 9-12.  Transmit Modulation Mask	39

9.6.2  Channel Data Rate.	39

9.6.3  Channel BER.	39

Table A  Hopping Sequence Set 1	40

Table A  Hopping Sequence Set 1	41

Table A  Hopping Sequence Set 1 continued	42

Table A  Hopping Sequence Set 1 continued	43

Table B  Hopping Sequence Set 2	44

Table B  Hopping Sequence Set 2	45

Table B  Hopping Sequence Set 2 continued	46

Table B  Hopping Sequence Set 2 continued	47

Table C  Hopping Sequence Set 3	48

Table C  Hopping Sequence Set 3	49

Table C  Hopping Sequence Set 3 continued	50

Table C  Hopping Sequence Set 3 continued	51

Appendix A	52

PMD Approved Motions	52

9.0 Physical Layer Specifications for

2.4 GHz Frequency Hopping Spread Spectrum

Wireless LAN

tc "9.1 Introduction"§9.1 Introduction

Editor's Note:  Sections 1.1 to 2.5.8.4 covering the PHY Layer Service Primitives will no longer be included in the FHSS PHY Specification.  The primitives will specified by a common set of primitives described in section 8.0 (formally section7) of the 802.11 Draft Standard.  I have shown the text to be removed using strike outs.  I will make a motion at the meeting to remove this text from our working document.

Editor's Note:  This will be the begin of Section 9 of the draft standard.  There is some text missing and I have inserted placeholders for that text.  Per the minutes of last meeting, I have re-number the following sections to reflect the 802.11 Draft Standard section numbering.

tc "9.1.1  Introduction to FHSS Phyiscal Layer."§9.1.1  Introduction to FHSS Phyiscal Layer. (new text)

(Editors to add text)

c1.9.2  FHSS PHY Specific Service Parameter Lists.;  (new text)

tc "9.2.1  Introduction."§9.2.1  Introduction. (new Text)

(Editors to add text):  Below is current text on subject.  We may need to do some more work in this section concerning data rates if data rates are determined on a per frame basis.

tc "9.2.2  PHY_DATA.request Parameter List."§9.2.2  PHY_DATA.request Parameter List.  (new text)

ANTSEL (optional)  The ANTSEL parameter is an optional parameter and is defined by the PMD_ANTSEL primitive in section 4.5.5 of the PMD sublayer specification.  This parameter allows the MAC entity to specify the antenna used by the PLCP to transmit on a per MPDU basis.

TXPWRLVL (optional)  The TXPWRLVL parameter is an optional parameter and is defined by the PMD_TXPWRLVL primitive in section 4.5.6 of the PMD sublayer specification.  This parameter allows the MAC entity to specify the transmit power level on a per MPDU basis.

tc "9.2.3  PHY_DATA.indicate Parameter List"§9.2.3  PHY_DATA.indicate Parameter List

ANTSEL (optional).  The ANTSEL parameter is an optional parameter and can be a value of 1 or 2.  This parameter is an indication by the PHY entity as to the antenna it used to receive the current MPDU it is transferring to the MAC sublayer.

RSSI  (optional).  The RSSI parameter is an optional parameter and can be a value of TBD.  This parameter is an indication by the PHY sublayer the value of the energy observed on the antenna used to receive the  as to the antenna it used to receive the current MPDU it is transferring to the MAC sublayer.

tc "9.3. FHSS Physical Layer Convergence Procedure"§9.3. FHSS Physical Layer Convergence Procedure

Sublayer;

tc "9.3.1  Introduction."§9.3.1  Introduction.  This section provides a convergence procedure to map MAC PDUs into a frame format designed for FHSS radio transceivers.  The procedures for transmission, carrier sense, and reception are defined for single and multiple antenna diversity radios.

tc "9.3.1.1  State Diagram Notation."§9.3.1.1  State Diagram Notation.  The operation of the procedures can be described by state diagrams.  Each diagram represents the domain of a function and consists of a group of connected, mutually exclusive states.  Only one state of a function is active at any given time.  Each state that the function can assume is represented by a rectangle as shown in Figure 9-1.  These are divided into two parts by a horizontal line.  In the upper part the state is identified by a name in capital letters.  The lower part contains the name of any ON signal that is generated by the function.  Actions are described by short phrases and enclosed in brackets.

tc "Figure 9-1.  State Diagram Notation Example"§Figure 9-1.  State Diagram Notation Example

All permissible transitions between the states of a function are represented graphically by arrows between them.  A transition that is global in nature (for example, an exit condition from all states to the IDLE or RESET state) is indicated by an open arrow.  Labels on transitions are qualifiers that must be fulfilled before the transition will be taken.  The label UCT designates an unconditional transition.  Qualifiers described by short phrases are enclosed in parentheses.

State transitions and sending and receiving of messages occur instantaneously.  When a state is entered and the condition to leave that state is not immediately fulfilled, the state executes continuously, sending the messages and executing the actions contained in the state in a continuous manner.

Some devices described in this standard (e.g., repeaters) are allowed to have two or more ports.  State diagrams capable of describing the operation of devices with an unspecified number of ports, require qualifier notation that allows testing for conditions at multiple ports.  The notation used is a term that includes a description in parentheses of which ports must meet the term for the qualifier to be satisfied (e.g., ANY and ALL).  It is also necessary to provide for term-assignment statements that assign a name to a port that satisfies a qualifier.  The following convention is used to describe a term-assignment statement that is associated with a transition:

(1)	The character ":" (colon) is a delimiter used to denote that a term assignment statement follows.

(2)	The character "_" (left arrow) denotes assignment of the value following the arrow to the term preceding the arrow.

The state diagrams contain the authoritative statement of the functions they depict; when apparent conflicts between descriptive text and state diagrams arise, the state diagrams are to take precedence.  This does not override, however, any explicit description in the text that has no parallel in the state diagrams.

The models presented by state diagrams are intended as the primary specifications of the functions to be provided.  It is important to distinguish, however, between a model and a real implementation.  The models are optimized for simplicity and clarity of presentation, while any realistic implementation may place heavier emphasis on efficiency and suitability to a particular implementation technology.  It is the functional behavior of any unit that must match the standard, not its internal structure.  The internal details of the model are useful only to the extent that they specify the external behavior clearly and precisely.

tc "9.3.2  Physical Layer Convergence Procedure Frame Format."§9.3.2  Physical Layer Convergence Procedure Frame Format.   The PLCP Frame Format provides for the asynchronous transfer of MAC Layer MPDU from any transmitting node to all receiving nodes within the wireless LAN.  The PLCP frame format illustrated in Figure 9-2 consists of three parts: a PLCP Preamble, a PLCP Header, and a PLCP_PDU.  The PLCP Preamble provides a period of time for several receiver functions.  These functions include antenna diversity, clock and data recovery and data delineation of the PLCP Header and the PLCP_PDU.  The PLCP Header is used to specify the length of the MPDU field and support any PLCP management information.  The PLCP_PDU contains the MPDU data modified by the MPDU data whitener.

tc "Figure 9-2.  PLCP Frame Format"§Figure 9-2.  PLCP Frame Format

tc "9.3.2.1  PLCP Preamble."§9.3.2.1  PLCP Preamble.  The PLCP preamble contains two separate sub-fields; the sync field and the start frame delimiter (SFD), to allow the PHY circuitry to reach steady state demodulation and synchronization of bit clock and frame start.

tc "9.3.2.1.1  Sync."§9.3.2.1.1  Sync.  The preamble sync field is a 10-octet field containing an alternating zero-one pattern, starting with zero and ending with one, to be used by the PHY sub-layer to detect a signal to receive, select an antenna if diversity is utilized, and to reach steady-state frequency offset correction and synchronization with the received packet timing.

tc "9.3.2.1.2  Start Frame Delimiter."§9.3.2.1.2  Start Frame Delimiter.  The start frame delimiter (SFD) consists of the 16-bit binary pattern 0000 1100 1011 1101 (left-most bit first).  The first bit of the start frame delimiter follows the last bit of the sync pattern.  The start frame delimiter defines the frame timing.

tc "9.3.2.2  PLCP Header."§9.3.2.2  PLCP Header.  The PLCP Header field contains three separate sub-fields: a 10-bit PLCP_PDU Length Word (PLW), a 6-bit PLCP Signaling Field (PSF), and a 16-bit PLCP Header Error Check (HEC) field.

tc "9.3.2.2.1  PLCP_PDU Length Word."§9.3.2.2.1  PLCP_PDU Length Word.   The PLCP_PDU Length Word (PLW) is passed down from the MAC as a parameter within the PHY_DATA.request primitive in the transmitting station.  The PLW represents the number of octets contained in the MPDU packet.  Its valid states are 000h - 3FFh, representing counts of zero to 1023 bytes.  The PLW is transmitted LSB first and MSB last.  The PLW is used by the receiving station to determine the last bit in the packet.

tc "9.3.2.2.2  PLCP Signaling Field."§9.3.2.2.2  PLCP Signaling Field.  The 6-bit PLCP Signaling Field (PSF) is defined in Table 9.1.  The PSF is transmitted LSB (bit 0) first and MSB (bit 5) last.

Bit

Parameter Name

Parameter Values

Description

0

Reserved

Reserved

Reserved

1

Reserved

Reserved

Reserved

2

Reserved

Reserved

Reserved

3

Reserved

Reserved

Reserved

4

Reserved

Reserved

Reserved

5

Reserved

Reserved

Reserved

tc "Table 9.1.  PLCP Signaling Field Bit Descriptions"§Table 9.1.  PLCP Signaling Field Bit Descriptions

tc "9.3.2.2.3  Header Error Check Field."§9.3.2.2.3  Header Error Check Field.  The Header Error Check (HEC) field is a 16-bit BCH type error detection and correction field.  The HEC uses the CCITT 16-bit CRC generator polynomial G(x) as follows:

G(x) = x16+x12+x5+1

The HEC shall be the ones complement of the sum (modulo 2) of the following:

1) The remainder of xk • (x16 + x15 + x14 + ... + x2 + x1 +1) divided (modulo 2) by G(x), where k is the number of bits in the PSF and PLW fields of the PLCP Header;

2) The remainder after multiplication by x16 and then division (modulo 2) by G(x) of the content (treated as a polynomial) of the PSF and PLW fields.

The HEC shall be transmitted with the coefficient of the highest term first.

As a typical implementation, at the transmitter, the initial remainder of the division is preset to all ones and is then modified by division of the PSF and PLW fields by the generator polynomial, G(x).  The ones complement of this remainder is inserted in the HEC field with the most significant bit inserted first.

At the receiver, the initial remainder of the division is again preset to all ones.  The division of the received PSF, PLW, and HEC fields by the generator polynomial, G(x), results, in the absence of transmission errors, in a unique nonzero value, which is the following polynomial R(x):

R(x) = x12 + x11 + x10 + x8 + x3 + x2 + x1 + 1

tc "9.3.2.3  PLCP_PDU Data Whitener."§9.3.2.3  PLCP_PDU Data Whitener.  The PLCP_PDU data whitener uses a length-127 frame synchronous scrambling followed by a 32/33 Bias Suppression Encoding to randomize the data from highly redundant patterns and to minimize the data DC bias and maximum run lengths.  Data bytes are placed in the transmit serial bit stream LSB first and MSB last.  The frame synchronous scrambler uses the generator polynomial S(x) as follows:

S(x) = x7+x4+1

and is illustrated in Figure 9-3.  The same scrambler is used to scramble transmit data and descramble receive data.  The data whitening starts with the first bit of the PLCP_PDU which follows the last bit of the PLCP Header.  The specific bias suppression encoding method used is defined in Figure 9-6a.  The format of the packet after data whitening is as shown in Figure 9-4.

tc "Figure 9-3  Frame Synchronous Scrambler/Descrambler."§Figure 9-3  Frame Synchronous Scrambler/Descrambler.

tc "Figure 9-4  PLCP_PDU Data Whitener Format."§Figure 9-4  PLCP_PDU Data Whitener Format.

tc "9.3.3  PLCP State Machines."§9.3.3  PLCP State Machines.  The PLCP consists of three state machines as illustrated in Figure 9-5: the transmit (TX), carrier sense/clear channel assessment (CS/CCA), and receive (RX) state machines.  Execution of the PLCP state machines normally transfers from the FH PLME state machine and begins at the CS/CCA state machine.  The PLCP exits back to the FH PLME state machine upon interrupt to service a PLME service request, e.g., PLME_SETFREQ, PLME_PHY_RESET, etc.

tc "Figure 9-5  PLCP Top Level State Diagram."§Figure 9-5  PLCP Top Level State Diagram.

tc "9.3.3.1  PLCP Transmit Procedure."§9.3.3.1  PLCP Transmit Procedure.  The PLCP transmit procedure is invoked by the CS/CCA procedure immediately upon receiving a PHY_DATA.request(Start_of_Activity, TXVECTOR) from the MAC layer.  The CSMA/CA protocol is performed by the MAC with the PHY PLCP in the CS/CCA procedure prior to executing the transmit procedure.

tc "9.3.3.1.1  Transmit State Machine."§9.3.3.1.1  Transmit State Machine.  The PLCP transmit state machine illustrated in Figure 9-6 includes functions that must be performed prior to, during, and after MPDU data transmission.  Upon entering the transmit procedure in response to a PHY_DATA.request (Start_of_Activity, TXVECTOR) from the MAC, the PLCP shall switch the PHY PMD circuitry from receive to transmit state; ramp on the transmit power amplifier in the manner prescribed in Section 9.5 (PMD specification); and transmit the preamble sync pattern and start frame delimiter.  The PLCP shall generate the PLCP header as defined in Section 9.3.2.2 (PLCP Header) in sufficient time to send the bits at their designated bit slot time.  The PLCP shall add the PLCP header to the start of the PLCP_PDU data.

Prior to transmitting the first MPDU data bit, the PLCP shall send a PHY_DATA.confirm message to the MAC indicating that the PLCP is ready to receive an MPDU data octet.  The MAC will respond by sending a PHY_DATA.request(Data, DATA).  This sequence of PHY_DATA. confirm and PHY_DATA.request(Data, DATA)shall be executed whenever the PLCP is ready for more data and until the last data octet is passed to the PLCP.  During transmission of the PLCP_PDU data, each bit of the MPDU passed down from the MAC shall be processed by the data whitener algorithm defined in Figure 9-6a and described in section 9.3.2.3 (PLCP_PDU Data Whitener).  The MPDU data bytes are processed and transmitted LSB first and MSB last.

After the last MPDU octet is passed to the PLCP, the MAC will respond to the following PHY_DATA. confirm with a PHY_DATA.request(End_of_Data_and_Activity, NULL).  After the last bit of the PLCP_PDU data has completed propagation through the radio and been transmitted onto the air, the PLCP shall ramp off the power amplifier in the manner prescribed in the section 9.5 (PMD), switch the PHY PMD circuitry from transmit to receive state, and complete the transmit procedure by sending a PHY_DATA.confirm to the MAC layer.  The execution will then return to the CS/CCA procedure.

tc "Figure 9-6  Transmit State Machine"§Figure 9-6  Transmit State Machine

Data Whitener Encoding Algorithm:

/*	If stuff bit = 1 = next block is inverted; 0 = not inverted */

/*	Accumulate PLCP Header; begin stuffing on first bit of the PLCP_PDU */

/********* Calculate number of 32-bit BSE blocks required to send MPDU;

no padding is necessary for number of bytes not multiple of 4 *********/

Input parameter: number_of_MPDU_bytes;

number_of_blocks_in_packet = truncate{(number_of_MPDU_bytes + 3) / 4 };

/********* Accumulate the bias in the header to use in calculating the inversion state of the 		first block of PLCP_PDU data *********/

Read in header {b(1),...,b(32)};				/* b(1) is first bit in */

header_bias = 2 [Sum{b(1),...,b(32)}] - (32);		/* calculate bias in header */

Transmit {b(1),...,b(32)};					/* no stuffing on header */

accum=header_bias;					/* initialize accum */

Initialize scrambler to all ones;

/********* Whiten the  PLCP_PDU data with scrambler and BSE encoder *********/

For n = 1 to number_of_blocks_in_packet

{

b(0) = 0;						/* b(0) is the stuff bit */

N = min(4, # of bytes remaining) * 8;		/* N= block size in bits */

Read in next block {b(1),...,b(N)};			/* b(n) = 0, 1 */

Scramble{b(1),...,b(N)};				/* see section 9.3.2.3 */

bias_next_block = 2 [Sum{b(0),...,b(N)}] - (N+1);	/* calculate bias with b(0)=0 */

/***** if accum and bias of next block has the same sign, then invert block; 			if accum=0, don't invert *****/

If {[accum * bias_next_block > 0] then

{

Invert {b(0),...,b(N)};

bias_next_block = - bias_next_block;

}

accum = accum + bias_next_block;

transmit {b(0),...,b(N)};				/* b(0) is first bit out */

}

tc "Figure 9-6a  Data Whitener Encoding Procedure"§Figure 9-6a  Data Whitener Encoding Procedure

tc "9.3.3.1.2  Transmit State Timing (OPEN)."§9.3.3.1.2  Transmit State Timing (OPEN).  The transmit timing illustrated in Figure 9-7 is defined from the instant that the PHY_DATA.request(Start_of_Activity, TXVECTOR) is received from the MAC layer.  The PMD circuitry shall be switched from receive to transmit and the power amplifier shall be turned on and settled within the limits about the final transmit power level as specified in section 9.5 (PMD) within 20 µs of receipt of the PHY_DATA.request.  The PLCP preamble shall be transmitted at 1 Mbps and be completed within 96 µs.  The PLCP header shall be transmitted at 1 Mbps and be completed within 32 µs.  The variable length PLCP_PDU shall be transmitted at the selected data rate.  After the last bit of the PLCP_PDU data has completed propagation through the radio and been transmitted onto the air, the PLCP shall turn off the power amplifier and be less than the specified off-mode transmit power in 8 µs.  At the end of the power amplifier ramp down period, the PLCP shall send the last PHY_DATA.confirm to the MAC layer and switch the PMD circuitry from transmit to receive.  The PHY_DATA.confirm and PHY_DATA.request(Data, DATA) exchanges shall occur prior to the points at which data octets are needed but do not have any additional time significance.

tc "Figure 9-7  Transmit State Timing"§Figure 9-7  Transmit State Timing

tc "9.3.3.2  Carrier Sense/Clear Channel Assessment Procedure (OPEN)."§9.3.3.2  Carrier Sense/Clear Channel Assessment Procedure (OPEN).  The PLCP carrier sense/clear channel assesment (CS/CCA) procedure is executed while the receiver is turned on and the station is not currently receiving or transmitting a packet.  The CS/CCA procedure is used for two purposes:  to detect the start of a network signal that can be received and to determine whether the channel is clear prior to transmitting a packet.

tc "9.3.3.2.1  Carrier Sense/Clear Channel Assessment State Machine (OPEN)."§9.3.3.2.1  Carrier Sense/Clear Channel Assessment State Machine (OPEN).  The carrier sense/clear channel assessment (CS/CCA) state machine is shown in Figure 9-8.  The PLCP shall perform a CS/CCA assessment at a minimum of one antenna within a slot time of 50 µs.  The PLCP shall provide a CS/CCA indication to the MAC at each slot time boundary which included in the assessment the data up to the end of the slot time and a minimum of 16 µs prior to the end of the slot time.  The CCA indication immediately prior to transmission must be performed on an antenna with essentially the same free space gain and gain pattern as the antenna to be used for transmission.  The method of determining CS/CCA is undefined except for the performance of a conformant method as specified in section 9.5 (PMD).

If a CS/CCA assessment returns a channel idle result, the PHY shall send a PHY_DATA.indicate (End_of_Activity, NULL) to the MAC.  If a PHY_DATA.request is received after a channel idle assessment, the CS/CCA procedure shall exit to the transmit procedure.

If a CS/CCA assessment returns a channel busy result, the PHY shall send a PHY_DATA.indicate(Start_of_Activity, NULL) to the MAC.  Upon a channel busy assessment, the PLCP shall stop any antenna switching prior to the earliest possible arrival time of the start frame delimiter (SFD) and detect a valid SFD.  If a valid SFD is detected, the CS/CCA procedure shall exit to the receive procedure.  The PLCP shall dwell and search for the SFD for a minimum period longer than the latest arrival time of the SFD.  Indication of a busy channel does not necessarily lead to the reception of a packet.

Upon exiting the CS/CCA procedure to receive a packet, the last indication of CS/CCA was BUSY.  The indication remains BUSY when returning from the receive procedure until the first CS/CCA assessment is performed and determines that the channel is IDLE.  If the MAC is sending an ACK following a valid received packet, the MAC should ignore the CS/CCA busy indication immediately following the end of the received packet.

The countdown timer may be a non-zero value when returning from the receive procedure if a signal in the process of being received was lost prior to the end as positively indicated in the length field of a valid PLCP header.  The countdown timer shall be used to force the CS/CCA indication to remain in the BUSY state until the predicted end of the packet regardless of actual CS/CCA indications.  However, if the CS/CCA procedure indicates the start of a new packet within the countdown timer period, it is possible to transition to the receive procedure prior to the end of the countdown timer period.  The initial value of the countdown timer upon entry of the CS/CCA procedure from power up or PHY reset is unspecified.

tc "Figure 9-8  CS/CCA State Machine."§Figure 9-8  CS/CCA State Machine.

tc "9.3.3.2.2  Carrier Sense/Clear Channel Assessment State Timing (OPEN)."§9.3.3.2.2  Carrier Sense/Clear Channel Assessment State Timing (OPEN).  Timing for priority (PIFS, DIFS), contention backoff (slot times), and CS/CCA assessment windows are defined relative to the end of the last packet on the air.  The PLCP shall perform a CS/CCA assessment at the end of each slot time including the PIFS and DIFS windows.  The indication shall be generated within 1 µs of the end of each 50 µs slot time.

If a station has not successfully received the previous packet, the perceived packet end time and slot boundary times will have a higher uncertainty for that station.

tc "Figure 9-9  CS/CCA State Timing"§Figure 9-9  CS/CCA State Timing

tc "9.3.3.3  PLCP Receive Procedure."§9.3.3.3  PLCP Receive Procedure.  The PLCP receive procedure is invoked by the PLCP carrier sense/clear channel assessment (CS/CCA) procedure upon detecting a portion of the preamble sync pattern followed by a complete start frame delimiter.

tc "9.3.3.3.1  Receive State Machine (OPEN)."§9.3.3.3.1  Receive State Machine (OPEN).  The PLCP receive procedure shown in Figure 9-10 includes functions that must be performed while receiving the PLCP header and the PLCP_PDU data.  The PLCP receive procedure begins upon detection of a complete start frame delimiter in the CS/CCA procedure.  The PLCP shall read in the 32-bit PLCP header and perform an error check using the header error check field.  Immediately upon detection of a valid PLCP header, the PLCP shall send a PHY_DATA.indicate (Start_of_Data, RXVECTOR) message to the MAC layer.  The PLCP shall set a PLCP_PDU byte/bit counter to indicate the last bit of the packet, receive the PLCP_PDU data bits and perform the data whitening decoding procedure shown in Figure 9-10a on each PLCP_PDU bit.  The PLCP shall pass correctly received data octets to the MAC with a PHY_DATA.indicate(Data, DATA).  After the last PLCP_PDU bit is received and the last octet is passed up to the MAC, the PLCP shall send a PHY_DATA.indicate(End_of_Data, RXERROR=no error) to the MAC layer.  Upon error-free completion of a packet reception, the PLCP shall exit the receive procedure and return to the PLCP CS/CCA procedure with TIME_REMAINING=0.

If the PLCP header contains uncorrectable errors, the PLCP shall immediately set the byte/bit counter to zero, complete the receive procedure with a PHY_DATA.indicate(End_of_Data, RXERROR=header error violation) to the MAC, and return to the CS/CCA procedure with TIME_REMAINING=0.  If after receiving a valid PLCP header, the signal is interrupted for any reason, the PLCP shall immediately complete the receive procedure with a PHY_DATA.indicate (End_of_Data, RXERROR=carrier lost) to the MAC, and return to the CS/CCA procedure with TIME_REMAINING= byte/bit count remaining.  If any format error was detected during the reception of the packet, the PLCP shall immediately complete the receive procedure with a PHY_DATA.indicate(End_of_DATA, RXERROR=format violation) to the MAC, and return to the CS/CCA procedure with TIME_REMAINING=byte/bit count remaining.

tc "Figure 9-10  Receive State Machine"§Figure 9-10  Receive State Machine

Data Whitener Decoding Algorithm:

/*	If stuff bit = 1 = next block is inverted; 0 = not inverted */

/*	Algorithm begins on first bit of PLCP Header following the start frame delimiter; */

/*	accumulate PLCP Header; begin stuffing on first bit of the PLCP_PDU */

/********* Calculate bias in header for format error checking *********/

Read in header {b(1),...,b(32)};				/* b(1) is first bit in */

accum = 2 [Sum{b(1),...,b(32)}] - (32);			/* calculate bias in header */

Verify header error check (HEC);				/*not part of BSE algorithm*/

Get number_of_MPDU_bytes from header;

number_of_blocks_in_packet = truncate{(number_of_MPDU_bytes + 3) / 4 };

Initialize scrambler to all ones;

/********* De-whiten the PLCP_PDU data with BSE decoder and de-scrambler *********/

For n = 1 to number_of_blocks_in_packet

{

N = min(4, # of bytes remaining) * 8;		/* N= block size in bits */

Read in next block {b(0),...,b(N)};			/* b(n) = 0, 1 */

bias_next_block = 2 [Sum{b(0),...,b(N)}] - (N+1);	/* calculate bias with b(0) */

accum = accum + bias_next_block;

If (accum < -32 or accum > +32) send bias error indication to MAC;

If {[b(0)=1] then Invert {b(1),...,b(N)};		/* if invert bit=true */

Descramble {b(1),...,b(N)};				/* see section 9.3.2.3 */

Send {b(1),...,b(N)} to MAC

}

tc "Figure 9-10a  Data Whitener Decoding Procedure"§Figure 9-10a  Data Whitener Decoding Procedure

tc "9.3.3.3.2  Receive State Timing (OPEN)."§9.3.3.3.2  Receive State Timing (OPEN).  The receive state timing shown in Figure 9-11 is defined to begin upon detection of a complete start frame delimiter in the carrier sense procedure.  The PLCP shall send a PHY_DATA.indicate (Start_of_Data, RXVECTOR) to the MAC layer within 1 µs of receiving the last bit of a valid PLCP header.  The PLCP shall begin receiving the variable length PLCP_PDU immediately after the end of the last bit of the PLCP header.  The PLCP shall send a PHY_DATA.indicate(End_of_Data, RXERROR) within 1 µs of receiving the last PLCP_PDU data bit.  There is no timing significance of the PHY_DATA.indicate (Data, DATA) indications other than the requirement that they occur after the time-significant PHY_DATA.indicate(Start_of_Data, RXVECTOR) and before the PHY_DATA.indicate (End_of_Data, RXERROR).

If there was an uncorrectable error in the PLCP header, the PLCP shall terminate the receive procedure within 8 µs of the end of the PLCP header.  If any other error was detected during the reception of the packet, the PLCP shall terminate the receive procedure within 8 µs of detecting the error.

tc "Figure 9-11  Receive Timing"§Figure 9-11  Receive Timing

tc "9.4 PLME_SAP Layer Management."§9.4 PLME_SAP Layer Management.

tc "9.4.1  Introduction"§9.4.1  Introduction

add Text

tc "9.4.2  PLME_SAP Sublayer Management Primitives"§9.4.2  PLME_SAP Sublayer Management Primitives  The following messages may be sent between the PHY sublayer entities and intralayer or higher Layer Management Entities (LME).

Primitive

Request

Indicate

Confirm

Response

PLME_RXRESET

X

X

PLME_TXRESET

X

X

PLME_SETCHNL

X

X

tc "Table 9.2. MPHY_SAP Sublayer Management Primitives"§Table 9.2. MPHY_SAP Sublayer Management Primitives

tc "9.4.2.1 PLME_SAP Management Service Primitive Parameters."§9.4.2.1 PLME_SAP Management Service Primitive Parameters.  The following table shows the parameters used by one or more of the PLME_SAP Sublayer Management Primitives.

Parameter

Associate Primitive

Value

SET

PLME_SETCHNL

0,1,2,3

PATTERN

PLME_SETCHNL

0, 2-23, 24-45,47-68

INDEX

PLME_SETCHNL

1-80

tc "Table 9.3.  PHY_SAP Service Primitive Parameters"§Table 9.3.  PHY_SAP Service Primitive Parameters

tc "9.4.2.2  PLME_SAP Detailed Service Specifications."§9.4.2.2  PLME_SAP Detailed Service Specifications.  The following section describes the services provided by each PLME_SAP Service Primitive.

tc "9.4.2.3.1  PLME_RXRESET.request"§9.4.2.3.1  PLME_RXRESET.request

9.4.2.3.1.1  Function.  This primitive is a request by the LME to reset the PHY sublayer receive state machine.

9.4.2.3.1.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_RXRESET.request

There are no parameters associated with this primitive.

9.4.2.3.1.3  When Generated.  This primitive can be generated at anytime to reset the receive state machine in the PHY sublayer.

9.4.2.3.1.4  Effect of Receipt.   Receipt of this primitive by the PHY sublayer will cause the PHY entity to reset the receive state machine to its idle state.

tc "9.4.2.3.2  PLME_RXRESET.confirm"§9.4.2.3.2  PLME_RXRESET.confirm

9.4.2.3.2.1  Function.  This primitive is a confirmation by the PHY sublayer to the local LME that the PLCP receive state machine was successfully reset.

9.4.2.3.2.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_RXRESET.confirm

There are no parameters associated with this primitive.

9.4.2.3.2.3  When Generated.  This primitive will be generated as a response to a PLME_RXRESET.request primitive once the PLCP has successfully completed the receive state machine reset.

9.4.2.3.2.4  Effect of Receipt.  The effect of receipt of this primitive by the LME is unspecified.

tc "9.4.2.3.3  PLME_TXRESET.request"§9.4.2.3.3  PLME_TXRESET.request

9.4.2.3.3.1  Function.  This primitive is a request by the LME to reset the PHY sublayer transmit state machine.

9.4.2.3.3.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_TXRESET.request

There are no parameters associated with this primitive.

9.4.2.3.3.3  When Generated.  This primitive can be generated at anytime to reset the transmit state machine in the PHY sublayer.

9.4.2.3.3.4  Effect of Receipt.   Receipt of this primitive by the PHY sublayer will cause the PHY entity to reset the transmit state machine to its idle state.

tc "9.4.2.3.4  PLME_TXRESET.confirm"§9.4.2.3.4  PLME_TXRESET.confirm

9.4.2.3.4 .1  Function.  This primitive is a confirmation by the PHY sublayer to the local LME that the PLCP transmit state machine was successfully reset.

9.4.2.3.4 .2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_TXRESET.confirm

There are no parameters associated with this primitive.

9.4.2.3.4 .3  When Generated.  This primitive will be generated as a response to a PLME_TXRESET.request primitive once the PLCP has successfully completed the transmit state machine reset.

9.4.2.3.4 .4  Effect of Receipt.  The effect of receipt of this primitive by the LME is unspecified.

tc "9.4.2.3.5  PLME_SETCHNL.request"§9.4.2.3.5  PLME_SETCHNL.request

9.4.2.3.5.1  Function.  This primitive is a request by the LME to the PHY sublayer to change frequency.

9.4.2.3.5.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_SETCHNL.request (SET,PATTERN,INDEX)

The SET parameter can be one of four values: 0, 1, 2, and 3.  This parameter selects the sets of hopping patterns as shown in the annex.  SET equals 0 is a special set used by the MAC to discover what LANs currently exist prior to associating with a given LAN.  The PATTERN parameter specifies one of 23 hop patterns within a given hop SET.  The hop patterns are listed in the annex.  PATTERN equal 0 is a special pattern used by the MAC to discover what LANs currently exist prior to associating with a given LAN.  The INDEX parameter can be one of 80 values ranging from 1 to 80.  The INDEX value is used to select a frequency within a given SET and PATTERN as shown in the annex.  If the SET and PATTERN parameters are both zero, then the INDEX value is equal to the frequency or channel desired.

9.4.2.3.5.3  When Generated.  This primitive can be generated at anytime to hop to a new channel or frequency.

9.4.2.3.5.4  Effect of Receipt.  Receipt of this primitive by the PHY sublayer will cause the PHY entity to start the channel change or hop state machine.

tc "9.4.2.3.4  PLME_SETCHNL.confirm"§9.4.2.3.4  PLME_SETCHNL.confirm

9.4.2.3.4 .1  Function.  This primitive is a confirmation by the PHY sublayer to the local LME that the frequency change or hop was successfully completed.

9.4.2.3.4 .2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_SETCHNL.confirm

There are no parameters associated with this primitive.

9.4.2.3.4 .3  When Generated.  This primitive will be generated as a response to a PLME_SETCHNL.request primitive once the PLME has successfully completed the frequency change.

9.4.2.3.4 .4  Effect of Receipt.  The effect of receipt of this primitive by the LME is unspecified.

tc "9.4.3  FH PHY Management Information Base"§9.4.3  FH PHY Management Information Base

tc "9.4.3.1  Introduction"§9.4.3.1  Introduction

tc "9.4.3.2  FH PHY Specific Paramters"§9.4.3.2  FH PHY Specific Paramters

tc "9.4.3.2.1   Introduction"§9.4.3.2.1   Introduction

tc "9.4.3.3  FH PHY Specific MAC Management State Machines"§9.4.3.3  FH PHY Specific MAC Management State Machines

tc "9.4.3.3.1   Introduction"§9.4.3.3.1   Introduction

tc "9.4.3.3  FH PHY Specific MAC Management State Machines"§9.4.3.3  FH PHY Specific MAC Management State Machines

tc "9.4.4  FH PHY Specific MAC Layer Management State Machines"§9.4.4  FH PHY Specific MAC Layer Management State Machines

tc "9.4.4.1   Introduction"§9.4.4.1   Introduction    This section describes the Mac Management state machines required for such things as frequency hopping.  this probably belongs in the Mac.

tc "9.4.4  FH PHY Layer Management State Machines"§9.4.4  FH PHY Layer Management State Machines

tc "9.4.4.1   Introduction"§9.4.4.1   Introduction   This section describes the Phy Layer Management state machines required for such things as changing to a new frequency, reseting the transmitter, reseting the receiver.

tc "9.5 FHSS Physical Medium Dependent Sublayer 1.0M Bit"§9.5 FHSS Physical Medium Dependent Sublayer 1.0M Bit

tc "9.5.1 Scope and Field of Application."§9.5.1 Scope and Field of Application.  This section describes the PMD services provided to the PLCP for the FHSS Physical Layer.  Also defined in this section are the functional, electrical and RF characteristics required for interoperability of implementations conforming to this specification.  The relationship of this specification to the entire FHSS Physical Layer is shown in figure 9-10.

tc "Figure 9-10  PMD layer Reference Model"§Figure 9-10  PMD layer Reference Model

tc "9.5.2  Overview of Services."§9.5.2  Overview of Services.  In general, The FHSS Physical Medium Dependent Sublayer accepts Physical Layer Convergence Procedure sublayer-service primitives and provides the actual means by which the signals required by these primitives are imposed onto the medium.   In the FHSS Physical Medium Dependent Sublayer at the receiver the process is reversed.  The combined function of the transmitting and receiving FHSS PMD Sublayers results in a data stream, timing information, and receive parameter information being delivered to the receiving Physical Layer Convergence Procedure Sublayer.

tc "9.5.3  Overview of Interactions."§9.5.3  Overview of Interactions. The primitives associated with the 802.11 PLCP sublayer to the FHSS PMD sublayer falls into two basic categories:

(1)  Service primitives that support PLCP peer-to-peer interactions

(2)  Service primitives that have local significance and support sublayer-to-sublayer interactions.

tc "9.5.4  Basic Service and Options."§9.5.4  Basic Service and Options.  All of the service primitives described in this section are considered mandatory unless otherwise specified.

tc "9.5.4.1  PMD_SAP Peer-to-Peer Service Primitives."§9.5.4.1  PMD_SAP Peer-to-Peer Service Primitives.  The following table indicates the primitives for peer-to-peer interactions.

Primitive

Request

Indicate

Confirm

Response

PMD_DATA

X

X

tc "Table 9.4. PMD_SAP Peer-to-Peer Service Primitives"§Table 9.4. PMD_SAP Peer-to-Peer Service Primitives

tc "9.5.4.2  PMD_SAP Sublayer-to-Sublayer Service Primitives."§9.5.4.2  PMD_SAP Sublayer-to-Sublayer Service Primitives.  The following table indicates the primitives for sublayer-to-sublayer interactions.

Primitive

Request

Indicate

Confirm

Response

PMD_TXRX

X

PMD_PARAMP

X

PMD_ANTSEL

X

PMD_TXPWRLVL

X

PMD_FREQ

X

PMD_RSSI

X

tc "Table 9.4. PMD_SAP Sublayer-to-Sublayer Service Primitives"§Table 9.4. PMD_SAP Sublayer-to-Sublayer Service Primitives

tc "9.5.4.3  PMD_SAP Service Primitives Parameters."§9.5.4.3  PMD_SAP Service Primitives Parameters.  The following table shows the parameters used by one or more of the PMD_SAP Service Primitives.

Parameter

Associate Primitive

Value

TXD_UNIT

PMD_DATA.request

ONE, ZERO,

TRISTATE

RXD_UNIT

PMD_DATA.indicate

ONE,ZERO

RF_STATE

PMD_TXRX.request

TRANSMIT,

RECEIVE

RAMP_STATE

PMD_PARAMP.request

ON, OFF

ANTENNA_

STATE

PMD_ANTSEL.request

1,2

TXPWR_LEVEL

PMD_TXPWRLVL.request

LEVEL1, LEVEL2

LEVEL3

CHNL_ID

PMD_FREQ.request

1 through 80 inclusive

STRENGTH

PMD_RSSI.indicate

TBD

tc "Table 9.5.  List of Parameters for PMD Primitives."§Table 9.5.  List of Parameters for PMD Primitives.

tc "9.5.5  PMD_SAP Detailed Service Specification."§9.5.5  PMD_SAP Detailed Service Specification.  The following section describes the services provided by each PMD primitive.

tc "9.5.5.1  PMD_DATA.request"§9.5.5.1  PMD_DATA.request

9.5.5.1.1  Function.  This primitive defines the transfer of data from the PLCP sublayer to the PMD entity.

9.5.5.1.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_DATA.request  (TXD_UNIT)

The TXD_UNIT parameter can take on one of three two values: ONE, or ZERO, or TRISTATE.  This parameter represents a single data bit.  The effect of this parameter is ******that the PMD will properly modulate the medium to represent ONEs or ZEROs as defined in the FHSS PMD Modulation Specifications for a given data rate.

9.5.5.1.3  When Generated.  This primitive is generated by the PLCP sublayer to request the transmission of a single data bit on the Physical Medium Dependent sublayer.  The bit clock is assumed to be resident or part of the PLCP and this primitive is issued at every clock edge once the PLCP has begun transmitting data.

9.5.5.1.4  Effect of Receipt.  The receipt of this primitive will cause the PMD entity to encode and transmit a single data bit.

tc "9.5.5.2  PMD_DATA.indicate"§9.5.5.2  PMD_DATA.indicate

9.5.5.2.1  Function.  This primitive defines the transfer of data from the PMD entity to the PLCP sublayer.

9.5.5.2.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_DATA.indicate  (RXD_UNIT)

The RXD_UNIT parameter can take on one of two values: ONE, or ZERO  This parameter represents the current state of the media as determined by the *****FHSS PMD Modulation Specifications for a given data rate.

9.5.5.2.3  When Generated.  The PMD_DATA.indicate is generate to all receiving PLCP entities in the network after a PMD_DATA.request is issued.

9.5.5.2.4  Effect of Receipt.  The effect of receipt of this primitive by the PLCP is unspecified.

tc "9.5.5.3  PMD_TXRX.request "§9.5.5.3  PMD_TXRX.request

9.5.5.3.1  Function.  This primitive is used to place the PMD entity into the transmit or receive function.

9.5.5.3.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_TXRX.request  (RF_STATE)

The RF_STATE parameter can take on one of two values: TRANSMIT or RECEIVE.  When the value of the primitive is TRANSMIT, the RF state of the radio is transmit.  If the value of the primitive is RECEIVE, the RF state of the radio is receive.

9.5.5.3.3  When Generated.  This primitive is generated whenever the mode of the radio needs to be set or when changing from transmit to receive or receive to transmit.

9.5.5.3.4  Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the mode of the radio to be in either transmit or receive.

tc "9.5.5.4  PMD_PARAMP.request"§9.5.5.4  PMD_PARAMP.request

9.5.5.4.1  Function.  This primitive defines the start of the ramp up or ramp down of the radio transmitter's Power Amplifier.

9.5.5.4.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_PARAMP.request  (RAMP_STATE)

The RAMP_STATE parameter can take on one of two values: ON or OFF.  When the value of the primitive is ON, the state of the transmit power amplifier is "on".  If the value of the primitive is OFF, the state of the transmit power amplifier is "off".

9.5.5.4.3  When Generated.  This primitive is issued only during transmit and to establish the initial state.  It is generated by the PLCP at the start of the transmit function to turn the transmitter's power amplifier "on".  A power amplifier ramp up period follows the change of state from "off" to "on".  After the PLCP has transferred all require data to the PMD entity, this primitive again will be issued by the PLCP to place the transmit power amplifier back into the "off" state.  A power amplifier ramp down period follows the change of state from "on" to "off".

9.5.5.4.4  Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the transmit power amplifier to be on or off.

tc "9.5.5.5  PMD_ANTSEL.request"§9.5.5.5  PMD_ANTSEL.request

9.5.5.5.1  Function.  This primitive is used to select which antenna the PMD entity will use to transmit or receive data.

9.5.5.5.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_ANTSEL.request  (ANTENNA_STATE)

The ANTENNA_STATE parameter can take on one of two values: ONE or TWO.  When the value of the primitive is a ONE, the PMD will switch to antenna 1 for receive or transmit.  If the value of the primitive is TWO, the PMD entity will switch to antenna 2 for receive or transmit.

9.5.5.5.3  When Generated.  This primitive is generate at various times by the PLCP entity to select an antenna.  During receive, this primitive can be used to do antenna diversity.  During transmit, this primitive can be use to select a transmit antenna.  This primitive will also be used during Clear Channel Assessment.

9.5.5.5.4  Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the radio to select the antenna specified.

tc "9.5.5.6  PMD_TXPWRLVL.request"§9.5.5.6  PMD_TXPWRLVL.request

9.5.5.6.1  Function.  This primitive defines the power level the PMD entity will use to transmit data.

9.5.5.6.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_TXPWRLVL.request  (TXPWR_LEVEL)

The TXPWR_LEVEL parameter can be one of the following values listed in table 9.6 below.

TXPWR_LEVEL

Level Description

LEVEL1

TBD

LEVEL2

TBD

LEVEL3

TBD

tc "Table 9.6.  Transmit Power Levels"§Table 9.6.  Transmit Power Levels

9.5.5.6.3  When Generated.  This primitive is generated as part of the transmit sequence.

9.5.5.6.4  Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the transmit power level to be modify.

tc "9.5.5.7  PMD_FREQ.request"§9.5.5.7  PMD_FREQ.request

9.5.5.7.1  Function.  This primitive defines the frequency the PMD entity will use to receive   or transmit data.  Since changing the radio frequency is not an immediate function, this primitive serves also as an indication of the start of this process.  The completion of this process is dictated by other PMD specifications.

9.5.5.7.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_FREQ.request  (CHNL_ID)

The CHNL_ID parameter can be one of the following values list in table 9.11.

9.5.5.7.3  When Generated.  This primitive is generated by the PLCP whenever a change to a new frequency is required.

9.5.5.7.4  Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the radio to change to a new frequency defined by the value of the CHNL_ID.

tc "9.5.5.8  PMD_RSSI.indicate"§9.5.5.8  PMD_RSSI.indicate

9.5.5.8.1  Function.  This primitive transfer a receiver signal strength indication of the physical medium from the PMD sublayer to the PLCP sublayer.  This value will be used by the PLCP to performing any diversity or clear channel assessment functions required by other sublayers.

9.5.5.8.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_RSSI.request  (STRENGTH)

The STRENGTH parameter can be ########.

9.5.5.8.3  When Generated.  This primitive is generated constantly by the PMD entity to transfer a receive signal strength indication to the PLCP.

9.5.5.8.4  Effect of Receipt.  The effect of receipt of this primitive by the PLCP is unspecified.

tc "9.5.6 MPMD_SAP Sublayer Management Primitives."§9.5.6 MPMD_SAP Sublayer Management Primitives.  The following messages may be sent between the PMD sublayer entities and intralayer or higher layer management entities.

Primitive

Request

Indicate

Confirm

Response

MPMD_PWRMGNT

X

MPMD_SYNLOCK

X

tc "Table 9.7.  MPMD Layer Management Primitives"§Table 9.7.  MPMD Layer Management Primitives

tc "9.5.6.1 MPMD_SAP Management Service Primitives Parameters."§9.5.6.1 MPMD_SAP Management Service Primitives Parameters.    The following table shows the parameters used by one or more of the MPMD_SAP Service Primitives.

Parameter

Associate Primitive

Value

MODE

MPMD_PWRMGNT.request

ON, OFF

STATUS

MPMD_SYNLOCK.indicate

LOCKED, UNLOCKED

tc "Table 9.8.  List of Parameters for PMD "§Table 9.8.  List of Parameters for PMD

Layer Management Primitives.;

tc "9.5.6.1  MPMD_PWRMGNT.request "§9.5.6.1  MPMD_PWRMGNT.request

9.5.6.1.1  Function.  This primitive is used by the higher layers entities to manage or control the power consumption of the PMD when not in use.  This allows higher layer entities to put the radio into a sleep or standby mode when not expecting to receive or send any data.

9.5.6.1.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

MPMD_PWRMGNT.request  (MODE)

The MODE parameter can be one of two values: ON or OFF.  When the value of the parameter is ON, the PMD entity will enter into a fully functional mode which allows it to send or receive data.  When the value of the parameter is OFF, the PMD entity will place itself in a standby or low power mode.  In the low power mode, the PMD entity is not expected to be able to perform any request by the PLCP nor is it expected to indicate any change in PMD state or status.

9.5.6.1.3  When Generated.  This primitive is delivered by the PLCP but actually is generated by a high layer management entity

9.5.6.1.4  Effect of Receipt.  Upon receipt of this primitive, the PMD entity will enter a fully functional or low power consumption state depending on the value of the primitive's parameter.

tc "9.5.6.2  MPMD_SYNLOCK.indicate"§9.5.6.2  MPMD_SYNLOCK.indicate

9.5.6.2.1  Function.  This primitive is a indication by the PMD entity to the PLCP that the radio synthesizer is locked to the frequency specified by the PMD_FREQ primitive.

9.5.6.2.2  Semantics of the Service Primitive.  The primitive shall provide the following parameters:

MPMD_SYNLOCK.indicate  (STATUS)

The STATUS parameter can be one of two values: LOCKED or UNLOCKED.  When the value of the parameter is LOCKED, the radio's synthesizer will be on the frequency specified by the PMD_FREQ primitive.  When the value of the parameter is UNLOCKED, the radio's synthesizer frequency will not match the frequency specified in the PMD_FREQ primitive.

9.5.6.2.3  When Generated.  The primitive will be issued whenever the PMD entity is required to change channels or hop.  The UNLOCKED value will always appear whenever the PMD_FREQ primitive is issued and the CHNL_ID value of that primitive doesn't match the current frequency of the synthesizer.  The LOCKED value will be issued whenever the frequency of the synthesizer and the CHNL_ID value become matched.  In the case which the radio's synthesizer is already at the frequency specified by PMD_FREQ primitive, the LOCKED indication will immediately be returned to the PLCP or high layer entity.

9.5.6.2.4  Effect of Receipt.  The effect of receipt of this primitive by the PLCP is unspecified.

tc "9.5.7  1M Bit PMD Operating Specifications General."§9.5.7  1M Bit PMD Operating Specifications General.  In general, the PMD accepts Convergence Layer-service primitives and provides the actual means by which the signals required by these primitives are imposed onto the medium.   In the Physical Medium Dependent sublayer at the receiver the process is reversed.  The combined function of the transmitting and receiving Physical Medium Dependent sublayers results in a data stream, timing information, and receive parameter information being delivered to the receiving Convergence Sublayer.

tc "9.5.7.1  Operating Frequency Range."§9.5.7.1  Operating Frequency Range.  A conformant PMD implementation shall be able to select the carrier frequency (Fc) from the full country-specific set of available set of carrier frequencies.  The set of carrier frequencies supported by an implementation is contained in the managed object detailing the available values of Fc. Table 9.9. summarizes these frequencies for a number of geographic locations:

Lower Limit

Upper Limit

Full Operating

Range

Geography

Status

2.402 GHz

2.482 GHz

2.400-2.483 GHz

USA

CLOSED

2.402 GHz

2.482 GHz

2.400-2.483 GHz

Europe

open

2.471 GHz

2.497 GHz

2.470-2.499 GHz

Japan

open

tc "Table 9.9.  Operating Frequency Range."§Table 9.9.  Operating Frequency Range.

tc "9.5.7.2  Number of Operating Channels."§9.5.7.2  Number of Operating Channels.  The number of transmit and receive frequency channels used for operating the PMD entity is 79. This is more fully defined in Table 9.11 of section 9.5.7.3 and section 9.5.7.6 for operation in the U.S.A.

Minimum*

Hopping Set

Geography

Status

75

79

USA

A.1.1

20

79

Europe

open

10

21

Japan

open

tc "Table 9.10.  Number of Operating Channels"§Table 9.10.  Number of Operating Channels

*The entry in the column labeled "Minimum" is per the regulatory bodies associated with each geographic area.

tc "9.5.7.3  Operating Channel Center Frequency."§9.5.7.3  Operating Channel Center Frequency. (CLOSED: A.2)  The channel center frequency is defined in sequential 1.0 MHz steps beginning with the first channel, channel 2.402 GHz for the U.S.A., as listed in the following table.

Channel #

Value

Channel #

Value

Channel #

Value

1

2.402

27

2.428

53

2.454

2

2.403

28

2.429

54

2.455

3

2.404

29

2.430

55

2.456

4

2.405

30

2.431

56

2.457

5

2.406

31

2.432

57

2.458

6

2.407

32

2.433

58

2.459

7

2.408

33

2.434

59

2.460

8

2.409

34

2.435

60

2.461

9

2.410

35

2.436

61

2.462

10

2.411

36

2.437

62

2.463

11

2.412

37

2.438

63

2.464

12

2.413

38

2.439

64

2.465

13

2.414

39

2.440

65

2.466

14

2.415

40

2.441

66

2.467

15

2.416

41

2.442

67

2.468

16

2.417

42

2.443

68

2.469

17

2.418

43

2.444

69

2.470

18

2.419

44

2.445

70

2.471

19

2.420

45

2.446

71

2.472

20

2.421

46

2.447

72

2.473

21

2.422

47

2.448

73

2.474

22

2.423

48

2.449

74

2.475

23

2.424

49

2.450

75

2.476

24

2.425

50

2.451

76

2.477

25

2.426

51

2.452

77

2.478

26

2.427

52

2.453

78

2.479

79

2.480

tc "Table 9.11  USA and Europe Requirements"§Table 9.11  USA and Europe Requirements:

(Values specified in GHz)

Channel #

Value

Channel #

Value

Channel #

Value

1

2.471

10

2.480

19

2.489

2

2.472

11

2.481

20

2.490

3

2.473

12

2.482

21

2.491

4

2.474

13

2.483

22

2.492

5

2.475

14

2.484

23

2.493

6

2.476

15

2.485

24

2.494

7

2.477

16

2.486

25

2.495

8

2.478

17

2.487

26

2.496

9

2.479

18

2.488

27

2.497

tc "Table 9.12.  Japan Requirements"§Table 9.12.  Japan Requirements:

(Values specified in GHz)

tc "9.5.7.4  Occupied Channel Bandwidth."§9.5.7.4  Occupied Channel Bandwidth.  (CLOSED: A.3)  The occupied channel bandwidth for the PMD is 1.0 MHz wide. as specified at the -20 dB points of the associated signal spectrum. This 1.0 MHz channel must contain 99% of the emitted engery. is measured at the ± 500 kHz frequency limits from the specified operating center frequency   The FCC may impose a further restriction on transmitted bandwidth requiring the 20 dB bandwidth, as measured with a sprectrum analyzer and referenced to the magnitude at the center of the transmitted bandwidth, to be less than 1 MHz.

The transmitter center frequency shall be within +/- 25ppm of one of the specified operating center frequencies listed in section 9.5.7.3. The following diagram illustrates the relationship of the operating channel transmitter center frequency (defined as Fc) to the occupied channel bandwidth.

tc "Figure 9-11.  Occupied Channel Bandwidth."§Figure 9-11.  Occupied Channel Bandwidth.

tc "9.5.7.5  Minimum Hop Rate."§9.5.7.5  Minimum Hop Rate.  (OPEN: A.4, A.5, A.6, A.7)  The rate at which the PMD entity is required to hop is governed by the MAC. Since the MAC must have the ability to maximize the use of each hop interval, the MAC must tell the PMD when to hop, thus defining the system hop rate. This precludes the notion of a maximum hop rate, which will most likely be determined by the transmit to receive duty cycle of the PMD.

The minimum hop rate, on the other hand, will be governed by the regulatory bodies in each geographic area. The minimum hop rate is specified by the number of channels visited divided by the total time spent on each of these channels. For the U.S.A, Part 15.247 of the Rules of the FCC states that a PMD must visit at least 75 channels in a 30 second period:

µ §

(Still need to specify this for Europe and Japan)

tc "9.5.7.6  Hop Sequences."§9.5.7.6  Hop Sequences.  (CLOSED: A.8)

The hopping sequence of an individual PMD entity is used to co-locate multiple PMD entities in similar networks in the same geographic area and to enhance the overall efficiency and throughput capacity of each individual network. The sequence is defined by the hopping length, p, and hopping pattern, Fi, where a family of (p-1) patterns is given by:

Fi = fi(0) fi(1) fi(2) ... fi(p-1)	with fi(j) = i*j mod(p)

Each of the Fi contains each p frequency channels equally often as each Fi is a permutation of 0, 1, 2 ..., (p-1). Given a pattern length of p and the criterion of minimal adjacent channel interference, the number of usable hopping sequences that can be derived is:

µ §

Where k = the number of adjacent channel interferers on each side of the channel frequency. For the 802.11 compliant FHSS PMD, there are three sets of hopping sequences with 22 patterns per sequence that meet the criterion of one adjacent channel interferer on each side of the desired channel.  The three sets of hopping sequences of 22 patterns each are listed below in order of preference. Set One contains patterns 24-45, set Two contains patterns 47-68 and set Three contains patterns 2-23. The channel numbers listed under each pattern refer to the actual frequency values listed in Table A, B & C in the Annex.

tc "9.5.7.7  Spurious In-Band Emissions."§9.5.7.7  Spurious In-Band Emissions.  (CLOSED: A.25)  Conformant PMD implementations of this FHSS standard shall limit their in-band spurious emissions, while transmitting or receiving any symbol pattern, to less than -55 dBc as measured with respect to the fundamental carrier frequency (Fc) within the operating frequency range as specified in Table 9.9. of section 9.5.7.1.  The following diagram is an example of this limit as related to the operating frequency range for the U.S. geographic location.

tc "Figure 9-12.  In-Band Spurious Response"§Figure 9-12.  In-Band Spurious Response

tc "9.5.7.8  Spurious Out-of-Band Emissions."§9.5.7.8  Spurious Out-of-Band Emissions.  (CLOSED: Regulatory)  Conformant PMD implementations of this FHSS standard shall limit the spurious emissions that fall outside of the operating frequency range, defined in Table 9.9 of section 9.5.7.1, to the geographically applicable regulations. For the U.S.A., the Rules of the FCC parts 15.247, 15.205 and 15.209 are the applicable regulations that govern these out-of-band emissions. For Europe, the governing regulations are ETS 300-328. {Japan TBD??}.

tc "9.5.7.9  Modulation."§9.5.7.9  Modulation.  (CLOSED: A.18)  The process of moving from the frequency representing one medium symbol to the frequency representing another shall be implemented as a continuous phase frequency modulation in a manner that results in the signal on the medium being that which would have been generated by modulating an ideal voltage controlled oscillator with a baseband control signal that falls within the mask detailed in Figure 9-13. Alternatively,  the time domain mask detailed in Figure 9-?? could be interpreted as the range of permissible baseband waveforms that could emerge from an ideal limiter-discriminator demodulator with the transmitter and receiver coupled together through a perfect channel exhibiting a VSWR of 1.0. The signal shall be such that the boundaries of the mask detailed in Figure 9-13 are not violated by any transmit symbol pattern.

The minimum set of requirements for a PMD to be compliant with the 802.11 FHSS PHY shall be that it is capable of operating using 2 level GFSK modulation with a nominial modulation index of BT=0.5. and a nominal peak deviation of 160 kHz.   The PMD will accept symbols from the set {{1},{0},{tristate}}from the PLCP.  The symbol {1} is encoded with a peak deviation of (+f), giving a peak transmit frequency of  (Fc+f), which is greater than the carrier center frequency (Fc).  The symbol {0} is encoded with a peak frequency deviation of (-f), giving a peak transmit frequency of (Fc-f).  The symbol {tristate} shall be encoded as the frequency (Fc) within the tolerance specifications detailed in section 9.5.7.5.

tc "Figure 9-13.  Transmit Modulation Mask"§

Figure 9-13.  Transmit Modulation Mask

tc "9.5.7.10  Channel Data Rate."§9.5.7.10  Channel Data Rate.  (CLOSED: A.15)  A compliant 802.11 FHSS PMD will be capable of transmitting and receiving at a nominal data rate of 1.0 MBps. as specified by the modulation parameters and criterion outlined in section 9.5.7.9. This data rate is considered to be the raw, over-the-air data rate as measured between the respective antenna ports of a transmitting PMD and the intended receiving PMD. This data rate is specified under clear medium conditions, where a clear medium is defined as a communication channel between a transmitting and receiving PMD that is void of interference, and where the BER specified in section 9.5.9.3 is achievable at minimum receiver sensitivity as specified in section 9.5.9.1.

tc "9.5.7.11  Channel Switching/Settling Time (OPENClosed August)"§9.5.7.11  Channel Switching/Settling Time (OPENClosed August)   The time to change from one operating channel frequency, as specified in section 9.5.7.3, is defined as 224 usec.  A conformant PMD is said to meet this switching time specification when the desired final operating channel center frequency has settled to within +/- 60 kHz of the operating channel center frequency as outlined in section 9.5.7.3.

(Editor's  NOTE:  To add more verbage to specify the conditions for switching.  Maybe add a graph to clarify)

tc "9.5.7.12  Transmit to Receive Switch Time (OPEN)"§9.5.7.12  Transmit to Receive Switch Time (OPEN)

Add Text

tc "9.5.7.13  Receive to Transmit Switching Time (OPEN)"§9.5.7.13  Receive to Transmit Switching Time (OPEN)

Add Text

tc "9.5.7.14  Operating Channel Availability (Closed per PAR)"§9.5.7.14  Operating Channel Availability (Closed per PAR)    A conformant PMD must provide availability to each operating channel center frequency at least 99.5% of the time with no interference present in the occupied operating channel bandwidth.

tc "9.5.7.15  Antenna Port Impedance/VSWR  (OPEN)."§9.5.7.15  Antenna Port Impedance/VSWR  (OPEN).

Add text.

tc "9.5.8  PMD Transmit Specifications."§9.5.8  PMD Transmit Specifications.  The following section describes the transmit functions and parameters associated with the Physical Medium Dependent sublayer. In general, these are specified by primitives from the PLCP and the Transmit PMD entity provides the actual means by which the signals required by the PLCP primitives are imposed onto the medium.

tc "9.5.8.1  Transmit Power Levels."§9.5.8.1  Transmit Power Levels.  (CLOSED: A.11, A.12)  In addition to the requirements imposed on the transmit signal by the baseband wave shape detailed in section 9.5.8.9, the signal shall also exhibit the characteristic that the maximum Equivalent Radiated Power (EIRP) of the PMD, as measured in accordance with the geographically applicable regulations, shall not exceed that listed in Table 9.13.  In addition, all conformant PMD implementations shall be capable of transmitting a minimum of 1.0 mW.

Maximum

EIRP [mW]

Geography

Status

1000

USA

CLOSED

100

Europe

CLOSED

10 / MHz

Japan

open

tc "Table 9.13.  Transmit Power Limits"§Table 9.13.  Transmit Power Limits

tc "9.5.8.2  Transmit Power Level Control."§9.5.8.2  Transmit Power Level Control.   (CLOSED: A.9, A.10, A.13)  If a conformant PMD implementation has the ability to transmit in a manner that results in the EIRP of the transmit signal exceeding the level of 100 mW, as measured by the geographically applicable regulations, at least one level of transmit power control shall be implemented.  This transmit power control shall be such that the level of the emission is reduced to a level below 100 mW under the influence of said power control.

As an optional PMD implementation, additional power level control will consist of four (4) discrete levels that are to be determined by the manufacturer. These levels must exist between the minimum transmit power level of 1.0 mW and the maximum of 1000 mW.

tc "9.5.8.3  Transmit Spectrum Shape."§9.5.8.3  Transmit Spectrum Shape.  (OPEN)  Conformant PMD implementations shall confine their emissions while transmitting any symbol pattern to be such that when measured by "the filter method" (EDITORS NOTE: WE WILL NEED TO DEFINE WHAT THIS IS) they will not exceed -20 dBc in any frequency range outside of Fc± 0.5 MHz, not exceed -45 dBc in the any frequency range outside of Fc± 2.0 MHz, not exceed -60 dBc in the any frequency range outside of Fc± 3.0 MHz. These are to be measured in a TBD bandwidth as specified by the following test conditions [NOTE: NEED TO SPECIFY CONDITIONS AND MEASUREMENT PROCEDURE].

tc "Figure 9-14.  Transmit Spectrum Mask"§Figure 9-14.  Transmit Spectrum Mask

tc "9.5.8.4  Transmit Center Frequency Tolerance."§9.5.8.4  Transmit Center Frequency Tolerance.  (OPEN: A.16)  An 802.11 FHSS compliant PMD shall have a transmit center frequency accuracy, as measured from Fc, of ±25.0 ppm. It shall maintain this stability over the following stated operating temperature ranges:

Minimum

Maximum

Conditions

Status

00 C

+400 C

Indoors

open

-200 C

+550 C

Outdoors

open

-150 C

+550 C

Portable Equipment

open

tc "Table 9.14.  Transmit Center Frequency Tolerance"§Table 9.14.  Transmit Center Frequency Tolerance

tc "9.5.9  PMD Receiver Specifications."§9.5.9  PMD Receiver Specifications.  The following section describes the receive functions and parameters associated with the Physical Medium Dependent sublayer. In general, these are specified by primitives from the PLCP and the Receive PMD entity provides the actual means by which the signals required by the PLCP primitives are recovered from the medium. The PMD sublayer monitors signals on the medium and will return symbols from the set {{1},{0},{tristate}} to the PLCP Sublayer.

tc "9.5.9.1  Spurious Free Dynamic Range."§9.5.9.1  Spurious Free Dynamic Range. (OPEN)  A conformant PMD implementation must be capable of recovering a conformant PMD signal from the medium, as described in related sections, whose level is between -80 dBm (defined as minimum sensitivity) and -20 dBm (defined as maximum allowable input level). The conformant PMD signal must maintain an Eb/No of 16.0 dB in the presence of Gaussian white noise at a BER of greater than or equal to 10-5.

tc "9.5.9.2  Selectivity."§9.5.9.2  Selectivity.  (OPEN)  A conformant PMD implementation must be capable of recovering a conformant PMD signal from the medium, as described in related sections, when a signal is offset from the center frequency (Fc) by greater than 2.0 MHz and has a power level that is 45 dB higher than that of the desired signal.

Conformance to this section is measured by inputting an in-channel signal at a level that provides a BER of 10-5. This signal is then increased in level by 1.0 dB. Simultaneously, an alternate channel signal with the same modulation characteristics, defined as Fc ± 2.0 MHz, is increased in level until the resultant BER is 10-5. The difference between the desired and undesired signal levels must be greater than 45 dB. This measurement is performed in a AWGN channel.

tc "9.5.9.3  Channel BER."§9.5.9.3  Channel BER.  (OPEN)  A conformant PMD implementation must provide a channel BER of at least 10-5 at an Eb/No of 16.0 dB in an AWGN channel for 1M bit per second data rate.

tc "9.5.9.4  Receive Center Frequency Tolerance."§9.5.9.4  Receive Center Frequency Tolerance.  (OPEN)  An 802.11 FHSS compliant PMD shall have a receive center frequency accuracy, as measured from Fc, of ±25.0 ppm. It shall maintain this stability over the following stated operating temperature ranges:

Minimum

Maximum

Conditions

Status

00 C

+400 C

Indoors

open

-200 C

+550 C

Outdoors

open

-150 C

+550 C

Portable Equipment

open

tc "Table 9.15.  Operating temperature Range"§Table 9.15.  Operating temperature Range

tc "9.5.9.5  Carrier Detect Response Time."§9.5.9.5  Carrier Detect Response Time.  (OPEN)  A conformant PMD implementation must be capable of providing to the PLCP within TBD µseconds an indication of whether the receiver has been able to determine if the channel has a carrier present on it. This will influence the time it takes for an 802.11 compliant PMD to acquire a signal that is present on the medium.

tc "9.5.9.6  Clock Recovery Time."§9.5.9.6  Clock Recovery Time  (OPEN)  A conformant PMD implementation must be capable of withstanding a data pattern of up to seven (7) continuos "1's" or seven (7) continuos "0's" with no degradation in output signal to noise ratio and BER.

tc "9.5.9.7  Receiver Data Jitter Tolerance(OPEN)."§ 9.5.9.7  Receiver Data Jitter Tolerance(OPEN)   A conformant PMD implementation must be capable of providing to the PLCP the following bit width restrictions as applied to a single logical "one" or logical "zero" data bit that meets the maximum run length and dc balance as specified in sections 9.5.9.x and 9.5.9.y.  The jitter associated with each individual data bi, as referenced from the center of a nominal dat bit period of 1.0 microsecond is outlined in Figure 9-15.

tc "Figure 9-15  Data Jitter Tolerance"§Figure 9-15  Data Jitter Tolerance

tc "9.5.9.8  Preamble Definition."§ 9.5.9.8  Preamble Definition

Add text

tc "9.5.9.8.1 Ramp Up Period (Closed per A19)"§ 9.5.9.8.1 Ramp Up Period (Closed per A19)   The purpose of a conformant PMD Ramp Up Time Period is to control the rate of change of the amplitude of the transmit signal durign its transition to the desired steady state transmit output level.  The ramp-up period is defined as a window consisting of 8-bit periods and is governed by the mask of Figure 9-16.

The following states are defined by the mask of diagram Figure 9-16.  The transmitter is considered "off" (i.e. less than -50 dBm EIRP) at the start of the first bit period and is less than 0 dBm at the end of the first bit period.  The output level at the end of the seventh bit period is within 3 dB of the desired steady state transmit power level and within 1 dB at the end of the eighth bit period.  The maximum magnitude of the rate of change of the output power level should be one volt per microsecond as measured by a wideband detector based on the RMS output voltage into a 50-ohm load of the conformant PMD.

tc "Figure 9-16  Ramp Up"§

Figure 9-16  Ramp Up

tc "9.5.9.8.2 Ramp Down Period (Closed per A19)"§ 9.5.9.8.2 Ramp Down Period (Closed per A19)   The purpose of a conformant PMD Ramp Down Time Period is to control the rate of change of the amplitude of the transmit signal durign its transition to the desired steady state transmit output level.  The ramp-down period is defined as a window consisting of 8-bit periods and is governed by the mask of Figure 9-17.

The following states are defined by the mask of Figure 9-17.  The transmitter is considered "off" (i.e. less than -50 dBm EIRP) at the start of the first bit period and is less than 0 dBm at the end of the first bit period.  The output level at the end of the seventh bit period is within 3 dB of the desired steady state transmit power level and within 1 dB at the end of the eighth bit period.  The maximum magnitude of the rate of change of the output power level should be one volt per microsecond as measured by a wideband detector based on the RMS output voltage into a 50-ohm load of the conformant PMD.

tc "Figure 9-17  Ramp Down"§

Figure 9-17  Ramp Down

tc "9.5.9.9 Receive Data DC Offset (Open)"§ 9.5.9.9 Receive Data DC Offset (Open)

Add text

tc "9.5.9.10 Received Data Maximum Run Length (DC Balance)  (Open)"§9.5.9.10 Received Data Maximum Run Length (DC Balance)  (Open)

Add text

tc "9.5.9.11 Receiver Sensitivity   (CLOSED per July 1994)"§ 9.5.9.11 Receiver Sensitivity   (CLOSED per July 1994) Sensitivity is defined as the minimum signal level required to produce a BER of 10-5.  A conformant PMD shall have the minimum signal level be less than or equal to -80 dBm across the operating frequency range

specified in 9.5.7.1.

tc "9.5.9.12 Intermodulation (CLOSED per July 1994)"§ 9.5.9.12 Intermodulation (CLOSED per July 1994)    Intermodulation protection (IMp) is defined as the ratio to measured sensitivity of the minimum amplitude of one of the two equal level interfering signals at 4 and 8 MHz removed from center frequency, both on the same side of center frequency, that cause the BER of the receiver to be reduced to 10-5, when the desired signal is 3 dB above sensitivity.  Each interfering signal is modulated with the FH PMD modulation uncorrolated in time to each other or the desired signal.  A conformant PMD shall have the IMp for the interfering signal at 4 and 8 MHz be greater than or equal to 30 dB.

tc "9.5.9.13  Desensitization (CLOSED per July 1994)"§ 9.5.9.13  Desensitization (CLOSED per July 1994)   Desensitization (Dp) is defined as the ratio to measured sensitivity of the minimum amplitude of an interfering signal that causes the BER of the receiver to be reduced to 10-5 when the desired signal is -77 dB (3 dB above sensitivity specified in section 9.5.9.11).  The interfering signal shall be modulated with the FHSS PMD modulation uncorrolated in time to the desired signal.

Dp for Interfering Signal 2 MHz removed                    	30 dB

Dp for Interfering Signal 3 or more MHz removed            	40 dB

tc "9.6 FHSS Physical Medium Dependent Sublayer 2.0M Bit"§9.6 FHSS Physical Medium Dependent Sublayer 2.0M Bit

tc "9.6.1  Introduction."§9.6.1  Introduction.

Editor's Note:  Add text introducing the 2.0 M bit 4 level FSK PMD.

tc "9.6.2  Modulation."§9.6.2  Modulation.

Editor's Note:  Add text describing modulation scheme.  This text is the responsibility of the HS-FHSS Phy group.

AS A SUGGESTION INSERT AN EYE-DIAGRAM MASK THAT IS BOUNDED BY GAUSSIAN FILTER IMPULSE RESPONSE OF BT=.5 and BT=.4 USING THE MAXIMUM PEAK DEVIATION THAT IS EXPECTED TO MEET THE FCC EMISSION MASK REQUIREMENTS.  THIS TEMPLATE CAN ALSO BE USED TO SPECIFY ALLOWABLE BIT TIMING VARIATIONS BY LABELING NOMINAL ZERO CROSSING COORDINATES

tc "Figure 9-12.  Transmit Modulation Mask"§Figure 9-12.  Transmit Modulation Mask

tc "9.6.2  Channel Data Rate."§9.6.2  Channel Data Rate.  A compliant 802.11 FHSS PMD will be capable of transmitting and receiving at a nominal data rate of 2.0 MBps as specified by the modulation parameters and criterion outlined in section 9.5.7.1. This data rate is considered to be the raw, over-the-air data rate as measured between the respective antenna ports of a transmitting PMD and the intended receiving PMD. This data rate is specified under clear medium conditions, where a clear medium is defined as a communication channel between a transmitting and receiving PMD that is void of interference, and where the BER specified in section 9.6.3 is achievable at minimum receiver sensitivity as specified in section 9.5.9.1.

tc "9.6.3  Channel BER."§9.6.3  Channel BER.   A conformant PMD implementation must provide a channel BER of at least 10-5 at an Eb/No of 22.5 dB in an AWGN channel for 2M bit per second data rate.

Annex

tc "Table A  Hopping Sequence Set 1"§Table A  Hopping Sequence Set 1

Pattern

Index

24

25

26

27

28

29

30

31

32

33

34

1

2

2

2

2

2

2

2

2

2

2

2

2

9

12

15

18

21

24

27

30

33

36

39

3

16

22

28

34

40

46

52

58

64

70

76

4

23

32

41

50

59

68

77

7

16

25

34

5

30

42

54

66

78

11

23

35

47

59

71

6

37

52

67

3

18

33

48

63

78

14

29

7

44

62

80

19

37

55

73

12

30

48

66

8

51

72

14

35

56

77

19

40

61

3

24

9

58

3

27

51

75

20

44

68

13

37

61

10

65

13

40

67

15

42

69

17

44

71

19

11

72

23

53

4

34

64

15

45

75

26

56

12

79

33

66

20

53

7

40

73

27

60

14

13

7

43

79

36

72

29

65

22

58

15

51

14

14

53

13

52

12

51

11

50

10

49

9

15

21

63

26

68

31

73

36

78

41

4

46

16

28

73

39

5

50

16

61

27

72

38

4

17

35

4

52

21

69

38

7

55

24

72

41

18

42

14

65

37

9

60

32

4

55

27

78

19

49

24

78

53

28

3

57

32

7

61

36

20

56

34

12

69

47

25

3

60

38

16

73

21

63

44

25

6

66

47

28

9

69

50

31

22

70

54

38

22

6

69

53

37

21

5

68

23

77

64

51

38

25

12

78

65

52

39

26

24

5

74

64

54

44

34

24

14

4

73

63

25

12

5

77

70

63

56

49

42

35

28

21

26

19

15

11

7

3

78

74

70

66

62

58

27

26

25

24

23

22

21

20

19

18

17

16

28

33

35

37

39

41

43

45

47

49

51

53

29

40

45

50

55

60

65

70

75

80

6

11

30

47

55

63

71

79

8

16

24

32

40

48

31

54

65

76

8

19

30

41

52

63

74

6

32

61

75

10

24

38

52

66

80

15

29

43

33

68

6

23

40

57

74

12

29

46

63

80

34

75

16

36

56

76

17

37

57

77

18

38

35

3

26

49

72

16

39

62

6

29

52

75

36

10

36

62

9

35

61

8

34

60

7

33

37

17

46

75

25

54

4

33

62

12

41

70

38

24

56

9

41

73

26

58

11

43

75

28

39

31

66

22

57

13

48

4

39

74

30

65

40

38

76

35

73

32

70

29

67

26

64

23

tc "Table A  Hopping Sequence Set 1"§Table A  Hopping Sequence Set 1

Pattern

Index

24

25

26

27

28

29

30

31

32

33

34

41

45

7

48

10

51

13

54

16

57

19

60

42

52

17

61

26

70

35

79

44

9

53

18

43

59

27

74

42

10

57

25

72

40

8

55

44

66

37

8

58

29

79

50

21

71

42

13

45

73

47

21

74

48

22

75

49

23

76

50

46

80

57

34

11

67

44

21

77

54

31

8

47

8

67

47

27

7

66

46

26

6

65

45

48

15

77

60

43

26

9

71

54

37

20

3

49

22

8

73

59

45

31

17

3

68

54

40

50

29

18

7

75

64

53

42

31

20

9

77

51

36

28

20

12

4

75

67

59

51

43

35

52

43

38

33

28

23

18

13

8

3

77

72

53

50

48

46

44

42

40

38

36

34

32

30

54

57

58

59

60

61

62

63

64

65

66

67

55

64

68

72

76

80

5

9

13

17

21

25

56

71

78

6

13

20

27

34

41

48

55

62

57

78

9

19

29

39

49

59

69

79

10

20

58

6

19

32

45

58

71

5

18

31

44

57

59

13

29

45

61

77

14

30

46

62

78

15

60

20

39

58

77

17

36

55

74

14

33

52

61

27

49

71

14

36

58

80

23

45

67

10

62

34

59

5

30

55

80

26

51

76

22

47

63

41

69

18

46

74

23

51

79

28

56

5

64

48

79

31

62

14

45

76

28

59

11

42

65

55

10

44

78

33

67

22

56

11

45

79

66

62

20

57

15

52

10

47

5

42

79

37

67

69

30

70

31

71

32

72

33

73

34

74

68

76

40

4

47

11

54

18

61

25

68

32

69

4

50

17

63

30

76

43

10

56

23

69

70

11

60

30

79

49

19

68

38

8

57

27

71

18

70

43

16

68

41

14

66

39

12

64

72

25

80

56

32

8

63

39

15

70

46

22

73

32

11

69

48

27

6

64

43

22

80

59

74

39

21

3

64

46

28

10

71

53

35

17

75

46

31

16

80

65

50

35

20

5

69

54

76

53

41

29

17

5

72

60

48

36

24

12

77

60

51

42

33

24

15

6

76

67

58

49

78

67

61

55

49

43

37

31

25

19

13

7

79

74

71

68

65

62

59

56

53

50

47

44

tc "Table A  Hopping Sequence Set 1 continued"§Table A  Hopping Sequence Set 1 continued

Pattern

Index

35

36

37

38

39

40

41

42

43

44

45

1

2

2

2

2

2

2

2

2

2

2

2

2

42

45

48

51

54

57

60

63

66

69

72

3

3

9

15

21

27

33

39

45

51

57

63

4

43

52

61

70

79

9

18

27

36

45

54

5

4

16

28

40

52

64

76

9

21

33

45

6

44

59

74

10

25

40

55

70

6

21

36

7

5

23

41

59

77

16

34

52

70

9

27

8

45

66

8

29

50

71

13

34

55

76

18

9

6

30

54

78

23

47

71

16

40

64

9

10

46

73

21

48

75

23

50

77

25

52

79

11

7

37

67

18

48

78

29

59

10

40

70

12

47

80

34

67

21

54

8

41

74

28

61

13

8

44

80

37

73

30

66

23

59

16

52

14

48

8

47

7

46

6

45

5

44

4

43

15

9

51

14

56

19

61

24

66

29

71

34

16

49

15

60

26

71

37

3

48

14

59

25

17

10

58

27

75

44

13

61

30

78

47

16

18

50

22

73

45

17

68

40

12

63

35

7

19

11

65

40

15

69

44

19

73

48

23

77

20

51

29

7

64

42

20

77

55

33

11

68

21

12

72

53

34

15

75

56

37

18

78

59

22

52

36

20

4

67

51

35

19

3

66

50

23

13

79

66

53

40

27

14

80

67

54

41

24

53

43

33

23

13

3

72

62

52

42

32

25

14

7

79

72

65

58

51

44

37

30

23

26

54

50

46

42

38

34

30

26

22

18

14

27

15

14

13

12

11

10

9

8

7

6

5

28

55

57

59

61

63

65

67

69

71

73

75

29

16

21

26

31

36

41

46

51

56

61

66

30

56

64

72

80

9

17

25

33

41

49

57

31

17

28

39

50

61

72

4

15

26

37

48

32

57

71

6

20

34

48

62

76

11

25

39

33

18

35

52

69

7

24

41

58

75

13

30

34

58

78

19

39

59

79

20

40

60

80

21

35

19

42

65

9

32

55

78

22

45

68

12

36

59

6

32

58

5

31

57

4

30

56

3

37

20

49

78

28

57

7

36

65

15

44

73

38

60

13

45

77

30

62

15

47

79

32

64

39

21

56

12

47

3

38

73

29

64

20

55

40

61

20

58

17

55

14

52

11

49

8

46

tc "Table A  Hopping Sequence Set 1 continued"§Table A  Hopping Sequence Set 1 continued

Pattern

Index

35

36

37

38

39

40

41

42

43

44

45

41

22

63

25

66

28

69

31

72

34

75

37

42

62

27

71

36

80

45

10

54

19

63

28

43

23

70

38

6

53

21

68

36

4

51

19

44

63

34

5

55

26

76

47

18

68

39

10

45

24

77

51

25

78

52

26

79

53

27

80

46

64

41

18

74

51

28

5

61

38

15

71

47

25

5

64

44

24

4

63

43

23

3

62

48

65

48

31

14

76

59

42

25

8

70

53

49

26

12

77

63

49

35

21

7

72

58

44

50

66

55

44

33

22

11

79

68

57

46

35

51

27

19

11

3

74

66

58

50

42

34

26

52

67

62

57

52

47

42

37

32

27

22

17

53

28

26

24

22

20

18

16

14

12

10

8

54

68

69

70

71

72

73

74

75

76

77

78

55

29

33

37

41

45

49

53

57

61

65

69

56

69

76

4

11

18

25

32

39

46

53

60

57

30

40

50

60

70

80

11

21

31

41

51

58

70

4

17

30

43

56

69

3

16

29

42

59

31

47

63

79

16

32

48

64

80

17

33

60

71

11

30

49

68

8

27

46

65

5

24

61

32

54

76

19

41

63

6

28

50

72

15

62

72

18

43

68

14

39

64

10

35

60

6

63

33

61

10

38

66

15

43

71

20

48

76

64

73

25

56

8

39

70

22

53

5

36

67

65

34

68

23

57

12

46

80

35

69

24

58

66

74

32

69

27

64

22

59

17

54

12

49

67

35

75

36

76

37

77

38

78

39

79

40

68

75

39

3

46

10

53

17

60

24

67

31

69

36

3

49

16

62

29

75

42

9

55

22

70

76

46

16

65

35

5

54

24

73

43

13

71

37

10

62

35

8

60

33

6

58

31

4

72

77

53

29

5

60

36

12

67

43

19

74

73

38

17

75

54

33

12

70

49

28

7

65

74

78

60

42

24

6

67

49

31

13

74

56

75

39

24

9

73

58

43

28

13

77

62

47

76

79

67

55

43

31

19

7

74

62

50

38

77

40

31

22

13

4

74

65

56

47

38

29

78

80

74

68

62

56

50

44

38

32

26

20

79

41

38

35

32

29

26

23

20

17

14

11

tc "Table B  Hopping Sequence Set 2"§Table B  Hopping Sequence Set 2

Pattern

Index

47

48

49

50

51

52

53

54

55

56

57

1

2

2

2

2

2

2

2

2

2

2

2

2

10

13

16

19

22

25

28

31

34

37

40

3

18

24

30

36

42

48

54

60

66

72

78

4

26

35

44

53

62

71

80

10

19

28

37

5

34

46

58

70

3

15

27

39

51

63

75

6

42

57

72

8

23

38

53

68

4

19

34

7

50

68

7

25

43

61

79

18

36

54

72

8

58

79

21

42

63

5

26

47

68

10

31

9

66

11

35

59

4

28

52

76

21

45

69

10

74

22

49

76

24

51

78

26

53

80

28

11

3

33

63

14

44

74

25

55

6

36

66

12

11

44

77

31

64

18

51

5

38

71

25

13

19

55

12

48

5

41

77

34

70

27

63

14

27

66

26

65

25

64

24

63

23

62

22

15

35

77

40

3

45

8

50

13

55

18

60

16

43

9

54

20

65

31

76

42

8

53

19

17

51

20

68

37

6

54

23

71

40

9

57

18

59

31

3

54

26

77

49

21

72

44

16

19

67

42

17

71

46

21

75

50

25

79

54

20

75

53

31

9

66

44

22

79

57

35

13

21

4

64

45

26

7

67

48

29

10

70

51

22

12

75

59

43

27

11

74

58

42

26

10

23

20

7

73

60

47

34

21

8

74

61

48

24

28

18

8

77

67

57

47

37

27

17

7

25

36

29

22

15

8

80

73

66

59

52

45

26

44

40

36

32

28

24

20

16

12

8

4

27

52

51

50

49

48

47

46

45

44

43

42

28

60

62

64

66

68

70

72

74

76

78

80

29

68

73

78

4

9

14

19

24

29

34

39

30

76

5

13

21

29

37

45

53

61

69

77

31

5

16

27

38

49

60

71

3

14

25

36

32

13

27

41

55

69

4

18

32

46

60

74

33

21

38

55

72

10

27

44

61

78

16

33

34

29

49

69

10

30

50

70

11

31

51

71

35

37

60

4

27

50

73

17

40

63

7

30

36

45

71

18

44

70

17

43

69

16

42

68

37

53

3

32

61

11

40

69

19

48

77

27

38

61

14

46

78

31

63

16

48

80

33

65

39

69

25

60

16

51

7

42

77

33

68

24

40

77

36

74

33

71

30

68

27

65

24

62

tc "Table B  Hopping Sequence Set 2"§Table B  Hopping Sequence Set 2

Pattern

Index

47

48

49

50

51

52

53

54

55

56

57

41

6

47

9

50

71

75

79

4

8

12

16

42

14

58

23

67

12

19

26

33

40

47

54

43

22

69

37

5

32

42

52

62

72

3

13

44

30

80

51

22

52

65

78

12

25

38

51

45

38

12

65

39

72

9

25

41

57

73

10

46

46

23

79

56

13

32

51

70

10

29

48

47

54

34

14

73

33

55

77

20

42

64

7

48

62

45

28

11

53

78

24

49

74

20

45

49

70

56

42

28

73

22

50

78

27

55

4

50

78

67

56

45

14

45

76

28

59

11

42

51

7

78

70

62

34

68

23

57

12

46

80

52

15

10

5

79

54

12

49

7

44

2

39

53

23

21

19

17

74

35

75

36

76

37

77

54

31

32

33

34

15

58

22

65

29

72

36

55

39

43

47

51

35

2

48

15

61

28

74

56

47

54

61

68

55

25

74

44

14

63

33

57

55

65

75

6

75

48

21

73

46

19

71

58

63

76

10

23

16

71

47

23

78

54

30

59

71

8

24

40

36

15

73

52

31

10

68

60

79

19

38

57

56

38

20

2

63

45

27

61

8

30

52

74

76

61

46

31

16

80

65

62

16

41

66

12

17

5

72

60

48

36

24

63

24

52

80

29

37

28

19

10

80

71

62

64

32

63

15

46

57

51

45

39

33

27

21

65

40

74

29

63

77

74

71

68

65

62

59

66

48

6

43

80

18

18

18

18

18

18

18

67

56

17

57

18

38

41

44

47

50

53

56

68

64

28

71

35

58

64

70

76

3

9

15

69

72

39

6

52

78

8

17

26

35

44

53

70

80

50

20

69

19

31

43

55

67

79

12

71

9

61

34

7

39

54

69

5

20

35

50

72

17

72

48

24

59

77

16

34

52

70

9

73

25

4

62

41

79

21

42

63

5

26

47

74

33

15

76

58

20

44

68

13

37

61

6

75

41

26

11

75

40

67

15

42

69

17

44

76

49

37

25

13

60

11

41

71

22

52

3

77

57

48

39

30

80

34

67

21

54

8

41

78

65

59

53

47

21

57

14

50

7

43

79

79

73

70

67

64

41

80

40

79

39

78

38

tc "Table B  Hopping Sequence Set 2 continued"§Table B  Hopping Sequence Set 2 continued

Pattern

Index

58

59

60

61

62

63

64

65

66

67

68

1

2

2

2

2

2

2

2

2

2

2

2

2

43

46

49

52

55

58

61

64

67

70

73

3

5

11

17

23

29

35

41

47

53

59

65

4

46

55

64

73

3

12

21

30

39

48

57

5

8

20

32

44

56

68

80

13

25

37

49

6

49

64

79

15

30

45

60

75

11

26

41

7

11

29

47

65

4

22

40

58

76

15

33

8

52

73

15

36

57

78

20

41

62

4

25

9

14

38

62

7

31

55

79

24

48

72

17

10

55

3

30

57

5

32

59

7

34

61

9

11

17

47

77

28

58

9

39

69

20

50

80

12

58

12

45

78

32

65

19

52

6

39

72

13

20

56

13

49

6

42

78

35

71

28

64

14

61

21

60

20

59

19

58

18

57

17

56

15

23

65

28

70

33

75

38

80

43

6

48

16

64

30

75

41

7

52

18

63

29

74

40

17

26

74

43

12

60

29

77

46

15

63

32

18

67

39

11

62

34

6

57

29

80

52

24

19

29

4

58

33

8

62

37

12

66

41

16

20

70

48

26

4

61

39

17

74

52

30

8

21

32

13

73

54

35

16

76

57

38

19

79

22

73

57

41

25

9

72

56

40

24

8

71

23

35

22

9

75

62

49

36

23

10

76

63

24

76

66

56

46

36

26

16

6

75

65

55

25

38

31

24

17

10

3

75

68

61

54

47

26

79

75

71

67

63

59

55

51

47

43

39

27

41

40

39

38

37

36

35

34

33

32

31

28

3

5

7

9

11

13

15

17

19

21

23

29

44

49

54

59

64

69

74

79

5

10

15

30

6

14

22

30

38

46

54

62

70

78

7

31

47

58

69

80

12

23

34

45

56

67

78

32

9

23

37

51

65

79

14

28

42

56

70

33

50

67

5

22

39

56

73

11

28

45

62

34

12

32

52

72

13

33

53

73

14

34

54

35

53

76

20

43

66

10

33

56

79

23

46

36

15

41

67

14

40

66

13

39

65

12

38

37

56

6

35

64

14

43

72

22

51

80

30

38

18

50

3

35

67

20

52

5

37

69

22

39

59

15

50

6

41

76

32

67

23

58

14

40

21

59

18

56

15

53

12

50

9

47

6

tc "Table B  Hopping Sequence Set 2 continued"§Table B  Hopping Sequence Set 2 continued

Pattern

Index

58

59

60

61

62

63

64

65

66

67

68

41

62

24

65

27

68

30

71

33

74

36

77

42

24

68

33

77

42

7

51

16

60

25

69

43

65

33

80

48

16

63

31

78

46

14

61

44

27

77

48

19

69

40

11

61

32

3

53

45

68

42

16

69

43

17

70

44

18

71

45

46

30

7

63

40

17

73

50

27

4

60

37

47

71

51

31

11

70

50

30

10

69

49

29

48

33

16

78

61

44

27

10

72

55

38

21

49

74

60

46

32

18

4

69

55

41

27

13

50

36

25

14

3

71

60

49

38

27

16

5

51

77

69

61

53

45

37

29

21

13

5

76

52

39

34

29

24

19

14

9

4

78

73

68

53

80

78

76

74

72

70

68

66

64

62

60

54

42

43

44

45

46

47

48

49

50

51

52

55

4

8

12

16

20

24

28

32

36

40

44

56

45

52

59

66

73

80

8

15

22

29

36

57

7

17

27

37

47

57

67

77

8

18

28

58

48

61

74

8

21

34

47

60

73

7

20

59

10

26

42

58

74

11

27

43

59

75

12

60

51

70

10

29

48

67

7

26

45

64

4

61

13

35

57

79

22

44

66

9

31

53

75

62

54

79

25

50

75

21

46

71

17

42

67

63

16

44

72

21

49

77

26

54

3

31

59

64

57

9

40

71

23

54

6

37

68

20

51

65

19

53

8

42

76

31

65

20

54

9

43

66

60

18

55

13

50

8

45

3

40

77

35

67

22

62

23

63

24

64

25

65

26

66

27

68

63

27

70

34

77

41

5

48

12

55

19

69

25

71

38

5

51

18

64

31

77

44

11

70

66

36

6

55

25

74

44

14

63

33

3

71

28

80

53

26

78

51

24

76

49

22

74

72

69

45

21

76

52

28

4

59

35

11

66

73

31

10

68

47

26

5

63

42

21

79

58

74

72

54

36

18

79

61

43

25

7

68

50

75

34

19

4

68

53

38

23

8

72

57

42

76

75

63

51

39

27

15

3

70

58

46

34

77

37

28

19

10

80

71

62

53

44

35

26

78

78

72

66

60

54

48

42

36

30

24

18

79

40

37

34

31

28

25

22

19

16

13

10

tc "Table C  Hopping Sequence Set 3"§Table C  Hopping Sequence Set 3

Pattern

Index

2

3

4

5

6

7

8

9

10

11

12

1

2

2

2

2

2

2

2

2

2

2

2

2

11

14

17

20

23

26

29

32

35

38

41

3

20

26

32

38

44

50

56

62

68

74

80

4

29

38

47

56

65

74

4

13

22

31

40

5

38

50

62

74

7

19

31

43

55

67

79

6

47

62

77

13

28

43

58

73

9

24

39

7

56

74

13

31

49

67

6

24

42

60

78

8

65

7

28

49

70

12

33

54

75

17

38

9

74

19

43

67

12

36

60

5

29

53

77

10

4

31

58

6

33

60

8

35

62

10

37

11

13

43

73

24

54

5

35

65

16

46

76

12

22

55

9

42

75

29

62

16

49

3

36

13

31

67

24

60

17

53

10

46

3

39

75

14

40

79

39

78

38

77

37

76

36

75

35

15

49

12

54

17

59

22

64

27

69

32

74

16

58

24

69

35

80

46

12

57

23

68

34

17

67

36

5

53

22

70

39

8

56

25

73

18

76

48

20

71

43

15

66

38

10

61

33

19

6

60

35

10

64

39

14

68

43

18

72

20

15

72

50

28

6

63

41

19

76

54

32

21

24

5

65

46

27

8

68

49

30

11

71

22

33

17

80

64

48

32

16

79

63

47

31

23

42

29

16

3

69

56

43

30

17

4

70

24

51

41

31

21

11

80

70

60

50

40

30

25

60

53

46

39

32

25

18

11

4

76

69

26

69

65

61

57

53

49

45

41

37

33

29

27

78

77

76

75

74

73

72

71

70

69

68

28

8

10

12

14

16

18

20

22

24

26

28

29

17

22

27

32

37

42

47

52

57

62

67

30

26

34

42

50

58

66

74

3

11

19

27

31

35

46

57

68

79

11

22

33

44

55

66

32

44

58

72

7

21

35

49

63

77

12

26

33

53

70

8

25

42

59

76

14

31

48

65

34

62

3

23

43

63

4

24

44

64

5

25

35

71

15

38

61

5

28

51

74

18

41

64

36

80

27

53

79

26

52

78

25

51

77

24

37

10

39

68

18

47

76

26

55

5

34

63

38

19

51

4

36

68

21

53

6

38

70

23

39

28

63

19

54

10

45

80

36

71

27

62

40

37

75

34

72

31

69

28

66

25

63

22

tc "Table C  Hopping Sequence Set 3"§Table C  Hopping Sequence Set 3

Pattern

Index

2

3

4

5

6

7

8

9

10

11

12

41

46

8

49

11

52

14

55

17

58

20

61

42

55

20

64

29

73

38

3

47

12

56

21

43

64

32

79

47

15

62

30

77

45

13

60

44

73

44

15

65

36

7

57

28

78

49

20

45

3

56

30

4

57

31

5

58

32

6

59

46

12

68

45

22

78

55

32

9

65

42

19

47

21

80

60

40

20

79

59

39

19

78

58

48

30

13

75

58

41

24

7

69

52

35

18

49

39

25

11

76

62

48

34

20

6

71

57

50

48

37

26

15

4

72

61

50

39

28

17

51

57

49

41

33

25

17

9

80

72

64

56

52

66

61

56

51

46

41

36

31

26

21

16

53

75

73

71

69

67

65

63

61

59

57

55

54

5

6

7

8

9

10

11

12

13

14

15

55

14

18

22

26

30

34

38

42

46

50

54

56

23

30

37

44

51

58

65

72

79

7

14

57

32

42

52

62

72

3

13

23

33

43

53

58

41

54

67

80

14

27

40

53

66

79

13

59

50

66

3

19

35

51

67

4

20

36

52

60

59

78

18

37

56

75

15

34

53

72

12

61

68

11

33

55

77

20

42

64

7

29

51

62

77

23

48

73

19

44

69

15

40

65

11

63

7

35

63

12

40

68

17

45

73

22

50

64

16

47

78

30

61

13

44

75

27

58

10

65

25

59

14

48

3

37

71

26

60

15

49

66

34

71

29

66

24

61

19

56

14

51

9

67

43

4

44

5

45

6

46

7

47

8

48

68

52

16

59

23

66

30

73

37

80

44

8

69

61

28

74

41

8

54

21

67

34

80

47

70

70

40

10

59

29

78

48

18

67

37

7

71

79

52

25

77

50

23

75

48

21

73

46

72

9

64

40

16

71

47

23

78

54

30

6

73

18

76

55

34

13

71

50

29

8

66

45

74

27

9

70

52

34

16

77

59

41

23

5

75

36

21

6

70

55

40

25

10

74

59

44

76

45

33

21

9

76

64

52

40

28

16

4

77

54

45

36

27

18

9

79

70

61

52

43

78

63

57

51

45

39

33

27

21

15

9

3

79

72

69

66

63

60

57

54

51

48

45

42

tc "Table C  Hopping Sequence Set 3 continued"§Table C  Hopping Sequence Set 3 continued

Pattern

Index

13

14

15

16

17

18

19

20

21

22

23

1

2

2

2

2

2

2

2

2

2

2

2

2

44

47

50

53

56

59

62

65

68

71

74

3

7

13

19

25

31

37

43

49

55

61

67

4

49

58

67

76

6

15

24

33

42

51

60

5

12

24

36

48

60

72

5

17

29

41

53

6

54

69

5

20

35

50

65

80

16

31

46

7

17

35

53

71

10

28

46

64

3

21

39

8

59

80

22

43

64

6

27

48

69

11

32

9

22

46

70

15

39

63

8

32

56

80

25

10

64

12

39

66

14

41

68

16

43

70

18

11

27

57

8

38

68

19

49

79

30

60

11

12

69

23

56

10

43

76

30

63

17

50

4

13

32

68

25

61

18

54

11

47

4

40

76

14

74

34

73

33

72

32

71

31

70

30

69

15

37

79

42

5

47

10

52

15

57

20

62

16

79

45

11

56

22

67

33

78

44

10

55

17

42

11

59

28

76

45

14

62

31

79

48

18

5

56

28

79

51

23

74

46

18

69

41

19

47

22

76

51

26

80

55

30

5

59

34

20

10

67

45

23

80

58

36

14

71

49

27

21

52

33

14

74

55

36

17

77

58

39

20

22

15

78

62

46

30

14

77

61

45

29

13

23

57

44

31

18

5

71

58

45

32

19

6

24

20

10

79

69

59

49

39

29

19

9

78

25

62

55

48

41

34

27

20

13

6

78

71

26

25

21

17

13

9

5

80

76

72

68

64

27

67

66

65

64

63

62

61

60

59

58

57

28

30

32

34

36

38

40

42

44

46

48

50

29

72

77

3

8

13

18

23

28

33

38

43

30

35

43

51

59

67

75

4

12

20

28

36

31

77

9

20

31

42

53

64

75

7

18

29

32

40

54

68

3

17

31

45

59

73

8

22

33

3

20

37

54

71

9

26

43

60

77

15

34

45

65

6

26

46

66

7

27

47

67

8

35

8

31

54

77

21

44

67

11

34

57

80

36

50

76

23

49

75

22

48

74

21

47

73

37

13

42

71

21

50

79

29

58

8

37

66

38

55

8

40

72

25

57

10

42

74

27

59

39

18

53

9

44

79

35

70

26

61

17

52

40

60

19

57

16

54

13

51

10

48

7

45

tc "Table C  Hopping Sequence Set 3 continued"§Table C  Hopping Sequence Set 3 continued

Pattern

Index

13

14

15

16

17

18

19

20

21

22

23

41

23

64

26

67

29

70

32

73

35

76

38

42

65

30

74

39

4

48

13

57

22

66

31

43

28

75

43

11

58

26

73

41

9

56

24

44

70

41

12

62

33

4

54

25

75

46

17

45

33

7

60

34

8

61

35

9

62

36

10

46

75

52

29

6

62

39

16

72

49

26

3

47

38

18

77

57

37

17

76

56

36

16

75

48

80

63

46

29

12

74

57

40

23

6

68

49

43

29

15

80

66

52

38

24

10

75

61

50

6

74

63

52

41

30

19

8

76

65

54

51

48

40

32

24

16

8

79

71

63

55

47

52

11

6

80

75

70

65

60

55

50

45

40

53

53

51

49

47

45

43

41

39

37

35

33

54

16

17

18

19

20

21

22

23

24

25

26

55

58

62

66

70

74

78

3

7

11

15

19

56

21

28

35

42

49

56

63

70

77

5

12

57

63

73

4

14

24

34

44

54

64

74

5

58

26

39

52

65

78

12

25

38

51

64

77

59

68

5

21

37

53

69

6

22

38

54

70

60

31

50

69

9

28

47

66

6

25

44

63

61

73

16

38

60

3

25

47

69

12

34

56

62

36

61

7

32

57

3

28

53

78

24

49

63

78

27

55

4

32

60

9

37

65

14

42

64

41

72

24

55

7

38

69

21

52

4

35

65

4

38

72

27

61

16

50

5

39

73

28

66

46

4

41

78

36

73

31

68

26

63

21

67

9

49

10

50

11

51

12

52

13

53

14

68

51

15

58

22

65

29

72

36

79

43

7

69

14

60

27

73

40

7

53

20

66

33

79

70

56

26

75

45

15

64

34

4

53

23

72

71

19

71

44

17

69

42

15

67

40

13

65

72

61

37

13

68

44

20

75

51

27

3

58

73

24

3

61

40

19

77

56

35

14

72

51

74

66

48

30

12

73

55

37

19

80

62

44

75

29

14

78

63

48

33

18

3

67

52

37

76

71

59

47

35

23

11

78

66

54

42

30

77

34

25

16

7

77

68

59

50

41

32

23

78

76

70

64

58

52

46

40

34

28

22

16

79

39

36

33

30

27

24

21

18

15

12

9

tc "Appendix A"§Appendix A

tc "PMD Approved Motions"§PMD Approved Motions

Item

Motion

Yes

No

Abstain

A.1

The 2.4 GHz frequency hop physical layer draft specification shall have 79 frequency channels. (May 1993)

26

0

6

A.2

The 2.4 GHz frequency hop physical layer draft specification shall have a channel center frequency of 1.0 MHz (May 1993)

26

0

6

A.3

The 2.4 GHz frequency hop physical layer draft specification shall have a maximum channel bandwidth of 1.0 MHz that contains 99% of the energy. (May 1993)

15

0

8

A.4

The hop rate shall be configurable in the MAC but fixed within a given BSA. It does not have to adapt. (Jan. 1993)

20

1

1

A.5

The 2.4 GHz frequency hop physical layer draft specification shall require the MAC to maximize the use of each hop interval.

(Jan. 1993)

15

4

3

A.6

The 2.4 GHz frequency hop physical layer draft specification shall have the maximum hop rate restriction removed.(May 1993)

21

1

3

A.7

The MAC will tell the PHY when to hop (Jan. 1993)

13

5

0

A.8

The FHSS PHY group accepts IBM's proposed hopping sequences, in document 93/60, for 802.11 compatible FHSS WLAN's (Jan. 1994)

16

0

2

A.9

The 2.4 GHz frequency hop physical layer draft specification shall require transmit power level control above a TBD level of transmit power. (Jan. 1993)

15

6

1

A.10

The threshold level referenced in Motion 4 shall equal 100 mW. (Jan. 1993)

11

9

0

A.11

The 2.4 GHz frequency hop physical layer draft specification shall be able to transmit at least a TBD level of power to conform to the standard. (Jan. 1993)

21

1

0

A.12

The level referenced in Motion 6 shall be 1.0 mW. (Jan. 1993)

15

6

0

A.13

When power control is required, then the number of bits provided to specify transmit power shall be 2 bits. (Jan. 1993)

9

9

0

A.14

The PHY shall not fragment frames/packets supplied by the MAC. (Jan. 1993)

16

3

2

A.15

The 2.4 GHz frequency hop physical layer draft specification shall have a channel data rate of 1.0 Mbps. (May 1993)

15

0

8

A.16

The 2.4 GHz frequency hop physical layer draft specification shall have a transmitter center frequency accuracy of ±25 ppm. (NOTE: This may be revised downward in the future). (May 1993)

26

0

6

A.17

The 2.4 GHz frequency hop physical layer draft specification shall receive the switching time requirement (Tx-Rx) from the MAC. (May 1993)

10

6

-

A.18

All 802.11 FHSS PHY shall be capable of operating using GFSK with BT=0.5 and a minimum deviation of 160 kHz with a data rate of 1.0 Mbps. Modulation techniques for higher data rates are for further study by 802.11 PHY committee. A means for negotiating a switch to higher data rate from GFSK is also for further study.

(July 1993)

39

5

5

A.19

Adopted the Jim McDonald proposal (Doc: 93/209) with language changed from dB to Watts, for both ramp up and ramp down, and modulation during the ramp is unspecified by 802.11, but must be specified by the manufacturer.

(November 1993)

13

0

0

A.20

The training sequence will be 80 bits in length and consist of a "01" pattern.

NOTE:  We did not vote on the unique word, length field length, PHY signaling field length or depth of protection on the length field. Peter Chadwick gave a summary of 8 bits PHY signaling, 16 bits packet length and 8 bits protection (I'm not sure I understand this?) and nobody objected. (November 1993)

9

2

3

A.21

The PHY subcommittee of IEEE 802.11 will continue to work on the international standardization process.  The first objective will be the release of a first-draft US standard by November 1994.

(Jan. 1994)

29

4

5

A.22

The DS, FHSS and HSFHSS subgroups shall prepare draft standards and template documents and their chairs shall present them to the full PHY. (Jan. 1994)

23

2

10

A.23

We shall remove from 93/161 all reference to the subject matter of line 16 of 93/83r2 (fall back data rates below 1.0 Mbps)

(Jan. 1994)

12

0

5

A.24

We shall remove from 93/161 all reference to lines 17 and 17a of 93/83r2 (baseband clock jitter & clock accuracy).

(Jan. 1994)

4

1

2

A.25

In-band spurious emissions shall be

-55 dBc. (Jan. 1994)

9

0

5