IEEE 802.11

Wireless Access Method and Physical Layer Specification

Title:		IR PHY Template

Authors:	Adriano J. C. Moreira, Cipriano Lomba, Rui Aguiar, Rui T. Valadas,

A. M. de Oliveira Duarte

Integrated Broadband Communications Group

Dept. of Electronics and Telecommunications

University of Aveiro					Tel: +351 34 381937

3800 AVEIRO					Fax: +351 34 381941

PORTUGAL						Email: adriano@ci.ua.pt

General Specifications

| Parameter | Value | Comments |
| --- | --- | --- |
| Data Rate | 1 and 2 Mbps |  |
| Wavelength | 850 - 950 nm |  |
| Modulation Method | PPM | 16-PPM for 1 Mbps 4-PPM for 2 Mbps |
| Number of Channels | 1 |  |
| Propagation Mode | Diffuse | Non aimed transceivers |

Transmitter Specifications

| Parameter | Value | Comments |
| --- | --- | --- |
| Output Peak Power | 2.0 W ± 20% | @900 nm |
| Pulse Format |  | See fig. 3 doc:94/96 |
| · tw (pulse width) | 250 ns ± 10 ns | 1 and 2 Mbps |
| · tr (rise time) | 40 ns (max) | 10 to 90% |
| · tf (fall time) | 40 ns (max) | 10 to 90% |
| · tj (jitter) | 10 ns (max) | absolute deviation |
| Emitter Radiation Pattern | TBD |  |

Receiver Specifications

| Parameter | Value | Comments |
| --- | --- | --- |
| Sensitivity (BER=10-9) | -47 dBm/cm2 (1 Mbps) | -10 dBm/cm2 ambient |
| (max) | -41 dBm/cm2 (2 Mbps) | light |
| Minimum Dynamic Range | 30 dB | On the irradiance at the receiver detector |
| Minimum Field of View | 150° | At the physical limit |
| Frame Error Rate (FER) | £ 4 ´ 10-5 | MAC frame = 512 octets |
| Carrier Sense out to MAC | £ 12 µs | After the preamble start |
| TX-RX turnaround time | TBD |  |
| RX-TX turnaround time | TBD |  |
| IR silence to Carrier Sense deassert | 16 µs | In case of EFD failure |

Frame Specifications I

| Parameter | Value | Comments |
| --- | --- | --- |
| Preamble | 57 - 73 time slots | See "Frame specs. II" |
| Start of Frame Delimiter | 4 time slots | See "Frame specs. II" |
| Data Rate Field | 3 time slots | See "Frame specs. II" |
| DC Level Adjustment | 32 time slots | See "Frame specs. II" |
| MAC Frame | TBD | Integer number of octets |
| End of Frame Delimiter | 16 time slots | See "Frame specs. II" |

Frame Specifications II

| Parameter | Format | Comments |
| --- | --- | --- |
| Preamble Format | 010101...0101010 | 1 and 2 Mbps |
| SFD Format | 1001 | 1 and 2 Mbps |
| DC Level Adjustment | 2 ´ symbol '8': | 1 Mbps / 16-PPM |
| Field Format | 8 ´ symbol '2': | 2 Mbps / 4-PPM |
| Data Rate Field Format | 000 | 1 Mbps |
|  | 001 | 2 Mbps |
|  | other formats: | TBD |
| EFD Format | 0000011011011011 | 1 and 2 Mbps |

PPM Mapping Table

| 4-PPM | 16-PPM | PPM Symbol |
| --- | --- | --- |
| 00 | 0000 | 1 - 0000000000000001 |
| 01 | 0001 | 2 - 0000000000000010 |
| 10 | 0010 | 3 - 0000000000000100 |
| 11 | 0011 | 4 - 0000000000001000 |
|  | 0100 | 5 - 0000000000010000 |
|  | 0101 | 6 - 0000000000100000 |
|  | 0110 | 7 - 0000000001000000 |
|  | 0111 | 8 - 0000000010000000 |
|  | 1000 | 9 - 0000000100000000 |
|  | 1001 | 10 - 0000001000000000 |
|  | 1010 | 11 - 0000010000000000 |
|  | 1011 | 12 - 0000100000000000 |
|  | 1100 | 13 - 0001000000000000 |
|  | 1101 | 14 - 0010000000000000 |
|  | 1110 | 15 - 0100000000000000 |
|  | 1111 | 16 - 1000000000000000 |