IEEE 802.11

Wireless Access Method and Physical Layer Specifications

Title:	Review of Distributed Time Bounded Services

Symbionics

St John's Innovation Park

Cowley Road

Cambridge, UK

CB4 4WS

Presented by:	Tim Phipps

Tel:	+44 223 421025

Fax:	+44 223 421031

Email:	tgcp@symbionics.co.uk

Abstract:	This paper reviews the way in which IEEE 802.11 will provide the ability to perform time critical data transfer. The Point Coordination Function method and the Distributed Priority method are considered.

Introduction

The addition of an optional Point Coordination Function (PCF) to the basic MAC currently defines how IEEE 802.11 (reference [1]) will provide support for an extra class of data transfer, as well as ordinary asynchronous data transfer. In the draft standard, this single additional class of data has been classified as "Time Bounded". There is an alternative way to provide different classes of data transfer, which is to use priority, this technique could be called "Distributed Time Bounded Services".

Claimed Advantages of the PCF

The Point Coordination Function supports data transfer with a MAC layer connection. The Contention Free Period (CFP) in a super-frame being used in a manner similar to Time Division Multiple Access (TDMA). This gives a guarantee that data will be transferred with a large delay, but with a low variance.

The advantage of a low variance is somewhat unclear, realistically a MAC user wants a high probability that the transfer delay of a given time bounded data packet will not exceed a certain maximum value. This effect could just as well be achieved by having a low average transfer delay, with a larger variance.

N.B. It is perhaps also worth stating that since the PCF was modified to allow for variable length data packets to be transferred (reference [2]), the variance in transfer delay experienced by data packets nearer to the end of the contention free period will be an accumulation of the variance in the length of the data packets that have already been transferred, this will not be zero (reference [3]).

A connection may guarantee access to the medium in a given "time-slot", but it cannot guarantee any level of service between two users. One reason for this is the unreliable (datagram) nature of all MAC data transfers, another reason is the inherent mobility of wireless systems, which can cause a connection to be lost if a user wanders out-of-range. Therefore, whatever scheme we choose for wireless, time bounded data transfer, we must accept that we will be dealing with a probabilistic schemes; the MAC that we specify will never be able to offer a true guarantee of service.

Problems with PCF

Medium access rules require that PCFs do not overlap. The PCF requires unique access to the channel and so necessarily operates in a contention free period without needing any contention resolution. This means that the medium cannot be shared by two PCFs.

One possible solution to this problem would be to time-synchronise all access points. This is not feasible, because super-frame periods may vary from one access point to another and because we would have to synchronise access points that are not in communication with each other, if they served the same area.

Another possible solution is for all access points to be separated, either physically (by distance) or by using different channels. This does not solve the problem, but it avoids trouble if we can keep a to an implementation with a large number of channels and a low density of users. (The last of these conditions sounds particularly unattractive).

The problem of separating overlapping PCFs is hardest to avoid in frequency hopping systems where neighbouring access points will normally be able to operate successfully, it will appear as if they have unique control of the medium, but this assumption will break down, giving unintentional contentions whenever the PCFs occasionally hop onto the same channel.

I also note that there has not been any method specified for (1) Requesting a connection, (2) For "tearing down" a connection, (3) For deciding when a connection can no longer be maintained and (4) For "hand-over" of an existing connection between access points. These are the sort of problems that make the specification of wireless telephony standards a difficult task, a task that we would do well to avoid if we possibly can.

It seems to be unwise to adopt an approach which tries to merge random access and time reserved access methods into one standard.

Distributed Time Bounded Services Method

The "Distributed" method for time bounded data proposes a simple extension to the existing asynchronous data transfer service.

µ §

Figure 1: Channel Access Method

It was demonstrated in reference [4] that priority is an acceptable way to provide time bounded data services. It has also been shown, in reference [5], that the baseline medium access method can support different medium access priorities. Simulations of prioritised medium access show that access delays are less than 20ms, a value which is less than that which would be achieved with the use of the PCF with a typical super-frame period.

All packets are treated as being "time bounded", though of course some (asynchronous data packets) have very lax transfer delay requirements. Packets are queued according to their quality of service requirements, that is those with the shortest required transfer delay will be transmitted first. By using "Queuing Priority" we have given correct order of transmission to packets at this station, however we must make sure that packets from different stations get sent in the correct order. To do this we use different "Channel Access Priorities", choosing a priority that is appropriate for the packet being transmitted. This gives a correct queuing between packets at all neighbouring stations, providing a distributed, virtual queue.

µ §

Figure 2: Prioritised Transmit Queue

The distributed method for time bounded services will send queued packets as soon as the medium is available, because of this they should arrive ahead of the time that they are actually needed by an application that wants isochronous data. If this is the case then it is not difficult for the creator of the application to re-time received data packets to fit the precise timing requirement.

µ §

Figure 3: Re-timing Received Data

Advantages of Distributed Method

The greatest advantage of the distributed method is that is does not rely on any specific placement of equipment, any number of stations and access points can share the same channel, each supporting the full range of services without failure of the medium access control.

The use of two medium access priorities (or possibly more than two) gives a better way to provide a "Quality of Service" (QoS) for any given packet. This allows us to specify a QoS that does not have to be a single type, as with the old "Time-Bounded" data, but can take a range of values. I believe that this flexibility will fit well with the wide variety of applications that could be supported when higher rate physical layers are used with this MAC. In the PCF model, we have only the option of two qualities of service, firstly asynchronous data and secondly data transfer service that has use of a reserved, regular "time-slot". This method of supporting "Time Bounded" data does not, serve all types of time dependant data equally well, though it is obviously well suited to processes that produce a single isochronous output stream, such as speech codecs.

Conclusion

I have given a short review of the arguments about how best to implement provision for a variety of qualities of service. I believe the "Distributed" method to be preferable to the "Point Coordination Function" method.

Therefore the following motion is suggested:

Move that the principle of Distributed Time Bounded Services be adopted into the standard, and that the Point Coordination Function be removed.

References

[1] "Draft Standard IEEE 802.11 Wireless LAN", Doc IEEE P802.11-93/20b0.

[2] Wim Diepstraten - NCR, Greg Ennis - Symbol Technologies, Phil Belanger - Xircom, "Distributed Foundation Wireless Medium Access Control", Doc IEEE P802.11-93/190 Nov. 1993.

[3] Chandos Rypinski - LACE, "Evaluation of the DFWMAC", Doc IEEE P802.11-94-12 Jan. 1994.

[4] Philip Rakity, Larry Taylor - Apple Computer, “Distributed Time Bound Services”, Doc RES10SAG93/62 Dec. 1993.

[5] Wim Diepstraten - AT&T GIS, “Priority in CSMA/CA to support distributed Time-Bounded Services”, Doc IEEE P802.11-94/58 Mar. 1994.