IEEE 802.11

Wireless Access Method and Physical Specification

Title:	Motion to Adopt FH PHY Packet Formatting

to Minimize Data DC Offset

Date:	May 9, 1994

Author:	Dean Kawaguchi

Symbol Technologies, Inc.

1101 S. Winchester Blvd., Suite B-110

San Jose, CA.  95128

Telephone:  (408)369-2629

FAX:  (408)446-4630

Motion

I move that the FH PHY sub-group adopts the FH packet formatting method presented in submission 94/069 with 32-bit block sizes and stuffing/inversion beginning with the first block following the PLCP header.

Details of Algorithm

Figure 1.  Stuffing/Inversion Packet Formatting Illustration

Stuffing Algorithm:  If stuff bit = 1 = next block is inverted; 0 = not inverted

Input parameter: number_of_MPDU_bytes;		/* can be even or odd */

number_of_blocks_in_packet = truncate{(number_of_MPDU_bytes + 3) / 2 };

/* no padding is necessary for number of bytes not multiple of 4 */

Nhdr = 32;						/* Number of bits in header */

Read in header {b(1),...,b(Nhdr)};			/* b(1) is first bit in */

accum = 2 [Sum{b(1),...,b(Nhdr)}] - (Nhdr);		/* calculate bias in header */

Transmit {b(1),...,b(Nhdr)};				/* no stuffing on header */

For n = 1 to number_of_blocks_in_packet

{

b(0) = 0;						/* b(0) is the stuff bit */

N = min(4, # of bytes remaining) * 8;		/* N= block size in bits */

Read in next block {b(1),...,b(N)};			/* b(n) = 0, 1 */

bias_next_block = 2 [Sum{b(0),...,b(N)}] - (N+1);	/* calculate bias with b(0) */

If {[accum * bias_next_block > 0] then		/* if accum and bias of next							block has the same sign, then invert block */

{

Invert {b(0),...,b(N)};

bias_next_block = - bias_next_block;

}

accum = accum + bias_next_block;

transmit {b(0),...,b(N)};				/* b(0) is first bit out */

}

Figure 2.  Stuffing/Inversion Packet Formatting Pseudo-Code