IEEE 802.11

Wireless Access Method and Physical Layer Specification

Title:		IR PHY Modulation Template

Editors:	Peter Blomeyer, Andromeda, Germany

Kamilo Feher, UC Davis

General Specifications

Parameter

Value

Comments

Spectrum Occupancy

15 MHz to 30 MHz

Data Rate

4 and 10 Mb/s

Wavelength

850 - 950 nm

Modulation Method

FQPSK

Number of Channels

1 to 6

Propagation Mode

Diffuse

Non aimed transceivers

Transmitter Specifications

Parameter

Value

Comments

Output Peak Power

2.0 W ± 20%

@900 nm

Pulse Format

doc: IEEE P802.11-93/97

• for 0 0 in

-1 to +1 sin

sin trans at 1/2 of symb

• for 0 1 in

+1 to -1 sin

• for 1 1 in

+1 dc

• for 1 0 in

-1 dc

• tj (jitter)

10 ns (max)

absolute deviation

Emitter Radiation Pattern

TBD

Receiver Specifications

Parameter

Value

Comments

Sensitivity (BER=10-9)

TBD dBm/cm2 (4 Mb/s)

-10 dBm/cm2 ambient

(max)

TBD dBm/cm2 (10 Mb/s)

light

Minimum Dynamic Range

30 dB

On the irradiance at the receiver detector

Minimum Field of View

150_

At the physical limit

Frame Error Rate (FER)

£ 4 ¥ 10-5

MAC frame = 512 octets

Carrier Sense out to MAC

TBD

After the preamble start

TX-RX turnaround time

TBD

RX-TX turnaround time

TBD

IR silence to Carrier Sense deassert

TBD

In case of EFD failure

Frame Specifications I

Parameter

Value

Comments

Preamble

50 - 100 bits

Start of Frame Delimiter

4 bits

Data Rate Field

3 bits

MAC Frame

TBD

Integer number of octets

End of Frame Delimiter

16 bits

Frame Specifications II

Parameter

Format

Comments

Preamble Format

010101...0101010

4 and 10 Mb/s

SFD Format

1001

4 and 10 Mb/s

Data Rate Field Format

000

4 Mb/s

001

10 Mb/s

other formats:

TBD

EFD Format

0000011011011011

4 and 10 Mb/s

Modulation Encoding Table

Digit Pattern d0

is first in time

Phase Change

0 0

0°

0 1

90°

1 1

180°

1 0

270°