IEEE 802.11

Wireless Access Method and Physical Layer Specifications

Title:	Bit Ordering

Symbionics

St John's Innovation Park

Cowley Road

Cambridge, UK

CB4 4WS

Presented by:	Tim Phipps

Tel:	+44 223 421025

Fax:	+44 223 421031

Email:	tgcp@symbionics.co.uk

Abstract:	This paper proposes a tightening up of the definition of bit ordering within IEEE 802.11.

1.	Introduction

I have taken the bit order principle from the IEEE 802.3 standard (section 3.3), which I have copied almost word-for-word.

I have worked from reference document [1], section 4.1, Basic Frame Format.

2.	Proposed Additions to the IEEE 802.11 specification document

#### 2.1.	Order of Transmission

Each octet of the MAC frame, with the exception of the Cyclic Redundancy Check (CRC), is transmitted low-order bit first, which is bit 0.

#### 2.2.	Type field format

Sub Type	bits 0 to 3,
Compressed	bit 4,
Encrypted	bit 5,
Type		bits 6 and 7.

#### 2.3.	Control Field Format

From AP	bit 0,
To AP		bit 1,
Retry		bit 2,
Power Management Mode	bits 3 and 4,
Elements present	bit 5,
More		bit 6,
Poll		bit 7,
CF ACK	bit 8.

#### 2.4	NID Format

BSSID		bits 0 to 9,
ESSID		bits 10 to 22,
Infra-structure	bit 23.

#### 2.5.	Frame Format Diagram

µ §

3.	Conclusion

I would like the following motion to be made that:

"All the changes proposed above be made to the IEEE 802.11 draft specification".

4.	References

[1] "Draft Standard IEEE 802.11 Wireless LAN", Doc IEEE P802.11-93/20b0.