# Multi Bit Rate Support

By Pablo Brenner, LANNAIR Ltd.

# Principles of Operation

## All stations support the ESS DEFAULT_RATE

## Control and Multicast in DEFAULT_RATE

## Unidata Message in DEFAULT_RATE

# Principles of Operation (ctd...)

## New Field in RTS: Requested_Rate

## New Field in CTS: Granted_Rate

## If RTS' Requested_Rate is acceptable for the receiving station's PHY, then

## Granted_Rate = Requested_Rate

## else

## Granted_Rate = DEFAULT_RATE

## Data Messages at Granted_Rate

# Principles of Operation (ctd...)

## RTS/CTS "duration" field is MPDU "Data Length" field

### The same value on the CTS regardless of the choosen speed.

### The NAV calculation uses the Requested_Speed/Granted_Speed to calculate the duration

# Draft Standard Changes Description

## Change Duration Element to Data_Length

## Add a Rate Element (RTS/CTS frames)

## Change RTS/CTS frames picture

## Change MAC Data Service Pseudo-Code

## Change MAC Management Service Pseudo-Code

## Change NAV Setting Procedure

## Describe how a CTS frame is built

### No formal description in the draft

# Rx State Machine Changes

## R20a: Change NAV Calculation Description

## R30a: Change NAV Calculation Description

## R12: Add Requested_Rate reading

## R13: Add Granted_Rate reading

# Ctrl State Machine Changes

## State C1, Tx RTS:

### Add that Requested_Rate shall be set to MAX_BITRATE

## State C5, Tx CTS:

### Add Granted_Rate Calculation Description

## State C3, Tx Data:

### Add: Set TX_BIT_RATE in PHY_DATA.request to Granted_Rate

These parameters will be part of the PLME_PARLIST passed on the PHY_DATA.request and PHY_DATA.indicate Service Primitives.