# SPM:

A Simple Power Management Proposal

By Pablo Brenner, LANNAIR Ltd.

# What's Broken in the current Proposal?

## Frame Misordering:

Multicast Messages and Unicast Message (to non-power saving stations) may arrive in reverse order.

From 802.1D (MAC Bridges):

"The service provided by the MAC Sublayer does not permit the reordering of frames transmitted with a given user priority"

From 802.11-93/20b2, Paragraph 7.2.1.5. :

"All Broadcast/Multicast frames shall be buffered", and

"Frames destined to stations in the CAM or TAM mode shall be directly transmitted"

# Example of problem

µ §

# What's out of the Scope of 802.11?

## Extreme Low Power Stations

Not Necessarily Wireless LAN Problem.

Real solution is Application Specific ("proxy approach")

# What Could be done better?

## Not Power Management Algorithm for non Power efficient Protocol, but Power efficient Protocol !

## The SPM: Simple Power Management Proposal.

# SPM Proposal

## Use NAV for Turning off receiver.

## Paradoxically: Efficient for high traffic, inefficient when low traffic.

## Rough Numbers:

## Fully loaded (400 byte long packets), receiver off for 500 out of 3500 microsec. (87.5 %)

## For (1500 byte long): 96%

# Trivial Solution

## Let one station (Probably the AP on infrastructure mode) "sacrifice" and transmit "dummy RTS", when no traffic (no pending tx, larger IFS).

# SPM Solution

## New Control Packet: PSS - Power Saving Start.

Contains "Duration" field.

Indicates No traffic will be send to PS stations during the "duration" period.

Any node willing to transmit to non-power-saving stations is allowed to transmit.

Stations are allowed to transmit packets with To_AP bit set.

This mechanism allows Power Saving Stations to turn off the receiver up to 80% of the time.

# Motion 1:

## Accept the SPM proposal, and add it to the Draft Standard.

# Motion:

## Remove existing Power Saving Mechanism from the Draft Standard.