IEEE P802.11
Wireless Access Method and Physical Layer Specifications

Title:	How Long is Long Enough,  or 
A proposed change to Sections 4.1.8 in the draft IEEE p802.11-93/20b1

Author:	Barry A. Dobyns
Photonics Corporation
2940 North First Street
San Jose, CA 95134
408-955-7930
408-955-7940 fax
76527,266@compuserve.com   or   dobyns@acm.org

Abstract:	What's Wrong:
The editor's note in section 4.1.8 near the top of page 47 of IEEE p802.11-93/20b1 reads:
[Editor's note (GE): how large is the duration field?]

How To Fix It:
Supply a definition, along with proposed text, like the one in this document.

Motions:
Resolved, that the proposed text changes in 11-94/178 be incorporated into the draft standard IEEE p802.11-93/20b1 section 4.1.8 in it's next revision by the editors.

# Proposed Change

The proposed text is shown below, with change bars, in the affected section from the draft standard, IEEE p802.11-93/20b1.

### 4.1.8	Type-Dependent

Additional type-dependent fields shall be present, depending upon the frame type.  The following are defined:

Duration: Time in microseconds of frame's transmission. Two octets in length.

[Editor's note (GE): how large is the duration field?]

Data: variable length data field

# Rationale

A one octet field is too small, only allowing for a duration sufficient for an 32 octet frame (at one Mbps), and there is no apparent need for a field larger than two octets at this time, which will allow for 8K frames at 1Mbps and 16K frames at 2Mbps.