IEEE 802.11

Wireless Access Method and Physical Specification

Title:	Proposed Revision to PHY_SAP Service Primitives

Date:	November 7, 1994

Author:	Dean Kawaguchi, FH PHY co-editor, Symbol Technologies Inc.

Ed Geiger, FH PHY co-editor, Apple Computer Inc.

Tim Blaney, FH PHY co-editor, Xircom

Keith Furuya, FH PHY co-editor, Xircom

Paul Struhsaker, DS PHY editor, Aironet Wireless Communications, Inc.

Barry Dobyns, BB-IR PHY editor, Photonics

Introduction

The PHY editors jointly propose to modify the PHY_SAP service primitives.  The current set of classes defined in section 8 are insufficient to support all PHYs or are not clearly defined.  A complete set of classes is proposed below with additions and clarifications of the existing classes.

Proposed Revision

PHY_DATA.request(class, data)

class=Start_of_Data, data=TXVECTOR

Starts the transmit state machine.  TXVECTOR contains the packet length and 			other packet control information which are PHY dependent.

class=Data, data=DATA

One octet of MPDU data

class=End_of_Data, data=NULL

Indicates end of data to transmit.

class=End_of_Activity, data=NULL

Resets the PLCP CS/CCA assessment timers to the state appropriate for the end 			of a received packet.  This is generated by the MAC at the end of a NAV timer.  			This request is used by some PHY implementations which may synchronize 			antenna diversity switching with the contention backoff timing.  The use of this 			request in the PHY is optional.

PHY_DATA.indicate(class, data)

class=Start_of_Activity, data=NULL

CS/CCA channel busy

class=Start_of_Data, data=RXVECTOR

Start of the MPDU portion of a packet following a valid SFD and PLCP Header.  			RXVECTOR contains the packet length and other packet information including 			RSSI.  RXVECTOR is PHY dependent.

class=Data, data=DATA

class=End_of_Data, data=RXERROR

End of a received packet.  RXERROR contains the condition of the received 			packet with states including no_error, header_violation, format_violation, 				carrier_lost.

class=End_of_Activity, data=NULL

CS/CCA channel idle

PHY_DATA.confirm

After all PHY_DATA.request(Data, DATA), it indicates the PLCP is ready to receive 	another byte.  After the PHY_DATA.request(End_of_Data, NULL), it indicates that 	transmission is complete.