IEEE 802.11

Wireless Access Method and Physical Layer Specification

Title:		Measurements of the Power Spectral Density of a 16-PPM @1Mbps

Optical Signal

Authors:	Luis M. V. Moreira, Adriano J. C. Moreira, Rui T. Valadas, A. M. de Oliveira

Duarte

Integrated Broadband Communications Group

Dept. of Electronics and Telecommunications

University of Aveiro					Tel: +351 34 381937

3800 AVEIRO					Fax: +351 34 381941

PORTUGAL						Email: lmig@ci.ua.pt

Summary

In [1] a transmit spectrum mask was proposed that specifies a -20dB point @ 15 MHz.

In this document we present experimental results on the measurements of the Power Spectral Density (PSD) of the optical signal transmitted by a baseband infrared transmitter, using 16-PPM at 1 Mbps. The presented experimental results are in good agreement with previously presented theoretical results [1] and show that the spectrum bandwidth of practical transmitters is narrower than 15 MHz, but can be higher than 10 MHz.

1. Introduction

In [1], the theoretical PSD of the optical signal generated by a baseband infrared transmitter operating at 2 Mbps (4-PPM) was presented, and a Transmit Spectrum Mask for a conformant transmitter was proposed. In [1], it was proposed that the spectrum bandwidth of the transmitted signal should be narrower than 15 MHz.

In this document, experimental results for the PSD of the optical signal generated by a transmitter operating at 1 Mbps (16-PPM) are presented. Since the transmitted pulse shape is the same for both data rates (1 and 2 Mbps), the resulting PSD is quite similar.

2. Measurement Setup

The experimental results presented above were obtained using the setup depicted in figure 1. The time domain measurements were performed using a fast sampling digital storage oscilloscope. The frequency domain measurements were performed using a spectrum analyser. The transmitted optical signal was generated by a conformant transmitter, using a very simple drive circuit.

Figure 1. Measurement setup.

3. Results

Experimental measurements were performed for three different types of commercially available LEDs. Table 1 summarises the switching times and spectrum bandwidth resulting for each of the used LEDs. Figure 2 to 4 present the basic pulse shape and PSD for each of the considered LED.

| LED Reference | Trise [ns] | Tfall [ns] | -20 dB cut off Frequency [MHz] |
| --- | --- | --- | --- |
| DN305 | 16 | 30 | 11 |
| HE8807SL | 20 | 36 | 10.25 |
| HE8812SG | 12 | 24 | 11 |

Table 1. Major characteristics of the transmitted optical signal.

a)							b)

Figure 2 - Optical signal emitted by DN305 LED: a) pulse shape b) PSD.

a)							b)

Figure 3 - Optical signal emitted by HE8807SL LED: a) pulse shape b) PSD.

a)							b)

Figure 4 - Optical signal emitted by HE8812SG LED: a) pulse shape b) PSD.

4. Conclusions

As expected faster LEDs produce broader spectra. However the differences between them are quite negligible. All of the three cut off frequencies are less than 15 MHz, but higher than 10 MHz. These results are in agreement with the theoretical studies and show that the spectrum of a practical transmitter using commercially available components is conformant with the specification proposed in [1].

References

[1]	Adriano J. C. Moreira et al. - Univ. of Aveiro - Portugal, "Definition of the Infrared Baseband

Transmit Spectrum Mask", doc: P802.11-94/154