# Proposed Encoding

# for the

# Management Information

# in

# MAC Frames

| Dave Bagby Advanced Micro Devices One AMD Place Sunnyvale, CA 94088 | Bob O'Hara Advanced Micro Devices One AMD Place Sunnyvale, CA 94088 | Dave Roberts Advanced Micro Devices One AMD Place Sunnyvale, CA 94088 |
| --- | --- | --- |

Abstract:  This paper presents the changes to section 4 required to encode the information in management frames with a minimum of overhead while providing all of the required functionality.

Action:  Adopt the changes in this paper to replace the relevent portions of Section 4 of the current draft.

# 1	General

# 2	General Description

# 3	MAC Service Definition

# 4	Frame and MPDU Formats

## 5	MAC Frame Formats

## 6	Format of Individual Frame Types

## 7	Frame Exchange Sequences

## 8	Frame Body Element Format Definitions

As described in section 4.2, each management frame type contains a number of information elements.  These information elements may be of fixed or variable length, depending upon the content of the information element.  This section defines the format of each of the individual information elements.

### 9	Fixed Length Information Fields

Fixed length information fields shall occur in management frames in a predefined order.  The identity of the element is known from its fixed position in the various frames in which it is used.

#### 10	Beacon Interval

This field shall represent the number of milliseconds between Beacon generations. The length of this element shall be one octet.

#### 11	DTIM Count

This field shall indicate how many TIMs (including the TIM in the current frame, if any) will appear before the next DTIM.  A DTIM Count of 0 shall indicate that the current TIM is a DTIM. The length of this element shall be one octet.

#### 12	DTIM Period

This field shall indicate the number of TIM intervals between successive DTIMs.  If all TIMs are DTIMs, the DTIM Period field shall have value 1. The length of this element shall be one octet.

#### 13	Broadcast Indicator

This field shall indicate that a broadcast or multicast frame will be transmitted by the Access Point following the next DTIM (or after the current frame if this frame includes a DTIM). The length of this element shall be one octet.

#### 14	Station ID (SID)

This field shall be a value assigned by an AP during association representing the 16-bit Station ID of a station. The length of this element shall be two octets.

#### 15	Time Stamp

This field shall represent the value of the TSFTIMER of a frame's source.  The most significant bit, when set, shall indicate that the station is synchronized within its BSS. The length of this element shall be four octets.

#### 16	Weight

This field shall indicate the degree to which a station is synchronized within its BSS.  Larger values shall indicate a greater degree of synchronization. The length of this element shall be two octets.

#### 17	Request/Response Indicator

This field shall be a boolean indicator.  When the value of this field is true (1), the indication is for a response.  When the value of this field is false (0), the indication is for a request. The length of this element shall be one octet.

#### 18	Privacy Algorithm Number

This field shall indicate a single privacy algorithm as identified in 802.10xx.  The length of this element shall be two octets.

#### 19	Status Value

This field shall be a boolean value indicating the success or failure of an operation.  When this field is true (1), the indication is for success.  When this field is false (0), the indication is for failure. The length of this element shall be one octet.

#### 20	Error Indicator

This field shall indicate a reason code for operations that resulted in a Status Value of failure. The length of this element shall be one octet.

#### 21	Current AP Address

This field shall be the MAC address of the access point with which the station is currently associated. The length of this element shall be six octets.

#### 22	Transaction Sequence

This field shall indicate the current state of progress through a multi-step transaction. The length of this element shall be one octet.

#### 23	Authentication Algorithm Number

This field shall indicate a single authentication algorithm as identified in 802.10xx.  The length of this element shall be two octets.

#### 24 Regulatory Domain

This field shall identify a regulatory domain from the following list:

1.

The length of this element shall be one octet.

[Editor's note (GE): Need list of domains]

#### 25 Set

This field shall identify the particular set of hop patterns.  The length of this element shall be one octet.

#### 26 Pattern

This field shall identify the individual pattern within a set of hop patterns.  The length of this element shall be one octet.

#### 27 Index

This field shall select the channel index within a pattern.  The length of this element shall be one octet.

#### 28 Hop Timing

This field shall provide a time reference to the station regarding the current hop sequence.  The first field is a 32 bit time in microseconds from the beginning of the hop pattern to the time reference of the frame that includes this element.  The second field is a 32 bit time in microseconds for the length of a hop.  The length of this element shall be eight octets.

### 29	Variable Length Information Fields

Variable length information fields shall occur in management frames in a predefined order.  The identity of the element is known from its fixed order in the various frames in which it is used.

Variable length information elements shall have a common general format consisting of a Length field and a variable-length element-specific field.  The value of the Length field shall be the number of octets in the element, exclusive of the Length field itself.  The value of the octets specific to each information element are dependent upon the element contents.

µ §

Figure 4-xx, Variable Length Information Element Format

The set of possible variable length elements is defined below. For each element type, the size of element-specific field is given.

#### 30	Traffic Indication Map (TIM)

The TIM Field shall contain a variable number of block groups, with each block group consisting of a block identifier followed by 0 to 7 one-octet blocks.  Each bit within a block shall indicate whether a frame is currently buffered for a station with a particular Station ID.  There is a one-to-one  mapping between the bits in a virtual bit map and the station IDs. The virtual bit map is maintained within the access point; the actual transmitted TIM is a compressed representation of the virtual bit map. The size of the length field for this element shall be one octet. The element-specific field length shall be between one and eight octets.

Block Group: 	Consists of a Block Identifier followed by from 0 to 7 Blocks.

µ §

BI: Block Identifier (1 octet)

µ §

Bit N (N = 1..7)	0 = Nth block in this group is absent

1 = Nth block in this group is present

M: More	0 = This is the last block group

1 = Another block group follows

Block (8 bits)	Each bit corresponds to a specific station within the block.  If this block represents the Nth block within the virtual bit map, then Bit M within the block shall correspond to the station with Station ID equal to 8*(N-1) + M.

Bit = 1: There is a frame pending for this station

Bit = 0: There is no frame pending for this station.

#### 31	Channel Sync Information

This field shall contain the information necessary for a station to be able to remain synchronized with a particular BSS that is using a multi-channel PHY.  The size of the length field for this element shall be one octet. The element-specific field length shall be a variable number of octets.

#### 32	ESS ID

This field shall indicate the identity of the Extended Service Set. The size of the length field for this element shall be one octet. The element-specific field length shall be a variable number of octets.

#### 33	Supported Algorithm List

This field shall indicate the list of privacy and/or authentication algorithms supported by a station. The size of the length field for this element shall be one octet. The element-specific field length shall be a variable number of octets.

#### 34	Identity Challenge

This field shall be the bit string used to challenge the identity of a station during the authentication process. The size of the length field for this element shall be two octets. The element-specific field length shall be a variable number of octets.

#### 35	Challenge Response

This field shall be the bit string resulting from the response to the Identity Challenge during the authentication process. The size of the length field for this element shall be two octets. The element-specific field length shall be a variable number of octets.

#### 36	Challenge Result

This field shall be a bit string resulting from the processing of the Challenge Response during the authentication process. The size of the length field for this element shall be two octets. The element-specific field length shall be a variable number of octets.