IEEE P802.11

Wireless Access Method and Physical Layer Specification

Liberating The “More” Function

Michael Fischer
Digital Ocean, Inc.
4242–3 Medical Drive
San Antonio, TX  78229

Telephone:  +1–210–614–4096
Facsimile:   +1–210–614–8192
email:   mfischer@CHILD.com

Abstract

The “More” function in the 20B3 draft is unnecessarily constrained.  Furthermore, the encoding of the More function can be changed to permit other frame header improvements pertaining to the Last Fragment indicator and the size of the Dialog Token or Sequence Number.

Category

The proposed changes restore some functionality that has been unnecessarily restricted, and provide improvements/optimizations in some related areas.

Action

Adopt the changes in this proposal to replace the relevant portions of sections 4 and 7 of the current draft standard.

Overview

The use of the More FC-bit is unnecessarily restricted in draft 20b3, by only being allowed to be set by APs.  This precludes a class of potential benefits.  The More indication can be merged into the Power Management FC–bits with no loss of power management functionality.  This frees up an FC–bit so that the Last Fragment (F) bit in the Fragment Number field can be relocated to the FC–field.  This permits the use of a 12–bit Dialog Token or Sequence number and a 4–bit Fragment Number, without giving up the reserved FC–bit.

Removing the unnecessary restriction

The current draft (93/20b3) text restricts the More function to indicate that additional frames are queued at an AP.  Stations are explicitly required to send the More bit as zero (page 48, lines 16-19).  There is no functional benefit from this restriction, since stations with more than one queued frame for the same wireless recipient should be allowed (not required) to indicate the presence of such frames.

The benefits of this approach are relevant in goth power management and CF–Async utilization.

•	The sender of an ATIM can use this function to indicate the presence of additional queued frames to the same recipient.

•	A PCF can use this function as a basis to give a station with more queued frames additional transmission opportunities during a given CF-period that is not fully utilized.

Merging More into Power Management

We can merge the More function into the Power Management subfield in the FC field.  The basis of this approach is that More is only relevant when the successor power management mode is "active."  (Why communicate about whether an about-to-be-sleeping station has additional queued frames?  If that traffic mattered the station would not be going to sleep!)  At the same time, the distinction between "continuously active mode" and "temporarily active mode" is only of external relevance at Association/Reassociation:  If a station is capable of using any form of power saving, the AP must be prepared to buffer traffic for that station and must monitor the Power Management FC-bits in frames from that station.  This is something that, like CF-awareness, can be communicated as part of the association process.  Once communicated, the AP does not need to distinguish between CAM and TAM.  Therefore, the two Power Management bits can encode the next mode as "active," "active with more traffic queued," "power save polling," and "power save non-polling."  The "active" code would be used by stations without more traffic as well as by stations that do not report the presence of more traffic.

•	The dynamic indication is “Active” vs. “PSxx”

•	The transition to Active from PSP or PSNP is the call for the AP to send buffered frames.

•	The transition from Active to PSP or PSNP is the signal for the AP to resume buffering frames.

•	The duration of “temporary” activity is controlled by the station

Relocating Last Fragment Bit into FC-field

This previous change allows the Last Fragment bit to be relocated into the FC field without giving up the reserved bit.  This makes architectural sense, as the Last  Fragment indication is logically as much a frame control indicator as the Retry bit.  This is also the only 1–bit subfield in the MAC header that is not located in the FC–field.

Once the Last Fragment bit is relocated, the Sequence Number field and the Fragment Number field can be merged into a single 2–octet field.  The advantage of doing so is to reallocate the bits to yield a 12–bit Dialog Token or Sequence Number and a 4–bit Fragment Number.

•	Increasing the size of the uniqueness information from 256 to 4096 values, whether as a Dialog Token or Sequence Number, reduces the probability that a new frame will be rejected as a duplicate.

•	Reducing the size of the fragment number from 128 to 16 states is not a problem, since 16 fragments are more than enough to convey a 2304–octet MSDU under existing guidelines that PHYs shall support 256–octet MPDUs.

Draft Standard Text Changes

Section 4

4.1.1. General Frame Format

< text unchanged, new drawing >

Figure 4–1:  MAC Frame Format

4.1.2.1. Frame Control Field

The Frame Control field shall consist of the following subfields:  Protocol Version, Type, Subtype, To AP, From AP, Last (Fragment), Power Management, and Elements Present.  The remaining subfields in the Frame Control field are reserved.  All reserved bits and fields shall be sent as “0.”  Reserved bits and fields shall be ignored on reception.

Figure 4–2:  Frame Control Field

4.1.2.1.5.  Last Fragment

This one bit field shall indicate that this frame is the last fragment of a fragmented MSDU or the sole fragment of an unfragmented MSDU.

4.1.2.1.7. Power Management

The two bit field shall indicate the power management state and buffered traffic state of the station.  The value sent shall indicate the state in which the station will be after the completion of the transmission of the frame.  The values for this field are given in table 4–2.

Value

Description

00

Active Mode (CAM or TAM), with More buffered frames

01

PSP - Power Save, Polling

10

PSNP - Power Save, No Polling

11

Active Mode (CAM or TAM), without More buffered frames

Table 4-2: Power Management Values

Section 7

7.2.1.5. Access Point Operation

< all text unchanged except for the final numbered subitem, which is replaced as follows>

8.	Whenever an AP is informed that a station changes from PSP or PSNP mode to
	Active mode, the the AP shall record that station as being in TAM mode and the AP 
	shall send buffered frames (if any exist) to that station immediately.

9.	If more than one buffered frame exists for a station receiving buffered frames from
	the AP, the AP shall set the Power Management bits in the Frame Control field to
	“11” to indicate that more buffered frames exist on all but the last buffered frame.

7.2.1.9. Station Operating in the TAM Mode

1.	An AP shall transmit all buffered frames for a TAM station immediately after the
	station has indicated a mode change to “Active” to the AP.

< remainder of section unchanged >

7.2.2.3. Frame Transmission

< all text unchanged, except for the addition of a new subitem >

6.	If more than one buffered frame is available for the same destination, transmitting
	station shall set the Power Management bits in the Frame Control field to “11” to
	indicate that more buffered frames exist on all but the last buffered frame.