IEEE 802.11

Wireless Access Method and Physical Layer Specifications

Title:	MAC Specification Issues

Symbionics Developments Ltd

St John's Innovation Park

Cowley Road

Cambridge, UK

CB4 4WS

Presented by:	Tim Phipps

Tel:	+44 1223 421025

Fax:	+44 1223 421031

Email:	tgcp@symbionics.co.uk

Abstract:	This paper aims to tidy-up the operation of infrastructure synchronisation,  probe response and power-management in the MAC specification.

# 1. 	Weight and Synchronisation

## Introduction

There are many references in 20b3 to a Weight field (e.g. section 7.1.3.2 Active Scanning), though there is no specification of what it is or how to use it. This is obviously a part of the specification that needs some attention.

The weight field was originally intended was to allow convergence of distributed timing in an ad-hoc network. There is no need to make any provision for complex timing synchronisation rules in an infrastructure network, since timing can be derived uniquely from the access point. So the following discussion applies solely on the case of ad-hoc network timing synchronisation.

The following events in the life-cycle of an ad-hoc network need to be considered:

·	Forming a new ad-hoc network

·	Joining an ad-hoc network

·	Steady-state ad-hoc network

·	Merging two unsynchronised ad-hoc networks

## To Form a new ad-hoc Network

This is already covered by section 7.1.3.6 of 20b3. "StartNet causes the station to establish the initial timing reference for the ad-hoc network. The station shall set its MACMGT_Sync_State variable to 'Synced' and begin sending beacons at the expected intervals". Note that, in this case, beacons are not sent before the station is synchronised.

## To Join an ad-hoc Network

Section 7.1.4 says that an unsynchronised station should adopt the time-stamp of any synchronised station. I propose that this rule be retained. There are also specified rules for what to do with a received time-stamp that is not synchronised. This rule applies only when unsynchronised timing information can be transmitted in a beacon, or a probe response. The unsynchronised state should only be transitory, until this station has acquired good timing synchronisation, therefore there is no benefit in allowing an unsynchronised station to transmit beacons or probe responses.  I propose that this is no longer permitted.

## Steady State ad-hoc network

But how can we ensure that timing synchroniation is stable in an ad-hoc network? I propose that once a station is in synchronisation it should no longer just adopt all received time-stamps, but should adjust its own timer to split the difference between the received time and the expected time.

New local time = (old local time + received time-stamp) / 2.

Since all stations within an ad-hoc network generate beacons the algorithm will stick to an average view of current time (and rate of passage of time), levelling drift caused by asynchronous clocks. This method is known to be very stable.

## Merging two unsynchronised ad-hoc networks

It is obviously possible that an ad-hoc network may be temporarily split into two completely independent physical groups, we should ensure that they can remerge into one. The method described above will ensure that re-merging can occur, though it will result in a temporary disruption to the orderly working of both sub-networks, whilst the timing information converges to a new value. This problem, of changing the timing reference of stations cannot be avoided if we want to implement a merge function, using a more complex timing synchronisation algorithm will not help.

## Motion:

"That no station be allow to transmit a time-stamp field (i.e. a beacon or probe response MPDU) before it is synchronised. That unsynchronised stations may adopt a received time-stamp to become synchronised. That timing synchronisation be maintained in an ad-hoc network, by adjusting a synchronised station timing reference to be the average of itself and any time-stamp received from another station in that network".

# 2. 	Estimate of Power Saving State

It is not be easy for a station to keep track of the power saving status of other stations in a network. This may causes no problems in an infrastructure network but introduces problems in an ad-hoc network. In the latter case, it is very likely that a station may be asleep when another station changes it power saving state. A station may think a destination is awake when it is,  in fact,  asleep.  This may result in a packet being sent to a sleeping station,  resent and eventually  discarded when the maximum number of retries is exceeded.

One way around this problematical behaviour is to allow a station change its estimate of the power saving state of any given node. Therefore, if a station thinks that a destination station should be continuously awake, or that it should be temporarily awake; but there is never any acknowledgement from that station, then the sender is not forced to keep retrying the transmission until it fails,  but could change to transmission using the PSNP rules.

## Motion:

"That the estimated power saving state of another station is based on the power management information transmitted by that station and additional information available locally (such as history of failed transmission attempts) which may be deemed relevant".

# 3. 	Probe Responder

If a probe response is to be issued, then in any network there should be at least one node that is awake at any given time. (This requirement seems to have slipped out of the specification). In an ad-hoc network of sleeping nodes, I make the recommendation that the station that sent the beacon is the one that stays awake for that beacon interval.

In practise,  at least one,  and possible a small number greater than one station will consider its last beacon transmission attempt to be successful.   There will be no region within the BSS which is not within range of one of these stations.

My recommendation is that this station/ these stations consider themselves to be Probe Responders.    They stay awake until the next beacon period,  at which time the identity of the probe responders may change.

There are two changes to the behaviour of stations with respect to probe responses.   Firstly,   only probe responders respond to probes.   Other stations which happen to be awake to observe a probe request ignore it.   Secondly,   probe responders do not need to cancel pending probe responses on observing some other station successfully respond to a probe request.    This is justified because each probing station is likely to have only one probe responder within range.    Even if this is not  the case,   it will receive a small number of probe responses,  which will cause it no problems.

## Motion:

"That in any network there shall be at least one node that is awake at any given time, to respond to probes.  Stations do not need to cancel a pending probe response transmission on observing a successful probe response from some other station".

# 4.	References

[1] "Draft Standard IEEE 802.11 Wireless LAN", Doc IEEE P802.11-93/20b3.