IEEE P802.11

Wireless Access Method and Physical Layer Specifications

Title:  Addition/Changes to DOC  P802.11-94/068 R5 to Support Multiple Rates

Author: Pablo Brenner

LANNAIR Ltd.

Atidim Technological Park

Devorah Haneviah St - Neve Sharet

Tel Aviv 61131 - ISRAEL

Voice: +972-3-6458375

FAX:  +972-3-5447146

EMail: pablo@lannet.com

Abstract:

This Submission describes a set of additions to Document 068/R5 in order to support Multiple Rate PHYs coexistance.

Introduction

This document describes the changes required on the current Frequency Hopping draft standard to support Mutliple Rates PHYs Coexistance.

In addition some new assignments for Reserved bits on the PLCP header are presented.

PLCP Header (paragraph 9.3.2.2.)

Currently the PLCP Header assigns 10 bits for PLCP Length and 6 Reserved bits for the PLCP Signaling Field.

This submission recommends the following changes:

·	Assign 2 bits (bits 4-5) of the PLCP signaling field for rate with the following assignment

00

1 MBit/s

01

2 MBit/s

10

3 MBit/s

11

4 MBit/s

·	Keep 4 bits reserved:

CCA State Machine (9.3.3.2.1)

Add text indicating that the Countdown Timer is initialized to the Duration of the packet (i.e. the Length divided by the Rate).

Add Rate  to the TX_Vector and RX_Vector (9.2.2, and 9.2.3)

Shouldn't Length be added to the TX_Vector too?

Add PMD_SAP Sublayer to Sublayer Primitives (9.5.4.2)

Add:

Primitive

Request

Indicate

Confirm

Response

PMD_RATE

X

X

Add Rate to the PMD_SAP Services Primitives Parameters (9.5.4.3)

Add:

RATE

PMD_RATE.indicate

PMD_RATE..request

00 for 1MBit/s

01 for 2MBit/s

10 for 3 Mbit/s

11 for 4 Mbit/s