Title:	DC Offset Control Algorithm for HS FH PHY

Date:	November 1994

Author:	Naftali Chayat

LANNAIR inc.

Atidim Industrial Park, Bldg. 3

Tel Aviv, 61131, ISRAEL

Tel: +972-3-645-9135

Fax: +972-3-648-7146

email: naftali@lannet.com

The DC control algorithm- the Higher Speed version

The DC offset control formatting presented in submission 94/068r5 can be readily modified to Higher-Speed FH PHY. The method of accomplishing so is to add one inversion control binary symbol for each 32 symbols (8 octets at 2 Mbit/sec). In this manner the timing structure, the amount of overhead and the octet-orientation of the method remain.

Some complication arises from the fact that the deviation factors of the regular and the higher speed portion of the packet are related by h4=0.45h2 and so exact computation of the accumulated DC would involve high resolution arithmetic. We propose to simplify the DC offset computation by assuming the simplified ratio h4=0.5h2 and summing integer numbers, mapped in the following way: -2 or +2 for a binary symbol, and -3,-1,1,3 for a quaternary symbol. We believe the effect of this rounded scaling to be negligible.

The modified stuffing algorithm pseudo-code is presented in figures 1 and 2. As seen, it extends seamlessly the current algorithm to higher rates.

MOTION:

Include the proposed modified algorithm into the Draft Standard.

Stuffing Algorithm:  If stuff bit = 1 = next block is inverted; 0 = not inverted

Input parameters: number_of_MPDU_bytes, rate;	/* rate is 1 or 2*/

number_of_symbols=number_of_MPDU_bytes *8/rate

number_of_blocks_in_packet = truncate{(number_of_symbols+ 31) / 32 };

Nhdr = 32;						/* Number of bits in header */

Read in header {b(1),...,b(Nhdr)};				/* b(1) is first bit in */

accum = Sum{map(b(1)),...,map(b(Nhdr))};			/* calculate bias in header */

/* map( ) is -2,2 for binary and -3,-1,1,3 for quaternary symbols */

Transmit {b(1),...,b(Nhdr)};					/* no stuffing on header */

For n = 1 to number_of_blocks_in_packet

{

b(0) = 0;						/* b(0) is the binary stuff bit */

N = min(32, # of symbols remaining);			/* N= block size in symbols */

Read in next block {b(1),...,b(N)};			/* b(n) = 0,1 or 0,1,2,3  */

bias_next_block = Sum{map(b(0)),...,map(b(N))};

/* calculate bias with b(0) */

If {[accum * bias_next_block > 0] then			/* if accum and bias of next

block has the same sign, then invert block */

{

Invert {b(0),...,b(N)};

bias_next_block = - bias_next_block;

}

accum = accum + bias_next_block;

transmit {b(0),...,b(N)};				/* b(0) is first bit out */

}

Figure 1.  Stuffing/Inversion Packet Formatting Pseudo-Code

Stuffing Algorithm:  If stuff bit = 1 = next block is inverted; 0 = not inverted

Input parameters: number_of_MPDU_bytes, rate;	/* rate is 1 or 2*/

Nhdr = 32;						/* Number of bits in header */

Read in header {b(1),...,b(Nhdr)};				/* b(1) is first bit in */

Receive {b(1),...,b(Nhdr)};					/* no stuffing on header */

accum = Sum{map(b(1)),...,map(b(Nhdr))};			/* calculate bias in header */

/* map( ) is -2,2 for binary and -3,-1,1,3 for quaternary symbols */

Decode PLCP header to determine  number_of_MPDU_bytes and rate

number_of_symbols=number_of_MPDU_bytes *8/rate

number_of_blocks_in_packet = truncate{(number_of_symbols+ 31) / 32 };

For n = 1 to number_of_blocks_in_packet

{

b(0) = 0;						/* b(0) is the binary stuff bit */

N = min(32, # of symbols remaining);	/* N= block size in symbols */

Receive {b(0),...,b(N)};				/* b(0) is first bit received */

accum = accum + Sum{map(b(0)),...,map(b(N))};

/* compute sum for violation check*/

If {[b(0)=1] then					/* if block inverted, invert back */

{

Invert {b(0),...,b(N)};

}

Pass block {b(1),...,b(N)} to MAC;			/* b(n) = 0,1 or 0,1,2,3  */

}

Figure 2.  Stuffing/Inversion Packet Reception Pseudo-Code