# A Compromise Proposal

# for Revisions to the

# MAC Frame Formats

# to

# Support Wireless Distribution Systems

| Dave Bagby Advanced Micro Devices One AMD Place Sunnyvale, CA 94088 | Bob O'Hara Advanced Micro Devices One AMD Place Sunnyvale, CA 94088 | Dave Roberts Advanced Micro Devices One AMD Place Sunnyvale, CA 94088 |
| --- | --- | --- |
| Wim Deipstraten AT&T GIS Nieuwegein The Netherlands | Michael Fischer Digital Ocean 4242-3 Medical Drive San Antonio, TX 78229 |  |

Abstract:  This paper presents a compromise proposal for the changes to section 4 required to enable the use of 802.11 LANs as a distribution system for other 802.11 LANs sharing the same channel.

Action:  Adopt the changes in this paper to replace the relevent portions of Section 4 of the current draft.

# 1	General

# 2	General Description

# 3	MAC Service Definition

# 4	Frame and MPDU Formats

## 5	MAC Frame Formats

### 6	General Frame Format

The MAC frame format comprises a set of fields that shall occur in a fixed order in all frames.  Some fields may be absent from some frame types.

A frame is an ordered octet string.  The order of transmission of the octets of a frame shall be from left to right in the figures shown in this section

Figure 4-1 depicts the general MAC frame format and field order.  The format of the MAC header for each of the frame types is defined subsequently.  Following sections define each of the fields of the MAC header.

µ §

Figure 4-1: MAC Frame Format

### 7	Frame Fields

#### 8	Frame Control Field

The Frame Control field shall consist of the following subfields: Protocol Version, Type, Subtype, To DS, From DS, More, Retry, Power Management and Elements Present.  The remaining subfields in the Frame Control field are reserved.  All reserved bits and fields shall be sent as '0'.  Reserved bits and fields shall be ignored on reception.

µ §

Figure 4-2: Frame Control Field

##### 9	Protocol Version

##### 10	Type and Subtype

##### 11	To DS

This one bit field shall indicate that the frame is destined for the distribution system in an infrastructure network.  This bit shall be transmitted as a one only if the frame Type = Data and the the frame is entering the distribution system.  It shall be transmitted as a zero, otherwise.

##### 12	From DS

This one bit field shall indicate that the frame is being distributed from the distribution system in an infrastructure network.  This bit shall be transmitted as a one only if the frame Type = Data and the frame is exiting the distribution system.  It shall be transmitted as a zero, otherwise.

The possible To/From DS bit combinations are given in the following table:

|  | To/From DS Values | Description |
| --- | --- | --- |
|  | To DS = False From = DS False. | A Frame direct from one Station to another Station. |
|  | To DS = True From DS = False | Data Frame entering the DS. |
|  | To DS = False From DS = True | Data Frame exiting the DS. |
|  | To DS = True From DS = True | WDS frame being distributed From one AP to another AP. |

Table 4-xx: To / From DS Combinations

##### 13	More

##### 14	Retry

##### 15	Power Management

##### 16	Elements Present

#### 17		Address Fields

There are four address fields in the MAC frame format.  These fields are used variously to indicate the BSSID, source address, destination address, transmitting station address and receiving station address.  The usage of the four address fields in each frame type will be indicated by the abbreviations BSSID, DA, SA, RA, TA indicating BSS Identifier, Destination Address, Source Address, Receiver Address and Transmitter Address, respectively.  Some frames may omit some of the address fields.

##### 18	Address Representation

Each Address field shall contain a 48-bit address as defined in section 5.2 of IEEE Std 802-1990.

##### 19	Address Designation

##### 20	BSS Identifier

##### 21	Destination Address

##### 22	Source Address

##### 23	Receiver Address

The Receiver Address (RA) field identifies the IEEE MAC address of the intended recipient of a wireless transmission.  The Individual/Group bit shall always be transmitted as a zero.

##### 24	Tranmsitter Address

The Transmitter Address (TA) field identifies the IEEE MAC address of the transmitter of a wireless transmission.  The Individual/Group bit shall always be transmitted as a zero.

#### 25	Sequence Number

#### 26	Fragment Number

#### 27	Duration

#### 28	Frame Body

#### 29	CRC

## 30	Format of Individual Frame Types

### 31	Control Frames

In the following descriptions, "immediately previous" frame means a frame, the reception of which concluded within the prior SIFS interval.

#### 32	RTS Frame Format

The frame format for an RTS frame is shown in Figure 4-xx.

µ §

Figure 4-xx: RTS Frame

The DA of this frame shall be the address of the immediate station receiving the frame.  In an infrastructure LAN, the DA shall be the address of the AP with which the station is associated.  In an ad hoc LAN, the DA shall be the destination of the subsequent data or management frame.  The SA shall be the address of the station transmitting the frame.

#### 33	CTS Frame Format

The frame format for an CTS frame is shown in Figure 4-xx.

µ §

Figure 4-xx: CTS Frame

The DA of the CTS frame shall be taken from the SA field of the RTS frame to which the CTS is a response.

#### 34	ACK Frame Format

The frame format for the ACK frame is shown in Figure 4-xx.

µ §

Figure 4-xx: ACK Frame

The DA of the ACK frame shall be the address contained in the Address 2 field of the immediately previous Data or Management frame.

#### 35	Poll Frame Format

The frame format for the Poll frame is shown in Figure 4-xx.

µ §

Figure 4-xx: Poll Frame

The BSSID shall be the address of the AP to which the Poll is directed.  The SA shall be the address of the station transmitting the frame.  The SID shall be the value assigned by the AP in the Associate Response frame.

### 36	 DATA Frame Format

The frame format for a Data frame is independent of subtype and is shown in Figure 4-xx.

µ §

Figure 4-xx: DATA Frame

The contents of the Address fields of the Data frame shall be dependent upon the values of the To DS and From DS bits and are defined in table 4-xx, below.  Where the content of a field is shown as N/A, the field shall be omitted.

|  | To DS | From DS | Address 1 | Address 2 | Address 3 | Address 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | DA | SA | BSSID | N/A |
|  | 0 | 1 | DA | BSSID | SA | N/A |
|  | 1 | 0 | BSSID | SA | DA | N/A |
|  | 1 | 1 | RA | TA | DA | SA |

Table 4-xx: Address Field Contents

A station shall use the contents of Address 1 to perform address matching for receive decisions.  The

The DA shall be the destination of the frame, i.e. the destination of the MSDU.

The SA shall be the address of the station transmitting the frame.

The RA shall be the address of the access point in the wireless distribution system that is the next immediate intended recipient of the frame.

The TA shall be the address of the access point in the wireless distribution system that is transmitting the frame.

The BSSID of the Data frame shall be determined as follows:

1)	If the station is an AP or is a associated with an AP, the BSS Identifier shall be the address of the AP.

2)	If the station is a member of an ad hoc LAN, the BSS Identifier shall be the the BSS ID of the ad hoc LAN.

The Frame Body shall be the MSDU or a fragment thereof.

### 37	Management Frames

The frame format for a Management frame is independent of subtype and is shown in Figure 4-xx.

µ §

Figure 4-xx: Management Frame Format

The address fields for Management frames shall not vary by frame subtype.

The BSS Identifier of the Management frame shall be determined as follows:

1)	If the station is an AP or is a associated with an AP, the BSS Identifier shall be the address of the AP.

2)	If the station is a member of an ad hoc LAN, the BSS Identifier shall be the the BSS ID of the ad hoc LAN.

The DA shall be the destination of the frame.

The SA shall be the address of the station transmitting the frame.

The Frame Body shall be the information elements described for each managment frame subtype, below.