IEEE 802.11

Wireless Access Method and Physical Layer Specifications

Title:	Outcome of MIB Ad-hoc Meeting

Symbionics Developments Ltd

St John's Innovation Park

Cowley Road

Cambridge, UK

CB4 4WS

Presented by:	Tim Phipps

Tel:	+44 1223 421025

Fax:	+44 1223 421031

Email:	tgcp@symbionics.co.uk

##### 1	oSMT

SMT MANAGED OBJECT CLASS

DERIVED FROM "ISO/IEC 10165-2":top;

CHARACTERIZED BY

pSMT_base	PACKAGE

BEHAVIOUR

bSMT_base BEHAVIOUR

DEFINED AS "The SMT object class provides the necessary support at the station to manage the processes in the station such that the station may work cooperatively as a part of an 802.11 network.";

ATTRIBUTES

aStation_ID	GET,

aActing_as_AP_Status	GET,

aCurrent_AP_MAC_Address	GET,

aCurrent_BSS_ID	GET,

aCurrent_ESS_ID	GET-REPLACE,

aKnown_APs	GET, (1 to N deep)

aAuthentication_Algorithms	GET,

aPrivacy_Algorithms	GET,

aSelected_Authentication_Algorithm	GET,

aSelected_Privacy_Algorithm	GET,

aAuthentication_Handshake_State	GET,

aPrivacy_Handshake_State	GET,

aAuthentication_State	GET,

aPrivacy_State	GET,

aMin_Authentication_Required	GET,

aMin_Privacy_Required	GET,

aAssociated_State	GET,

aBeacon_Period	GET-REPLACE,

aPower_Mgt_State	GET-REPLACE,

aPower_Mgt_Capability	GET;

ATTRIBUTE GROUPS

agStation_Config_grp,

agAuthentication_grp,

agPrivacy_grp;

ACTIONS

acSMT_init,

acSMT_reset;

NOTIFICATIONS

nAssociate,

nDissociate;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) smt(0) };

#### 2	MAC Object Class

##### 3	oMAC

MAC MANAGED OBJECT CLASS

DERIVED FROM "ISO/IEC 10165-2":top;

CHARACTERIZED BY

pMAC_base	PACKAGE

BEHAVIOUR

bMAC_base BEHAVIOUR

DEFINED AS "The MAC object class provides the necessary support for the access control, generation and verification of frame check sequences, and proper delivery of valid data to upper layers.";

ATTRIBUTES

aMAC_Address	GET,

aGroup_Addresses	GET-REPLACE,

aPromiscuous_Status	GET,

aTransmitted_Frame_Count	GET-REPLACE,

aOctets_Transmitted_Count	GET-REPLACE,

aMulticast_Transmitted_Frame_Count	GET-REPLACE,

aBroadcast_Frame_Count	GET-REPLACE,

aFailed_Count	GET-REPLACE,

aFrame_Exchange_Error_CountaCollision_Count	GET-REPLACE,

aSingle_Frame_Exchange_Error_CountaSingle_Collision_Count	GET-REPLACE,

aMultiple_Frame_Exchange_Error_CountaMultiple_Collision_Count	GET-REPLACE,

aReceived_Frame_Count	GET-REPLACE,

aOctets_Received_Count	GET-REPLACE,

aMulticast_Received_Frame_Count	GET-REPLACE,

aBroadcast_Received_Frame_Count	GET-REPLACE,

aReceived_Frame_Error_Count	GET-REPLACE,

aFCS_Error_Count	GET-REPLACE,

aLength_Mismatch_Count	GET-REPLACE,

aFrame_Too_Long_Count	GET-REPLACE,

aFrame_With_Protocol_Error_Count	GET-REPLACE,

aMAC_Enable_Status	GET,

aTransmit_Enable_Status	GET,

aNAV	GET,

aNAV_max	GET,

aRate_Factor	GET,

aHandshake_Overhead	GET,

aSIFS	GET, Note: May need modification to  include 94/124

aPIFS	GET, Note: May need modification to  include 94/124

aDIFS	GET, Note: May need modification to  include 94/124

aRTS_Threshold	GET-REPLACE,

aTotal_Accumulated_Backoff_Time	GET-REPLACE,

aSlot_Time	GET,

CW_max	GET-REPLACE,

aCW_min	GET-REPLACE,

aCTS_Time	GET,

aACK_Time	GET,

aRTS_Retry_max	GET-REPLACE,

aDATA_Retry_max	GET-REPLACE,

aMax_Frame_Length	GET,

aFragmentation_Threshold	GET-REPLACE,

aManufacturer_ID	GET,

aProduct_ID	GET;

ATTRIBUTE GROUPS

agCapabilities_grp,

agConfig_grp,

agAddress_grp,

agOperation_grp,

agCounters_grp,

agFrame_Error_Condition_grp,

agStatus_grp;

ACTIONS

acMAC_init,

acMAC_reset;

NOTIFICATIONS

nFrame_Error_Rate_Exceeded;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) mac(1) };