Preamble Modification for Improved Selection Diversity

Ronald Mahany

Norand Corporation

550 Second Street SE

Cedar Rapids, IA  52401

Telephone: (319)369-3552

Facsimile: (319)369-3299

email: mahanyrl@norand.com

Abstract

The Frequency Hopping and Direct Sequence PHY  groups have selected preamble characteristics  with the expectation that some 802.11 compliant implementations will use  some form of antenna diversity, or some other form of multipath compensation. In both cases, preambles with uniform data content were chosen -- a dotting pattern  of 1-0-1-0... for FH,  and a  marking pattern  of 1-1-1-1... (at the input to the scrambler) for DS. While these uniform patterns meet the basic requirements of receiver synchronization and channel busy detection,  they may not be the most appropriate  for implementing  antenna diversity. Insertion of a  time stamp within the preamble is proposed to improve performance of antenna selection diversity algorithms, with potential application to other multipath compensation or channel optimization alternatives as well.

Introduction

A number of papers related to preamble characteristics,  synchronization words,  clear channel assessment,  have been submitted before the FH and DS PHY ad hoc groups. The subject of antenna diversity was discussed in some detail in a recent contribution -- IEEE P802.11-94/70, by Dean Kawaguchi.   Document 94/70 provides excellent groundwork for further work in this area. While it was presented within the FH group, its content is not unique to frequency hopping, and concepts presented within should be considered relevant to the DS PHY as well.

The implications of antenna diversity may not have been fully considered when the respective RF PHY groups selected preambles for their PHY headers. This paper is intended to highlight this issue, and offer an alternative that provides more flexibility  in addressing this  problem.

Assumptions:

The discussion of diversity within this paper is based upon a particular view of  diversity's place within the standard.  The  perspective of the author can be summarized  in the following list of assumptions.

1.	While most contributors who have discussed methods of channel optimization within the working groups tend to focus on  two-antenna selection diversity, the standard should not preclude alternatives  such as  n antenna selection diversity, adaptive equalization,  maximum  ratio combining, or RAKE approaches for  direct sequence. All of these techniques generally benefit  from training  during the preamble period. The preamble should provide a reasonable amount of flexibility to implement alternative approaches to address the wireless channel.

2.	Use of diversity, or any of the alternative  methods,  is not a requirement of either 802.11 RF PHY.   The purpose of including preamble support  for them  is to provide means for manufacturers whose target customers can benefit from (and will pay for) them to use them, not to dictate  use, or mandate algorithms.

3.	Because of the broad interest in selection diversity,  it is a good litmus test for evaluating whether we have selected satisfactory preambles.

4.	CCA and diversity approaches are interrelated, and cognizance of diversity issues is important in specifying CCA detect time, Pdet and Pfa requirements for preamble reception. As a fairness issue, CCA conformance  testing   must not be specified  in a way that favors either diversity, or  single antenna implementations.

5.	Diversity selection on a per transmission basis is appropriate given the propagation characteristics of the channel, and the boundless variety of potential interference scenarios that need to be considered with like and unlike devices operating in proximity, in an ISM environment. Diversity must be viewed in both noise-limited and interference-limited contexts.

6.	Throughput using the foundation MAC protocol, like other LBT protocols,  can be expected to be sensitive to increases in transmission overhead. In both FH and DS, PHY overhead  exceeds MAC payload for some message types. It is desirable to enable channel compensation techniques within the PHYs, yet it is undesirable to extend preamble length boundlessly to do so. (Note that in the case of DS,  PHY overhead is now approaching 200 symbols per packet.)

7.	Given the usual implementation constraints, of size, cost, power consumption, etc., anything that can be done at the transmitter to make the receiver's job easier, should be done.

Background:

Document  94/70 details two basic classes of two-antenna  selection diversity algorithms. The best antenna algorithm requires that both  antennas be scanned  and assessed for signal quality during the preamble period,  with the receiver returning to the best antenna with sufficient time remaining in the preamble to attain synchronization prior to the unique word.  The satisfactory antenna class of algorithm stops scanning as soon an antenna with acceptable signal quality is found.

For asynchronous traffic, the foundation MAC does not provide prior knowledge of frame  boundaries, therefore antenna sampling algorithms must consider the worst case timing relationships between start of transmission, and antenna sampling.  This results in the preamble period being divided into five sampling periods for the best antenna algorithm, and three sampling  periods for the satisfactory antenna  algorithm. These sampling periods, less switching and synchronization times for the receiver,  specify an observation window in which to make an assessment of the presence, and/or quality of the received signal.

Frequency Hopping

Discussion of   94/70 -- Noise Limited Channel

The previously approved 80 bit preamble length for  FH places severe constraints on the best antenna class of algorithms, allowing (after receiver settling time) only  eight to ten symbols   of  preamble for clock synchronization and channel  assessment. The antenna selection settling time indicated in 94/70 is realistic,  and clock lock  within the allotted number of bits within the Pdet, Pfa  limits indicated  is  possible under the conditions simulated.  However, it is not realistic to expect to make signal quality comparisons between antennas  when such a short sampling period is available. This is clearly the case in figure 3 of 94/70, where the simulation shows  Pfa in noise of 5% to 10%. The  Pdet statistics for the ten-symbol observation  window are pretty good, but may  substantially degrade  if the clock detector bandwidth is widened to contend with multipath induced jitter. More significantly, the ten-symbol observation window  Pdet results in  94/70 were based on a noise-limited channel providing a 1 x 10-5 BER.  Pdet  at lower channel BER is also important, if the ability to detect clock is to be used as a means of comparing two or more antennas. With short observation windows, measurable Pdet will be attained at poor channel bit error rates that will not sustain payload transfer. Detection of clock alone cannot be used with certainty to determine the best antenna.

The best antenna algorithms in figures 5 and 6 of 94/70 implicitly  recognize the detection uncertainty issue, and  attack it by requiring clock lock to initially ascertain presence of desired signal, then using energy in band (RSSI) comparisons to determine the best antenna. With the preamble length and content already selected, there aren't many degrees of freedom. Given these constraints, the illustrated algorithms do as well as any in solving the problem for the noise limited case.

Discussion of  94/70--Interference  Limited Channel

The weaknesses of energy-based antenna selection  have been discussed previously within the PHY ad hoc groups. Even within the noise-limited case, there are issues with using RSSI for determining best antenna. The difference in signal strength between a 1 x 10-3 channel and a
1 x 10-5 channel is two to three  dB. This places a burden on RSSI measurement accuracy.

Document 94/70 did not address detection performance in the presence of interference. The C/I limited case is similar to the C/N limited case--short observation windows introduce more detection uncertainty. Again, within short windows, clock will be detected in interference limited channels with unsatisfactory  BER.  In this case,  RSSI based decisions  can result in the wrong antenna being selected, since using RSSI optimizes for C+I, not C/I or pre-detection SNR. (The FH PHY chairman's immortal example of locking to the strongest microwave comes to mind.)

It is clearly  desirable to use the longest observation window possible for channel characterization. This provides better detection certainty, and potentially allows qualitative,  SNR  based antenna selection algorithms. Low modulation index FM is known to have substantial intrinsic jitter that must be averaged out. Interference and multipath induced jitter complicate this problem, again requiring longer observation windows to make comparative  measurements. Personal experience with clock recovery  for discriminator demodulated  GMSK, at similar data rate and BT to that of the draft standard GFSK  modulation,   indicated that about  20 bits of transitions are needed  to  obtain a useful indication of recovered SNR.   This  appears to be  reasonably consistent with the low uncertainty in the 20msec window  clock detector simulation statistics in figure 3 of 94/070.

Truncated Preambles

Antenna selection algorithms also need to function  in cases where some of the beginning of the preamble is obscured on both antennas  by interference, or the end of another transmission. The five-window, best antenna scheme is extremely vulnerable in this case, and will frequently miss transmissions that single antenna systems will receive.

Direct Sequence Issues

While 94/70 was not presented in the context of the direct sequence PHY, there are significant parallels between FH and DS  selection diversity   requirements.  The best antenna algorithm in 94/70 is  similar to that discussed in a DS context in 93/37.   DS diversity algorithms must  also contend with  noise limited, interference limited, and truncated preamble cases.

For DS,  acquisition and clock detection are a combined process, and  Pdet, Pfa  are replaced by Pacq,, P false acq . Like their FH counterparts, DS receivers will acquire and track signal under low BER conditions unsuitable for data transfer. In general, it can be expected that DS receivers  will take longer than FH receivers  to acquire signal  each time an antenna is selected, but a shorter period to evaluate the received signal after acquisition. The approved 128 bit preamble for DS is about 50% longer than that for FH.  The difference in preamble length between the two PHYs is  necessary to account for the incremental requirements  of DS synchronization (AGC training, code acquisition, scrambler flushing, etc.), and may   already reflect a perceived need  within  the DS community  for longer observation periods for acceptable  preamble detection  performance

The best antenna algorithm of 94/70 could be applied directly in the DS case, using  pre-despreader signal strength measurement to compare antennas, if a quick measurement is desired.

Alternatively,  post-despreader signal quality estimation  may be performed readily in a DS receiver, e.g.,  as the measure of correlated to uncorrelated energy at the depreader output,  integrated over several adjacent bit intervals.  Some of the  implementation issues differ, but the choice of best antenna vs. satisfactory antenna approaches for DS is the same conceptually, as it is for FH.

An Alternative  Preamble Concept

Preamble length, asynchronicity between antenna scanning and start of transmission, and antenna switching time dictate the  duration of preamble observation windows. As stated in the assumptions, it is undesirable from an overall system performance standpoint to make the preamble any longer. It is also not possible to reduce  antenna switching time below some minimum number of symbol times for  either DS or FH PHYs.

The one area where is some recourse, is in reduction of the timing ambiguity between antenna scanning and start of message.  With the currently specified uniform preambles in both PHYs, a  receiver, synchronizing to preamble for the first time, has no knowledge of whether it has synchronized to the first bits of the preamble, the center, or the end. The worst possible timing relationship between start of transmission and antenna sampling must be assumed in designing  switching  algorithms. For example, in the best case, where antenna scanning and start of transmission happen to coincide,  the best antenna algorithm requires only  two or  three observation  periods to make a decision, not five. Each period could be longer increasing confidence in the information obtained.  The assumption of worst case timing can  be eliminated by the giving the receiver diversity algorithm more information. This information can easily be inserted into the preamble as a timestamp, as shown in figure 1.  The timestamp information can be used to facilitate a hybrid best or satisfactory (BOS) diversity  algorithm.

A proposed timestamped preamble for frequency hopping is shown in figure 2. It consists of 32 bits of  alternating 1-0 followed by 48 bits of  0-1.  The time stamp is centered  at the boundary between bits 32 and 33.  It can be easily distinguished  by simple pattern recognition circuits keyed to 1001 (distance 2, from preamble) or 010010 (distance 3 from preamble). This preamble represents minimal departure from the current FH PHY dotting pattern preamble.

The proposed timestamp for direct sequence is shown in figure 3. DS differs from FH, in that clock can  be extracted directly from the despreader, eliminating the need for transitions in the data for timing recovery. The preamble below would be scrambled to provide spectral whitening, per the DS PHY draft.

¯ Timestamp

| m bits preamble | T | n bits preamble | 16 bits Unique Word |
| --- | --- | --- | --- |

¬¾¾¾¾¾¾¾ 80 bits FH or 128 Bits DS¾¾¾¾¾¾¾¾®

Figure 1:  Timestamp Concept

| m bits Preamble A | T | n bits Preamble B | Frame Sync |  |
| --- | --- | --- | --- | --- |
| 1010101........010101010 | 010101010101................01010101010101 |  |  | Unique Word |
| 32 bits | 48 bits |  |  | 16 bits |

Figure 2: Proposed FH Preamble with Timestamp

| m bits Preamble A | T | Preamble B | Frame Sync |  |
| --- | --- | --- | --- | --- |
| 00000000........00000000 | 111111111111................11111111111111 |  |  | Unique Word |
| 48 bits | 80 bits |  |  | 16 bits |

Figure 3: Proposed DS Preamble with Timestamp

An SNR based Best or Satisfactory (BOS)  Diversity Algorithm Example

The flow chart in figure 4 illustrates a possible application of the timestamp in a hybrid algorithm  improve  selection diversity performance. For comparative purposes, it is modelled in the context of frequency hopping.   This is an adaptation of figure  4 of 94/70.  Assumed system parameters, illustrated in Figure 5, were also selected for consistency with 94/70. Also, the flow chart assumes a circuit implementation with continuously available clock lock and SNR outputs. In other words,  once clock lock and SNR outputs are valid, their status is updated continuously until the detection circuits are reinitialized.

| Parameter: | Value | Notes |
| --- | --- | --- |
| Antenna Switching Time: | 8 msec |  |
| SYNC (Clock) Detect Observation Interval: | 10 msec |  |
| Minimum Length SNR Observation Interval until SNR valid: | 20 msec | (Concurrent with SYNC detect) |
| RSSI Threshold | Arbitrary |  |

Figure 5 Assumed System Switching and Detection Parameters

µ §

Operation

This algorithm is presented  in the context of the frequency hopping PHY in accordance with 94/70. Like those in 94/70 it  is intended as an example of an appropriate algorithm, not as a requirement for conformance with the standard.

Operational Description

A) The receiver will switch antennas at intervals = Switching time + SYNC detect until a signal exceeding the RSSI threshold is detected. Using the system parameters indicated, switching occurs at increments of  18ms per antenna.

B) Presence of energy and absence of clock may trigger an opposite PHY test if required by the standard.

C) A receiver detecting clock for the duration of the Sync observation period will cease switching, and monitor the selected antenna  until either:

1) SNR on the selected antenna  is determined to be unacceptable. The antenna is rejected, and the receiver switches to the other antenna.

2) SNR is determined to be acceptable. The receiver continues to monitor the selected antenna. The framing word is received and normal  frame reception is initiated. In this case the preamble was initially  detected too late in the preamble period to detect the timestamp, so the current (satisfactory) antenna was selected.

Note that in cases (1) and (2), the timestamp may have occurred within the SNR monitoring interval. Clock recovery and SNR assessment circuitry must be tolerant of this change in preamble content, so that it is not construed as clock loss or poor SNR. A timestamp transition occurring prior to [SNR valid]  is ignored.

3) SNR is acceptable and a timestamp is received after the SNR monitoring interval. The algorithm   switches  antennas and evaluates recovered SNR through the second antenna. If the first antenna provides better SNR, the receiver will return to the first antenna. Using  six-symbol timestamp pattern recognition, the second antenna is selected at symbol 36 in the preamble. Allotting 8 msec for antenna switching, the second antenna is evaluated over symbols 44 through 64, and if necessary, the receiver returns to the first antenna by symbol 72.  This  leaves a short period (8 symbols) for bit Sync prior to the framing word, if a switch back to the first antenna is required.

Best Antenna Selection--Timing  Analysis:

There are three  cases to consider when evaluating the value of  the preamble timestamp in a two antenna, selection diversity implementation.  The first, with no signal present at either antenna, is trivial.  The remaining two are more interesting, and have been evaluated using a sliding window timing analysis to test the potential benefit of the timestamp. These cases are:

·	Signal is discernible  on one antenna, but not the other.

In this case,  the algorithm selects the satisfactory antenna, as expected. For the specified timing parameters,  a sufficient portion of  Preamble A  to execute the best antenna branch of the flow chart  for 36% of the timing relationships between sampling and start of transmission. After detecting the timestamp and switching antennas, failure to detect clock on the other antenna causes a rapid return to the original antenna prior to the framing word. For the other 64% of cases,  the satisfactory branch of the algorithm is used.

·	Signal is discernible on both antennas.

The results of the  sliding window analysis for 8ms switching times, and various observation windows is summarized in the table of  figure: 6.

| Preamble A: Minimum SNR Interval | Preamble A: Average SNR Interval | Preamble B: Observation Interval | Best Antenna Branch Execution Percentage |
| --- | --- | --- | --- |
| 20 msec | 26 msec | 20 msec | 72% |
| 18 msec | 25 msec | 18 msec | 83% |
| 16 msec | 24 msec | 16 msec | 94% |

Figure 6    Best Antenna Selection Vs.  Observation Time

Note that these results, do not include the effects of less than ideal clock Pdet, Pfa statistics, or imperfect SNR comparisons.  Shortening the SNR observation window  improves the likelihood that the best antenna branch will be executed, but also increases the possibility of making a wrong decision based on imperfect detection. Even with  pessimistic assumptions about detection performance, the  presence of  the timestamp allows best antenna selection  a significant majority of the time, while still allowing the longer observation windows of the satisfactory class of algorithm in 94/70.

Other Implications:

Specific Issues with DS

With appropriate modifications, the sample algorithm can be applied to direct sequence. DS systems are capable of SYNC recovery and signal quality estimation independently of data content. This has been recognized in proposing  a SPACE/MARK transition as a timestamp boundary for DS. In FH, the timestamp boundary must be observed to ascertain the time remaining until the unique word.  In DS, it is possible to determine that the timestamp has not yet occurred based on data content.  This allows modifications to the sample algorithm to switch antennas prior to the timestamp boundary if signal quality determination  is completed early. For the 128 symbol DS preamble,  a timestamp boundary at the seventh octet is appropriate. Based on certain assumptions of DS acquisition performance, it is conceivable that use of a timestamp and BOS algorithm  could allow reduction in the DS preamble length  of four octets, should the DS PHY group feel that sufficient system performance gains would result.

Unique Word Recognition -- Distance Requirements

The preamble with timestamp maintains required distances from the FH and DS unique words. For the case of FH, the alternative unique word (frame Sync)  of 94/49 is a slightly better alternative for the particular time stamp illustrated in figure 2A. The unique word in 93/209r provides a worst case distance of 5 with the preamble containing the time stamp, the proposal in 94/49 has a worst case distance of 6.

CCA -- Preamble Detection

A CCA requirement based on this type of algorithm might require CCA on preamble within two switching periods, and two clock detect windows, for a total of 36 ms for FH. Alternatively, 18 ms would provide detection on one antenna. This approach can be applied to DS as well with the appropriate timing.

Bit Errors During  the Preamble

Bit errors in the preamble may create a false timestamp. Using the suggested combination of 6 symbols  for FH (preamble bits: 30,31,32,33,34,35 in figure 2) requires 3 bit errors for this to occur.  This is not catastrophic, and  has the desired effect of instructing the receiver to switch antennas to avoid channel errors. The DS preamble timestamp can make additional use of the fact that  the preamble content changes completely  at the timestamp boundary. Errors in preamble content during either the Preamble A or Preamble B period can be used as an indication to switch antennas.

Truncated Preamble

Interference obscuring the early part of the preamble on both antennas can be expected to occur occasionally, due to classical hidden node problems, and burst interference sources.  Note that this is potentially catastrophic for  the best antenna algorithm in 94/70.  The sample algorithm illustrated here, and the satisfactory antenna algorithm in 94/70 will usually catch the frame Sync word with better than 50% of the beginning of the preamble obliterated.

Impact on Single Antenna, Non Diversity Implementations

The presence of the timestamp should not be disruptive to single antenna implementations. If a particular clock synchronizer cannot cope with the apparent clock slip introduced in the timestamp, there is  substantial opportunity for clock synchronization after the time stamp is received.

Implementation of the Sample Algorithms of 94/70, or Others

Presence of a timestamp as illustrated in figure 2A does not preclude implementation of the algorithm types described in 94/70, or a variety of other algorithms. The only constraint on these implementations is that clock detection  must be tolerant of the change in preamble content at the timestamp boundary. It is anticipated that Pfa for the noise only case will increase slightly, since a larger number of noise patterns will correlate with allowed preamble content.

Impact on Alternative Channel Compensation Methods

Techniques such as equalization and maximum ratio combining will generally rely on the transition content and timing  in the FH preamble. The proposed timestamp results in the loss of a single transition, which should not be significant.  For DS,  RAKE and equalization approaches can be implemented using  correlation only, without consideration of data content.  The availability of time information within the preamble can be used to advantage in both PHYs in implementing these alternatives.

Alternative Preambles For FH

Figures 7A and 7B provide alternatives where additional timestamp information can be extracted from preamble content. This provides FH with the same advantage that the DS preamble content provides--preambles A and B can be differentiated before or after the timestamp -- at the expense of fewer transitions for clock recovery. Because of their substantial departure from the presently approved FH preamble, the impact of switching to these preamble types requires more assessment before they are proposed for  serious consideration.

| m bits Preamble A | T | n bits Preamble B | Frame Sync |  |
| --- | --- | --- | --- | --- |
| 1010........................1010 | 110011001100....................110011001100 |  |  | Unique Word |
| 32 bits | 48 bits |  |  | 16 bits |

Figure 7A

| m bits Preamble A | T | n bits Preamble B | Frame Sync |  |
| --- | --- | --- | --- | --- |
| 0110......................0110 | 010101010101................01010101010101 |  |  | Unique Word |
| 32 bits | 48 bits |  |  | 16 bits |

Figure 7B

Conclusions

Addition of a timestamp to the respective PHY preambles can be generally beneficial to selection diversity algorithms. The illustrated example shows one method of applying the timestamp. Inclusion of the timestamp does not preclude, and may benefit other diversity algorithms or alternative channel compensation approaches.

References:

IEEE Doc  P802.11- 93/37,  "Proposal for 2 Mbit/s DSSS PHY", Jan Boer

IEEE Doc  P802.11-93/209r  "Preamble Proposal for 2.4 GHz Frequency Hop Standard (With Revisions)", Jim McDonald

IEEE Doc  P802.11-94/49  "Unique Word Proposal for the 802.11 PHY Preamble", Naftali Chayat

IEEE Doc  P802.11-94/70 "Carrier Sense with Diversity Model for FH PHY", Dean Kawaguchi