Comments lost in editing

| 9.1.2 | N. Silberman | T | managed object for Jabber control is missing. |  |
| --- | --- | --- | --- | --- |
| 10.2.X | N. Silberman | T | PHY_TX_Power parameter is needed for stations with optional power control | Power control is specified in the standard |
| Table 10-8 | N. Silberman | T | Antenna state should allow for more than two antennae for diversity. | Implementations of APs that utilize sectored antennae could utilize a higher number of antennae. |
| Table 10-8 | N. Silberman | T | Add parameter to indicate Hopping sequence acquisition / Lock | There is no indication that the unit is synchronized with the hopping sequence. |
| 10.5.5.5 | N. Silberman | T | Allow for more than two antennae in the ANTENNA_State primitive | See comment on table 10-8 |
| 10.5.5.9 | N. Silberman | E | It is not clear whether there is difference between standby and low power mode. If there is difference it should be explained explicitly. |  |
| 10.6.6 | N. Silberman | E | Make the paragr. Starting with “The minimum hop rate, a NOTE: |  |
| 10.6.9 | N. Silberman | T | Specify if the zero crossing error is on a bit by bit basis, average or peak. | Otherwise will be left to “flexible interpretations. |
| 10.6.10 | N. Silberman | T | Specify data rate tolerance. E.g 1 Mbps +/- 5% | If tolerance too loose it might be a n interoperability issue. |
| 10.6.18 | N. Silberman | E | The definition of the frequency offset is not clear. Define DF=+xMhz from the center of the desired channel. | Definiton too confuding |
| 10.6.18 | N. Silberman | T | -40 dB at 2 MHz away is too high of a signal. I propose at least -55dB | For a receiver that’s on a different sequence but within 1 meter from the transmitting station: it will be able to receive only signals above - 60 dBm. This represents a 20 dB desensitization from the minimum sensitivity specified (-80 dBm). |
| 10.6.27 | N. Silberman | T | Specify Signal to noise ratio at antenna port for the specified BER | Otherwise specification meaningless. |
| 10.7.10 | N. Silberman | T | Specify data rate tolerance. | See comment of par 10.6.10 |
| Annex A | bdobyns | T | PICS ProForma for MAC, all three PHY's |  |
| Index | bdobyns | E | Add an Index |  |