IEEE P802.11
Wireless Access Method and Physical Layer Specifications

Title:	IR PHY Response to Technical Comments on Section 12

Authors: 	Barry A. Dobyns	Tom Baumgartner

Photonics Corporation	Spectrix Corporation

2940 North First Street	906 University Pl

.	San Jose, CA 95134, USA	Evanston, IL 60201

Voice: 408-944-7930	Voice: 708-491-4534

FAX: 408-944-7950	Fax: 708-467-1094

EMail:  76527,266

Adriano Moreira	Hirohisa Wakai
University of Aveiro	Sharp
3800 Aveiro, Portugal	1385 Shattuck Av #102
Tel: +351 34 381937	Berkeley, CA 94709
Fax: +351 34 381941	Voice: 510 644 9267
adriano@ua.pt	Fax: 510 644 1126

wakai@sharp.co.jp

Abstract:	What's Wrong:  
Some people inexplicably voted against the IR PHY Section with comments.

How To Fix It:
Accept the reasonable comments and reject the bogus ones.

Motions:
As shown below, and in the minutes 802.11-95/34 and 802.11-95/42.

Resolved that 802.11-95/45 is a correct and true record if the Infrered PHY resonse to ballot comments on draft D1 section 12, and should be presented to the commentatiors.

IR PHY VOTE 3-0-0

IR PHY RESPONSES TO COMENTS ON SECTION 12

| SECTION | AUTHOR | TYPE | PROPOSED CHANGE | RATIONALE | ED | RESPONSE |
| --- | --- | --- | --- | --- | --- | --- |
| 12.2.5.1 | C. Thomas Baumgartner | t | Change Start_of_Data to Start_of_Activity | Paragraph 8. defines Start_of_Activity, not Start_of_Data | Y | ACCEPTED This still does not match section 8.1.1, which is internally inconsistent. IR PHY VOTE 4-0-0 |
| 12.2.5.1 | C. Thomas Baumgartner | t | confirm with authors of Section 8. that there is an End_of_Data class. | Section 8. has contradictions about this | Y | ACCEPTED WITH CHANGE Pending the disobfuscation of the confusatron in section 8; Change 12.2.5.1 to "End_of_Data_and_Activity" IR PHY VOTE 4-0-0 |
| 12.2.5.1 | C. Thomas Baumgartner | t | describe the procedure in more realistic terms | Even though the introduction says that the procedure description is theoretical and based on actions taking place with no delay time the real PHY has to work with a real MAC. With an exposed interface the PHY developer will not have control of the MAC. It is not possible to send the headers, then send a confirm to the MAC, and wait for data from the MAC. | N | WITHDRAWN BY COMMENTATOR |
| 12.3.3.3 | C. Thomas Baumgartner | t | Change to "The mask represents the irradiance normalized to the total emitted power..." | Isn't it more accurate than average emitted power? | Y | ACCEPTED WITH CHANGE change "average emitted" to "total peak emitted" IR PHY VOTE 4-0-0 |
| 12.3.3.3 | C. Thomas Baumgartner | t | Must add at least one other emitter radiation pattern now for portable handheld device. Should not cover entire azimuth in recognition that the handheld is likely to be positioned in certain way relative to ceiling. | The pattern in the spec is for a ceiling mounted device in the middle of a room. Must have at least one other pattern for a handheld or mobile device where the perfectly circular pattern is not nearly so useful. This pattern is probably not so wide, therefore the total power might reasonably be reduced in paragraph 12.3.3.1. | N | ACCEPTED PROVISIONALLY When another pattern is presented, this subcommittee will gladly consider it for inclusion. IR PHY VOTE 4-0-0 |
| 12.3.4.2 | C. Thomas Baumgartner | t | Change 3rd sentence to "Conforming PHY are required to assert this condition within the first 12 microseconds of signal reception, when the received signal level is between the receiver sensitivity defined in 12.3.3.7 "Receiver Sensitivity" and the maximum set by the dynamic range defined in 12.3.3.8 "Receiver Dynamic Range", and the background IR signal is at the level defined in 12.3.3.7 "Receiver Sensitivity." | more accurate to define this parameter over the range of received level | Y | ACCEPTED WITH CHANGES replace "at a signal level" with "at the minimum signal level" IR PHY VOTE 4-0-0 |
| 12.4 | C. Thomas Baumgartner | t | in table Channel_Transit_Delay should be larger than 25 nsec for 10 meter range. Whatever change is made the same change required for Channel_Transit_Variance. | I think that IR propagation speed is about 1.7 nsec per foot | Y | ACCEPTED Change delay and variance to 100 ns, to allow for future more sensitive receivers with greater range. IR PHY VOTE 4-0-0 |
| 12.4 | C. Thomas Baumgartner | t | delete MPDU_Current_Maximum attribute | Unnecessary complication in an already too complex protocol. The only use I know would be for PHY to know that its error rate is high so a smaller packet could get through better. But the MAC has responsibility for making this decision and MAC doesn’t have to tell PHY it just sends smaller MPDU. In Section 5.1.4 the attribute is called Fragmentation_Threshold. | Y | ACCEPTED WITH CHANGES Change to "... aMPDU_Maximum, STATIC, identical for all conformant phy." IR PHY VOTE 2-0-2 |
| 12 | Wim Diepstraten | T | There is no means specified with which a MAC can evaluate the quality of an IR link, so that it is not possible to determine which AP is the best candidate for reassociation. An RSSI and / or SQ type of indication could provide with the relative information that can be used by the MAC Management entity to determine the best candidate. |  | N | REJECTED RSSI and/or SQ are not produced as a easy or uncomlicated side effect of the operation of the IR-PHY, unlike Radio PHY's. We believe that mandating the added complexity of RSSI or SQ is not justified for the single use in selecting an AP. IR PHY VOTE 4-0-0 |
| 12. | bdobyns | T | Add thermal operating range. |  | Y | DEFERRED Commentator defers to R.Valadas comment 12.3.2 which proposes text for thermal operating range. |
| 12.1, 5th paragraph | Fischer, Mike. | T | Ò . . . may sufferÓ ÐÐ add statement of what the symptoms of this suffering may be | clarity | Y | ACCEPTED reword to "... performance of a Baseband Infrared PHY system may suffer reduced range." IR PHY VOTE 4-0-0 |
| 12.2, also 10.1, 10.5, 11.1, 11.4, and 2.9 | Fischer, Mike. | T | The reference model in figure 2Ð11 should be replaced with one that matches the remainder of the standard. A recommended replacement drawing appears in document 95/16. To the extent that it makes editorial sense to include reference model drawings in subsequent (e.g. PHY) chapters, those drawings should be copies of, or subsets of, the drawing in section 2.9. | There should be a consistent reference model for all sections of the specification, and for all PHYs; otherwise the concept of a reference model is of dubious value. The existing drawings in 4 chapters are all different, and none fully match the description of the MAC and PHY elsewhere in this document. | Y | ACCEPTED Delete figure 12-2, and refer instead to figure 2-11 in section 2.9 "reference model" IR PHY VOTE 4-0-0 |
| 12.2.3 | Fischer, Mike. | T | The behavior of the IR PHY which does not implement the 2Mbps transmission option when requested to transmit at 2Mbps needs to be specified. (I donÕt particularly care about what is specified as long as the result is not the transmission of the MPDU at 1Mbps, which would foul up the MACÕs sense of time by remaining on the medium twice as long as the duration field indicates.) | completeness | Y | ACCEPTED Two possilble interpretations of comment: (1) Two stations A, B. Station A requests Station B to transmit at 2mbps but Station B cannot. This is an error in the rate negotiation protocol at the MAC level, and not an error, per se, in the PHY. If the MAC in station B requests 2mbps from the PHY in station B, then it will be handled as in case 2. (2) MAC request of the PHY in it's own station to transmit at 2mbps, but PHY cannot. This is an error in the MAC, since the PHY MIB in that station should indicate that only 1mbps is possible. Nevertheless, the PHY will fail to transmit and will give an error indication to the MAC. IR VOTE 4-0-0 |
| 12.2.3 | Fischer, Mike. | T | What modulation isused for the DCLA field? Should be stated, as this falls after the data rate field and the length field is at the higher rate if so indicated. | completeness | Y | ACCEPTED This field is not symbol data in any modulation format. Clarifying text "The Synce SFD, DR and DCLA are not symbols which are modulated in L-PPM" will be added to 12.2.3, 12.2.4.1, 12.2.4.2, 12.2.4.3 and 12.2.4.4 IR VOTE 4-0-0 |
| 12.2.4.1 | Fischer, Mike. | T | How is theÓabsence of a pulseÓ distinguished from theÓempty slotÓ that ends the SYNC field? Either specify the distinction or use consistent terminology for what is really the same thing. | clarity | Y | ACCEPTED Second sentence will be changed to read "... shall terminate with the absence of a pulse in the last slot." IR VOTE 4-0-0 |
| 12.2.4.2 | Fischer, Mike. | T MAJOR ISSUE | How does this SFD meet the 802 Hamming distance requirement? The SYNC field appears to be of the form 10101010. . ., so the hamming distance to this sequence is only 2. Recommend use of a 16Ðbit unique word, as is done by the other PHYs. | Reliable startÐofÐframe delimitation is one of the most important functions of any PHY. With the current type of PLCPs, the importance is even greater because there is no endÐofÐframe delimiter, just a length in the PLCP header, so the importance of reliable SFD detection is even more important. | N | REJECTED. The choice of 4bits SFD is an optimum choice for IR PHY, as it is the minimum of the probability of false detection of the SFD and the probility of correct detection. In the case of false SFD detection, the PLCP CRC will fail with hamming distance much greater than required. IR PHY VOTE 4-0-0 |
| 12.2.4.3 12.2.4.5 12.3.4.1 | Wim Diepstraten | T | The rate field is not specified such that the CCA based on the Length field can support coexistance with a possible future extended rate. in addition, the Length field is specified to be transmitted at the target datarate, so that it can not be interpreted by all receivers that do only support the basic rate. | This may not be an issue when no in band extended rate extension of this standard can be expected. | N | REJECTED. Length (timed) CCA is protection for fading, which is not a problem for the IR PHY. The IR PHY CCA only considers energy-detect and modulation-detect. As a consequence of our design, future higher data rates must be modulated in such a way that current CCA mechnanism asserts a 'detection.' IR PHY VOTE 4-0-0 ***** In conversation with commentatior, Wim is concerned with EFD REJECTED AGAIN AFTER RECONSIDERATION Reliable EFD is necessary for proper location of the frame CRC, which is not readable anyway at new unknown modulations techniques. Therefore this is still unimportant. IR PHY VOTE 3-0-0 |
| 12.2.4.4 | Fischer, Mike. | T | Does the encoding in the DCLA field allow for future introduction of 6 more data rates? If so, this should be stated. If not, the last sentence of 12.2.4.3 shold be replaced with a statement that other data rate codes are possible, but may not be usable because of limitations imposed by the (length?, format?) of the DCLA field. | consistency | Y | REJECTED For the data rates that we can conceive of attempting to add to this PHY, the DCLA field is adaquate. IR PHY VOTE 4-0-0 delete the 8 rate sentence in 12.2.4.3 for clarity |
| 12.2.4.6, also 11.2.3.6, and 10.3.2.2.3 | Fischer, Mike. | T | The CRC polynomial does not match its name. The listed polynomial is ÒCRCÐCCITT.Ó There is a polynomial named ÒCRCÐ16Ó but its polynomial is (X^16)+(X^15)+(X^2)+1. Either of these polynomials is acceptable for PLCP header checking, but the name and the polynomial should be consistent (and uniform across all of these PHYs). Please choose 1. The description of the algorithm in 10.3.2.2.3 is the clearest, and should be replicated for all of the other HEC sections (or adapted for all if the CRCÐ16 polynomial is desired and the error was in the polynomial rather than the name of the polynomial). | consistency, technical correctness | Y | ACCEPTED Change name of the polynomial to CRC-CCITT. IR PHY VOTE 4-0-0 |
| 12.2.5.2, 10.2.3.1, 11.2.7, | Fischer, Mike. | T MAJOR ISSUE | It is imperative that all PHYs explicitly constrain the length reported in the RXVECTOR of the PHY_DATA.indicate(Start_of_Data) to equal the length sent from MAC to PHY in the TXVECTOR of the PHY_DATA.request(Start_of_Data) at the peer PHY entity that placed the PhPDU onto the WM. This needs to be true even if the unification of TXVECTOR and RXVECTOR formats and encodings recommended in another of my comments is not adopted. | If the receiving MAC cannot rely upon the length indicated in the RXVECTOR to be an accurate copy of the MPDU length from the peer MAC entity, the entire fragmentation/reassembly model needs to be reexamined. The absence of a fragment length field in the MAC header has been discussed extensively, both regarding fragmentation and regarding WEP (especially WEP, which applies to MSDUs, in conjunction with fragmentation, which generates MPDUs after WEP has encrypted the MSDU). In several of these discussions, the ability to omit this fragment length indication was justified on the basis of this property of the length indication from the RXVECTOR ÐÐ but the current PHY drafts do not explicitly require that this property is true. Note that if this property can be relied upon (in cases that the HEC is valid on reception), the use of the PLCP length reported in the RXVECTOR is superior to a length field in the MAC header, because a MAC implementation may use the length from the RXVECTOR as a validated (rather than speculative) quantity prior to receipt and validation of the CRC at the end of the MAC frame. | Y | ED. ACCEPTED IR PHY does not modify the length of the MPDU or the contents of the the length field as given to the PHY by the MAC in the PHY_DATA.request(Start_of_data). Does commentator want a specific notation to that effect in the standard, or is it obvious that the PHY must operate this way or nothing will work. ***** Specific notation provided by Editor in 12.2.5.2 d). |
| 12.3.1 figure 12-2 | bdobyns | T | PHY LME shown connecting to MAC. figure 12-2 should be deleted and the text should refer instead to figure 2-11. |  | Y | ACCEPTED Already handled in a previous comment. by Mike Fischer. Delete figure 12-2 and refer figure 2-11 |
| 12.3.1 (Figure 12-2) | Bob O'Hara | T | this figure must match all other architectural figures | inconsistent | Y | ACCEPTED Already handled in a previous comment by Mike Fischer. Delete figure 12-2 and refer figure 2-11 |
| 12.3.2 PMD Operating Specifications General | Rui Valadas | T | There should exist a section specifying the operating environment, with the following text: 12.3.2.3 Operating Environment The IR-PHY will operate only in indoor environments. IR-PHY interfaces can not be exposed to direct sun light. The IR-PHY does not require a line-of-sight between emitter and receiver in order to work properly. The performance of the system will vary with the geometry of the environment and with the natural and artificial illumination conditions. | It is not clear from the current standard, the conditions required for an IR-PHY to work properly. | Y | ACCEPTED WITH CHANGE Proposed text added to section 12.1, with apppropriate editorial changes to fit the tone and style, what little there is. IR PHY VOTE 4-0-0 ED. ACCEPTED AS PROPOSED. |
| 12.3.2 PMD Operating Specifications General | Rui Valadas | T | There should exist a specification for the "operating temperature range", with the following text: 12.3.2.4 Operating Temperature Range The temperature range for full operation compliance with the IR PHY is specified as 0 to 40 degrees centigrade. | There should exist a specification for the "operating temperature range". | Y | ACCEPTED Added to standard, with one change "The minimum temperature range ..." IR PHY VOTE 4-0-0 |
| 12.3.3.2 | Bob O'Hara | T | specify method to determine jitter | is jitter for pulse as a whole or for each edge independenly? | Y | REJECTED Method to determine jitter is more properly a subject of a test suite. We believe that the prose and the picture are sufficiently clear. Jitter is for pulse edges independently, but, as prose states, pulse width must be +- 10ns as well. IR PHY VOTE 4-0-0 ED. NOTE: Will change wording in p1, line 4,5 "pulse" to "edge" |
| 12.3.3.7 | Samdahl | T | Para 1 line 2: Add “unmodulated” before “background” | This is intended as a measure of immunity to a background ‘DC’ source of IR in the passband of the receiver. There may also be a need to specify the noise performance, but that was not the original intent of this section. | Y | ACCEPTED Add word to 12.3.3.7 and 12.3.3.8 IR PHY VOTE 4-0-0 |
| 12.3.3.7 | Samdahl | T | Para 2 line 1: Use “background” instead of “noise” | Same as above. | N | REJECTED Can't find word 'noise' anywhere in section 12, except one occurence in section 12.2.4.1 "signal-to-noise-ratio" IR PHY VOTE 4-0-0 |
| 12.3.3.9 | Samdahl | T | Para 3 line 1: Add a new paragraph: “The receiver sensitivity will be greater than 10% of its maximum value at +/- 85 degrees from the normal.” | 9Specifying 90 degrees will result in very inefficient operation. In a diffuse system, a substantial fraction of the available energy in the vicinity of the receiver will occur at entry angles greater than +/- 45 degrees from the normal vertical. As defined, a conformant receivers acceptance angle could fall to zero at any point outside the +/- 45 degree cone, as long as its sensitivity was greater than 50% at the +/- 45 degree point. Such a conformant receiver would exhibit performance much inferior to units with sensitivity out to, say, +/-85 degrees, which I believe to be a more realistic value. A true Lambertian receiver will have 50% sensitivity at +/- 60 degrees, so specifying +/- 45 degrees may not be too poor a choice. The problem is that the extinction point or perhaps the angle at which the sensitivity falls to 10% should also be specified, probably to a value of +/-85 degrees. It can also be argued that a superior implementation might have a receiver that has sensitivity enhanced at shallow angles, since it is at these shallow angles that radiation from most distant sources will arrive. High sensitivity in the cone above the receiver (at narrow receptor angles) is of relatively little value, since radiation arriving in this cone typically originates nearby and is of high intensity. | Y | ACCEPTED WITH CHANGE Replace 12.3.3.2 paragraph 2 and 3 with: Define the receiver FOV as more than 65% of the maximum received optical power for angles for angles less than 20', 55% for angles less than 40', 35% for angles less than 60' and 10% for angles less than 80'. IR PHY VOTE 3-0-1 |
| 12.3.3.9 Receiver Field-of-View (FOV) | Rui Valadas | T | Define the receiver axis as the direction of incidence of the optical signal at which the received optical power is maximum. Define the receiver FOV as twice the angle measured between the receiver axis and the direction of incidence at which the received optical power is equal to 1% of the maximum received optical power. For incident angles smaller than half the FOV, the received optical power should always be higher than 1% of the maximum received power. The receiver FOV of a conformant receiver shall be greater than or equal to 150° . | The receiver FOV should be as wide as possible to minimise the hidden station problem. | Y | ACCEPTED WITH CHANGES 12.3.3.9 Samdahl |
| 12.3.4.3 Energy Detect | Rui Valadas | T | For further study | The sensitivity of the Energy Detection mechanism is too low. With the indicated threshold there is a high probability that one or more transmissions from a like-PHY will not produce enough energy to assert the Energy Detection Signal. | Y | REJECTED CS is an indepenent function from ED, and a like phy below the ED threshold may still cause CS to be asserted. IR PHY VOTE 4-0-0 ED. clarifying text added instead |
| 12.4 | Bob O'Hara | T | MIB definition is required in ASN.1 format | definition is incomplete | Y | ACCEPTED Added text to 12.4 "This section does not provide the definion of the MIB, but only provides the phy-specific values for the ASN.1 in section 9." IR PHY VOTE 4-0-0 |
| 12.3 | K.C.Chen | T | open | Iti is suggested that IR PHY define the PMD_SAP (not a factor to decide my vote) | N | REJECTED Definition of a PMD_SAP does not improve the clarity, readability or implemetability of the standard. IR PHY VOTE 4-0-0 |
| 12.3.2.1 | K.C.Chen | T | Data in table 12-1 and table 12-2 should be gray coded. For example 00 01 11 10 for 4-PPM | This can minimize the distance of neighboring signal constellations under ISI and provide advantages for error detection and correction for future speed expansion. | Y | ACCEPTED Change Table, and add text in 12.3 "The data in these tables has been arranged (gray coded) so that a single out-of-position-by-one error in the medium, caused, for example, by intersymbol interference, results in only a single bit error in the received data, rather than a multiple bit error. " IR PHY VOTE 4-0-0 |
| 12.3.3.1 | K.C.Chen | T | The peak optical power of an emited pulse shall not be greater than 2W | Emitting power should be defined the upper-limit ONLY for IR PHY | N | REJECTED This increases the hidden-node problem by permitting very weak conformant devices. This also permits asymmetric communications opportunities, which affects interoperability. IR PHY VOTE 4-0-0 |
| 12.3.3.3 | K.C. Chen | T | <delete> | Unless this can be well defined without ambiguity no reason to keep this. We may define a nondirective conformance test in the future. For safety concern the radiation can be defined a minimum value for decline angle 30 degrees. | N | REJECTED Pattern mask is defined for diffuse interoperablity, not for safety. The current definition is sufficient. Additional conformant masks may be added before approval. IR PHY VOTE 4-0-0 |
| 12.1 | C. Thomas Baumgartner | e | 10 paragraph change to (placing it in a different room) delete "is usually sufficient" | placing it in another room from the LAN coverage is always sufficient. | Y | ED. ACCEPTED |
| 12.1.3 | C. Thomas Baumgartner | e | Definitions missing | Fill in section or delete if nothing to put here | Y | ED. ACCEPTED Section deleted. Definitions belong in section 1 not here |
| 12.1.4 | C. Thomas Baumgartner | e | Add Acronyms SYNC, SFD,DR, FER | These acronyms are equally in need of explanation | N | ED. REJECTED Work for Section 1 |
| 12.2.2 | C. Thomas Baumgartner | e | Change title of Figure 12-1: to PDU Frame Format | The diagram shows the entire PDU Frame | Y | ED. ACCEPTED |
| 12.2.3 | C. Thomas Baumgartner | e | missing number 1 as in 1 Mbps bit rate | typo | Y | ED. ACCEPTED |
| 12.2.4.2 | C. Thomas Baumgartner | e | Add note that 1=pulse and 0=no pulse in slot | Not clear what binary digits mean regarding energy in slots. | Y | ED. ACCEPTED |
| 12.2.5.1 | C. Thomas Baumgartner | e | Add before a) "Following is the transmit procedure:" | List of steps needs an introduction. | Y | ED. ACCEPTED |
| 12.2.5.2 | C. Thomas Baumgartner | e | Add before a) "Following is the receive procedure:" | List of steps needs an introduction. | Y | ED. ACCEPTED |
| 12.2.5.2 | C. Thomas Baumgartner | e | in a) correct to End_of_Activity, delete 1 | typo | Y | ED. ACCEPTED |
| 12.2.5.2 | C. Thomas Baumgartner | e | in a) rewrite 2nd and 3rd sentences to "When PHY senses activity on the medium it indicates that the medium is busy with a PHY_DATA.Indicate class=Start_of_Activity. This will normally occur during the SYNC field of the PLCP preamble. | more clear, concise and accurate and same as CCA description in 12.2.5.3 | Y | ED. ACCEPTED |
| 12.2.5.2 | C. Thomas Baumgartner | e | in d) correct to Start_of_Data | typo | Y | ED. ACCEPTED |
| 12.2.5.3 | C. Thomas Baumgartner | e | Add before a) "Following is the CCA procedure:" | List of steps needs an introduction. | Y | ED. ACCEPTED |
| 12.3.1 | C. Thomas Baumgartner | e | delete Figure 12-2: PMD Layer Reference Model, add reference to Layer Reference Model in another part of document | This is a general model of the interaction of the layers and should be somewhere in the general specification not in the IR section. There is more detail in Figure 10-1 so this is the one that should survive. | Y | ED. ACCEPTED |
| 12.3.2.2 | C. Thomas Baumgartner | e | show LSB on the right, not left | section 1.6 "Conventions" says that is the way to do it in this standard | Y | ED. ACCEPTED |
| 12.3.3.6 | C. Thomas Baumgartner | e | change paragraph number to 12.3.4 and ripple changes through rest of section 12 as follows: 12.3.3.7 to 12.3.4.1; 12.3.3.8 to 12.3.4.2; 12.3.3.9 to 12.3.4.3; 12.3.4 to 12.3.5; 12.3.4.1 to 12.3.5.1; 12.3.4.2 to 12.3.5.2; 12.3.4.3 to 12.3.5.3. Don't for get to change the reference in 12.3.4.2. | The PMD Transmit Spec were 12.3.3. The PMD Receiver Spec should be at same level, ie 12.3.4, not a subset of Tx spec. | Y | ED. ACCEPTED |
| 12.3.4.1 | C. Thomas Baumgartner | e | Italics not necessary at end of first paragraph | Italics not used other places for primitives | Y | ED. ACCEPTED |
| 12.3.4.1 | C. Thomas Baumgartner | e | Change last sentence in 2nd paragraph to "The CCA may remain "BUSY" after the end of data if some form of energy is still being detected. The PHY will signal PHY_DATA.Indicate class=End_of_Activity only when the CCA goes "CLEAR". | more accurately states the case | Y | ED. ACCEPTED |
| 12.4 | C. Thomas Baumgartner | e | in table CCA_Watchdog_Timer_Min needs units, I believe microseconds | missing units required | Y | ED. ACCEPTED |
| 8. | C. Thomas Baumgartner | e | Global replace of Ph with PHY | Need to be consistent with rest of document in referring to Physical Layer |  |  |
| 12 (general) | Fischer, Mike. | E | There are far too many paragraphs in this section that read like they are in a marketing document rather than a draft standard. There is no reason to mention IrDA, discuss Òtrue LAN systemÓ in qualitative terms, state Ò. . . without the possibility of eavesdroppingÓ (do I hear product liability lawyers lining up outside?), etc. | It would be nice if the result of this work was a standard where the different chapters appeared to have been written on the same planet, maybe even the same continent. | Y | ED. REJECTED This section was produced at the specific request of the 802.11 Chair. |
| 12, ch 10,11,12 | PFS | E | PLCP general descriptions should use similar language and text for all phy’s and should speak to the MAC layer primitives in the same way |  |  | ED. DEFERRED Wait until section 8 updated. |
| 12.1 | Bob O'Hara | E | replace "insure" with "ensure" |  | Y | ED. ACCEPTED |
| 12.1.1 | Bob O'Hara | E | replace "by" with "to", "for" with "by", "might" with "may" |  | Y | ED. ACCEPTED |
| 12.1.1 | Mahany | E | Replace “Nodes” with “Stations” | Term Node not in earlier definitions. | Y | ED. ACCEPTED |
| 12.1.3, 12.1.4, 12.1.5 | Fischer, Mike. | E | these should be merged into the relevant portions of section 1 | uniformity of notation and nomenclature | Y | ED. ACCEPTED Deleted here |
| 12.1.4 | bdobyns | E | Merge with section 1.3, Abbreviations | don't need a Distributed Abbreviation Function. | Y | ED. ACCEPTED Deleted here |
| 12.1.4 Acronyms | Rui Valadas | E | None. | The Acronyms list should be only one, common to the MAC and all the PHYs. | Y | ED. ACCEPTED deleted here |
| 12.2.1 | Bob O'Hara | E | replace "appended" with "prepended", "MPDU" with "MPDU (PSDU)" |  | Y | ED. ACCEPTED |
| 12.2.2 | Bob O'Hara | E | replace "MPDU" with "PSDU" |  | Y | ED. ACCEPTED |
| 12.2.2 (Figure 12-1) | Bob O'Hara | E | replace "MPDU" with "PSDU" |  | Y | ED. ACCEPTED |
| 12.2.3 | Bob O'Hara | E | replace "MPDU" with "PSDU", "rates: Mbps" with "rates: 1 Mbps" |  | Y | ED. ACCEPTED |
| 12.2.3 | Fischer, Mike. | E | ÒwordsÓ is not a term used elsewhere in this standard for this purpose ÒMbps and 2MbpsÓ should be Ò1 Mbps and 2MbpsÓ The use of LÐPPM here is valid, but in several later subsections ÒLÐPPMÓ appears where I believe the correct usage would be either 16ÐPPM or 4ÐPPM Ñ please clarify | consistency | Y | ED. ACCEPTED |
| 12.2.3 | Geiger | E | Mbps s/b/ 1Mbps | Spelling | Y | ED. ACCEPTED |
| 12.2.3 | Samdahl | E | Para 2 line 4: Should be “1 Mbps” instead of “Mbps” |  | Y | ED. ACCEPTED |
| 12.2.4.3 | Bob O'Hara | E | replace "which will" with "that shall" |  | Y | ED. ACCEPTED |
| 12.2.4.3 | Bob O'Hara | E | replace "is" with "shall be" |  | Y | ED. ACCEPTED |
| 12.2.4.4 | Bob O'Hara | E | replace "is" with "shall be" |  | Y | ED. ACCEPTED |
| 12.2.4.4 | Bob O'Hara | E | replace "MPDU" with "PSDU" |  | Y | ED. ACCEPTED |
| 12.2.4.6 | Bob O'Hara | E | replace "is" with "shall be" |  | Y | ED. ACCEPTED |
| 12.2.4.7 | Bob O'Hara | E | replace "MPDU" with "PSDU" |  | Y | ED. ACCEPTED (everywhere in sec 12) |
| 12.2.4.7 | Bob O'Hara | E | replace "are" with "shall be" |  | Y | ED. ACCEPTED |
| 12.2.5.1 | Bob O'Hara | E | replace "MPDU" with "PSDU" |  | Y | ED. ACCEPTED |
| 12.3.1 | Bob O'Hara | E | replace "from" with "on" |  | Y | ED. REJECTED Sorry, that sentence deleted |
| 12.3.1, 2.9, 11.1.2, 10.1.2, | Isabel Lin | E | Make them consistent. | The Reference Models in those sections are not consistent. What needs to be done: Make them consistent. | Y | ACCEPTED Already handled in a previous comment by Mike Fischer. Delete figure 12-2 and refer figure 2-11 |
| 12.3.3.2 figure 12-3 | bdobyns | E | figure 12-3 could be smaller, without loss of information. |  | Y | ED. ACCEPTED |
| 12.3.3.3 figure 12-4 | bdobyns | E | figure 12-4 uses grey shades for lines, should use dotted, dashed or otherwise non-colored lines for clarity. |  | Y | ED. ACCEPTED |
| 12.3.3.7 | Bob O'Hara | E | replace "an MPDU" with "a PSDU" |  | Y | ED. ACCEPTED |
| 12.3.3.7 | Bob O'Hara | E | replace "an MPDU" with "a PSDU" |  | Y | ED. ACCEPTED |
| 12.3.3.7 | Samdahl | E | “FER” (Frame Error Rate) should be defined if it hasn’t been done earlier |  | N | ED. REJECTED work for Section 1 |
| 12.3.4.1 | Bob O'Hara | E | delete ":", replace "will" with "shall" |  | Y | ED. ACCEPTED But - Couldn't find colon. |
| 12.3.4.2 | Bob O'Hara | E | delete ":" |  | N | ED. what colon? |
| 12.3.4.3 | Bob O'Hara | E | delete ":" |  | N | ED. can't find colon |
| 12.4 | Wim Diepstraten | E E | A clear Slot time specification should be provided. The meaning of the PHY_SAP_delay is unclear. | It is difficult to assess which parameters do add up to the Slot Time. | N | ED. DEFER Wait and see if Section 9 changes. |
| 12.2.5.2 b | A. Moreira | E |  | Change name of CRC | Y | ED. ACCEPTED |