IEEE P802.11
Wireless Access Method and Physical Layer Specifications

Title:	Proposed changes to D1 Section 12 based on Ballot Changes

Authors: 	Barry A. Dobyns	Tom Baumgartner

Photonics Corporation	Spectrix Corporation

2940 North First Street	906 University Pl

.	San Jose, CA 95134, USA	Evanston, IL 60201

Voice: 408-944-7930	Voice: 708-491-4534

FAX: 408-944-7950	Fax: 708-467-1094

EMail:  76527,266

Adriano Moreira	Hirohisa Wakai
University of Aveiro	Sharp
3800 Aveiro, Portugal	1385 Shattuck Av #102
Tel: +351 34 381937	Berkeley, CA 94709
Fax: +351 34 381941	Voice: 510 644 9267
adriano@ua.pt	Fax: 510 644 1126

wakai@sharp.co.jp

# 1	Ignore this text

# 2	Ignore this text

# 3	Ignore this text

# 4	Ignore this text

# 5	Ignore this text

# 6	Ignore this text

# 7	Ignore this text

# 8	Ignore this text

# 9	Ignore this text

# 10	Ignore this text

# 11	Ignore this text

Abstract:	What's Wrong:  
Some people inexplicably voted against the IR PHY Section with comments.

How To Fix It:
Accept the corrected Section 12 text as shown below

Motions:
Resolved that the text in document 802.11-95/46 be incorporated into the next draft of the standard as section 12 as shown.

IR PHY VOTE 3-0-0

# 12	Baseband Infrared Physical Layer Specification

## 13	Introduction

This section describes the physical layer for the Baseband Infrared system.  The Baseband IR PHY uses near-visible light in the 850 nanometer to 950 nanometer range for signaling.  This is similar to the spectral usage of both common consumer devices such as infrared remote controls, as well as other data communications equipment such as IrDA (Infrared Data Association) devices.

However, unlike many other infrared devices the Baseband IR PHY is not directed.  That is, the receiver and transmitter do not have to be aimed at each other and do not need a clear line-of-sight.  This permits the construction of a true LAN system, whereas with an aimed system, it would be difficult or impossible to install a LAN because of physical constraints.

A pair of conformant Baseband Infrared devices would be able to communicate in a typical environment at a range up to about ten meters.  The standard allows conformant devices to have more sensitive receivers, and this may increase range up to about twenty meters.

The Baseband Infrared PHY relies on both reflected infrared energy as well as line-of-sight infrared energy for communications.  Most designs anticipate that ALL of the energy at the receiver is reflected energy.  This reliance on reflected infrared energy is called “Diffuse Infrared” transmission.

The standard specifies the transmitter and receiver in such a way to ensure insure that a conformant design will operate well in most environments where there is no line-of-sight path from the transmitter to the receiver.  However, in an environment which has few or no reflecting surfaces, and where there is no line-of-sight, performance of a Baseband Infrared PHY system may suffer reduced range.

The Infrared PHY will operate only in indoor environments.  Infrared radiation does not pass through walls, and is significantly attenuated passing through most exterior windows.  This characteristic can be used to “contain” a Baseband Infrared PHY in a single physical room, like a classroom or conference room.  Different LANs using the Baseband Infrared PHY can operate in adjacent rooms separated only by a wall without interference, and without the possibility of eavesdropping.

At the time of this standard’s preparation, the only known regulatory standards which apply to the use of infrared radiation are safety regulations, such as IEC 825-1 and ANSI Z - 136.1. While a conformant Baseband Infrared PHY device can be designed in a way to also comply with these safety standards, conformance with this standard does not ensure insure conformance with other standards.

Worldwide, there are currently no frequency allocation or bandwidth allocation regulatory restrictions on infrared emissions.

Emitter (typically LED) and detector (typically Pin-Diode) devices for infrared communications are relatively inexpensive at the infrared wavelengths specified in the Baseband Infrared PHY, and at the electrical operating frequencies required by this PHY.

While many other devices in common use also use infrared emissions in the same optical band, these devices usually transmit infrared intermittently and do not interfere with the proper operation of a compliant Baseband Infrared PHY.  If such a device does interfere, by transmitting continuously and with a very strong signal, it can be physically isolated (placing it in a different room is usually sufficient) from the 802.11 LAN.

### 14	Scope

This section describes the physical layer services provided to by the 802.11 wireless LAN MAC by for the Baseband Infrared (IR) system. The Baseband IR PHY layer consists of two protocol functions as follows:

a)	A physical layer convergence function which adapts the capabilities of the physical  medium dependent system into the Physical Layer service. This function is supported by the Physical Layer Convergence Procedure (PLCP) which defines a method of mapping the 802.11 MAC layer Protocol Data Units (MPDU) into a framing format suitable for sending and receiving user data and management information between two or more stations nodes using the associated physical medium dependent system.

b)	A Physical Medium Dependent (PMD) system whose function defines the characteristics of, and method of transmitting and receiving data via wireless media between two or more stations nodes.

Each physical medium dependent sublayer for the Baseband IR PMD may require the definition of a unique PLCP. If the PMD sublayer already provides the defined Physical Layer services, the physical layer convergence function may might be null.

### 15	Baseband IR Physical Layer Functions

The Baseband IR physical layer contains three functional entities: the physical medium dependent function, the physical layer convergence function, and the layer management function. Each of these functions is described in detail in the following subsections.

The Baseband IR Physical Layer service is provided to the Media Access Control entity at the node through a Service Access Point (SAP) as described in Section 8, Physical Service Specification.    For a visual guide to the relationship of the Baseband IR Physical Layer to the remainder of a system, refer to figure 2-11, Portion of the ISO Basic Reference Model Covered in this Standard.

#### 16	Physical Layer Convergence Procedure Sublayer

In order to allow the 802.11 MAC to operate with minimum dependence on the PMD sublayer, a physical layer convergence sublayer is defined. This function simplifies the physical layer service interface to the 802.11 MAC services.  The PHY specific preamble is normally associated with this convergence layer.

#### 17	Physical Medium Dependent Sublayer

The physical medium dependent sublayer provides a clear channel assessment mechanism, transmission mechanism and reception mechanism which are used by the MAC via the PLCP to send or receive data between two or more stations nodes.

#### 18	Physical Layer Management Entity (LME)

The Physical LME performs management of the local Physical Layer Functions in conjunction with the MAC Management entity.

### 19	Definitions

This section defines the terms used in this standard. Words in italics indicate terms that are defined elsewhere in the lists of definitions

### 20	Acronyms

CRC	=	Cyclic Redundancy Check

DCLA	=	DC Level Adjustment

FCS	=	Frame Check Sequence

IR	=	Infrared

IrDA	=	Infrared Data Association

LME	=	Layer Management  Entity

MAC	=	Media Access Control

MPDU	=	MAC Protocol Data Unit

PDU	=	Protocol Data Unit

PHY_SAP	=	Physical Layer Service Access Point

PLCP	=	Physical Layer Convergence Procedure

PMD	=	Physical Medium Dependent

PMD_SAP	=	Physical Medium Dependent Service Access Point

PPM	=	Pulse Position Modulation

SAP	=	Service Access Point

### 21	Service Specification Method and Notation

The models represented by figures and state diagrams are intended as the illustrations of functions provided. It is important to distinguish between a model and a real implementation. The models are optimized for simplicity and clarity of presentation, the actual method of implementation is left to the discretion of the 802.11 Baseband IR PHY compliant developer.  Conformance to the standard is not dependent on following the model, and an implementation which follows the model closely may not be conformant.

The service of layer or sublayer is a set of capabilities that it offers to a user in the next higher layer (or sublayer). Abstract services are specified here by describing the service primitives and parameters that characterize each service. This definition is independent of any particular implementation.  In particular, the PHY_SAP operations are defined and described as instantaneous, however, this may be difficult to achieve in an implementation.

## 22	Baseband IR Physical Layer Convergence Procedure Sublayer

While the Physical Layer Convergence Procedure (PLCP) sublayer and the Physical Medium Dependent (PMD) sublayer are described separately, the separation and distinction between these sublayers is artificial, and is not meant to imply that the implementation must separate these functions. This distinction is made primarily to provide a point of reference from which to describe certain functional components and aspects of the PMD.  The functions of the Physical Layer Convergence Procedure can be subsumed by a Physical Medium Dependent sublayer:  In this case, the PMD will incorporate the PHY_SAP as it's interface, and will not offer a PMD_SAP.

### 23	Introduction

This section provides a convergence procedure in which MPDUs are converted to and from PDUs. During transmission, the MPDU (PSDU) is prepended appended with a PLCP Preamble and PLCP Header to create the PDU. At the receiver, the PLCP Preamble is processed and the internal data fields are processed to aid in demodulation and delivery of the MPDU (PSDU).

### 24	Physical Layer Convergence Procedure Frame Format

Figure 12-1 shows the format for the PDU including the PLCP Preamble, the PLCP Header and the PSDU MPDU. The PLCP Preamble contains the following fields: Synchronization (SYNC) and Start Frame Delimiter (SFD). The PLCP Header contains the following fields: Data Rate (DR), DC Level Adjustment (DCLA), Length (LENGTH) and CRC. Each of these fields will be described in detail in section 12.2.4.

µ §

Figure 12-1: PDU PLCP Frame Format

### 25	PLCP Modulation and Rate Change

The PLCP Preamble shall be transmitted using the basic pulse defined  in section 12.3.3.2. The PSDU MPDU, LENGTH and CRC fields are transmitted using Pulse Position Modulation (PPM). PPM maps bits in the octet words into symbols: a 16-PPM maps four bits into a 16 position symbol, and 4-PPM maps two bits into a 4 position symbol. word with n bits is mapped into one of the L=2n positions of a symbol (L-PPM). The basic L-PPM time unit is the slot. A slot corresponds to one of the L positions of a symbol and has a 250 ns duration. The PSDU MPDU, LENGTH and CRC fields are transmitted in one of two bit rates: 1 Mbps and 2 Mbps. The Data Rate field indicates the data rate which will be used to transmit the PSDU MPDU, LENGTH and CRC fields. The 1 MBPS data rate uses 16-PPM (Basic Access Rate) and the 2 MBPS data rate uses 4-PPM (Enhanced Access Rate). The transmitter and receiver will initiate the modulation or demodulation indicated by the DR field starting with the first 4 bits in 16-PPM or 2 bits in 4-PPM of the LENGTH field. The PSDU MPDU transmission rate is set by the RATE parameter in PHY_DATA.request primitive specifying START-OF-ACTIVITY. Any conformant Baseband IR PHY shall be capable of receiving at 1 MBPS and 2 MBPS. Transmission at 2 MBPS is optional.

A PHY_DATA.request which specifies a data rate which is not supported by an PHY instance will cause the PHY to indicate an error to it's MAC instance.  A PHY is not permitted in any circumstance to transmit at a different rate than the requested rate.

### 26	PLCP Field Definitions

#### 27	PLCP Synchronization (SYNC )

The SYNC  field consists of a sequence of alternated presence and absence of a pulse in consecutive slots. The SYNC  field has a minimum length of 57 L-PPM slots and a maximum length of 73 L-PPM slots and shall terminate with the absence of a pulse in the last an empty slot. This field is provided so that the receiver can perform clock recovery (slot synchronization), automatic gain control (optional), signal-to-noise ratio estimation (optional) and diversity selection (optional).

The SYNC field is not modulated using L-PPM, but instead consists of  transitions in L-PPM slots which would otherwise constitute an illegal symbol.

#### 28	PLCP Start Frame Delimiter (SFD)

The SFD field length is 4 L-PPM slots and consists of the binary sequence 1001, where 1 indicates a pulse in the L-PPM slot and 0 indicates no pulse in the L-PPM slot.  The left most bit shall be is transmitted first. The SFD field is provided to indicate the start of the PLCP Preamble and to perform bit and symbol synchronization.

The SFD field is not modulated using L-PPM, but instead consists of  transitions in L-PPM slots which would otherwise constitute an illegal symbol.

#### 29	PLCP Data Rate (DR)

The DR field indicates to the PHY the data rate that shall which will be used for the transmission or reception of the PSDU MPDU, LENGTH and CRC fields. The transmitted value shall be is provided by the PHY_DATA.request primitive specifying START_OF_ACTIVITY as described in section 8.  The DR field has a length of 3 L-PPM slots.  The left most bit, as shown below, shall be is transmitted first. The Baseband IR Infrared PHY currently supports two data rates defined by the slot pattern following shown for the three L-PPM slots following the SFD, where 1 indicates a pulse in the L-PPM slot and 0 indicates no pulse in the L-PPM slot. binary words:

1 MBPS:	000

2 MBPS:	001

The DR field is not modulated using L-PPM, but instead consists of  transitions in L-PPM slots which would otherwise constitute an illegal symbol.

This field allows for the future introduction of a maximum of 8 different data rates.

#### 30	PLCP DC Level Adjustment (DCLA )

The DCLA  field is required to allow the receiver to stabilize the DC level after the SYNC  , SFD and DR fields. The left most bit, as shown below, shall be is transmitted first. The length of the DCLA  field is 32 L-PPM slots and consists of the contents shown, where 1 indicates a pulse in the L-PPM slot and 0 indicates no pulse in the L-PPM slot binary words:

1 MBPS:	00000000100000000000000010000000

2 MBPS:	00100010001000100010001000100010

The DCLA  field is not modulated using L-PPM, but instead consists of  transitions in L-PPM slots which would otherwise constitute an illegal symbol.

#### 31	PLCP LENGTH

The LENGTH field is an unsigned 16 bit integer which indicates the number of octets to be transmitted in the PSDU. The transmitted value shall be is provided by the PHY_DATA.request primitive specifying Start_of_Activity  as described in section 8.  The LSB (least significant bit) shall be transmitted first in time.  This field is modulated and sent in L-PPM format.  This field is protected by the CRC16 Frame Check Sequence described in the next section.

#### 32	PLCP CRC

The LENGTH field shall be protected by a 16-bit CRC-CCITT CCITT CRC16 FCS (Frame Check Sequence). The CRC-CCITT CRC16 FCS is the ones compliment of the remainder generated by the modulo module 2 division of the LENGTH field by the polynomial:

x16+x12+x5+1

The protected bits will be processed in transmit order.  The LSB (least significant bit) of the 16-bit CRC-CCITT shall be transmitted first in time.  This field shall be is modulated and sent in L-PPM format.  All CRC-CCITT FCS calculations shall be made prior to L-PPM encoding on transmission and after L-PPM decoding on reception.

#### 33	PSDU MPDU Field

This field is composed of a variable number of octets.  The minimum is 0 (zero) and the maximum is 2500.  The LSB (least significant bit) of each octet shall be transmitted first in time.  All the octets of this field shall be are modulated and sent in L-PPM format.

### 34	PLCP Procedures

#### 35	PLCP Transmit Procedure

All commands issued by the MAC require PHY_DATA.confirm primitives to be issued by the PHY. The PHY_DATA.confirm primitives provide flow control between the MAC and the PHY.

The steps below are the transmit procedure:

a)	Based on the status of CCA the MAC will determine if  the channel is clear.

b)	If the channel is clear, transmission of the PSDU MPDU is initiated by a PHY_DATA.request class=Start_of_Activity Start_of_Data  with DATA parameters LENGTH and RATE.

c)	The PHY entity will immediately initiate transmission of the PLCP preamble and PLCP header based on the LENGTH and RATE parameters passed in the PHY_DATA.request class=Start_of_Activity Start_of_Data . Once the PLCP preamble and PLCP header transmission is completed the PHY entity issues a PHY_DATA.confirm.

d)	Each octet of the PSDU is passed from Data is then exchanged between the MAC to and the PHY by a series of PHY_DATA.request class=DATA.  Each PHY_DATA.request must be answered by the PHY with a PHY_DATA.confirm before the next request is made.

e)	At the PHY layer each PSDU octet is divided into symbols (2 or 4 bits). The symbols are modulated using L-PPM and transmitted into the medium.

f)	Transmission is terminated by the MAC through the primitive PHY_DATA.request  class=End_of_Data_and_Activity.   The PHY indicates the end of the transmission with a PHY_DATA.Confirm.

#### 36	PLCP Receive Procedure

The steps below are the receive procedure:

a)	CCA is provided to the MAC via the PHY_DATA.indicate class=Start_of_Activity and the PHY_DATA.Indicate class=End_of_Activity.  When PHY senses activity on the medium it indicates that the medium is busy with a PHY_DATA.Indicate class=Start_of_Activity. This will normally occur during the SYNC field of the PLCP preamble.  1Reception is initiated by a PHY_DATA.Indicate  class=Start_of_Activity  indicating that the medium is busy.  This will occur during the SYNC field of the PLCP preamble.

b)	The PHY entity will then begin searching for the SFD field. Once the SFD field is detected the PHY entity will receive the PLCP header. After receiving the DR and DCLA fields the CRC processing is initiated and LENGTH field is received. The change indicated in the DR  field is initiated with the first symbol of the LENGTH field.  The CRC-CCITT CRC16 FCS will be processed.

c)	If the CRC-CCITT CRC16 FCS check fails or no match is found for DR  field then NO  PHY_DATA.Indicate  class=Start_of_Data  will be issued.  When the medium is again free, the PHY will issue PHY_DATA.Indicate class=End_of_Activity.

d)	If the PLCP preamble and PLCP header reception is successful, the PHY sends a PHY_DATA.Indicate class=Start_of_Data Start-Of-Data, including the parameters RATE and LENGTH.

A PHY must guarantee that the length reported to it's MAC in the RXVECTOR of PHY_DATA.Indicate class=Start_of_Data is equal to the length sent from the peer MAC to the peer PHY entity in the TXVECTOR of PHY_DATA.request class=Start_of_Data.

e)	The received PSDU MPDU L-PPM symbols  are assembled into octets and presented to the MAC using  a series of PHY_DATA.Indicate  class=DATA..  The PHY proceeds with PSDU MPDU reception.

f)	Reception is terminated after the reception of the final symbol  of the last PSDU MPDU octet indicated by the PLCP header LENGTH field. After the PHY_DATA.Indicate class=Data for that octet is issued, the PHY will issue PHY_DATA.Indicate class=End_of_Data.

g)	After the PHY_DATA.Indicate class=End_of_Data, when the medium is no longer busy, the PHY will issue PHY_DATA.Indicate class=End_of_Activity.  .

#### 37	CCA Procedure

CCA is provided to the MAC via the PHY_DATA.Indicate class=Start_of_Activity and the PHY_DATA.Indicate class=End_of_Activity

The steps below are the CCA procedure:

a)	When the PHY senses activity on the medium a PHY_DATA.Indicate class=Start_of_Activity is issued.  This will normally occur during the SYNC field of the PLCP preamble.

b)	When the PHY senses the medium is free, a PHY_DATA.Indicate class=End_of_Activity is issued.

c)	At any time, the MAC may issue a PHY_DATA.request class=End_of_Activity, which will reset the PHY’s internal CCA detection mechanism to the medium not busy (free) state.  The PHY_DATA.confirm to this request shall indicate that the PHY now believes the medium is free.

## 38	Baseband IR Physical Medium Dependent Sublayer

The Baseband IR Physical Medium Dependent Sublayer does not define PMD Service Access Primitives.   The mechanism for communications between the PLCP and PMD sublayers, as well as the distinction between these two sublayers, if any, are left to the implementors.   In particular, it is possible to design and implement in a conformant way a single sublayer which subsumes the functions of both the PLCP and PMD, presenting only the PHY_SAP.

### 39	Introduction

This section describes the PMD  functional, electrical, and optical characteristics required for interoperability of implementations conforming to this specification. The relationship of this specification  to the entire Baseband IR PHY Layer is shown in figure 2-11 in section 2.9 Reference Model. 12-2.

µ §

Figure 12-2: PMD Layer Reference Model

The Baseband IR Physical Medium Dependent Sublayer  provides the actual means by which data is transmitted or received from the media.

### 40	PMD Operating Specifications General

The following sections provide general specifications for the Baseband IR Physical Medium Dependent sublayer. These specifications apply to both the receive and transmit functions and general operation of a compliant Baseband IR PHY.

#### 41	Modulation and Channel Data Rates

Two modulation formats and data rates are specified for the Baseband IR PHY: a Basic Access Rate and an Enhanced Access Rate. The Basic Access Rate is based on 1 MBPS 16-PPM modulation. The 16-PPM encoding is specified in Table 12-1. Each group of 4 data bits is mapped in one of 16-PPM symbols. The Enhanced Access Rate is based on 2 MBPS 4-PPM. The 4-PPM encoding is specified in Table 12-2. Each group of 2 data bits is mapped into one of 4-PPM symbols.  Transmission into the medium is from left to right, as shown below.

The data in these tables has been arranged (gray coded) so that a single out-of-position-by-one error in the medium, caused, for example, by intersymbol interference, results in only a single bit error in the received data, rather than a multiple bit error.

| Data | 16-PPM Symbol |
| --- | --- |
| 0000 | 0000000000000001 |
| 0001 | 0000000000000010 |
| 0011 | 0000000000000100 |
| 0010 | 0000000000001000 |
| 0110 | 0000000000010000 |
| 0111 | 0000000000100000 |
| 0101 | 0000000001000000 |
| 0100 | 0000000010000000 |
| 1100 | 0000000100000000 |
| 1101 | 0000001000000000 |
| 1111 | 0000010000000000 |
| 1110 | 0000100000000000 |
| 1010 | 0001000000000000 |
| 1011 | 0010000000000000 |
| 1001 | 0100000000000000 |
| 1000 | 1000000000000000 |

Table 12-1: MBPS 16-PPM Basic Rate Mapping

| Data | 4-PPM Symbol |
| --- | --- |
| 00 | 0001 |
| 01 | 0010 |
| 11 | 0100 |
| 10 | 1000 |

Table 12-2: MBPS 4-PPM Enhanced Rate Mapping

#### 42	Octet Partition and PPM symbol generation procedure

Since PPM is a block modulation method, with the block size less than a full octet, octets have to be partitioned prior to modulation (mapping into PPM symbols).

Octet partition depends on the PPM order being used.

Assume an octet is formed by eight bits numbered 7 6 5 4 3 2 1 0 0 1 2 3 4 5 6 7, where bit 0 is the LSB (least significant bit).  Partition the octet as follows:

For 16-PPM, create two PPM symbols:
the symbol using bits   3 2 1 0 0 1 2 3   transmitted onto the medium first 
the symbol using bits   7 6 5 4 4 5 6 7   transmitted onto the medium last

For 4-PPM, create four PPM symbols:
the symbol using bits   1 0 0 1   transmitted onto the medium first
the symbol using bits   3 2 2 3   transmitted onto the medium second
the symbol using bits   5 4 4 5   transmitted onto the medium third
the symbol using bits   7 6 6 7   transmitted onto the medium last

#### 43	Operating Environment

The Infrared PHY will operate only in indoor environments. Infrared PHY interfaces can not be exposed to direct sun light. The Infrared PHY relies on reflected infrared energy and does not require a line-of-sight between emitter and receiver in order to work properly. The range and bit-error-rate of the system may vary with the geometry of the environment and with the natural and artificial illumination conditions.

#### 44	Operating Temperature Range

The temperature range for full operation compliance with the Infrared PHY is specified as 0 to 40 degrees centigrade.

### 45	PMD Transmit Specifications

The following sections describe the transmit functions and parameters associated with the Physical Medium Dependent sublayer.

#### 46	Transmitted Peak Optical Power

The peak optical power of an emitted pulse shall be 2 W ± 20% averaged over the pulse width.

#### 47	Basic Pulse Shape and Parameters

The basic pulse width measured between the 50% amplitude points, shall be 250 ns ± 10 ns. The pulse rise time, measured between the 10% and 90% amplitude points, shall be lower than or equal to 40 ns. The pulse fall time, measured between the 10% and 90% amplitude points, shall be lower than or equal to 40 ns. The edge pulse jitter, defined as the absolute deviation in time of the edge pulse from its correct position, shall be lower than or equal to 10 ns.

µ §
Figure 12-3: Basic Pulse Shape

#### 48	Emitter Radiation Pattern Mask

The emitter radiation pattern mask is defined in table 12-3. Position the conformant device in its recommended attitude. Define the conformant device axis as the axis passing through the emitter center and having the direction of the vertical from the floor. The mask represents the irradiance normalized to the total peak average emitted power, as a function of the angle between the conformant device axis and the axis from the emitter center to the test receiver center (declination angle). The distance between emitter and test receiver is 1 meter. The test receiver normal is always aimed at the emitter center. The azimuth angle is a rotation angle on the conformant device axis.

| DECLINATION ANGLE | NORMALIZED IRRADIANCE |
| --- | --- |
| a £ 60° | > 3.5e-6 |
| a £ 29° | £ 2.2e-5 |
| 29° < a £ 43° | £ -1.06e-4 + (0.44e-5) a |
| 43° < a £ 57° | £ 1.15e-4 - (7.1e-7) a |
| 57° < a £ 74° | £ 2.98e-4 - (3.9e-6) a |
| 74° < a £ 90° | £ 4.05e-5 - (4.5e-7) a |

Table 12-3: Definition of the Emitter Radiation Pattern Mask

A device is conformant if for any azimuth angle its radiation pattern as a function of declination angle falls within the pattern mask.

µ §

Figure 12-4: Emitter radiation pattern mask.

Other radiation patterns may be added at a later time.

#### 49	Optical Emitter Peak Wavelength

The optical emitter peak wavelength shall be between 850 and 950 nm.

#### 50	Transmit Spectrum Mask

Define the transmit spectrum of a transmitter as the Fourier Transform, or equivalent, of a voltage (or current) signal whose amplitude, as a function of time, is proportional to the transmitted optical power.

The transmit spectrum of a conformant transmitter shall be 20 dB below its maximum for all frequencies above 15 MHz. The transmit spectrum mask is shown in figure 12-5.

µ §

Figure 12-5: Transmit spectrum mask.

### 51	PMD Receiver Specifications

The following sections describe the receive functions and parameters associated with the Physical Medium Dependent sublayer.

#### 52	Receiver Sensitivity

The Receiver Sensitivity defined as the minimum irradiance (in mW/cm2) at the photodetector plane required for a FER (Frame Error Rate) of 4x10-5 with an PSDU MPDU of 512 octets and with  an unmodulated background IR source between 800 nm and 1000 nm with a level of 0.1 mW/cm2 , shall be:

1 MBPS: 2.x10-5mW/cm2

2 MBPS: 8.x10-5mW/cm2

[Editors note: for conformance testing we need to specify a noise source (a tungsten lamp seems a good candidate) and a calibrated receiver].

#### 53	Receiver Dynamic Range

The receiver dynamic range, defined as the ratio between the maximum and minimum irradiance at the plane normal to the receiver axis that assures a FER lower than or equal to 4x10-5 with an PSDU MPDU of 512 octets and with an unmodulated background IR source between 800 nm and 1000 nm with a level of 0.1 mW/cm2 , shall be greater or equal to 30 dB.

#### 54	Receiver Field-of-View (FOV)

Define the receiver axis as the direction of incidence of the optical signal at which the received optical power is maximum.

The received optical power shall be greater than the values given in the table below, at the angles indicated,  where "Angle of Incidence" is the angle of  incidence of the optical signal relative to the receiver axis, and "Received Power" is the received optical power as a percentage of that measured at the receiver axis.

| ANGLE OF INCIDENCE | RECEIVED POWER |
| --- | --- |
| a £ 20° | ³ 65% |
| a £ 40° | ³ 55% |
| a £ 60° | ³ 35% |
| a £ 80° | ³ 10% |

Table 12-4: Definition of the Receiver Field Of View

Define the receiver FOV as twice the angle measured between the receiver axis and the direction of incidence at which the received optical power is 50%  of the received optical power on the receiver axis. For incident angles smaller than half the FOV, the received optical power should always be higher than 50%  of the received power on the receiver axis.

The receiver FOV of a conformant receiver shall be greater than or equal to 90°

### 55	Clear Channel Assessment, Carrier Detect and Energy Detect Definitions

#### 56	Clear Channel Assessment

Clear Channel Assessment (CCA) shall will be asserted “CLEAR” by the PHY when the Carrier Detect Signal (CS) and the Energy Detect Signal (ED) are both false, or when ED has been continuously asserted for a period of time defined by the product of CCA_WATCHDOG_TIMER and CCA_WATCHDOG_COUNT without CS becoming active.  When either CS or ED go true, CCA is indicated as “BUSY” to the MAC via the primitive PHY_DATA.Indicate class=Start_of_Activity .

Normally, CCA will be held “BUSY” throughout the period of the PLCP Header.  After receiving the last PLCP bit and the first data octet, the PHY shall will signal PHY_DATA.Indicate class=Start_of_DATA with the parameters LENGTH and RATE.  CCA shall will be held “BUSY” until the number of octets specified in the decoded PLCP Header are received.  At that time the PHY shall will signal PHY_DATA.Indicate class=End_of_Data.   The CCA may remain "BUSY" after the end of data if some form of energy is still being detected.  The PHY will signal PHY_DATA.Indicate class=End_of_Activity only when the CCA goes "CLEAR".  If CCA remains “BUSY”, indicating some form of  the PHY will signal PHY_DATA.Indicate class=End_of_Activity only when CCA goes “CLEAR”.

The transition of CCA from "BUSY" to "CLEAR"  is indicated to the MAC via the primitive PHY_DATA.Indicate, class=End_of_Activity.

If CS and ED go false before the PHY signals PHY_DATA.Indicate class=DATA, CCA is set to “CLEAR” and immediately signaled to the MAC via PHY_DATA.Indicate class=End_of_Activity. If CS and ED go false after the PHY has signaled PHY_DATA.Indicate class=DATA implying that the PLCP Header has been properly decoded, then the PHY shall will not signal a change in state of CCA until the proper interval has passed for the number of bytes indicated by the received PLCP LENGTH.  At that time, the PHY shall will signal PHY_DATA.indicate class=End_of_Data. followed by PHY_DATA.indicate class= End_of_Activity.

The transition of  CCA from “CLEAR” to “BUSY” resets the timer and counter associated with CCA_WATCHDOG_TIMER and CCA_WATCHDOG_COUNT, respectively.  CCA_WATCHDOG_TIMER and CCA_WATCHDOG_COUNT are parameters available via MIB table entries and can be read and set via the LME.

Rise and fall times of CCA relative to the OR’ing of the CS and ED signals shall will be less than 30 nanoseconds.  CS and ED are both internal signals to the PHY and are not available directly to the MAC, nor are they defined at any exposed interface.

#### 57	Carrier Detect Signal

The Carrier Detect Signal (CS) is asserted by the PHY when it detects and locks on to  an incoming PLCP Preamble signal.  This signal is not directly available to the MAC.  Conforming PHY are required to assert this condition within the first 12 microseconds of signal reception, at the minimum a signal level  equal to the receiver sensitivity specified in 12.3.3.7 “Receiver Sensitivity”,  with a background IR level as specified in 12.3.3.7 “Receiver Sensitivity”.

The Carrier Detect Signal (CS) is de-asserted by the PHY when the receiving conformant device loses carrier lock.

Note that the 12 microseconds specification is somewhat less than the minimum length of PLCP SYNC interval which is 14.25 usec.

The Carrier Detect Signal (CS) shall operate independently of Energy Detect (ED) and shall not require a prior ED before the acquisition and assertion of CS.  This permits reception of signals at the minimum signal level specified in 12.3.3.7 “Receiver Sensitivity”, even though these signals fall below the ED level.

This signal is not directly available to the MAC.

#### 58	Energy Detect

The Energy Detection Signal (ED) is set true when IR energy variations in the band between 1 MHz and 10 MHz exceed  0.001 mW/cm2.

The Energy Detect (ED) shall operate independently of Carrier Detect Signal(CS).  ED shall not be asserted at the minimum signal level specified in 12.3.3.7 “Receiver Sensitivity”, which is below the level specified here..

This signal is not directly available to the MAC.

## 59	PHY Managed Objects

PHY Managed objects have default values, or allowed values which are PHY dependent.  This section describes those values, and further specifies whether they are permitted to vary from implementation to implementation.

This section does not provide the definition of the MIB objects, but only provides the Infrared PHY - specific values for the MIB Objects whose definitions and ASN.1 are in Section 9 of this Standard.

| PHY MIB Object | Default Value | Operational Semantics | Operational Behavior |
| --- | --- | --- | --- |
| aCCA_Rise_Time | 5 usec | Static | Identical for all conformant PHY |
| aCCA_Fall_Time | 1 usec | Static | Identical for all conformant PHY |
| aRxTx_Turnaround_Time | 0 usec | Static | Identical for all conformant PHY |
| aRx_Propagation_Delay | 1.5 usec | Static | Identical for all conformant PHY |
| aTx_Propagation_Delay | 3.5 usec | Static | Identical for all conformant PHY |
| aPLCP_Time | 60 usec 1mbps 40 usec 2mbps | Static | Identical for all conformant PHY |
| aPHY_SAP_Delay | 13.6 usec 1mbps 9.6 usec 2mbps | Static | Identical for all conformant PHY |
| aCCA_Watchdog_Timer_Max | implementation dependent | Dynamic | A conformant PHY may set this via the LME |
| aCCA_Watchdog_Count_Max | implementation dependent | Dynamic | A conformant PHY may set this via the LME |
| aCCA_Watchdog_Timer_Min | 22 ms. | Static | Identical for all conformant PHY |
| aCCA_Watchdog_Count_Min | 1 | Static | Identical for all conformant PHY |
| aChannel_Transit_Delay | 100 25 nsec | Static | Identical for all conformant PHY |
| aChannel_Transit_Variance | 100 25 nsec | Static | Identical for all conformant PHY |
| aPSDU_Maximum | 2500 octets | Static | Identical for all conformant PHY |
| aPSDU_Minimum | 0 octets | Static | Identical for all conformant PHY |
| aPSDU_Current_Maximum | aPSDU_Maximum implementation dependent | Static Dynamic | Identical for all conformant PHY A conformant PHY may set this via the LME |
| aSupported_Tx_Rates | implementation dependent | Static | All conformant PHY must include the value 000 (1 Mbps). |
| aSupported_Rx_Rates | implementation dependent | Static | All conformant PHY must include the values 000 (1 Mbps) and 001 (2 Mbps). |
| aBSS_Basic_Rate_Set | 000, 001 | Static | Identical for all conformant PHY |
| aStation_Basic_Rate | implementation dependent | Dynamic | A conformant PHY may set this via the LME |
| aExtended_Rate_Set | implementation dependent | Static | Rates not in the BSS_Basic_Rate_Set which are supported by the PHY |
| aPLCP_Rate | 000, 001 | Static | The PLCP rate must be a member of the BSS_Basic_Rate_Set, and if the data rate is also a member of the BSS_Basic_Rate_Set, then the PLCP_Rate is the same. |
| aPreferred_Tx_Rate | implementation dependent | Dynamic | A conformant PHY may set this via the LME. |
| aPreferred_Rx_Rate | implementation dependent | Dynamic | A conformant PHY may set this via the LME. |

## 60	PHY Parameters

This section does not provide the definition of the MIB objects, but only provides the Infrared PHY - specific values for the MIB Objects whose definitions and ASN.1 are in Section 9 of this Standard.

| Parameter | Value | Comments |
| --- | --- | --- |
| aCCA_Rise_Time | 5 msec maximum | conformant PHY may implement a faster CCA mechanism, but must present 5 msec in their MIB for interoperability |
| aCCA_Fall_Time | 1 msec maximum | conformant PHY may implement a faster CCA mechanism, but must present 1 msec in their MIB for interoperability |
| aRx_Propogation_Delay | 1.5 msec maximum | conformant PHY may implement a faster receive processing mechanism, but must present 1.5 msec in their MIB for interoperability |
| aTx_Propogation_Delay | 3.5 msec maximum | conformant PHY may implement a faster transmit processing mechanism, but must present 3.5 msec in their MIB for interoperability |