Adding Some Meaningful Status Codes

Aug. 31, 1995

Schaumburg IL

Author:	Tom Baumgartner

Abstract:

The Status Code sub-clause, 4.3.1.9, has not been fleshed out with meaningful items. This paper suggests some of the obvious codes that are useful. In addition, this work made it obvious that the Status Code field, as defined, does not belong in the Disassociation and Deauthentication Frames. A solution to this problem is proposed. [Bold comments in brackets are not part of the proposed text and are included for explanation only.]

Body:

Add the following Status Codes to sub-clause 4.3.1.9, Status Code:

Status Code	Meaning

10. 	CanÕt support all requested capabilities in Capability Information field

11. 	STA requesting (Re)Association is not Authenticated with responding STA

12. 	Reassociation is denied because canÕt confirm that Association exists

13. 	Association denied due to not recognizing the requesting STA as valid

14. 	Open System Authentication is not acceptable to the responding STA

15. 	Responding STA does not support the specified Authentication Algorithm

16. 	Received an Authentication Frame with Authentication Transaction Sequence Number out of expected sequence

17. 	Authentication rejected because of challenge failure

Status Code definition of sub-clause 4.3.1.9 says that the Status Code is an indication of the success or failure of an operation. In the Disassociation and the Deauthentication Frames there is no operation that has happened that can be reported upon; Disassociation and Deauthentication are in fact commands to someone to take an action, similar to the Association Request. It was suggested by an random body of Schaumburg attendees that a Reason Code should be created for use in the Disassociation and Deauthentication Frames. This paper makes no attempt to list meaningful Reason Codes.