IEEE 802.11

Wireless Access Method and Physical Specification

Proposed Text for Section 7 (Layer Management)

July 11, 1995

| Greg Ennis Symbol Technologies, Inc. | Bob O’Hara Advanced Micro Devices | Mike Trompower Aironet Wireless Communications |
| --- | --- | --- |
| Dean Kawaguchi Symbol Technologies, Inc. | Michael Fischer Digital Ocean Inc. |  |

# 1

# 2

# 3

# 4

# 5

# 6

7

Background

Layer Management section:

was Section 6 (Draft 1.1), now Section 7 (Draft 1.2)

Section currently has no content

Small group volunteered at Salt Lake City meeting to work on this

Goal: develop basic framework consistent with other sections

Layer Management Entities and Service Access Points

GET and SET Operations

GET/SET.REQUEST and CONFIRM

XX = PM or MM:

XX_GET.request (MIB_parameter)

XX_GET.confirm (status, MIB_parameter, 							  MIB_parameter_value)

XX_SET.request (MIB_parameter, MIB_parameter_value)

XX_SET.confirm (status, MIB_parameter)

Other Primitives

Everything could be represented as MIB operations

May be more natural to represent certain operations with dedicated primitives

- Example: complex operations involving multiple parameters

Different PHYs currently take different approaches

Current PHY Management Primitives (Non-MIB)

FH:

Reset, Power, SetChannel

DS:

Reset

IR:

(None)

Reset

Applicable to both MAC and PHY

MM_RESET.request:

MM_RESET.confirm

PM_RESET.request:

PM_RESET.confirm

MAC Management Non-MIB Primitives

MM_SCAN.request (scan_parameters)

MM_SCAN.confirm

MM_POWER_MGT.request (power_mgt_parameters)

MM_POWER_MGT.confirm

Summary: PHY Management SAP Primitives

PM_GET.request

PM_GET.confirm

PM_SET.request

PM_SET.confirm

PM_RESET.request

PM_RESET.confirm

Summary: MAC Management SAP Primitives

MM_GET.request			MM_SCAN.request

MM_GET.confirm			MM_SCAN.confirm

MM_SET.request			MM_POWER_MGT.request

MM_SET.confirm			MM_POWER_MGT.confirm

MM_RESET.request

MM_RESET.confirm

Motion: accept Document 11-95/118 for inclusion in the 802.11 draft.