IEEE 802.11

Wireless Access Methods and Physical Layer Specifications

TITLE:                Proposed Text Change for MIB Variables

Section 9 of Draft 1

DATE:		            July 10-13th,1995

AUTHOR:                        Ed Geiger

Apple Computer

One Infinite Loop

Cupertino, CA 95014

edg@apple.com

Introduction

At the last joint PHY meeting a number of changes were made to the Managed Object list.  The intent of these changes were to define a common list of MIB parameters to be used by the MAC and LME when interfacing to the PHY and PHY LME.  This contribution provides the updates for section 9 which are in concert with the efforts in the joint PHY.

# Physical Layer Management

This section covers those  the attribute definitions and templates aspects of PHY layer management. which are independent of the specific PHY type.  This includes both the PHY-dependent and the PHY-independent portions of the PHY-layer Management Information Base.  PHY-specific MIB definitions are included within the subsequent PHY-specific layer descriptions.  Not all attributes in the following section are supportted by every PHY.  Each PHY contains a Managed Object list which defines the PHY-specific values required for each PHY implementation.

## PHY-Independent PHY Management Information Base

### Phy Attributes

#### agPhyOperation_grp

aCCA_Rise_Time,
aCCA_Fall_Time,
aPHY_Type,
aReg_Domains_Suprt,
aCurrent_Reg_Domain,
aSlot_time,
aCCA_Asmnt_Time,
aRxTx_Turnaround_Time,
aTxRx_Turnaround_Time,
aRx_Propagation_Delay,
aTx_Propagation_Delay,
aPHY_SAP_Delay,
aPLCP_Time,
aTx_PLCP_Delay,
aRxTx_Switch_Time,
aTxRamp_On_time,
aTx_RF_Delay,
aSIFS_Time,
aRx_RF_Delay,
aRx_PLCP_Delay,
aMAC_Prc_Delay,
aTxRamp_Off_Time,
aCCA_Watchdog_Timer_Max,
aCCA_Watchdog_Count_Max,
aCCA_Watchdog_Timer_Min,
aCCA_Watchdog_Count_Min,
aChannel_Transit_Delay,
aChannel_Transit_Variance,
aMPDU_Maximum,
aMPDU_Minimum,
aMPDU_Current_Maximum;

#### agPhyRate_grp

aSuprt_Data_Rates,
aMPDU_Max_Lngth_1M,
aMPDU_Max_Lngth_2M,
aSupported_Tx_Rates,
aSupported_Rx_Rates,
aBSS_Basic_Rate_Set,
aStation_Basic_Rate,
aExtended_Rate_Set,
aPLCP_Rate,
aPreferred_Tx_Rate,
aPreferred_Rx_Rate;

#### agPhyAntenna_grp

aSuprt_Tx_Antennas,
aCurrent_Tx_Antenna,
aSuprt_Rx_Antennas,
aDiversity_Suprt,
aDiversity_Slct_Rx;

#### agPhyTxPwr_grp

aNbr_Suprtd_Pwr_Lvls,
aTx_Pwr_Lvl1,
aTx_Pwr_Lvl2,
aTx_Pwr_Lvl3,
aTx_Pwr_Lvl4,
aTx_Pwr_Lvl5,
aTx_Pwr_Lvl6,
aTx_Pwr_Lvl7,
aTx_Pwr_Lvl8,
aCurrent_Tx_PwrLvl;

#### agPhyFHSS_grp

aHop_Time,
aCurrent_Channel_Nbr,
aMax_Dwell_Time,
aCurrent_Dwell_Time,
aCurrent_Set,
aCurrent_Pattern,
aCurrent_Index;

#### agPhyStatus_grp

aSynthesizer_Locked;

#### Not Grouped

none

### PHY Object Class

PHY MANAGED OBJECT CLASS

DERIVED FROM "ISO/IEC 10165-2":top;

CHARACTERIZED BY

pPHY_base		PACKAGE

BEHAVIOR

bPHY_base BEHAVIOR

DEFINED AS "The PHY object class provides the necessary support for the timing  all the required PHY operational information, and rate change information, which may vary from PHY to PHY and from STA to STA to be communicated to upper layers."

ATTRIBUTES

aCCA_Rise_Time	GET,

aCCA_Fall_Time	GET,

aPHY_Type	GET,
aReg_Domains_Suprt	GET,
aCurrent_Reg_Domain	GET-REPLACE,
aSlot_Time	GET,
aCCA_Asmnt_Time	GET,
aRxTx_Turnaround_Time	GET,
aTx_PLCP_Delay	GET,
aRxTx_Switch_Time	GET,
aTxRamp_On_Time	GET,
aTx_RF_Delay	GET,
aSIFS_Time	GET,
aRx_RF_Delay	GET,
aRx_PLCP_Delay	GET,
aMAC_Prc_Delay	GET,
aTxRamp_Off	GET,
aTxRx_Turnaround_Time	GET,
aRx_Propagation_Delay	GET,
aTx_Propagation_Delay	GET,
aPHY_SAP_Delay	GET,

aPLCP_Time	GET,
aCCA_Watchdog_Timer_Max	GET-REPLACE,
aCCA_Watchdog_Count_Max	GET-REPLACE,
aCCA_Watchdog_Timer_Min	GET,
aCCA_Watchdog_Count_Min	GET,
aChannel_Transit_Delay	GET,
aChannel_Transit_Variance	GET,
aMPDU_Maximum	GET,
aMPDU_Minimum	GET,
aMPDU_Current_Maximum	GET-REPLACE,

aSuprt_Data_Rates	GET,
aMPDU_Max_Lngth_1M	GET,
aMPDU_Max_Lngth_2M	GET,
aSupported_Rx_Rates	GET,
aSupported_Tx_Rates	GET,
aBSS_Basic_Rate_Set	GET,
aStation_Basic_Rate	GET-REPLACE,
aExtended_Rate_Set	GET,
aPLCP_Rate	GET,
aPreferred_Tx_Rate	GET-REPLACE,
aPreferred_Rx_Rate	GET-REPLACE,

aSuprt_Tx_Antennas	GET,
aCurrent_Tx_Antenna	GET-REPLACE,
aSuprt_Rx_Antennas	GET,
aDiversity_Suprt	GET,
aDiversity_Slct_Rx	GET-REPLACE,
aNbr_Suprt_Pwr_Lvls	GET,
aTx_Pwr_Lvl_1	GET,
aTx_Pwr_Lvl_2	GET,
aTx_Pwr_Lvl_3	GET,
aTx_Pwr_Lvl_4	GET,
aTx_Pwr_Lvl_5	GET,
aTx_Pwr_Lvl_6	GET,
aTx_Pwr_Lvl_7	GET,
aTx_Pwr_Lvl_8	GET,
aCurrent_Tx_PwrLvl	GET-REPLACE,
aHop_Time	GET,
aCurrent_Channel_Nbr	GET-REPLACE,
aMax_Dwell_Time	GET,
aCurrent_Dwell_Time	GET-REPLACE,
aCurrent_Set	GET-REPLACE,
aCurrent_Pattern	GET-REPLACE,
aCurrent_Index	GET-REPLACE,
aSynthesizer_Locked	GET;

ATTRIBUTE GROUPS

agPhyOperation_grp,

agPhyRate_grp,

agPhyAntenna_grp,

agPhyTxPwr_grp,

agPhyFHSS_grp,

agPhyStatus_grp;

ACTIONS

acPHY_init,

acPHY_reset;

NOTIFICATIONS

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) };

### PHY Attribute Group Templates

#### agPhyOperation_grp

PhyOperation_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aCCA_Rise_Time,
aCCA_Fall_Time,
aPHY_Type,
aReg_Domains_Suprt,
aCurrent_Reg_Domain,
aSlot_time,
aCCA_Asmnt_Time,
aRxTx_Turnaround_Time,
aTxRx_Turnaround_Time,
aRx_Propagation_Delay,
aTx_Propagation_Delay,
aPHY_SAP_Delay,
aPLCP_Time,
aTx_PLCP_Delay,
aRxTx_Switch_Time,
aTxRamp_On_time,
aTx_RF_Delay,
aSIFS_Time,
aRx_RF_Delay,
aRx_PLCP_Delay,
aMAC_Prc_Delay,
aTxRamp_Off_Time,
aCCA_Watchdog_Timer_Max,
aCCA_Watchdog_Count_Max,
aCCA_Watchdog_Timer_Min,
aCCA_Watchdog_Count_Min,
aChannel_Transit_Delay,
aChannel_Transit_Variance,
aMPDU_Maximum,
aMPDU_Minimum,
aMPDU_Current_Maximum;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(3) PhyOperation_grp(0) };

#### agPhyRate_grp

PhyRate_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aSuprt_Data_Rates,
aMPDU_Max_Lngth_1M,
aMPDU_Max_Lngth_2M,
aSupported_Tx_Rates,
aSupported_Rx_Rates,
aBSS_Basic_Rate_Set,
aStation_Basic_Rate,
aExtended_Rate_Set,
aPLCP_Rate,
aPreferred_Tx_Rate,
aPreferred_Rx_Rate;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(3) PhyRate_grp(1) };

#### agPhyAntenna_grp

PhyAntenna_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aSuprt_Tx_Antennas,
aCurrent_Tx_Antenna,
aSuprt_Rx_Antennas,
aDiversity_Suprt,
aDiversity_Slct_Rx;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(3) PhyAntenna_grp(1) };

#### agPhyTxPwr_grp

PhyTxPwr_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aNbr_Suprtd_Pwr_Lvls,
aTx_Pwr_Lvl1,
aTx_Pwr_Lvl2,
aTx_Pwr_Lvl3,
aTx_Pwr_Lvl4,
aTx_Pwr_Lvl5,
aTx_Pwr_Lvl6,
aTx_Pwr_Lvl7,
aTx_Pwr_Lvl8,
aCurrent_Tx_PwrLvl;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(3) PhyTxPwr_grp(1) };

#### agPhyFHSS_grp

PhyFHSS_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aHop_Time,
aCurrent_Channel_Nbr,
aMax_Dwell_Time,
aCurrent_Dwell_Time,
aCurrent_Set,
aCurrent_Pattern,
aCurrent_Index;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(3) PhyFHSS_grp(1) };

#### agPhyStatus_grp

PhyStatus_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aSynthesizer_Locked;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(3) PhyStatus_grp(1) };

### PHY Attribute Templates

#### aCCA_Rise_Time

CCA_Rise_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"Time (in hundreds of nanoseconds) from the disappearance of a transmission in the medium to the assertion of Clear Channel.  The decay time of the CCA detector is included in this value.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) CCA_Rise_Time(11) };

#### aCCA_Fall_Time

CCA_Fall_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"Time (in hundreds of nanoseconds) from the appearance of a transmission in the medium to the assertion of Busy Channel.  The attack time of the CCA detector is included.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) CCA_Fall_Time(22) };

#### aPHY_Type

PHY_Type ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This is a 8 bit integer value which identifies the PHY Type supported by the attached PLCP and PMD"

FHSS 2.4GHz = 01 , DSSS 2.4GHz = 02, IR Baseband = 03";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) PHY_Type(33) };

#### aReg_Domains_Suprt

Reg_Domains_Suprt ATTRIBUTE

WITH APPROPRIATE SYNTAX

Null Terminated list of byte integers;

BEHAVIOR DEFINED AS

"There are different operational requirements dependent on the regulatory domain.  This attribute list describes the regulatory domains the PLCP and PMD support in this implementation.   Each integer is an 8 bit value as defined below:

FCC = 10, DOC = 20, ETSI = 30, MKK = 40";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Reg_Domains_Suprt(33) };

#### aCurrent_Reg_Domain

Current_Reg_Domain ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This octet defines the current regulatory domain this  implementation of the PMD is supportting.  This octet corresponds to one of the Reg_Domains list in the Reg_Domains_Suprt attribute list";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Current_Reg_Domain(33) };

#### aSlot_Time

Slot_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The time in microseconds the MAC will use for defining the PIFS and DIFS periods.  The Slot_Time is defined as a function of the following the equation:

CCA_Asmnt_Time + RxTx_Turnaround_Time + Air_Propagation_Time.

Air_Propagation_Time is defined as 1 usec.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Slot_Time(33) };

#### aCCA_Asmnt_Time

CCA_Asmnt_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The minimum time in microseconds the CCA mechanism has available to assess the media within every slot to determine whether the media is clear or busy";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) CCA_Asmnt_Time(33) };

#### aRxTx_Turnaround_Time

RxTx_Turnaround_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"Minimum required time (in hundreds of nanoseconds) between the PHY_DATA.indicate(END_OF_ACTIVITY) and the earliest possible transmission of a new packet.  This does not include any recovery time required by the receiver. The maximum time in mircoseconds the PHY requires to change from receive to transmiting the start of the first symbol out on the air.  The following equation is used to derive the RxTx_Turnaround_Time:

aTx_PLCP_Delay + aRxTx_Switch_Time + aTxRamp_On_Time + aTx_RF_Delay. ";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) RxTx_Turnaround_Time(44) };

#### aTxRx_Turnaround_Time

TxRx_Turnaround_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"Minimum required time (in hundreds of nanoseconds) between the transmission of the last symbol of an outgoing frame and the recovery of the receiver to within 3 dB of its nominal gain and noise performance parameters.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) TxRx_Turnaround_Time(44) };

#### aRx_Propagation_Delay

Rx_Propagation_Delay ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The electronic delay time (in hundreds of nanoseconds) in the receiver from the reception of a signal at the antenna to a PHY_DATA.indicate. Includes the conversion of eight bits from a serial stream to an octet. Startup delays introduced by the PLCP header are not included in this value."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Rx_Propagation_Delay(55) };

#### aTx_Propagation_Delay

Tx_Propagation_Delay ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The electronic delay time (in hundreds of nanoseconds) in the transmitter from the reception of a PHY_DATA.indicate to the generation of a signal at the antenna. Startup delays introduced by the PLCP header are not included in this value."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Propagation_Delay(66) };

#### aPHY_SAP_Delay

PHY_SAP_Delay ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The delay introduced (in hundreds of nanoseconds) from PHY_DATA.request(class DATA) to PHY_DATA.indicate(class DATA) in a peer PHY.  This includes propagation delays, channel delays, and paallel-serial-parallel conversion of information."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) PHY_SAP_Delay(77) };

#### aPLCP_Time

PLCP_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of integer;

BEHAVIOR DEFINED AS

"The set of times required (in hundreds of nanoseconds) by the PHY to construct and transmit the PLCP header and the PLCP corresponding to each aPLCP_Rate.  Does not include aTx_Propagation_Delay.  The receive time is assumed to be identical to or less than the transmit time."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) PLCP_Time(88) };

#### aTx_PLCP_Delay

Tx_PLCP_Delay ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The nominal time in microseconds the PLCP uses to deliver a symbol from the MAC interface to the tranmit data path of the PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_PLCP_Delay(88) };

#### aRxTx_Switch_Time

RxTx_Switch_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The nominal time in mircoseconds the PMD takes to switch from Receive to Transmit";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) RxTx_Switch_Time(88) };

#### aTxRamp_On_Time

TxRamp_On_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The maximum time in microseconds the PMD takes to turn the Transmitter on";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) TxRamp_On_Time(88) };

#### aTx_RF_Delay

Tx_RF_DelayWITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The nominal time in nanoseconds the PMD uses to transfer a symbol through the Transmit path of the PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_RF_Delay(88) };

#### aSIFS_Time

SIFS_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The time in microseconds the MAC and PHY will require to receive the last symbol of a frame at the air interface, process the frame and response with the first symbol on the air interface of the earliest possible response.  The following equation is used to determine the SIFS_Time:

aRx_RF_Delay + aRx_PLCP_Delay + aMAC_Prc_Delay +

aRxTx_Turnaround_Time";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) SIFS_Time(88) };

#### aRx_RF_Delay

Rx_RF_Delay ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The nominal time in nanoseconds the PMD uses to deliver a symbol from the antenna to the PLCP.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Rx_RF_Delay(88) };

#### aRx_PLCP_Delay

Rx_PLCP_Delay ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The nominal time in nanoseconds the PLCP uses to deliver a bit from the PMD receive path to the MAC";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Rx_PLCP_Delay(88) };

#### aMAC_Prc_Time

MAC_Prc_Delay ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The nominal time in miroseconds the MAC uses to process a frame and prepare a response to the frame";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) MAC_Prc_Delay(88) };

#### aTxRamp_Off_Time

TxRamp_Off_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PMD takes to turn the Transmit PA off";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) TxRamp_Off_Time(88) };

#### aCCA_Watchdog_Timer_Max

CCA_Watchdog_Timer_Max ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The time (in hundreds of nanoseconds) at which the watchdog timer in the PMD CCA mechanism generates an alarm. The product of aCCA_Watchdog_Timer_Max and the aCCA_Watchdog_Count_Max must be set to match the length of the maximum length undecoded packet that is to be allowed time to operate."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) CCA_Watchdog_Timer_Max(99) };

#### aCCA_Watchdog_Count_Max

CCA_Watchdog_Count_Max ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The count of alarms which the watchdog counter in the PMD CCA is permitted to generate before an undecoded energy detection in the medium is ignored."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) CCA_Watchdog_Count_Max(1010) };

#### aCCA_Watchdog_Timer_Min

CCA_Watchdog_Timer_Min ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The minimum time (in hundreds of nanoseconds) to which the aCCA_Watchdog_Timer_Max may be set."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) CCA_Watchdog_Timer_Min(1111) };

#### aCCA_Watchdog_Count_Min

CCA_Watchdog_Count_Min ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The minimum time (in hundreds of nanoseconds) to which the aCCA_Watchdog_Count_Max may be set."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) CCA_Watchdog_Count_Min(1212) };

#### aChannel_Transit_Delay

Channel_Transit_Delay ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"Antenna to antenna signal transit time measured in hundreds of nanoseconds."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Channel_Transit_Delay (1313) };

#### aChannel_Transit_Variance

Channel_Transit_Variance ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"Expected variance in the channel transit time, measured in hundreds of nanoseconds."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Channel_Transit_Variance(1414) };

#### aSupported_Rx_Rates

Supported_Rx_Rates ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of integer;

BEHAVIOR DEFINED AS

"This attribute shall be a set of all the reception rates supported by the PHY.  The values of the numbers for the reception rates are defined by each PHY."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Supported_Rx_Rates (1515) };

#### aSupported_Tx_Rates

Supported_Tx_Rates ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of integer;

BEHAVIOR DEFINED AS

"This attribute shall be a set of all the transmission rates supported by the PHY.  The values of the numbers for the reception rates are defined by each PHY."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Supported_Tx_Rates (1616) };

#### aBSS_Basic_Rate_Set

BSS_Basic_Rate_Set ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of integer;

BEHAVIOR DEFINED AS

"This attribute shall be a set of all the reception rates that must be supported by all PHY in the BSS of which the STA is a member.  The values of the numbers for the reception rates are defined by each PHY."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) BSS_Basic_Rate_Set (1717) };

#### aStation_Basic_Rate

Station_Basic_Rate ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute shall be the transmission rate, preferred by the PHY in this STA. This rate must be a member of aBSS_Basic_Rate_Set.  The values of the numbers for the reception rates are defined by each PHY."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Station_Basic_Rate (1818) };

#### aExtended_Rate_Set

Extended_Rate_Set ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of integer;

BEHAVIOR DEFINED AS

"This attribute shall be a set of all the reception rates supported by the PHY, which rates are not also a member of the aBSS_Basic_Rate_Set."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Extended_Rate_Set (1919) };

#### aPLCP_Rate

PLCP_Rate ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of integer;

BEHAVIOR DEFINED AS

"This attribute shall be the set of rates at which the PHY in this STA will always transmit the PLCP header.  These values must be members of the aBSS_Basic_Rate_Set.  The values of the numbers for the rates are defined by each PHY."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) PLCP_Rate (2020) };

#### aPreferred_Tx_Rate

Preferred_Tx_Rate ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute shall be the distinguished rate at which the PHY in this STA would prefer to transmit in the data phase.  This value is not constrained to be a member of the aBSS_Basic_Rate_Set.  The values of the numbers for the reception rates are defined by each PHY."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Preferred_Tx_Rate (2121) };

#### aPreferred_Rx_Rate

Preferred_Rx_Rate ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute shall be the distinguished rate at which the PHY in this STA would prefer to receive in the data phase.  This value is not constrained to be a member of the aBSS_Basic_Rate_Set.  The values of the numbers for the reception rates are defined by each PHY."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Preferred_Rx_Rate (2222) };

#### aMPDU_Maximum

MPDU_Maximum ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute shall be the maximum MPDU, in octets,  that the PHY shall ever be capable of  accepting.  This maximum MPDU is distinguished from the maximum size which is currently acceptable to the PHY."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) MPDU_Maximum (2323) };

#### aMPDU_Minimum

MPDU_Minimum ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute shall be the Minimum MPDU, in octets,  that the PHY shall ever be capable of  accepting.  "

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) MPDU_Minimum (2424) };

#### aMPDU_Current_Maximum

MPDU_Current_Maximum ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute shall be the current maximum MPDU, in octets,  that the PHY is capable of  accepting at the time the object is inspected.  The value shall always be greater than or equal to aMPDU_Minimum, and less than or equal to aMPDU_Maximum.  The PHY shall replace this value at it's discretion as the maximum deliverable MPDU changes."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) MPDU_Current_Maximum(2525) };

#### aSuprt_Data_Rates

Suprt_Data_Rates ATTRIBUTE

WITH APPROPRIATE SYNTAX

Null Terminated list of byte integers;

BEHAVIOR DEFINED AS

"The bit rates supported by the PLCP and PMD.  The following list defines the supported data rates:

1M bps = 01, 2M bps = 02";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Suprt_Data_Rates (2424) };

#### aMPDU_Max_Lngth_2M

MPDU_Max_Lngth_2M ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The maximum number of octets in a MPDU that can be load into the PLCP_PDU when transmitting at 2M bit per second";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) MPDU_Max_Lngth_2M (2424) };

#### aSuprt_Tx_Antennas

Suprt_Tx_Antennas ATTRIBUTE

WITH APPROPRIATE SYNTAX

Null terminated list of integers;

BEHAVIOR DEFINED AS

"This attribute represent a list of one or more antennas which can be used as the tranmit antenna.  Each antenna is define as an integer starting with antenna 1 to antenna N where N is < or = 255";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Suprt_Tx_Antennas (2424) };

#### aCurrent_Tx_Antenna

Current_Tx_Antenna ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The current antenna being used to transmit.  This value is one of the attributes appearing in the list defined as the Suprt_Tx_Antennas.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) MPDU_Max_Lngth_1M (2424) };

#### aSuprt_Rx_Antennas

Suprt_Rx_Antennas ATTRIBUTE

WITH APPROPRIATE SYNTAX

Null terminated list of integers;

BEHAVIOR DEFINED AS

"This attribute represent a list of one or more antennas which can be used as the tranmit antenna.  Each antenna is define as an integer starting with antenna 1 to antenna N where N is < or = 255.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Suprt_Rx_Antennas (2424) };

#### aDiversity_Suprt

Diversity_Suprt ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute defines this implementation's support for diversity.  This attribute can have one of three values.  01h indicates that diversity is available and performed over a fixed list of antennas define in attribute Diversity_Slct_Rx.  02h indicates that diversity is not supportted.  03h indicates that diversity is supportted and that control of diversity is also available.  Diversity control indicates that the Diversity_Slct_Rx can be dynamically modified by the LME.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) MPDU_Max_Lngth_1M (2424) };

#### aDiversity_Slct_Rx

Diversity_Slct_Rx ATTRIBUTE

WITH APPROPRIATE SYNTAX

Null terminated list of integers;

BEHAVIOR DEFINED AS

"This attribute represent a list of one or more antennas which can be used as receive antennas.  Each antenna is define as an integer starting with antenna 1 to antenna N where N is < or = 255.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Diversity_Slct_Rx (2424) };

#### aNbr_Suprt_Pwr_Lvls

Nbr_Suprt_Pwr_Lvls ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the number of power levels supported by the PMD.  This attribute can have a value of 1 to 8.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Nbr_Suprt_Pwr_Lvls (2424) };

#### aTx_Pwr_Lvl_1

Tx_Pwr_Lvl_1 ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL1 in mWatts.  This is also the default power level.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Pwr_Lvl_1 (2424) };

#### aTx_Pwr_Lvl_2

Tx_Pwr_Lvl_2 ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL2 in mWatts.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Pwr_Lvl_2 (2424) };

#### aTx_Pwr_Lvl_3

Tx_Pwr_Lvl_3 ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL3 in mWatts.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Pwr_Lvl_3 (2424) };

#### aTx_Pwr_Lvl_4

Tx_Pwr_Lvl_4 ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL4 in mWatts.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Pwr_Lvl_4 (2424) };

#### aTx_Pwr_Lvl_5

Tx_Pwr_Lvl_5 ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL5 in mWatts.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Pwr_Lvl_5 (2424) };

#### aTx_Pwr_Lvl_6

Tx_Pwr_Lvl_6 ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL6 in mWatts.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Pwr_Lvl_6 (2424) };

#### aTx_Pwr_Lvl_7

Tx_Pwr_Lvl_7 ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL7 in mWatts.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Pwr_Lvl_7 (2424) };

#### aTx_Pwr_Lvl_8

Tx_Pwr_Lvl_8 ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL8 in mWatts.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Tx_Pwr_Lvl_8 (2424) };

#### aCurrent_Tx_PwrLvl

Current_Tx_PwrLvl ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The Tx_Pwr_Lvl_N currently being used to transmit data.  Some PHYs also use this value to determine the receiver sensitivity requirements for CCA. ";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Current_Tx_PwrLvl (2424) };

#### aHop_Time

Hop_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds for the PMD to change from channel 2 to 80";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Hop_Time (2424) };

#### aCurrent_Channel_Nbr

Current_Channel_Nbr ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This defines the current channel number of the frequency loaded in the RF synthesizer";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Current_Channel_Nbr (2424) };

#### aMax_Dwell_Time

Max_Dwell_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The maximum time in nanoseconds that the radio can operate on a single channel";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Max_Dwell_Time (2424) };

#### aCurrent_Dwell_Time

Current_Dwell_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"The current time in nanoseconds that the radio shall operate on a single channel set by the MAC";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Current_Dwell_Time (2424) };

#### aCurrent_Set

Current_Set ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute represents the current set of patterns the PHY LME is using to determine the hop sequence.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Current_Set (2424) };

#### aCurrent_Pattern

Current_Pattern ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute represents the current pattern the PHY LME is using to determine the hop sequence.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Current_Pattern (2424) };

#### aCurrent_Index

Current_Index ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This attribute represents the current index value the PHY LME is using to determine the Current_Channel_Nbr.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Current_Index (2424) };

#### aSynthesizer_Locked

Synthesizer_Locked ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOR DEFINED AS

"This is an indication that the PMD's synthesizer is locked to the current channel speciifed in the Current_Channel_Nbr.  00h represents unocked while FFh represents locked. ";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(3) attribute(7) Synthesizer_Locked (2424) };

A default product of 22 msec allows time for 802.11 type maximal length packets.