IEEE P802.11

Wireless Access Method and Physical Layer Specification

Comment on RTS/CTS Intellectual Property Issue

Michael Fischer
Digital Ocean, Inc.
4242Ð3 Medical Drive
San Antonio, TX  78229

Telephone:  +1Ð210Ð614Ð4096
Facsimile:   +1Ð210Ð614Ð8192
email:   mfischer@CHILD.com

# Abstract

An 802.11 D1 Letter Ballot comment by Ed Geiger on Section 5.2 (postponed for later discussion at the March, 1995 meeting, see document 95/067) identifies a potential intellectual property issue over the RTS/CTS mechanism proposed for use in the 802.11 MAC.  This submission discusses the scope of the subject patent, and states the reasons which the submitter believes that no conflict exists.

# Desired Outcome

Resolution or this letter ballot comment, which calls for removal of RTS/CTS mechanism from the standard, as Ònot acceptedÓ due to nonÐrelevance of the cited intellectual property to the 802.11 MAC as currently defined.

# Disclaimer

The author of this submission is not an attorney, and this document is not a legal opinion concerning the scope of the patented subject matter nor the relevance of this patent to any proposal under consideration by the P802.11 working group.  The statements herein regarding the scope of the patent are technical opinions from an individual who is familiar with the patented subject matter through the use of Apple Computer equipment and AppleÕs published documentation, and is familiar with technical interpretation of networking patents from other work as a technical expert on several patent infringement cases.

# Background

In 1993, Apple Computer identified U.S. patents 4,661,902 (Òthe Ô902 patentÓ) and 4,689,786 (Òthe Ô786 patent) as potentially relevant to proposals before P802.11.  Both of these patents were filed on March 21, 1985, based on the same specification.  This specification describes the physical and link layer implementation of AppleÕs LocalTalkª network.  The abstracts mention the use of a Òthree step handshake, wherein the sending agent transmits an ÒRTSÓ signal and within a predetermined time must receive a ÒCTSÓ signal from the receiving agent.Ó

In the D1 letter ballot, Ed Geiger made the comment ÒRemove the usage of RTS/CTS in the standardÓ with the stated reason that ÒApple Computer supplied the committee with a statement which indicated that the RTS/CTS reservation mechanism may infringe upon a specific patent.  Apple has never submitted any licensing statement regarding the use of any of their patented technology which might appear in the StandardÓ  [doc 95/18-5/1R1, March, 1995, p. 33].

Resolution of this comment was Òpostponed to later discussion.Ó  This submission provides a proposed basis upon which to resolve this comment.

# The Ô786 Patent

U.S. Patent 4,689,786 is entitled ÒLocal Area Network with Self Assigned Address Method.Ó  While both the abstract and specification of this patent mention the use of RTS/CTS, the claims do not ÐÐ so it can be assumed that the letter ballot comment, which did not indicate a specific patent, is not referring to Ô786.  An additional basis for nonÐrelevance is that, of this patentÕs 19 claims, only 2 are independent claims, and those each begin by requiring an apparatus or method Ò... for assigning a unique address to a data processing device ...Ó Ñ which is not done by any IEEE 802 LAN.

# The Ô902 Patent

U.S. Patent 4,661.902 is entitled ÒLocal Area Network with Carrier Sense Collision Avoidance.Ó  Dependent claims under two of the three independent claims of this patent refer to the use of RTS and CTS frames, wherein the absence of the later in a defined IFG time is used to generate a Òcollision assumptionÓ or a Òcollision signal.Ó  Read in isolation, these claims appear broadly to cover the use of the RTS/CTS mechanism on a LAN.  However, by 1985, when this patent application was filed, the various mechanisms mentioned in the claims were in widespread use on other networks.  Therefore, for these claims to be valid (rather than being invalid due to ÒobviousnessÓ or Òprior useÓ) they must be interpreted quite narrowly.  This narrow interpretation does not cover the 802.11 MAC protocol, because 802.11Õs use of RTS/CTS differs in several, distinct areas from LocalTalkÕs use of RTS/CTS.

Certainly the Ô902 patent is not a fundamental patent:

- CSMA dates back to the Aloha network in the early 1970s;

- collision avoidance was used on Network SystemsÕ HyperChannelª & HyperBusª, developed in the midÐ1970s, as well as on several other networks (mostly Aloha derivatives) operational well before 1984;

- RTS and CTS packets (rather than discrete, outÐofÐband RTS & CTS signals) were used in wide area networks at least as far back as some of the first airline reservation systems in the midÐ1960s, and on local area networks since the introduction of ARCNET¨ in 1977; and

- synchronization pulses at the beginning of frames are used in ARCNET (as well as many other preÐ1984 LANs), and the use of missing clock transitions to render the synchronization signal unique dates back, at least, to the early 1970s with the IBM 3274 Control Unit/Terminal signaling scheme.

Of greatest significance to possible applicability to the 802.11 MAC are the instances of prior art using RTS/CTS frames.

## Prior Use of RTS/CTS Frames

The Ô902 patent claims an RTS frame Òincluding at least one synchronization flag byte having a predefined bit sequences and a type field containing an RTS byteÓ (claims 7 & 30), and a CTS frame Òincluding at least one synchronization flag byte and a type field containing a CTS byteÓ (claims 9 & 32).  An almost identical mechanism, with different nomenclature, is used by ARCNET.  From a prior art standpoint, ARCNET constitutes public disclosure of RTS/CTS, not later than 1983.  Datapoint Corp. never filed any patent applications on ARCNET.  General information on the ARCNET protocol appeared in the descriptive literature distributed at the time of original product introduction (1977, this author received such information at the Datapoint booth of the 1977 National Computer Conference).  Some aspects of the ARCNET protocol remained Datapoint trade secrets until 1981, when Standard Microsystems Corp. (SMC) was licensed to sell the ARCNET controller chip as a standard product.  The ARCNET MAC (and PHY) protocols were described in the data sheets and application notes on SMCÕs ÒCOM9026,Ó some of which were published in 1981 and 1982.  An detailed description of the ARCNET MAC appeared in the ARCNET DesignerÕs Handbook, the first edition of which was published by Datapoint in 1983 (publication #61610).  This handbook was widely distributed, initially at no charge, later for a nominal fee.

ARCNET uses a 4Ðstage frame exchange sequence for directed data packets:  (1) The ÒRTSÓ frame, which is called a ÒFree Buffer EquiryÓ (FBE), and is comprised of an Alert Burst (establishes synchronization), an ENQ frame type byte, and a destination address.  (2) The ÒCTSÓ frame, which is called an ÒAcknowledgmentÓ (ACK), and is comprised of an Alert Burst and an ACK frame type byte.  (3) The data packet frame.  (4) An Acknowledgment to the data frame (only if received without errors).  The ACK frames must be received within 74.6 microseconds after the FBE or data packet frames, or a response timeout occurs.  Broadcast packets are sent without FBE/ACK and are not acknowledged by the recipients.

The ARCNET FBE/ACK mechanism is functionally equivalent to the LocalTalk RTS/CTS mechanism, but was in commercial use 8 years earlier.  There is an operational difference because ARCNET uses token passing to control the right to transmit, so the results of the RTS/CTS exchange are used for flow control and detection of absent/inactive destination stations (thereby conserving medium bandwidth for deliverable data frames).  On LocalTalk, the lack of a CTS may occur because of a data error on the medium, an absent/inactive station, or a collision.  The relative frequency of collisions versus medium errors on the preferred LocalTalk medium leads to a presumption of collision when a CTS is not received.

The use of RTS/CTS frame exchanges prior to 1984 was not limited to ARCNET (a token bus LAN), nor to tokenÐbased networks.  Another widely known local network that used such frame exchange sequences was the Cambridge Ring (a slotted ring LAN), under its Single Shot Protocol and its Byte Stream Protocol.  Because of the Cambridge RingÕs fineÐgrain interleaving of Òminipackets,Ó its RTSÐequivalent mechanism was needed to detect ÒcollisionsÓ as well as for flow control.  A ÒcollisionÓ on a minipacket network occurs when the addressed recipient is unable to receive the data frame because the assembly of minipackets of a frame from another sender is currently in progress.  One of many publications describing the Cambridge Ring is the monograph The Cambridge Distributed Computing System, R.M. Needham & A. J. Herbert, AddisonÐWesley, 1982 (ISBN 0Ð210Ð14092Ð6).

## Differences Between the RTS/CTS Mechanisms in 802.11 and the Ô902 Patent

Because the use of RTS/CTS frame exchanges on LANs, for flow control, medium availability determination, medium usage optimization, and collision detection, preÐdated the development of LocalTalk and the filing of the Ô902 patent by many years, the scope of the Ô902 claims related to RTS/CTS is much narrower than an isolated reading might indicate.  The embodiment described in the patent specification is quite different than that defined for the 802.11 MAC:

- The Ô902 patent describes a protocol where RTS frames are used before broadcasts (without eliciting a CTS response) as well as before directed data frames.  802.11, like ARCNET, only sends RTS frames before directed data frames.

- The Ô902 patent describes a 3Ðstage handshake (RTSÐCTSÐData), whereas 802.11, like ARCNET, uses a 4Ðstage handshake (RTSÐCTSÐDataÐACK).

- Despite the term Òcollision avoidanceÓ in its title, the network described in the Ô902 patent does not actually have a collision avoidance mechanism.  Instead, failure to receive a timely CTS response is used as a collision detection mechanism, in lieu of PHYÐlayer collision detection as is used by 802.3 (Ethernet).  The Òcollision avoidanceÓ of the Ô902 patent is actually avoidance of collisions involving the data frames.  This is confirmed in the text of the specification, which states Ò... the purpose of the three step handshake protocol described above is to avoid collisions by restricting the periods in which collisions are highly likely (typically during the RTS and CTS frame exchanges) ...Ó  (column 8, lines 49Ð53).  The 802.11 protocol includes an actual collision avoidance mechanism (the NAVÕs Òvirtual carrier senseÓ), which operates in parallel with the physical carrier sense and RTS/CTS handshake.  Basing backoff and deferral decisions, prior to the transmission of the RTS, on information obtained from MAC fields in preceding frames (including those addressed to other stations and even those from other BSSes) is distinctly different from the Ô902 protocol (as well as from most other collision avoidance protocols).

- The 802.11 protocol, like the Cambridge Ring, uses the RTS/CTS exchange to establish the instantaneous presence of the physical link Ñ a very important function when trying to run a LAN over wireless media, but one that is not mentioned in the Ô902 specification.

- The Ô902 protocol uses an isolated pulse, followed by missing clocks, to establish synchronization (largely due to idiosyncrasies of the 8530 SCC used as the LocalTalk controller), and a unique abort sequence as an endÐofÐframe delimiter.  802.11 uses a unique word, sent using the normal modulation technique of the active PHY as a starting delimiter, and determines endÐofÐframe using a CRCÐprotected length indicator in the frame header, rather than an ending delimiter.  Reliance on positive detection of a unique ending delimiter is likely to yield poor operational results with some wireless media, especially RF media in ISM bands.

# Conclusion

The Ô786 patent is not relevant to 802.11 because of its requirement for dynamicallyÐassigned network addresses.

The Ô902 patent is not relevant to 802.11 because its claims must be construed quite narrowly due to prior art networks which use RTS/CTS packet exchange mechanisms, and there are significant functional and operational differences (some of which are directly relevant to the successful operation over a wireless medium) between 802.11Õs RTS/CTS mechanism and LocalTalkÕs RTS/CTS mechanism.