IEEE 802.11

Wireless Access Method and Physical Layer Specifications

Title:	Proposed Updates to the D1 Draft, Section 3

Presented by:	Tom Siep

Texas Instruments Incorporated

13510 N. Central Expressway, m/s 446

Dallas, Texas  75243

Tel:	(214) 995-3675

Fax:	(214) 995-6194

Email:	siep@hc.ti.com

Abstract:	This paper proposes changes to the D1 Draft to reflect the resolution of comments on Section 3 during the May 95 meeting.

Action:	Adopt the changes in this paper to update the relevant portions of P802.11/D1

Introduction

The text of section 3 is modified by this document to reflect work done at the May 1995 meeting.  The changes are described by the following categories

- Removal of references to 802.10

- Removal of “dead” editorial text

- Restructing of the section to reflect its simpler content

Other Contributors

David Bagby

Michael Fischer

Leon Scaldeferri

Jeff Abramowitz

# 1	MAC Service Definition

## 2	Overview of MAC Services

### 4	Asynchronous Data Service

This service provides peer LLC entities with the ability to exchange MAC Service Data Units.  To support this service, the local MAC will use the underlying PHY-level services to transport an MSDU to a peer MAC entity, where it will be delivered to the peer LLC.  Such asynchronous MSDU transport is performed on a best-effort connectionless basis.  There are no guarantees that the submitted MSDU will be delivered successfully.  Broadcast and multicast transport is part of the asynchronous data service provided by the MAC. All Stations are required to support the Asynchronous Data Service.

### 5	Time-bounded Services

Time-Bounded services are implemented within the Point Coordination Function (PCF) as connection based data transfers. The access point adds connections to the polling-list in a best attempt to maintain the requested connection.  Maintaining time bounded services within an ESS shall be supported.

Since the PCF is optional, support for Time-bounded Services are also optional.

### 6	Security Services

{For d1.1: any changes to section 5.4 take precedence to text contained in this section}

Security services in 802.11 shall be provided by the Wired Equivalency Privacy mechanism], hereafter referred to as WEP.  The scope of the security services provided is limited to Station-to-Stationdata exchange.  The confidentiality service offered by an 802.11 WEP implementation shall be the encipherment of the MSDU payload.  For the purposes of this standard, the WEP is viewed as a logical sub-layer located at the top of the MAC sub-layer as shown in the reference model - section 2.4.  Actual implementations of the WEP sub-layer is considered transparent to the LLC or other layers above the MAC sub-layer.

The security services provided by the WEP in 802.11 are:

1)	confidentiality;

2)	authentication; and

3)	access control in conjunction with layer management.

Threats protected against are:

1)	unauthorized disclosure;

2)	unauthorized resource use; and

3)	masquerade.

During the authentication exchange, parties A and B exchange authentication information as described in section y.y.

The Layer 2 security services provided by the WEP rely on information from non-Layer 2 management or system entities. Management entities communicate the information to the WEP entity through a set of MIB variables.

8	Reordering of MPDUs

The services provided by the MAC Sublayer permit the reordering of MSDUs. The MAC does not intentionally reorder MSDUs. However, since MSDUs can transit a DS, and a DS might reorder MSDUs, it is not possible for the MAC to guarantee MSDU ordering.

### Simple Example of Security Management Information Base (SMIB)

Note 1 - The station ID could be 48 bit "Ethernet like" address or other type (this is not necessicarily the same as the SID)

Note 2 - The Data Privacy Mask can be either fixed or variable (max) length to accommodate a variety of algorithms.

Note 3 - Algorithm Number is an algorithm number registered per IEEE 802.10.

(editor note: When the Algorithm are defined, the text "Annex 2 contains an example of the transmission and reception procedures using specific algorithms" will be placed here - July 1993).

10	Detailed Service Specification

11	MAC Data Services

#### 12	MA_UNITDATA.request

Function

This primitive defines the transfer of a MSDU from a Local LLC sublayer entity to a single peer LLC sublayer entity, or multiple peer LLC sublayer entities in the case of group addresses.

Semantics of the Service Primitive

The semantics of the primitive are as follows:

MA_UNITDATA.request (

source_address,

destination_address,

routing_information,

data,

priority,

service_class

)

The source_address parameter  (SA) shall specify an individual MAC sublayer entity address, this SA shall be replaced in the MPDUs resulting from this request with the individual MAC sublayer address of the MAC entity to which the request is made.  The destination_address parameter  (DA) shall specify either an individual or a group MAC sublayer entity address. The routing_information parameter specifies the route desired for the data transfer (a null value indicates source routing is not to be used).  The data parameter specifies the MAC service data unit (MSDU) to be transmitted by the MAC sublayer entity.  The length of the MSDU shall  be less-than or equal to 2304 octets.  The priority parameter specifies the priority desired for the data unit transfer (contention or contention-free).  The service_class  parameter specifies the service_class  desired for the data unit transfer (asynchronous or time-bounded).

{For d1.2:need enumeration types for priority and service_class}

When Generated

This primitive is generated by the LLC sublayer entity whenever a MSDU must be transferred to a peer LLC sublayer entity or entities.  This can be as a result of a request from higher layers of protocol, or from a MSDU generated internally to the LLC sublayer, such as required by Type 2 operation.

Effect of Receipt

The receipt of this primitive shall cause the MAC sublayer entity to append all MAC specified fields, including DA, SA, and any fields that are unique to the particular media access method, and pass the properly formatted frame to the lower layers for transfer to peer MAC sublayer entity or entities.

#### 17	MA_UNITDATA.indication

Function

This primitive defines the transfer of a MSDU from the MAC sublayer entity to the LLC sublayer entity, or entities in the case of group addresses.  In the absence of error, the contents of the data parameter are logically complete and unchanged relative to the data parameter in the associated MA_UNIT_DATA-Request primitive.

Semantics of the Service Primitive

The semantics of the primitive are as follows:

MA_UNITDATA.indication(

source_address,

destination_address,

routing_information,

data,

reception_status,

priority,

service_class

)

The source_address parameter must be an individual address as specified by the SA field of the incoming frame.  The destination_address parameter shall be either an individual or a group address as specified by the DA field of the incoming frame.  The routing_information parameter specifies the route desired for the data transfer (null for 802.11 MACs).  The data parameter specifies the MAC service data unit (MSDU) as received by the local MAC entity, and shall  be less than or equal to 2304 octets in length.  The reception_status parameter indicates the success or failure of the incoming frame. The priority parameter specifies the priority desired for the data unit transfer (contention or contention-free).  The service_class parameter specifies the service_class desired for the data unit transfer (asynchronous or time-bounded).

{For d1.2: need enumeration types for reception_status, priority, and service_class}

When Generated

The MA_UNIT_DATA-Indication primitive is passed from the MAC sublayer entity to the LLC sublayer entity or entities to indicate the arrival of a frame at the local MAC sublayer entity.  Frames are reported only if at the MAC sublayer they are validly formatted, received without error, received with valid (or null) privacy encryption, and their destination address designates the local MAC sublayer entity as either an individual or group member.  When the receiving MAC sublayer entity is operating with a null privacy function, frames that are received in error may be reported, at the option of LLC; however, when operating with WEP enabled, erroneous reception (e.g. CRC failure) precludes  validation of the ICV, so to report such frames when operating with WEP enabled could constitute a breach of security.

Effect of Receipt

The effect of receipt of this primitive by the LLC sublayer is dependent on the validity and content of the frame.

### 22 MAC Management Services

{For d1.2: text needed}

To facilitate the three distribution system services:

a) Association

b) Reassociation

c) Disassociation - including the detection of link outage

### 23 Contention Free Connection Services

Contention Free Connections (CFCs) Services is a set of optional connection-oriented services.  Connection setup is done once per association with an ESS, and is maintained across BSS transitions (reassociations) but must  be reestablished if a disassociation occurs (either due to explicit disassociation or timeout).

{For d1.2: need to define connection setuptime-out}

#### 24 MA_CONNECTION_START.request

{For d1.2: text needed}

Function

{For d1.2: text needed}

Semantics of the Service Primitive

{For d1.2: text needed}

MA_CONNECTION_START.request	(

maximum_msdu_size,

normal_request_interval

)

When Generated

{For d1.2: text needed}

Effect of Receipt

{For d1.2: text needed}

#### 25 MA_CONNECTION_END.request

{For d1.2: text needed}

Function

{For d1.2: text needed}

Semantics of the Service Primitive

{For d1.2: text needed}

MA_CONNECTION_END.request	(

connection_id

)

When Generated

{For d1.2: text needed}

Effect of Receipt

{For d1.2: text needed}

#### 26 MA_CONNECTION_END.indication

{For d1.2: text needed}

Function

{For d1.2: text needed}

Semantics of the Service Primitive

{For d1.2: text needed}

MA_CONNECTION_END.indication	(

connection_id

)

When Generated

{For d1.2: text needed}

Effect of Receipt

{For d1.2: text needed}

#### 27 MA_CONNECTION_GRANT.indication

{For d1.2: text needed}

Function

{For d1.2: text needed}

Semantics of the Service Primitive

{For d1.2: text needed}

MA_CONNECTION_GRANT.indication	(

connection_id

)

When Generated

{For d1.2: text needed}

Effect of Receipt

{For d1.2: text needed}

#### 28 MA_CONNECTION_NOT_GRANTED.indication

{For d1.2: text needed}

Function

{For d1.2: text needed}

Semantics of the Service Primitive

{For d1.2: text needed}

MA_CONNECTION_NOT_GRANTED.indication()

When Generated

{For d1.2: text needed}

Effect of Receipt

{For d1.2: text needed}

#### 29 Access Point Initiates Connection Set-up Illustration

The following exchange will be used when an AP wants to establish a connection.

1. AP MAC user makes Start Connection Request. If the AP MAC believes that it can support this connection then the AP MAC generates Start Connection Request frame (otherwise the AP MAC asserts a Connection Not Granted Indication).

2. If the STA MAC can support this connection then it generates a Grant Connection frame and a Grant Connection Indication. On receipt of the Grant Connection Frame a Grant Connection Indication is generated.

Note: Only one connection request may be outstanding, with any one station, at any given time. The exchange fails if no response is received before a time-out (connection set up time-out). This will result in a Connection Not Granted Indication.

{For d1.2: Should “connection not requested due to traffic congestion” be indicated back to the requester?}

Figure 3-5: Connection Initiated by AP

#### 30 Station Initiates Connection Set-up Illustration

The following exchange will be used when a STA wants to establish a connection.

1. STA MAC user makes a Start Connection Request. If the STA MAC can support this connection then it generates a Start Connection Request frame (otherwise it will assert the Connection Not Granted Indication).

2. If the AP MAC believes that it can support this connection request then it will generate a Grant Connection frame and a Grant Connection Indication.

Note: Only one connection request may be outstanding at any given time. The exchange fails if no response is received before a time-out (connection set up time-out).

Figure 3-6: Connection Initiated by STA

#### 31 End Connection

Either an AP or a station may end a connection in the following way:

1. End Connection.

No MAC layer negotiation is needed to end a connection.

Figure 3-7: End Connection