IEEE 802.11

Wireless Access Method and Physical Layer Specifications

Title:	Proposed Revisions to Section 7.1, 7.2, and 7.3 of 	P802.11/D1.1

Many authors participated in the creation of these proposed changes.

Abstract:  This paper presents the changes to section 7.1, 7.2, and 7.3 to address a large number of letter ballot comments..

Action:  Adopt the changes in this paper to replace the relevent portions of Section 7.1, 7.2, and 7.3 of P802.11/D1.1.

# 1

# 2

# 3

# 4

# 5

# 6

# 7	MAC Layer Management Entity

## 8	Synchronization

### 9	Basic Approach

#### 10	TSF for Infrastructure Networks

In an infrastructure network, the AP shall be the timing master and shall perform the Timing Synchronization Function. The AP shall initialize the TSF timer such that simultaneously started APs are not synchronized. To synchronize the other stations in a BSS, the AP shall periodically transmit special frames called Beacons that contain a copy of its Synchronization Timer. Receiving stations shall always accept the timing information in Beacons sent from the AP servicing their BSS. If the station's Synchronization Timer is different from the timestamp in the received Beacon, they shall set their local timer to the received timestamp value.

Beacons shall be generated for transmission by the AP once every aBeacon_Interval time units.

### 11	Maintaining Synchronization

#### 12	Beacon Generation in Infrastructure Networks

#### 13	Beacon Generation in Ad Hoc Networks

Beacon generation in an ad hoc network is distributed. All members of the BSS participate in Beacon generation. Each station shall maintain its own TSF timer which is used for aBeacon_Interval timing. This defines a series of Target Beacon Transmission Times (TBTTs) exactly aBeacon_Interval time units apart, time zero is defined to be a TBTT.  At each TBTT the station shall 1) calculate a random delay, 2) wait for the period of the random delay, 3) if no Beacon has arrived during the delay period, send a Beacon.  See Figure 7-2.

Figure 7-2 — Beacon transmission in an Ad Hoc network.

The Beacon transmission will always occur during the Awake Period of stations that are operating in a low power mode. This is described in more detail in Section 7.2.

## 14	 Power Management

### 15	Power Management in an Infrastructure Network

#### 16	Overview

Stations changing power management mode shall inform the AP of this fact (via the Power Management bits within transmitted frames).  The AP shall not arbitrarily transmit frames to station operating in a power saving mode, but shall buffer frames and only transmit them at designated times.

The stations which currently have buffered frames within the AP are identified in a Traffic Indication Map (TIM), which shall be included as an element within all Beacons generated by the AP.  A station shall determine that a frame is buffered for it by receiving and interpreting a TIM.

Stations operating in power save modes shall periodically listen for Beacons, as determined by the Station's aListen_Interval parameter.

Upon determining that a frame is currently buffered in the AP, a Station operating in the Power Save Polling mode (PSP) shall transmit a short Poll frame to the AP, which will respond with the corresponding buffered frame. If any station in its BSS is in power saving mode, the AP shall buffer all broadcast and multicast frames and deliver them to all stations following a Delivery TIM (DTIM) transmission.

A station shall remain in its current power management mode until it informs the AP of a power management mode change via a successful frame exchange.  Power management mode shall not change during any single frame exchange sequence, as described in section 4.3.

#### 17	Station Power Management Modes

A station can be in two different power states:

Awake:	Station is fully powered.

Doze: Station is not able to transmit or receive and consumes very low power. Some circuitry (such as timers) may still be active.

The manner in which a station transitions between these two power states shall be determined by the station's Power Management Mode (aPower_Management_Mode). These modes are summarized in the table below.

| Active Mode or AM | Station may receive frames at any time, no AP buffering of frames. In Active Mode, a station shall be in the Awake state. A station on the polling list of a PCF shall be in Active Mode for the duration of the contention free period. |
| --- | --- |
| Power Save Polling or PSP | Station listens to selected TIMs (based upon its aListen_Interval) and polls the AP if the TIM indicates a frame is buffered for that station (except for broadcasts). The AP shall transmit buffered unicast frames only in response to a poll. In PSP mode, a station shall be in the Doze state and shall enter the Awake state to listen for selected TIMs and to transmit. |

To change power management modes, a station shall inform the AP through a successful frame exchange initiated by the station.

#### 18	Access Point TIM Transmissions

The TIM will identify the stations for which traffic is pending and buffered in the AP.  This information is coded in a virtual bitmap, as described in Section 4.  In addition the TIM contains an indication whether Broadcast/multicast traffic is pending. Every station is assigned a Station ID code (SID) by the AP as part of the association process (see Section 7.3).  SID 0 (zero) shall be reserved to indicate the presence of buffered broadcast/multicast frames.  The AP shall identify those stations for which frames are buffered by setting bits in the TIM's virtual bitmap that correspond to the appropriate SIDs.

#### 19	TIM Types

Two different TIM types are distinguished: TIM and DTIM. After a DTIM, the AP shall send out the stored Broadcast/Multicast frames using normal frame transmission rules.

The AP shall transmit TIMs with every Beacon.  Every aDTIM_Interval, a TIM of type "DTIM" is transmitted within a Beacon rather than an ordinary TIM.

The following figure illustrates the AP and station activity under the assumption that a DTIM is transmitted once every three TIMs.

Figure 7-5: Infrastructure Power Management Operation (No PCF Operating)

#### 20	Access Point Operation During the Contention Period

Access Points shall maintain a Power Management status for each currently associated station that indicates in which Power Management mode the station is currently operating. An AP shall, depending on the Power Management mode of the station, buffer the frame destined to the station temporarily. No frames received for stations operating in the Active mode shall be buffered.

a)	Frames destined for PSP stations shall be temporarily buffered in the AP.

b)	Frames destined to stations in the Active mode shall be directly transmitted.

c)	At every Beacon Interval, the AP shall assemble the virtual bitmap containing the buffer status per destination for stations in the PSP mode, and shall send this out in the TIM field of the Beacon. The bit for SID 0 (zero) shall be set  whenever broadcast or multicast traffic is buffered.

d)	All Broadcast/Multicast frames shall be buffered if any associated stations are in PSP mode.

e)	After every DTIM, the AP shall transmit all buffered broadcast/multicast frames.

f)	Buffered frames for stations in the PSP mode shall be forwarded to the station after a Poll has been received from that station.

g)	An AP shall  have an aging function to delete pending traffic when it was buffered for an excessive time period.

h)	Whenever an AP is informed that a station changes to the Active Mode, then the AP shall send buffered frames (if any exist) to that station immediately.

#### 21	Access Point Operation During the Contention Free Period

Access Points shall maintain a Power Management status for each currently associated CF Aware station that indicates in which Power Management mode the station is currently operating. An AP shall, for stations in PSP Mode, buffer the frame destined to the station temporarily.

a)	Frames destined for PSP stations shall be temporarily buffered in the AP.

b)	Frames destined to stations in the Active mode shall be transmitted as defined in Section 5.

c)	Prior to every Contention Free Period, the AP shall assemble the virtual bitmap containing the buffer status per destination for stations in the PSP mode, set the bits in the virtual bitmap for stations the Point Coordinator is intending to poll in this Contention Free Period, and shall send this out in the TIM field of the DTIM. The bit for SID 0 (zero) shall be set  whenever broadcast or multicast traffic is buffered.

d)	Buffered frames for stations in the PSP mode shall be forwarded to the CF Aware stations under control of the Point Coordinator.

e)	An AP shall  have an aging function to delete pending traffic when it was buffered for an excessive time period.

23	 Receive Operation for Stations in PSP Mode During the Contention Period

Stations in PSP mode shall operate as follows to receive a frame from the AP when not participating in the contention free period:

a)	Stations shall wake-up so as to receive the next scheduled Beacon after Listen Interval from the last received Beacon.

b)	When a station detects that the bit corresponding to its SID is set in the TIM, the station shall issue a Poll to retrieve the buffered frame.  If more than one bit is set in the TIM, the Poll shall be transmitted after a random delay.

c)	If the Power Management bits in the received frame indicate that more traffic for that station is buffered,  the station shall Poll until no more frames are buffered for that station.

d)	To receive broadcast frames, the station shall wake up so as to receive every DTIM.

#### 24	 Receive Operation for Stations in PSP Mode During the Contention Free Period

Stations in PSP mode that are associated as CF Aware shall operate as follows to receive a frame from the AP during the contention free period:

a)	Stations shall wake-up so as to receive the next scheduled DTIM.

b)	When a station detects that the bit corresponding to its SID is set in the DTIM, the station shall remain awake for the duration of the contention free period to receive the frame(s) buffered for it in the AP or until a frame is received with the Power Management bits indicating no further traffic is buffered.

c)	If the Power Management bits in the received frame indicate that more traffic for that station is buffered and the contention free period ends,  the station may send a Poll to the AP to request the delivery of additional buffered frames.

27	Stations Operating in the Active Mode

Stations operating in this mode shall have their transceivers activated continuously, so they do not need to interpret the traffic announcement part of the Beacons.

### [Editor's note (GE): The timing of the start of the first ATIM frame should be randomized (backoff before transmission), to prevent multiple ATIMs from colliding.]

## 32	Association and Reassociation

This section defines how a station associates and reassociates with an access point.

### 33	Station Association Procedures

A station shall associate with an access point via the following procedure:

a)	The station shall transmit a ASSOCIATION Request frame to an access point.

b)	If transmission fails, the station shall scan for a different access point with which to attempt association.

c)	If an ASSOCIATION Responseframe is received with status value of “successful”, the station is now associated with the access point.

d)	If an ASSOCIATION Response frame is received with status value of  “failed”, the station shall scan for a different access point with which to attempt association.

### 34	Access Point Association Procedures

An access point shall operate as follows in order to support the association of stations.

a)	Whenever an ASSOCIATION Request frame is received from a station, , the access point shall transmit a ASSOCIATION response with a status value of  “successful” or “failed”.  If the status value is “successful”, the assigned Station ID to the station is included in the response..

b)	When the ASSOCIATION Response with a status value of  “successful” frame is acknowledged by the station, the station is considered to be associated with this access point.

c)	The AP shall inform the Disribution System  of the association.

### 35	Station Reassociation Procedures

A station shall reassociate with an access point via the following procedure:

a)	The station shall transmit a REASSOCIATION Request frame to an access point.

b)	If transmission fails, the station shall scan for a different access point with which to attempt reassociation.

c)	If an REASSOCIATION Responseframe is received with status value of “successful”, the station is now associated with the access point.

d)	If an REASSOCIATION Response frame is received with status value of  “failed”, the station shall scan for a different access point with which to attempt reassociation.

### 36	Access Point Reassociation Procedures

An access point shall operate as follows in order to support the reassociation of stations.

a)	Whenever an REASSOCIATION Request frame is received from a station, , the access point shall transmit a REASSOCIATION response with a status value of  “successful” or “failed”.  If the status value is “successful”, the assigned Station ID to the station is included in the response..

b)	When the REASSOCIATION Response with a status value of  “successful” frame is acknowledged by the station, the station is considered to be associated with this access point.

c)	The AP shall inform the Disribution System  of the reassociation.