Changes to Section 6.2.6.3

Authors:	Tom Baumgartner, Bob O’Hara

#### 6.2.6.3.	RTS/CTS Recovery Procedure and Retransmit Limits

Many circumstances may cause an error to occur in a RTS/CTS exchange.

For instance, CTS may not be returned after the RTS transmission. This can happen due to a collision with another RTS or a DATA frame, or due to interference during the RTS or CTS frame. It can however also be that CTS fails to be returned because the remote station has an active carrier sense condition, indicating a busy medium time period.

If after an RTS is transmitted, the CTS Timeout expires, then a new RTS shall be generated while following the basic access rules for backoff.  Since this pending transmission is a retransmission attempt, the CW shall be doubled as per the backoff rules. This process shall continue until the number of attempts exceeds the aShort_Retry_Limit.

The same backoff mechanism shall be used when no ACK frame is received within a predetermined ACK_Timeout after a directed DATA frame has been transmitted.  The ACK_Timeout value is the time required to transmit the ACK frame plus a SIFS interval.  Since this pending transmission is a retransmission attempt the CW will be doubled as per the backoff rules.  This process shall continue until the number of attempts exceeds the aLong_Retry_Limit for DATA frames the length of which exceed aRTS_Threshold or aShort_Retry_Limit for DATA frames the length of which do not exceed aRTS_Threshold.