# 1	MAC Service Definition

## 2	Overview of MAC Services

### 3	Asynchronous Data Service

This service provides peer LLC entities with the ability to exchange MAC Service Data Units.  To support this service, the local MAC will use the underlying PHY-level services to transport an MSDU to a peer MAC entity, where it will be delivered to the peer LLC.  Such asynchronous MSDU transport is performed on a best-effort connectionless basis.  There are no guarantees that the submitted MSDU will be delivered successfully.  Broadcast and multicast transport is part of the asynchronous data service provided by the MAC. All Stations are required to support the Asynchronous Data Service.

### 4	Time-bounded Services

Time-Bounded services are implemented within the Point Coordination Function (PCF) as connection based data transfers. The access point adds connections to the polling-list in a best attempt to maintain the requested connection.  Maintaining time bounded services within an ESS shall be supported.

Since the PCF is optional, support for Time-bounded Services are also optional.

### 5	Security Services

Security services in 802.11 shall be provided by the Authentication service and the Wired Equivalency Privacy mechanism.  The scope of the security services provided is limited to Station-to-Station data exchange.  The privacy service offered by an 802.11 WEP implementation shall be the encryption of the MSDU payload.  For the purposes of this standard, WEP is viewed as a logical sub-layer located within the MAC sub-layer as shown in the reference model - figure X.X.  Actual implementations of the WEP sub-layer are transparent to the LLC or other layers above the MAC layer.

The security services provided by the WEP in 802.11 are:

1)	confidentiality;

2)	authentication; and

3)	access control in conjunction with layer management.

Threats protected against are:

1)	unauthorized disclosure;

2)	unauthorized resource use; and

3)	masquerade.

During the authentication exchange, parties A and B exchange authentication information as described in  X.X.

The Layer 2 security services provided by WEP rely on information from non-Layer 2 management or system entities. Management entities communicate information to WEP through a set of MIB variables.

### 6	Reordering of MPDUs

The services provided by the MAC Sublayer permit the reordering of MSDUs. The MAC does not intentionally reorder MSDUs. However, since MSDUs can transit a DS, and a DS might reorder MSDUs, it is not possible for the MAC to guarantee MSDU ordering.

## 7	Detailed Service Specification

### 8	MAC Data Services

#### 9	MA_UNITDATA.request

Function

This primitive defines the transfer of a MSDU from a Local LLC sublayer entity to a single peer LLC sublayer entity, or multiple peer LLC sublayer entities in the case of group addresses.

Semantics of the Service Primitive

The semantics of the primitive are as follows:

MA_UNITDATA.request (

source_address,

destination_address,

routing_information,

data,

priority,

service_class

)

The source_address parameter  (SA) shall specify an individual MAC sublayer entity address, this SA shall be replaced in the MPDUs resulting from this request with the individual MAC sublayer address of the MAC entity to which the request is made.  The destination_address parameter  (DA) shall specify either an individual or a group MAC sublayer entity address. The routing_information parameter specifies the route desired for the data transfer (a null value indicates source routing is not to be used).  The data parameter specifies the MAC service data unit (MSDU) to be transmitted by the MAC sublayer entity.  The length of the MSDU shall  be less-than or equal to 2304 octets.  The priority parameter specifies the priority desired for the data unit transfer (contention or contention-free).  The service_class  parameter specifies the service_class  desired for the data unit transfer (asynchronous or time-bounded).

{For d1.2:need enumeration types for priority and service_class}

When Generated

This primitive is generated by the LLC sublayer entity whenever a MSDU must be transferred to a peer LLC sublayer entity or entities.  This can be as a result of a request from higher layers of protocol, or from a MSDU generated internally to the LLC sublayer, such as required by Type 2 operation.

Effect of Receipt

The receipt of this primitive shall cause the MAC sublayer entity to append all MAC specified fields, including DA, SA, and any fields that are unique to the particular media access method, and pass the properly formatted frame to the lower layers for transfer to peer MAC sublayer entity or entities.

#### 10	MA_UNITDATA.indication

Function

This primitive defines the transfer of a MSDU from the MAC sublayer entity to the LLC sublayer entity, or entities in the case of group addresses.  In the absence of error, the contents of the data parameter are logically complete and unchanged relative to the data parameter in the associated MA_UNIT_DATA-Request primitive.

Semantics of the Service Primitive

The semantics of the primitive are as follows:

MA_UNITDATA.indication(

source_address,

destination_address,

routing_information,

data,

reception_status,

priority,

service_class

)

The source_address parameter must be an individual address as specified by the SA field of the incoming frame.  The destination_address parameter shall be either an individual or a group address as specified by the DA field of the incoming frame.  The routing_information parameter specifies the route desired for the data transfer (null for 802.11 MACs).  The data parameter specifies the MAC service data unit (MSDU) as received by the local MAC entity, and shall  be less than or equal to 2304 octets in length.  The reception_status parameter indicates the success or failure of the incoming frame. The priority parameter specifies the priority desired for the data unit transfer (contention or contention-free).  The service_class parameter specifies the service_class desired for the data unit transfer (asynchronous or time-bounded).

{For d1.2: need enumeration types for reception_status, priority, and service_class}

When Generated

The MA_UNIT_DATA-Indication primitive is passed from the MAC sublayer entity to the LLC sublayer entity or entities to indicate the arrival of a frame at the local MAC sublayer entity.  Frames are reported only if at the MAC sublayer they are validly formatted, received without error, received with valid (or null) privacy encryption, and their destination address designates the local MAC sublayer entity as either an individual or group member.  When the receiving MAC sublayer entity is operating with a null privacy function, frames that are received in error may be reported, at the option of LLC; however, when operating with WEP enabled, erroneous reception (e.g. CRC failure) precludes  validation of the ICV, so to report such frames when operating with WEP enabled could constitute a breach of security.

Effect of Receipt

The effect of receipt of this primitive by the LLC sublayer is dependent on the validity and content of the frame.

### 11 MAC Management Services

{For d1.2: text needed}

To facilitate the three distribution system services:

a) Association

b) Reassociation

c) Disassociation - including the detection of link outage