IEEE P802.11
Wireless LANs

Comment on Back-off in MAC

Date:	March 11, 1997

Author:	Harold Teunissen
	Lucent Technologies WCND B.V.
	Zadelstede 1-10
	3431 JZ  Nieuwegein, the Netherlands
	Phone: +31 306 097 6510
	Fax: +31 306 097 6556
	e-Mail: hteunis@wcnd.lucent.com

802.11 does not specifiy the status of the medium while a frame is being transmitted. The status is used to decrement the backoff.

rev. 802.11/clause 9.2.1, 9.2.4, 9.2.5, ....)

Scenario:

- Backoff > 0 and decrementing (waiting to transmit MPDU_x).

- Medium sensed busy -> Backoff suspended -> receiving MPDU_y.

- Medium is free again, thus backoff is resumed. However received MPDU_y needs Ack_y, which is transmitted after a SIFS period. During the transmission of the Ack_y the Backoff expires and the MAC is ready to transmit MPDU_x. In worst case MPDU_x will be transmitted following a SIFS period after the Ack_y  -> DCF fails.

My suggestion is to add to the medium status the status of the MAC. Medium busy if:

- physical carrier sensed,

- NAV > 0 (virtual carrier sensed),

- MAC/PHY is transmitting.