IEEE P802.11
Wireless LANs

PROPOSAL FOR HIGH-DATA-RATE 2.4-GHz PHY

Date:	July 10, 1998

Author:	John H. Cafarella
	MICRILOR, Inc.
	17 Lakeside Office Park, Wakefield, MA 01880
	Phone: 781-246-0103
	Fax: 781-246-0157
	e-Mail: JohnCafarella@worldnet.att.net

Abstract

This is the July revision of MICRILOR’s proposal for the 2.4-GHz High-Data-Rate PHY standard.

High-Rate Direct-Sequence Spread Spectrum Physical Layer Specification for the 2.4 GHz ISM Band

## 1.1	Introduction

### 1.1.1	Scope

### 1.1.2	High-Rate DSSS Physical Layer Functions

## 1.2	DSSS Physical Layer Convergence Procedure Sub-Layer

### 1.2.1	Introduction

During transmission the MPDU shall be prepended with a PLCP Preamble and PLCP Header to create the PPDU.  At the receiver the PLCP Preamble and Header are processed to aid in demodulation and delivery of the MPDU.

### 1.2.2	Physical Layer Convergence Procedure Frame Format

Figure 1 shows the format for the PPDU, including the DSSS PLCP preamble, the DSSS PLCP header and the MPDU frame.  The PLCP Preamble contains the Synchronization (SYNC) and Start Frame Delimiter (SFD) fields.  The PLCP Header contains the 802.11 signaling (SIGNAL) and length (LENGTH) fields.  Each of these is described in clause .

Figure 1 - PLCP Frame Format

### 1.2.3	PLCP Field Definitions

The PHY Preamble consists of the SYNC and SFD fields, while the PHY Header contains the SIGNAL, LENGTH and R/S-CRC fields.  These are transmitted as a sequence of 5-bit channel symbols.  See clause  for a description of the modulation formats, as well as the meaning of the symbol-modulator bits, interpreted as the 5-bit pattern [DBPSK,R1,R2,R3,R4], and as used below.

#### 1.2.3.1	PLCP Synchronization (SYNC)

The synchronization field shall consist of forty (40) symbols which employ one of eight (8) search codes, but which contain all zeroes for the data, i.e., [00000] as the 5-bit modulator-symbol pattern.  This field is provided so that the receiver can perform necessary operations to initiate reception of a frame (e.g., signal detection, timing alignment).

#### 1.2.3.2	PLCP Start Frame Delimiter (SFD)

After the SYNC field a single symbol will be transmitted which carries a DBPSK “1”, i.e. [10000] as the 5-bit modulator-symbol pattern, with the leftmost bit leading.  This field provides for reliable detection of the end of the PHY Preamble.

#### 1.2.3.3	PLCP Signal Field (SIGNAL)

After the SFD, the 1-symbol SIGNAL field will be transmitted; the DBPSK bit must be zero, i.e., [0xxxx] as the 5-bit modulator-symbol pattern, with the leftmost bit leading and equal to zero, followed by s1, etc., as a serial stream.  Table 1 shows the correspondence between the bits of the signal field and the symbol-modulator bits.

Table 1 - SIGNAL Symbol

| Modulator bit position | SIGNAL Field bit |
| --- | --- |
| DBPSK | 0 |
| R1 | S1 |
| R2 | S2 |
| R3 | S3 |
| R4 | S4 |

This SIGNAL field determines the modulation used in the main body of the frame.  The SIGNAL field shall be interpreted according Table 2.

Table 2 - SIGNAL Field Interpretation

| s1 | S2 | s3 | s4 |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 10-Mbps (16-ary DBOK) |
| 1 | 0 | 0 | 0 | 8.7-Mbps (16-ary DBOK with (15,13) FEC) |
| 0 | 1 | 0 | 0 | 18-Mbps (Constrained 4x4-ary DBOK) |
|  |  |  |  | All other patterns reserved |

#### 1.2.3.4	PLCP Length Field (LENGTH)

The LENGTH field is three symbols with the DBPSK bit equal to zero, i.e., [0xxxx] as the 5-bit symbol pattern, with the leftmost bit leading and equal to zero.  The 12 non-zero-bits form a binary number which specifies the length of the MPDU in bytes.  The relationship between the bits of the 3 LENGTH symbols and the 12-bit Length number is shown in Table 3.  The lsb of the Length is b0.

Table 3 - Correspondence Between LENGTH Symbols and Number

| Modulator bit position | 1st Symbol after SIGNAL | 2nd Symbol after SIGNAL | 3rd Symbol after SIGNAL |
| --- | --- | --- | --- |
| DBPSK | 0 | 0 | 0 |
| R1 | B0 | b4 | b8 |
| R2 | B1 | b5 | b9 |
| R3 | B2 | b6 | B10 |
| R4 | B3 | b7 | B11 |

#### 1.2.3.5	PLCP CRC Field (R/S-CRC)

The SIGNAL and SERVICE fields shall be protected with a Reed-Solomon (15,11) code, shortened to 8 symbols, for CRC.  This is encoded using the polynomial (x+1)(x+)(x+2)(x+3)=x4+3x+1,

(x+1)(x+α)(x+α2)(x+α3)=x4+α12x3+α4x2+x+α6

where  is a primitive root of x4+x+1 over GF{24}.  The four check symbols of the r/s-CRC field provide the minimum Hamming distance of 4, as required for control fields by IEEE 802 Functional Requirements Document, Draft 6.10, 12 November, 1991.  All modulator symbols for the SIGNAL, SERVICE and R/S-CRC fields are in GF{24}.

### 1.2.4	PLCP/DSSS PHY Data Scrambler and Descrambler

Scrambling is optional.  It offers users the ability to randomize inter-BSA interactions at the modulation level.  Scrambling operates on the spreading codes during the MPDU portion of the PPDU.  While this is equivalent to data scrambling in the 5-bit/symbol mode, it must actually operate on the spreading codes, rather than the data symbols, in order to preserve the constant-envelope property in the 9-bit/symbol mode.

Scrambling is intended for use with scrambled code channels (SCC), as described in , although nothing prohibits its use with cyclic code channels (CCC) as described in .  When implemented, scrambling will be frame-synchronous and commence with the first channel symbol of the MPDU.  Scrambling shall not be used on the PHY Preamble or Header.

The scrambler shall employ a 17-stage (binary) Linear Feedback Shift Register (LFSR) generator using polynomial x17+x3+1.  This is clocked once per channel symbol when scrambling.  The LFSR generator shall be seeded with a 16-bit SCC with a most-significant “1” added to prevent the possibility of all “0”s as initial condition for the LFSR;  the lsb of the seed vector is the lsb of the SCC.

The bits of the LFSR generator form a 17-bit binary word [w0w1w2w3w4w5w6w7w8w9w10w11w12w13w14w15w16] whose bits obey the recursion

w16+=w0−⊕w3−

Where the - and + superscripts indicate the instances just before and after, respectively, the symbol clock which updates the LFSR generator.  Bits w1 and w0 are used for code selection when using optional SCC, as described in .  Bits w7 to w2 form a 5-bit scrambling symbol which is combined with the nth-symbol spreading code Cn according to

Cn'=R1w0R2w1R3w2R4w3Cn

where Cn’ replaces Cn as the spreading code to be combined with the data symbol as specified in .  (See  for explanation of notation in the above equation.)

### 1.2.5	PLCP Data Modulation and Modulation Rate Change

### 1.2.6	PLCP Transmit Procedure

### 1.2.7	PLCP Receive Procedure

## 1.3	High-Rate DSSS Physical Layer Management Entity

## 1.4	High-Rate DSSS Physical Medium Dependent Sublayer

### 1.4.1	Scope and Field of Application

### 1.4.2	Overview of Service

### 1.4.3	Overview of Interactions

### 1.4.4	Basic Service and Options

### 1.4.5	PMD_SAP Detailed Service Specification

### 1.4.6	PMD Operating Specifications General

The following clauses provide general specifications for the high-rate DSSS Physical Medium Dependent sub-layer.  These specifications apply to both transmit and receive functions and general operation of the high-rate DSSS PHY.

#### 1.4.6.1	Operating Frequency Range

The DSSS PHY shall operate in the frequency range of 2.4 to 2.4835 GHz as allocated by regulatory bodies in the USA and Europe or in the 2.471 to 2.497 GHz frequency band as allocated by regulatory authority in Japan.

#### 1.4.6.2	Number of Operating Frequency Channels

The channel center frequencies and CHNL_ID numbers shall be the same as for the lower-rate DSSS PHY standard, as shown in the following table.  (See clause 15.4.6.2 for discussion.)

Table 4 - DSSS PHY Frequency Channel Plan

|  |  | Regulatory Domains |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHNL_ID | Frequency | 10h FCC | 20h IC | 30h ETSI |  |  | 40h MKK |
| 1 | 2412 MHz | X | X | X |  |  |  |
| 2 | 2417 MHz | X | X | X |  |  |  |
| 3 | 2422 MHz | X | X | X |  |  |  |
| 4 | 2427 MHz | X | X | X |  |  |  |
| 5 | 2432 MHz | X | X | X |  |  |  |
| 6 | 2437 MHz | X | X | X |  |  |  |
| 7 | 2442 MHz | X | X | X |  |  |  |
| 8 | 2447 MHz | X | X | X |  |  |  |
| 9 | 2452 MHz | X | X | X |  |  |  |
| 10 | 2457 MHz | X | X | X |  |  |  |
| 11 | 2462 MHz | X | X | X |  |  |  |
| 12 | 2467 MHz |  |  | X |  |  |  |
| 13 | 2472 MHz |  |  | X |  |  |  |
| 14 | 2484 MHz |  |  |  |  |  | X |

#### 1.4.6.3	Number of Operating Code Channels

Preamble acquisition is performed using a repeated, unmodulated search spreading code, while data transfer employs data spreading codes combined with Walsh-function codes as selected by the data.  Code channels are formed by using various possible combinations of a search code and data codes.

Table 5 - Search Code Channels

| SRCH_ID | Search Code | SRCH_ID | Search Code |
| --- | --- | --- | --- |
| 0 | S0 | 4 | S4 |
| 1 | S1 | 5 | S5 |
| 2 | S2 | 6 | S6 |
| 3 | S3 | 7 | S7 |

##### 1.4.6.3.1	Search Code Channels

Search code channels SRCH_ID are enumerated 0-7, these correspond to the subscript of the search codes as in table 5.

##### 1.4.6.3.2	Data Code Channels: Cyclic

The spreading codes shall be from the set {C0C1C2C3C4C5C6C7}.  The spreading code patter can be a single code for all symbols, any two codes in alternation from symbol to symbol, or any four codes used cyclicly over four symbols.  Because the change from search to data spreading codes happens at the same place in the PPDU, all permutations of the two groups constitute distinct channels.  These are 1744 cyclic code channels (CCC); they are designated as code channel (CCC_ID) “abcd,” where a, b, c and d are integers 0 to 7 corresponding to the subscripts of the codes (C0-C7).  For example, code channel “1111” uses C1 for all symbols.  Code channels using C0-C3 have best multipath properties.  Because codes C0 to C3 reflect the best multipath delay-spread tolerance, the 40 data channels which use only these first 4 codes are preferred when multipath performance is critical.

##### 1.4.6.3.3	Data Code Channels: Scrambled

The scrambled code channels (SCC) are optional, and can only be used when the optional data-scrambling mode described in  is in effect.  The spreading codes shall be four from the set {C0C1C2C3C4C5C6C7}.  The sequence with which specific codes are used, beginning on the first channel symbol of the MPDU portion of the frame, shall be determined by the two lowest-order bits [w1,w0] of the LFSR described in .  These shall have the correspondence shown in Table 6, where set {CA,CB,CC,CD} is four distinct codes selected from {C0C1C2C3C4C5C6C7}.

Table 6 - SCC Selection

| W5 | W4 | Code |
| --- | --- | --- |
| 0 | 0 | CA |
| 0 | 1 | CB |
| 1 | 0 | CC |
| 1 | 1 | CD |

To effect communications, it is necessary for users to agree upon the 16-bit SCC as well as a permutation of four eight total codes.  This constitutes 1680x216, or approximately 110 million distinct scrambled-code channels.  These are intended for deployments (such as SOHO) in which no system administrator can coordinate all BSAs.

#### 1.4.6.4	Channelization Concepts

There are 3 frequency channels; there are code channels corresponding to combinations of the 8 SRCH_Ids as well as 1744 CCC_IDs.  (The optional SCC are not considered in this section.)  This provides a total of 3x8x1744=41856 combinations with which to effect BSA isolation.  The “channel” identification is the concatenation of the frequency, search-code and data-code channels:

CH_ID = CHNL_ID / SRCH_ID/ CCC_ID

For example, channel 3/5/1212 transmits on a center frequency of 2422 MHz using search code S5 during acquisition, and cyclic data code sequence C1C2C1C2 during the data portion of the frame.

When substantial overlap of BSA coverage is required, adjacent BSAs should be operated on CHNL_IDs separated in frequency to provide good isolation.  When BSAs have frequency isolation, it is still good practice to employ different SRCH_IDs and CCC_IDs to avoid spurious injection of frames between BSAs.

When overlapped operation of BSAs is not required, then it is sufficient to employ different SRCH_IDs and CCC_IDs for BSA isolation.  When it is desired to coordinate (share capacity, but avoid near/far problems) then the same codes should be used, as in the original DSSS 802.11 PHY.

Figure 2 - Overlapped and Non-Overlapped BSA Coverage

Non-Overlapped: STAs connect only to nearest AP

Overlapped: STAs can connect to more-distant AP

#### 1.4.6.5	Spreading Sequences

All codes contain 16 binary values within a channel symbol.  One of 8 “search” spreading codes (Table 7) is used for synchronization (SYNC, SFD and SIGNAL fields).  During the remainder of a frame a sequence of 4 “data” spreading codes (Table 8) is used for data transmission; the 8 such data codes are called “coset leaders”, and are grouped as two sets of four codes.  The Walsh functions, combined with the coset leaders to carry the data in the transmitted waveform, are also presented below (Table 9), along with the Rademacher functions (Table 10) used to construct the Walsh functions.  The codes are presented in three representations: hexadecimal, binary logical, and binary algebraic.  The leftmost value is the first element of a transmitted symbol waveform.

Table 7 - Search-Mode  Spreading Codes

| Code | Hex | Binary logical | Binary algebraic |
| --- | --- | --- | --- |
| S0 | 44BC | 0100010010111100 | +1,-1,+1,+1,+1,-1,+1,+1,-1,+1,-1,-1,-1,-1,+1,+1 |
| S1 | A0DC | 1010000011011100 | -1,+1,-1,+1,+1,+1,+1,+1,-1,-1,+1,-1,-1,-1,+1,+1 |
| S2 | D223 | 1101001000100011 | -1,-1,+1,-1,+1,+1,-1,+1,+1,+1,-1,+1,+1,+1,-1,-1 |
| S3 | 0A76 | 0000101001110110 | +1,+1,+1,+1,-1,+1,-1,+1,+1,-1,-1,-1,+1,-1,-1,+1 |
| S4 | 425C | 0100001001011100 | +1,-1,+1,+1,+1,+1,-1,+1,+1,-1,+1,-1,-1,-1,+1,+1 |
| S5 | 23A4 | 0010001110100100 | +1,+1,-1,+1,+1,+1,-1,-1,-1,+1,-1,+1,+1,-1,+1,+1 |
| S6 | 245C | 0010010001011100 | +1,+1,-1,+1,+1,-1,+1,+1,+1,-1,+1,-1,-1,-1,+1,+1 |
| S7 | A243 | 1010001001000011 | -1,+1,-1,+1,+1,+1,-1,+1,+1,-1,+1,+1,+1,+1,-1,-1 |

codes S0-S3 have better multipath performance than codes S4-S7.

Table 8 - Data-Mode Spreading Codes

| Code | Hex | Binary logical | Binary algebraic |
| --- | --- | --- | --- |
| C0 | 0653 | 0000011001010011 | +1,+1,+1,+1,+1,-1,-1,+1,+1,-1,+1,-1,+1,+1,-1,-1 |
| C1 | 1247 | 0001001001000111 | +1,+1,+1,-1,+1,+1,-1,+1,+1,-1,+1,+1,+1,-1,-1,-1 |
| C2 | 141B | 0001010000011011 | +1,+1,+1,-1,+1,-1,+1,+1,+1,+1,+1,-1,-1,+1,-1,-1 |
| C3 | 1427 | 0001010000100111 | +1,+1,+1,-1,+1,-1,+1,+1,+1,+1,-1,+1,+1,-1,-1,-1 |
| C4 | 0563 | 0000010101100011 | +1,+1,+1,+1,+1,-1,+1,-1,+1,-1,-1,+1,+1,+1,-1,-1 |
| C5 | 065C | 0000011001011100 | +1,+1,+1,+1,+1,-1,-1,+1,+1,-1,+1,-1,-1,-1,+1,+1 |
| C6 | 114B | 0001000101001011 | +1,+1,+1,-1,+1,+1,+1,-1,+1,-1,+1,+1,-1,+1,-1,-1 |
| C7 | 1274 | 0001001001110100 | +1,+1,+1,-1,+1,+1,-1,+1,+1,-1,-1,-1,+1,-1,+1,+1 |

codes C0-C3 have better multipath performance than codes C4-C7.

Table 9 - Walsh-Function Codes

| Func | Hex | Binary logical | Binary algebraic |
| --- | --- | --- | --- |
| W0 | 0000 | 0000000000000000 | +1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1 |
| W1 | 00FF | 0000000011111111 | +1,+1,+1,+1,+1,+1,+1,+1,-1,-1,-1,-1,-1,-1,-1,-1 |
| W2 | 0F0F | 0000111100001111 | +1,+1,+1,+1,-1,-1,-1,-1,+1,+1,+1,+1,-1,-1,-1,-1 |
| W3 | 0FF0 | 0000111111110000 | +1,+1,+1,+1,-1,-1,-1,-1,-1,-1,-1,-1,+1,+1,+1,+1 |
| W4 | 3333 | 0011001100110011 | +1,+1,-1,-1,+1,+1,-1,-1,+1,+1,-1,-1,+1,+1,-1,-1 |
| W5 | 3366 | 0011001101100110 | +1,+1,-1,-1,+1,+1,-1,-1,+1,-1,-1,+1,+1,-1,-1,+1 |
| W6 | 3636 | 0011011000110110 | +1,+1,-1,-1,+1,-1,-1,+1,+1,+1,-1,-1,+1,-1,-1,+1 |
| W7 | 3663 | 0011011001100011 | +1,+1,-1,-1,+1,-1,-1,+1,+1,-1,-1,+1,+1,+1,-1,-1 |
| W8 | 5555 | 0101010101010101 | +1,-1,+1,-1,+1,-1,+1,-1,+1,-1,+1,-1,+1,-1,+1,-1 |
| W9 | 55AA | 0101010110101010 | +1,-1,+1,-1,+1,-1,+1,-1,-1,+1,-1,+1,-1,+1,-1,+1 |
| W10 | 5A5A | 0101101001011010 | +1,-1,+1,-1,-1,+1,-1,+1,+1,-1,+1,-1,-1,+1,-1,+1 |
| W11 | 5AA5 | 0101101010100101 | +1,-1,+1,-1,-1,+1,-1,+1,-1,+1,-1,+1,+1,-1,+1,-1 |
| W12 | 6666 | 0110011001100110 | +1,-1,-1,+1,+1,-1,-1,+1,+1,-1,-1,+1,+1,-1,-1,+1 |
| W13 | 6633 | 0110011000110011 | +1,-1,-1,+1,+1,-1,-1,+1,+1,+1,-1,-1,+1,+1,-1,-1 |
| W14 | 6363 | 0110001101100011 | +1,-1,-1,+1,+1,+1,-1,-1,+1,-1,-1,+1,+1,+1,-1,-1 |
| W15 | 6336 | 0110001100110110 | +1,-1,-1,+1,+1,+1,-1,-1,+1,+1,-1,-1,+1,-1,-1,+1 |

Table 10 - Rademacher-Function Codes

| Func | Hex | Binary logical | Binary algebraic |
| --- | --- | --- | --- |
| R0 | 0000 | 0000000000000000 | +1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1,+1 |
| R1 | 00FF | 0000000011111111 | +1,+1,+1,+1,+1,+1,+1,+1,-1,-1,-1,-1,-1,-1,-1,-1 |
| R2 | 0F0F | 0000111100001111 | +1,+1,+1,+1,-1,-1,-1,-1,+1,+1,+1,+1,-1,-1,-1,-1 |
| R3 | 3333 | 0011001100110011 | +1,+1,-1,-1,+1,+1,-1,-1,+1,+1,-1,-1,+1,+1,-1,-1 |
| R4 | 5555 | 0101010101010101 | +1,-1,+1,-1,+1,-1,+1,-1,+1,-1,+1,-1,+1,-1,+1,-1 |

The Rademacher codes are used to construct the algebraic form of the Walsh codes.  For WN we employ the standard binary representation of N, i.e.,

N=k=0kmax2kbk bk∈0,1

then

WN=k=1kmaxRkbk

The notation SN, CN, WN and RN, etc., refers to the identity of the code, or the corresponding function.  Where needed, to index the sub-elements (bits) of the codes a second subscript is added e.g. WNn.

#### 1.4.6.6	Modulation and Channel Data Rates

The FSD field of the PHY Preamble is transmitted at 1 bit/symbol using DBPSK for high reliability.  The SIGNAL, LENGTH and R/S-CRC fields of the PHY Header are transmitted at 4 bit/symbol using 16-ary BOK.

The MPDU is transmitted at 5 or 9 bits/symbol, respectively, with optional rate 13/15 FEC on the 5-bit/ssymbol modulation.  The primary data-transfer modulation is 5 bits/symbol using 16-ary DBOK to effect 10-Mbps data rate at 32-Mchip/s OQPSK spreading.  FEC provides the “degraded-speed” mode, used, e.g., under poor propagation conditions.  The 9-bit/symbol modulation is the “enhanced-speed” mode, used, e.g., at short range.  All modes are shown in Table 11.

Table 11 - Combinations of Chip Rate, Data Rate and Modulation

| Chip Rate | Data Rate | Modulation (& Coding) | Used For | Regulatory |
| --- | --- | --- | --- | --- |
| 32 Mchip/s OQPSK | 2 Mbps | DBPSK | FSD | FCC, IC, ETSI,MKK |
|  | 8 Mbps | 16-ary OK | SIGNAL, LENGTH, R/S-CRC |  |
|  | 10 Mbps | 16-ary DBOK | MPDU |  |
|  | 8.7 Mbps | 16-ary DBOK/(15,13)FEC | MPDU |  |
|  | 18 Mbps | Constrained 4x4-ary DBOK | MPDU |  |

##### 1.4.6.6.1	DBPSK Signaling

Differential Bi-Phase Shift Keying (DBPSK) conveys a single bit of data per channel symbol by altering the carrier phase by 0 or  radians between symbols.  The encoding of DBPSK data  determines the nth symbol baseband pattern (just prior to the OQPSK modulator) to be

−1d0npn−1Cn

where

d0n is the nth DBPSK data bit,

pn−1=m=0n−1−1d0nis the previous carrier polarity (1 depending upon the history of bits d0n for all previous symbols), and

Cn is the spreading code used on the nth symbol.  The “0” subscript on the DBPSK data bits is maintained for later reference to other modulations.

##### 1.4.6.6.2	16-ary OK Signaling

16-ary Orthogonal Keying (16-ary OK) conveys 4 bits of information by transmitting one of 16 possible orthogonal waveforms for each symbol waveform.  The form of orthogonal keying is chip-wise combination of a spreading code with one of a set of 16 Walsh functions.  The combination of codes is performed by multiplication of the algebraic {1} values.  The encoding of 16-ary OK data determines the nth symbol baseband pattern (just prior to the OQPSK modulator) to be

R1dn4R2dn3R3dn2R4dn1Cn

where

[dn1,dn2,dn3,dn4] is the 4-bit data pattern for the nth symbol (dn1 is first in), and

Cn is the spreading code used on the nth symbol.  Note that here the second subscript on the symbol data bits runs 1 to 4, not 0 to 3.

Demodulation of this signaling technique comprises estimating which of the 16 possible waveforms was transmitted (i.e., selecting the correlator channel with the largest magnitude), which yields 4 bits of information.

##### 1.4.6.6.3	16-ary DBOK Signaling

Differential Bi-Orthogonal Keying (DBOK) combines DBPSK modulation from symbol-to-symbol with orthogonal keying for each symbol waveform.  The form of orthogonal keying is chip-wise combination of a spreading code with one of a set of Walsh functions.  For 16-ary DBOK the spreading code is combined with one of 16 Walsh functions (codes), the selection of which conveys 4 bits of information.  The combination of codes is performed by multiplication of the algebraic {1} values.  The encoding of 16-ary DBOK data determines the nth symbol baseband pattern (just prior to the MOQPSK modulator) to be

R1dn4R2dn3R3dn2R4dn1pn−1−1dn0Cn

where

[dn0,dn1,dn2,dn3,dn4] is the 5-bit data pattern for the nth symbol (dn0 is first in), and

pn−1=m=0n−1−1d0n is the previous carrier polarity (1 depending upon the history of bit dn0 for all previous symbols), and

Cn is the spreading code used on the nth symbol.

Demodulation of this signaling technique comprises:

- estimating which of the 16 possible waveforms was transmitted (i.e., selecting the correlator channel with the largest magnitude), which yields 4 bits of information, then

- performing a DBPSK demodulation between the largest-magnitude complex correlator outputs for the current and previous symbols, which yields the 5th bit.

##### 1.4.6.6.4	(15,13)  Forward Error Correction

The FEC option encodes 13 5-bit information symbols into 15 5-bit channel symbols.  The two check symbols enable correction of any single-symbol error due to a failure of the orthogonal signaling component.  Single-error-correction capability enables fixed block decoding in hardware; combined with the small block size this offers low latency decoding.

The FEC technique recognizes that the DBPSK component of 16-ary DBOK signaling requires 4-dB lower symbol SNR than does the orthogonal-signaling component.  In the absence of orthogonal-signaling errors the probability of error for the DBPSK component is truly negligible.  On the other hand, when a demodulation error is made in the selection of the orthogonal waveform, then the DBPSK error probability is approximately 50%; furthermore, for two adjacent symbols the DBPSK decisions are so effected.   Table 12 shows how these statistics are accommodated in the coding technique.

check

symbols

Table 12 - FEC Coding Block

| d00 | D10 | d20 | d30 | D40 | D50 | d60 | d70 | d80 | d90 | dA0 | dB0 | dC0 | dD0 | DE0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d01 | D11 | d21 | d31 | D41 | D51 | d61 | d71 | d81 | d91 | dA1 | dB1 | dC1 | dD1 | DE1 |
| d02 | D12 | d22 | d32 | D42 | D52 | d62 | d72 | d82 | d92 | dA2 | dB2 | dC2 | dD2 | DE2 |
| d03 | D13 | d23 | d33 | D43 | D53 | d63 | d73 | d83 | d93 | dA3 | dB3 | dC3 | dD3 | DE3 |
| d04 | D14 | d24 | d34 | D44 | D54 | d64 | d74 | d84 | d94 | dA4 | dB4 | dC4 | dD4 | DE4 |

Note: hexadecimal notation used for symbol subscripts of this section to avoid commas in subscripts, e.g., dD3 instead of d13,3.

Encoding

- The 13 4-bit input symbols [dn1,dn2,dn3,dn4], 0£n£12, are encoded using a (15,13) Reed-Solomon code over GF{24}, producing the check symbols [dD1,dD2,dD3,dD4] and [dE1,dE2,dE3,dE4];

- The code polynomial is x2+11x+1, where  is a primitive root of x4+x+1;

- The DBPSK bits dn0 are separated into even and odd streams, and parity on these is computed independently, producing dD0 and dE0.

Decoding

- As the symbols emerge from the demodulator on receive the syndrome is computed, while the information symbols are buffered;

- If the syndrome is [0,0,0,0,0],[0,0,0,0,0] then the buffered information symbols are read out unchanged;

- If the syndrome is not identically all zeros, then the information symbols are read out and corrected as follows;

- The 4-bit symbol in error (the “error symbol”) is changed to its correct value;

- Parity is computed on the even bits dn0, and, if in error the bit closest to the error symbol is complimented;

- Parity is computed on the odd bits dn0, and, if in error the bit closest to the error symbol is complimented.

This single-error-correcting code cannot detect multiple errors within a coding block; the frame CRC provides the overall integrity check.

##### 1.4.6.6.5	Constrained 4x4-ary DBOK Signaling

The 16 Walsh functions used for 16-ary DBOK can be decomposed into 4 sub-groups of 4 each Walsh functions, i.e., X0={W0,W1,W2,W3}, X1={W4,W5,W6,W7}, X2={W8,W9,W10,W11}, X3={W12,W13,W14,W15}.  Each of these can be used to effect 4-ary DBOK signaling.  4x4-ary DBOK is a modulation which results from performing 4-ary DBOK in each of the above sub-groups, then algebraically summing the resulting waveforms.  Such a modulation conveys 12 bits per symbol, but this incurs a 6-dB peak-to-average envelope fluctuation.

Constrained 4x4-ary DBOK uses 4-ary DBOK in 3 sub-groups, then modulates in the fourth sub-group based upon the data for the other sub-groups.  The result is a constant-envelope waveform consistent with saturated (efficient) power amplification.

The encoding of constrained 4x4-ary DBOK data  determines the nth symbol envelope to be

( −1dn0R1dn1R2dn2p0,n−1+−1dn3R1dn4R2dn5R3p1,n−1+−1dn6R1dn7R2dn8R4p2,n−1+−1F(d0,d3,d6)R1G(d1,d4,d7)R2G(d2,d5,d8)R3R4p3,n−1 ) Cn

where

[dn0,dn1,dn2,dn3,dn4,dn5,dn6,dn7,dn8] is the current 9-bit data pattern (dn0 is first in),

p0,n−1=m=0n−1−1d0m is the previous polarity for sub-group X0 (1 depending upon the history of bit dn0 for all previous symbols),

p1,n−1=m=0n−1−1d3m is the previous polarity for sub-group X1 (1 depending upon the history of bit dn3 for all previous symbols),

Table 13 -  F and G

| a | B | c | F(a,b,c) | G(a,b,c) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 |

p2,n−1=m=0n−1−1d6m is the previous polarity for sub-group X2 (1 depending upon the history of bit dn6 for all previous symbols),

p3,n−1=m=0n−1−1d9m is the previous polarity for sub-group X3 (1 depending upon the history of dn9=F(dn0,dn3,dn6)for all previous symbols), and

Cn is the spreading code used on the nth symbol.  The functions F(a,b,c) and G(a,b,c) are given in Table 13.  The function F demands that the pattern [a,b,c,F] have odd parity, while the function G demands that the pattern [a,b,c,G] have even parity.

#### 1.4.6.7	Transmit and Receive In-Band and Out-of-Band Spurious Emissions

The high-rate DSSS PHY shall conform with in-band and out-of-band spurious emissions as set by regulatory bodies.  For the USA, refer to FCC 15.247, 15.202, and 15.209.  For Europe, refer to ETS 300-328.

#### 1.4.6.8	Transmit to Receive Turnaround Time

The transmit-to-receive turn-around time shall be less than 2 s, including the power-down ramp specified in clause .

#### 1.4.6.9	Receive to Transmit Turnaround Time

The receive-to-transmit turn-around time shall be less than 2 s, including the power-up ramp specified in clause .

#### 1.4.6.10	Slot Time

The slot time for the high-rate DSSS PHY shall be the sum of twice the carrier-detect time specified in clause  and the receive-to-transmit turn-around time specified in clause .

#### 1.4.6.11	Transmit and Receive Antenna Port Impedance

When exposed, the transmitter and receiver antenna port(s) shall be of nominal impedance 50.

#### 1.4.6.12	Transmit and Receive Operating Temperature

### 1.4.7	PMD Transmit Specifications

The following clauses describe the transmit functions and parameters associated with the Physical Medium Dependent sub-layer.

#### 1.4.7.1	Maximum Transmit Power Levels

The maximum allowable output power, as measured in accordance with practices specified by the regulatory bodies, is shown in Table 2.  In the USA, the radiated emissions should also conform to the ANSI uncontrolled radiation emission standards (ANSI/IEEE C95.1-1992).

Table 14 - Maximum Transmit Power Levels

| Maximum output power | Geographic location | Compliance Document |
| --- | --- | --- |
| 1000 mw (GT <6 dBi) | USA | FCC 15.247 |
| 100 mW (EIRP) | Europe | ETS 300-328 |
| 200 mW (£10 mW/MHz) | Japan | MPT ordinance 79 |

In the USA, the output power must be reduced from 1000 mW by 1/3 (in dB) of the amount by which the antenna gain exceeds 6 dBi.

#### 1.4.7.2	Minimum Transmitted Power level

The minimum transmit power shall be no less than 1 mW.

#### 1.4.7.3	Transmit Power Level Control

Power control shall be provided for transmitted power greater than 100 mW.  A maximum of 4 power levels may be provided. A radio capable of transmitting more than 100 mW shall at least be capable of switching down to 100 mW.

#### 1.4.7.4	Transmit Spectrum Mask

The peak of the transmitter power spectrum in taken to be 0 dBr.  The transmit power spectral density PT(f) (including spurious products) shall be:

PT(f) < -30 dBr		12 MHz £ |f-fc| £ 20 MHz

PT(f) < -50 dBr		    |f-fc| > 20 MHz

Where fc is the carrier frequency and f is the spectral variable, both in MHz.  The transmitter spectral mask is shown in Figure 3.  The measurements shall be made using 100 kHz resolution bandwidth and 30 kHz video bandwidth.

Figure 3 - Transmit Spectrum Mask (f relative to fc)

The mask shown satisfies the FCC protected bands for 100-mW transmitter power, as well as supporting the ACR specification; to also satisfy MKK requirements the main spectral lobe must be flattened to expand the 90% energy bandwidth to ³20MHz.

#### 1.4.7.5	Transmit Center Frequency Tolerance

The transmit center-frequency tolerance shall be 10 ppm.

#### 1.4.7.6	Chip Clock Frequency Tolerance

The chip-clock-frequency tolerance shall be  ppm.

#### 1.4.7.7	Transmit Power On and Power Down Ramp

The transmitter power-on and power-off, measured from 10%-to-90% and 90%-to-10% of steady-state power, respectively, shall not exceed 2 s.  The transmit power ramps must be consistent with the out-of-band emissions as specified in clause .

#### 1.4.7.8	RF Carrier Suppression

The RF carrier suppression, measured at the channel center frequency, shall be at least 10 dB below the peak of the OQPSK power spectrum.  The RF carrier suppression shall be measured while transmitting randomized data.  The peak of the OQPSK spectrum shall be referenced to 100-kHz resolution.

#### 1.4.7.9	OQPSK Spreading Modulation

The form of OQPSK employed incorporates a phase rotation of ejnπ2=jn on transmit and receive, where n is the index of the chip.  This definition leaves the chip code values the same as they would be for a PSK transmission for a specified code.  On receive, processors can employ PSK-type matched filters or correlators for simplicity, avoiding I/Q channel cross-coupling; this technique is common for MSK waveforms.  In the transmitter, the serial chip stream fed to the modulator is separated into even and odd streams, destined, respectively, for the I and Q modulation channels; these streams further toggle the signs of alternate bits.  This is equivalent to multiplying the original code by the third Rademacher function R3.

### 1.4.8	PMD Receiver Specifications

#### 1.4.8.1	Receiver Minimum Input Level Sensitivity

The frame error rate (FER) shall be less than 5% at an MPDU length of 1024 bytes for an input level of –80 dBm measured at the antenna connector (or equivalent specification, if antenna is built-in).  This FER shall pertain to the 10-Mbps (16-ary DBOK) modulation, and shall be exclusive of frame retransmission protocol(s).

#### 1.4.8.2	Receiver Maximum Input Level

The frame error rate (FER) shall be less than 5% at an MPDU length of 1024 bytes for an input level of -4 dBm measured at the antenna connector (or equivalent specification, if antenna is built-in).  This FER shall pertain to the 10-Mbps (16-ary DBOK) modulation, and shall be exclusive of frame retransmission protocol(s).

#### 1.4.8.3	Receiver Adjacent Channel Rejection

Adjacent channel rejection is defined between two channels (CHNL_ID) as specified in clause , and is the same as the legacy DSSS specification.  The adjacent-channel-rejection specification applies to any two channels separated by ³30 MHz.  The adjacent rejection shall be 35 dB, or greater, at a FER of 5% using the 16-ary DBOK modulation described in clause  and an MPDU length of 1024 bytes.  The adjacent channel rejection shall be measured as follows:

The input signal shall use the 5-bit/symbol, 10-Mbps mode at a level 6-dB greater than that specified in clause .  The adjacent-channel signal shall use the 10-Mbps mode at a level 41-dB greater than that specified in clause .  The adjacent channel signal shall be derived from a separate signal source.  SRCH_IDs and CCC_IDs shall be different for the two channels.  The FER shall be no worse than 5%.

Additionally, ACI between high-rate and legacy 1-/2-Mbps DSSS equipment will also be 35-dB for any two channels separated by ³30 MHz.

#### 1.4.8.4	Clear Channel Assessment

The high-rate DSSS PHY shall provide the capability to perform Clear channel Assessment (CCA) according to the following Carrier-Sense methods:

Mode 1 (required): DSSS under 802.11-18.  CCA shall report a busy medium upon detecting the preamble portion of a DSSS transmission specified under 802.11-18 (this document), using the search code in effect within the BSA.

Mode 2 (optional): DSSS under 802.11-15. CCA shall report a busy medium upon detecting any portion of a 1- or 2-Mbps DSSS transmission using 11-Mchip/s PSK spreading and the Barker code specified in 802.11-15.

Mode 3 (optional): FH under 802.11-14. CCA shall report a busy medium upon detecting any portion of a 1- or 2-Mbps FHSS transmission using 1-symbol/s on center frequencies and with modulation specified in 802.11-14.

Modes 2 and 3, if implemented, must be maskable.

The CCA shall be TRUE if none of the un-masked modes indicates carrier sense condition.  CCA operation is subject to the following criteria:

- Carrier sense must occur for signals above the minimum receive signal level indicated in clause ;

- Carrier sense must occur within the CCA carrier-sense time of 4 s after initiation of signal;

- If an 802.11-18 PLCP Header is received correctly, then the high-rate DSSS PHY shall hold the CCA signal inactive (channel busy) for the full duration as indicated by the PLCP LENGTH field, even if loss of carrier sense should occur before the end of reception;

- If an optional (Mode 2 or Mode 3) carrier sense occurs, then the CCA signal inactive (channel busy) shall persist until the signal is no longer detectable.

### 1.4.9	Interoperability With Legacy Equipment (Optional)

The optional CCA modes for coordinating with legacy 1-/2-Mbps DSSS and FHSS transmissions described in  can be augmented with optional header-transmission and frame transmission capability for DSSS and/or FHSS legacy equipment.  To enable CSMA of the high-rate DSSS by one of the legacy systems (DSSS or FHSS, not both at the same time), the appropriate legacy PLCP header will be transmitted just prior to the actual high-rate DSSS frame.

When the interoperable header format is elected, full physical-level exchange of legacy frames may also be implemented, but is not required.