IEEE P802.11
Wireless LANs

Performance of the Proposed 5-GHz PHY

Date:	Month Day, Year

Author:	John H. Cafarella
	MICRILOR, Inc.
	17 Lakeside Office Park, Wakefield, MA 01880
	Phone: 617-246-0103
	Fax: 617:246-0157
	e-Mail: JohnCafarella@worldnet.att.net

Abstract

# This is a first draft documenting the performance of the 5-GHz proposal.   It is currently incomplete, especially in the area of multipath & noise performance.  It is to be re-issued in approximately one month.

# 1	Performance Summary

## 1.1	Implementation

# RF/IF nothing special relative to other implementations

Baseband chip  25K gates modem function + 10K gates control

Diversity not considered, but no major impact on circuitry.

## 1.2	Immunity or Multipath and Noise

Multipath analysis still in progress.

## 1.3	Overhead Related Parameters

PLCP Preamble & Header < 30 s (diversity not included)

Slot size 25 s; similar to 802-11.15

SIFS could be short, depending upon processor.

## 1.4	Spectral Efficiency and Cell Density Related Parameters

Channelization:	Six 24-Mbps frequency channels

Eight search code channels

48 cyclic data code channels

64K pseudorandom code channels

Cell Planning: BSAs operated with substantial spatial overlap of user areas if on different frequency channels; BSAs on same frequency channel is reasonable separation; always separate code channels as a matter of practice.

Adjacent channel interference:  On same frequency channel, desired signal must exceed interfering signal by  8 dB.  For different frequency channels there is extra 35 dB isolation ( as in low-rate standard); could be 40-45 dB with margin.

Interference Immunity

>14-dB processing gain against CW

12-dB processing gain against >25% BW Gaussian noise

## 1.5	Critical Points

- Phase noise not an issue

- Power consumption attractive

- Complexity not an issue

- RF PA backoff 6 dB; not required for 18 Mbps mode

- Antenna diversity separable issue

## 1.6	Intellectual Property

Letter to IEEE in preparation.  Patents pending.  Will comply with IEEE guidelines.  Policy by 1 March; call Dr. Stanley Reible.

# 2	Multipath Models

## 2.1	Infinitely Diffuse Rayleigh (IDR)

The Infinitely Diffuse Rayleigh path model assumes that the channel impulse response comprises Rayleigh-distributed paths at discrete, uniformly spaced delays, filled in delay

hc(t)=k=0¥akd(t−kts)

where ts is the delay spacing, and ak is a complex amplitude whose real and imaginary components are Gaussian-distributed, with the following equivalent distributions:

p(Reak)=12πσαke−Reak22σαk2p(Imak)=12πσαke−Imak22σαk2	real/imaginary components

- - - - -

p(ak)=akσαk2e−ak22σαk2p(Argak)=12π		magnitude & phase

The path strengths are exponentially distributed in amplitude vs. delay

σαk2=σ02e−kτsTRMS with σ02=1−e−τsTRMS2

where TRMS is the multipath RMS delay spread.  The normalization of the path strengths holds the total received signal power constant as other parameters are varied.  The multipath-delay spacing is the shorter of the spread-spectrum chip time Tc or half the delay spread TRMS.

# 3	Signal Model

The transmit waveform has the baseband representation

s(t)=mn=0MjnCmnpT(t−(n+16m)Tc)

where

Tc is the inverse of the chip frequency,

n is the chip index within a symbol,

there are M=16 chips per symbol,

m is the symbol index,

Cmn is the chip code, and

pT(t) is the single-chip waveform:

During the acquisition preamble the chip value is

Cmn=CSn

where

CSn is the search PN code, 0£n<M-1.

In the data portion of a frame the chip value is

Cmn=14s=03dmsPmnWKmsn

where

dms is the polarity specified by the DBPSK component of the 4-ary DBOK signaling in sub-channel s,

Pmn is the PN code during the mth symbol,

WKmsn is the sub-channel s Walsh function during the mth symbol, and

Kms is specified by the 4-ary OK signaling component in sub-channel s.

When transmitted, s(t) is convolved with the channel impulse response to yield the received waveform

r(t)=k=0∞akmn=0M−1jnCmnpT(t−kτs−(n+mM)Tc)

At the receiver complex Gaussian noise z(t) is added, which obeys

z(t)=0z(t)z(t')=0z(t)z(t')=2N0δ(t−t')

where N0 is the one-sided noise spectral density.  The waveform is then convolved with the aggregate receive filter to form x(t)=w(t)+q(t).  The deterministic part w(t) (i.e., r(t) filtered) is

r(t)=k=0∞akmn=0M−1jnCmnpR(t−kτs−(n+mM)Tc)

where pR(t) is the chip-pulse waveform after receive filtering.

The noise component q(t) (i.e., z(t) filtered) has variance

q(t)2=σN2=2N0BN

where BN is the noise bandwidth of the receiver.

The filtered receive waveform is sampled at the chip rate to produce the sequence xn=wn+qn.

wn=r(t0+nTc)=k=0∞akmn'=0M−1jn'Cmn'pR(t0+nTc−kτs−(n'+mM)Tc)

The statistics of samples of the noise qn will be the same as q(t).  During acquisition the strongest path (at some delay k0s) is selected for demodulation.  We explicitly define the sampling time reference to correspond to the correct sampling time for the first chip of symbol m0 on this strongest signal component; that is, we select t0=k0s+m0MTc.  Thus, the signal sequence is

wn=k=0∞akmn'=0M−1jn'Cmn'pR((k0−k)τs+(n−n'+(m0−m)M)Tc)

The correlator reference sample sequence is j-nBm0n; the correlator output is

YBm0=n=0M−1j−lBm0nwn=k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'pR((k0−k)τs+(n−n'+(m0−m)M)Tc)

A number of chip values can contribute to the nth sample because of the width of pR(t) and also because of the multipath delay spread.  It is most natural to measure the delay spread in multiples, or sub-multiples, of the chip time because it simplifies analysis and simulations.  For 0 delay spread we have a Gaussian channel; when the delay spread is a single chip time, then it is necessary to half-chip sampling of the multipath profile; for integer multiples greater than 1 the multipath profile is sampled at the chip time.  The multipath will thus be sampled at Tc or ½Tc; the possible sampled values of pR(t) are

pR0=pR(0)=1pR12=pR(T2)=pR(−T2)pR1=pR(T)=pR(−T)

Samples at 1.5Tc or greater from the time of the peak are approximately zero.  We can make the replacement

pR((k0−k)τs+(n−n'+(m0−m)M)Tc)=δn−n'−(m0−m)M+(k0−k)τsTc+pR12δn−n'−(m0−m)M+(k0−k)τsTc+12+δn−n'−(m0−m)M+(k0−k)τsTc−12+pR1δn−n'−(m0−m)M+(k0−k)τsTc+1+δn−n'−(m0−m)M+(k0−k)τsTc−1

where the function x=1 if the index equals 0, and x=0 otherwise).

YBm0=k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc+pR12k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc+12+pR12k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc−12+pR1k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc+1+pR1k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc−1

Observations:

-  The first term is the principal contribution for each path, i.e., samples at the peak of the chip waveform.

-  The second two terms (having coefficients pR½) are introduced when the multipath is sampled at half the chip time.

-  The last two terms (coefficients pR1) reflect the inter-chip effects of using, MSK or filtered PSK.

This may be further developed by specializing to multipath sampling at the chip time or half that value.  In the following two equations, the expression in braces {} is a required constraint on remaining variables to be consistent with the original limits on the summation over n’.  Note that in all cases this constraint has the form {0 £ n+£ M-1}.

For s=Tc

YBm0=k=0∞akmn=0M−1j−(m0−m)M−(k0−k)Bm0nCm,n+(m0−m)M+(k0−k) 0≤n+(m0−m)M+(k0−k)≤M−1+pR1k=0∞akmn=0M−1j−(m0−m)M−(k0−k)−1Bm0nCm,n+(m0−m)M+(k0−k)+1 0≤n+(m0−m)M+(k0−k)+1≤M−1+pR1k=0∞akmn=0M−1j−(m0−m)M−(k0−k)+1Bm0nCm,n+(m0−m)M+(k0−k)−1 0≤n+(m0−m)M+(k0−k)−1≤M−1

For s=½Tc

k−k0even∞n=0M−1j−(m0−m)M−(k0−k)2+1Bm0nCmn+(m0−m)M+(k0−k)2−1 0≤n+(m0−m)M+(k0−k)2−1≤M−1

We introduce the acyclic correlation function RBC as follows:

RBmCm'Δ=0 Δ≤−M=j−Δn=−ΔM−1BmnCm',n+Δ −M<Δ<0j−Δn=0M−1−ΔBmnCm',n+Δ 0≤Δ<M0 Δ≥M

This is a standard definition of the crosscorrelation of code sequences, except that the j- factor introduced by MSK signaling has also been absorbed into the definition.  With this, the above equations become

These equations represent the outputs Y of correlators matched to reference codes B, due to input codes C at various multipath delays.  It is necessary to further specialize these to consider signal demodulation and detection performance; this will be done in the next two sections.

The receiver filtering will be intentionally broadened relative to ideal matched filtering to avoid excessive inter-chip ISI (keep pR1 small); thus, the noise will be uncorrelated from sample to sample.  The noise variances of correlator outputs is

σY2=MσN2=2MN0BN

For s=Tc

YBm0=k=0∞akmRBm0Cm,(m0−m)M+(k0−k)+pR1k=0∞akmRBm0Cm,(m0−m)M+(k0−k)+1+pR1k=0∞akmRBm0Cm,(m0−m)M+(k0−k)−1

When multiple correlator outputs are considered, the correlator noise is uncorrelated between correlators because the reference functions are orthogonal.

For s=½Tc

k−k0even∞

In the following sections, we shall focus on delay spreads greater than or equal to 2Tc.  This avoids considerable analytical complexity, since sampling at s=Tc avoids handling correlations induced by sampling the multipath at half the chip rate.  This restricts our attention to delay spreads ³62.5 ns for the 32-Mchip/s modulation.

# 4	Demodulation Performance

## 4.1	Demodulation Mode Formulation

We begin with the expression for the correlator outputs specialized to the case of 4x4-ary signaling, where the transmitted waveforms are Cmn=14s=03dmsPmnWKmsn and the correlators are matched to Bm=PmWK.  The correlator outputs for symbol m0 are

YPm0WK=k=0∞ak14s=03mdmsRPm0WK,PmWKms,(m0−m)M+(k0−k)+pR1k=0∞ak14s=03mdmsRPm0WK,PmWKsm,(m0−m)M+(k0−k)+1+pR1k=0∞ak14s=03mdmsRPm0WK,PmWKms,(m0−m)M+(k0−k)−1

From here on we shall neglect the terms in pR1.  These reflect inter-chip effects due to bandwidth reduction, for example as  encountered when PSK signals are filtered, and in any case for MSK-like signals.  The value of pR1 is approximately .2; by contrast we must consider correlation side lobes of order .5 or higher relative to the autocorrelation peak, and multipath amplitudes which can be equal to the path being demodulated.  Thus, the effects reflected by carrying the pR1 terms are of second-order in significance, and carrying them would complicate the analysis without benefit.  The correlator outputs are

YPm0WK=k=0∞ak14s=03mdmsRPm0WK,PmWKms,(m0−m)M+(k0−k)

We expect that symbol waveform m0 can be significantly effected by the two immediately preceding symbols and the following symbol, for any multipath model.  For 2 Msymbol/s signaling this is a span of 2 s.  Over this range of symbols we expand the sum

YPm0WK=k=0∞ak14s=03[dm0+1RPm0WK,Pm0+1WKm0+1,s,−M+(k0−k)+dm0RPm0WK,PmWKms,(k0−k)+dm0−1RPm0WK,Pm0−1WKm0−1,s,M+(k0−k)+dm0+2RPm0WK,Pm0−2WKm0−2,s,2M+(k0−k)]

In addition to the above signal components there is thermal noise.  The correlator outputs are Rician-distributed with SNR

γK=YPm0WK2σY2=YPm0WK22MN0BN

The probability of symbol error, conditioned on the specific waveforms s=s=03Pm0+1WKm0+1,s,s=03Pm0WKm0s,s=03Pm0−1WKm0−1,s,s=03Pm0−2WKm0−2, and the multipath profile a=α0,α0,α0 . . ., is

Pe|s,a=1−S0∞drKm0s rKm0sσ2 e−rKm0s22σ2−γKm0s I0(rKm0sσ22γKm0s)K≠Km0s0rKm0drKs rKsσ2 e−rKs22σ2−γKs I0(rKsσ22γK)

In general it is necessary to average this probability of error over relevant distributions for s and a.

## 4.2	Gaussian channel

For the Gaussian channel we have |0|2=1 and (hence k0=0) and k=0 (k>0), so the signal component of the detection correlator outputs are

YPm0WJ=a014RPm0WJ,Pm0WKm0,0=14α0δJKm0s

where nm is the conventional Kronecker delta.  These are the outputs of 16 correlators matched to the waveform set generated by combining the PN code used for the mth symbol Pm0 with each of the 16 Walsh functions WJ.  The output SNR is

γ=YPm0WKm02σY2=M2α022MN0BN=M4 SNRIN

Figure 1 – Pe  Approximation

Union

Bound

Numerical

We may evaluate the error probability for 4x4-ary OK using the union bound

Pe(γ)≈1−(1−32 e−γ2)4

The exact OK probability of error is given by

Pe=1−0∞dr1 r1σ2 e−r122σ2−γ I0(r1σ22γ)0r1dr2 r2σ2 e−r222σ234

where r1 is the Rician variate of the correct correlator output in a sub-group of four, and r2 represents the Rayleigh variates of the other three incorrect correlator outputs.  This has no closed-form solution, which is, of course, why the approximation is needed.  Figure 1 compares numerical evaluation of the integrals to the approximation.

## 4.3	IDR Channel, TRMS³2Tc

THIS SECTION IN PROGRESS

# 5	Detection Performance

## 5.1	Acquisition Mode Formulation

The signal model was formulated to support analysis of Noncoherent BiOrthogonal Keying with PN codes changing from symbol to symbol.  In this section we specialize to the acquisition signal; i.e., a 16-chip code CSn repeated from symbol to symbol.  The codes and their cyclic correlations are tabulated in section . The cyclic correlation of two codes is

RBmCm'Δ=j−Δn=0M−1BmnCm', n+Δ(modM)

Consider the primary contribution (peak of chip waveform) to a correlator output YB for symbol m0 due to a unit-amplitude signal component at delay ks

YB=mRBmCm',(m0−m)M+(k0−k)=RBmCm',(k0−k)+RBmCm',(M+k0−k) k>k0=RBmCm',(k0−k)+RBmCm',(−M+k0−k) k<k0RBmCm',(k0−k)

where we have recognized that the two acyclic correlations in each case actually make up the full cyclic correlation (using jM=1).  The correlator output, with the reference matched to and aligned with the signal at lag k0c, is

YB=k=0∞akRSS,(k0−k)+pR1k=0∞akRSS,(k0−k)+1+pR1k=0∞akRSS,(k0−k)−1

During the search phase, the signal is repeated continually; the cyclic correlation function, in effect, aliases multipath components delayed by one or multiple symbols, whereas during demodulation these become intersymbol interference.  We now explore this fold-over effect.

There are only MTc possible sample times within the periodic symbol timing (assuming delay spread ³2Tc).  Multipath at lag (mM+n)Tc appears summed with multipath at lag nTc, for all m.  Because of this, we may re-cast the multipath description by summing multipath components separated by multiples of MTc.  For path models having strengths which are complex Gaussian, such a summation will also be complex Gaussian (Rayleigh amplitude).  For example, the re-cast (cyclic) IDR channel impulse response is

Hc(t)=k=0M−1βkδ(t−kτs) τs=Tc

where

σβk2=σ02e−kTcTRMS1−e−MTcTRMS

## 5.2	Gaussian channel

For the Gaussian channel we have |0|2=1 and (hence k0=0) and k=0 (k>0), so the signal component of the detection correlator output is

YB=RSS0=Mα0

and the output SNR is

γ=YB2σY2=M2α022MN0BN=M SNRIN

The probability of detection vs.  for a given probability of false alarm may be found as the non-fluctuating-target case in books on radar, e.g., DiFranco and Rubin, although the reader is cautioned that radar texts generally use R=2 as the signal to noise, whereas communication texts use .

## 5.3	IDR Channel, TRMS³2Tc

Although the search correlator employs the same single-sample-per-chip computation as does the demodulation correlator, this is actually stepped along at half-chip intervals to limit the “straddling loss” to a fraction of a dB.  During demodulation k0 corresponds to the strongest multipath; during acquisition k0 can be considered a hypothesis to be tested, i.e., whether the signal at delay k0s exceeds the threshold.  Thus, we may compute the probability of detection as one minus the probability that correlator outputs for k0=0 to M-1 all fall below the threshold.  The correlator outputs for the k0 are

YBk0=k=0M−1βkRSS,(k0−k)+pR1k=0M−1βkRSS,(k0−k)+1+pR1k=0M−1βkRSS,(k0−k)−1

We ignore autocorrelation side lobes, assuming

RSSΔ=Mδ0Δ

which results in

YBk0=βk0+pR1βk0+1+pR1βk0−1βk0

To the same level of approximation, we can set pR1=0 for detection since it is lower than expected autocorrelation side lobes.  For a single-component Rayleigh-fluctuating signal the probability of detection vs. mean signal-to-noise ratio and probability of false alarm is given by

Pd=Pfa1(1+γ)

Where  is the mean SNR.  For detection we test correlator amplitudes for M times of arrival, and the signal is missed only if detection fails on all M.  For signal timing corresponding to k0, the mean path SNR for the IDR channel is

γk0=YBk02σY2=M2βk022MN0BN=M2σβk02 SNRTOT

The definition of SNRTOT comes from

k0=0M−1YBk02=k0=0M−12σβk02=2σ021−e−MTcTRMSk0=0M−1e−kTcTRMS=2σ021−e−TcTRMS=1

independent of TRMS for the IDR channel.  Thus, the probability of detection in multipath is

Pd=1−k0=0M−11−Pfa1(1+M2σβk02 SNR)

## 5.4	Performance

Figure 2 shows the probability of missed detection vs. mean input SNR for the IDR channel, and vs. SNR for the Gaussian channel, for a detection criterion requiring threshold crossings on three successive symbols, at overall probability of false alarm of 10-6. Requiring multiple threshold crossings enables improvement of detection performance, relative to single-symbol detection, without incurring the complexity of video combining before testing against a threshold.  If we budget 1% of frame loss to missed detection, then it is clear from the figure that IDR multipath spreads from about 60 to 250 ns will incur 4- to 5-dB of fading loss, but that detectability improves monotonically with SNR.

Figure 2 - Detection Performance

# 6	Interference Rejection

Previous sections assessed link performance in noise and multipath.  This section considers interference rejection in the absence of multipath propagation effects.

## 6.1	Interference Rejection Formulation

From  the signal response for the mth  symbol is

YPmWJ=a014RPWJ,PWKm,0=M4α0δJKm

where Pm is the PN code, J represents the Walsh-function correlator outputs and Kms are the signal variants transmitted, respectively, 0 is the signal strength and nm is the Kronecker delta.

The baseband representation of the interfering signal is x(t)ej2πυt, where x(t) is the envelope and  is the offset frequency.  It is assumed that the bandwidth of x(t) is small enough, and that  is restricted to be sufficiently close to the signal carrier frequency, that no substantial roll-off due to receive filtering can be included in the processing gain calculation.  The interference is sampled and applied to the correlator bank to produce

YPmWJ=RPWJ,X=n=0M−1PmnWJnxnej2πnυTc

We consider next the cases of narrowband CW interference and of band-limited Gaussian interference.

## 6.2	Narrowband CW Interference

We assume the interference to be I ej2πυt, where I is the complex amplitude of the interference.  The correlator outputs are

YPmWJ=RPWJ,X=I n=0M−1PmnWJnej2πnυTc

As shown in document IEEE P802.11-97/116, for CW interference there is an abrupt drop to zero probability of error for some CW interference level when the largest possible interference output can no longer influence the data decision.  We may determine this threshold by finding

Max over m,J n=0M−1PmnWJnej2πnυTc

Figure 3 - Correlator output responses.

for the (M=16 bit) PN codes to be used.  The 8 PN codes (coset leaders) have identical statistics in this regard.  We show here the case of Pmn=0158H.  Figure 1 shows YPmWJ2 (measured in dB relative to coherent response) vs. Tc for all 16 correlator outputs.  The worst-case situation occurs for  within a symbol bandwidth of center frequency, where 2 correlator outputs produce outputs only 6-dB down from a coherent signal.  As the CW interference level approaches the zero-error threshold, it is only the two channels having –6-dB responses for the interference which are of interest.  With approximately ¼ probability both of these channels fall in the same sub-set of Walsh functions.  The signal would emerge from one of these two channels, and it is possible for the CW interference to produce an amplitude out of an incorrect channel while simultaneously lowering the amplitude of the correct channel.  An error will occur when

I2+α04≤I2

Taking the arbitrary phase angle between the interference and signal to be , an error occurs for

2Iα0+1≤2Iα0 or 1+4Iα02+2Iα0cos(Θ)≤4Iα02

If a solution to this inequality exists, we may identify a boundary angle B between angles corresponding to making an error and angles for which no error occurs.  The solution for this angle is

cos(ΘB)=−2Iα0−1=−γ4 where γ≡α02I2

is the signal-to-interference ratio.  There clearly can be no error for >4; for <4 the solution for B is

ΘB=±cos−1(−γ4)

The random phase  is distributed over 2, so the probability of error is simply the fraction of 2 for which an error will occur times the 1/4 probability that the correct channel is one of the two channels under consideration, or

Pe(γ)=14πcos−1(γ4)

It must be remembered that this is the asymptotic behavior near =4; for low signal-to-interference ratio the probability of error is larger than indicated by the above expression.

## 6.3	Narrowband Gaussian Interference

In this case we assume that x(t) is a Gaussian random variable.  The correlator outputs due to noise are

YPmWJ=n=0M−1PmnWJnxnej2πnυTc

It is assumed that the noise bandwidth is small enough, and that the frequency offset is confined, such that no substantial roll-off due to receive filtering is included as processing gain.

The processed interference remains Gaussian, since the correlation process is linear, so it remains only to find the mean-square correlator outputs.

YPmWJ2=n=015PmnWJnznn'=015Pmn'WJn'zn'=n=015n'=015PmnWJnPmn'WJn'znzn'=2σ2 n=015n'=015PmnWJnPmn'WJn' e−an−n'Tc

In the case of white Gaussian noise, the correlator outputs are equal-variance, uncorrelated Gaussian variables.  The above equation can be used to explore the effect of the bandwidth of the Gaussian interference.  Pm=0158H was used, combined with the 16 Walsh functions to calculate the correlator outputs shown in Figure 4.

Figure 4 - Correlator output varainces, normalized to 22, in dB relative to coherent response.

For very narrowband Gaussian interference (a®0) anomalous results are obtained.  Two of the correlator outputs produce interference outputs suppressed only 6 dB relative to the coherent signal; this is due to lack of one-zero  balance in the code for those correlator channels.  On the other hand, six of the correlator outputs provide essentially infinite rejection of narrowband interference.  The remaining eight outputs yield the same 12-dB suppression of narrowband interference as is the case for wideband interference.

Asymptotically (a®) all 16 correlator outputs produce the 12-dB suppression of the Gaussian interference expected from the 16 chips per symbol.  It is clear that for noise bandwidths greater than approximately 25% of the chip rate (or 4 times the symbol rate) the processing gain is essentially the nominal 12 dB.

As shown above, 4x4-ary OK splits the signal between the four sub-set channels, incurring 6-dB reduction of signal amplitude.

## 6.4	Performance

The S/I performance is summarized in Figure 5.  CW interference produces a large error probability until the S/I approaches 0 dB, at which point the probability of error drops abruptly to zero.  For Gaussian interference the probability of error was estimated using the union bound, with the signal emerging from the correlator channel having the smallest variance; that is, summing the binary-orthogonal error probabilities for S/I set by the (largest) individual channel responses to the interference.  This was done for 5%, 10% and 20% noise bandwidth relative to the chip rate (fnTc).  Also shown is the asymptotic behavior if the noise were decorrelated from chip to chip.  At 20% noise bandwidth the performance is within a fraction of a dB of the asymptotic behavior for large noise bandwidth.

Figure 5 – Symbol error probability for CW and Gaussian interference

GAUSSIAN

FnTc=5%

CW

=10%

=20%

Interpreting this curve in terms of the “processing gain” test, the dashed curve (decorrelated-noise limit) represents the 12-dB which is ten times log10 of the number of chips per symbol; CW interference would appear to have PG of 14 dB, while Gaussian noise at 10% and 5% would yield 10.9-dB and 9.7-dB, respectively.

# 7	Appendix: Approximate MSK

The 12-bit/symbol modulation exhibits a 6-dB peak/average amplitude fluctuation.  The individual chip waveforms will have the shape of MSK chip pulses, although their amplitudes will fluctuate.  The result will be an average power spectrum which appears the same as for MSK.  In the 9- and 5-bit/symbol modes the waveforms are truly of constant envelope, hence these will be MSK transmissions in fact.  The approximation to MSK to be implemented departs from an ideal, matched MSK system in the following:

- the generation process combines a staircase approximation to a cosine pulse followed by a filter to offer an inexpensive implementation;

- the transmit spectrum has lower side bands than ideal MSK;

- the receive processor does not exactly match the chip waveform, preferring to keep the bandwidth somewhat higher for better multipath resolution.

The ideal MSK waveform has the baseband representation

s(t)=njncnpMSK(t−nTc)

where

Tc is the inverse of the chip frequency,

n is the chip index,

cn is the chip value, and

pMSK(t) is the single-chip MSK waveform:

pMSK(t)=cos(pt2Tc)t<Tc0t<Tc

Ideal processing of the MSK waveform at the receiver begins with the (analog) chip matched filter (CMF) for the MSK pulse.

pCMF(t)=1Tccos(pt2Tc)t<Tc0t<Tc

which results in the baseband waveform

w(t)=k=0¥aknjncnRMSK(t−kts−nTc)

where the chip autocorrelation function is

RMSK(t)=(1−t2Tc)cos(pt2Tc)+1psin(pt2Tc)t<2Tc0t>Tc

By contrast, the proposed implementation begins with a generator for the chip waveform which produces

pG(t)=1 t<.5Tc.37 .5Tc<t<Tc0 t>Tc

The transmit filtering is designed to minimize the energy taken out of the main spectral lobe while suppressing the side lobes.  This may be accomplished using, e.g., 5th–order baseband filters plus some IF bandpass filter.  As a result of this filtering, the generated pulse shape is replaced by the transmitter (equivalent-baseband) pulse shape pT(t), where

pT(t)=pG(t)∗hT(t)

so that the baseband representation of the transmitter waveform is

s(t)=njncnpT(t−nTc)

On receive, the waveform is passed through equivalent IF and baseband filtering.  The resulting baseband complex signal is

s(t)=njncnpR(t−nTc)

where pR(t) is

pR(t)=pG(t)∗hT(t)∗hR(t)

Of significance is that the pulse shape of PR(t) is of considerably less time extent that would be the case for a true MSK pulse passed through an exact chip matched filter.  Chip-to-chip amplitude overlap with this approach is typically less than .2, compared to .5 for true MSK.  This is important for good multipath performance.

# 8	Appendix: Search Codes

The 16-bit codes selected for use during search and CSn are given in the following table, along with their cyclic autocorrelation functions, symmetric about the main lobe.

| CSn (hex) | Rss0 | Rss1 | Rss2 | Rss3 | Rss4 | Rss5 | Rss6 | Rss7 | Rss8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44BC | 16 | 0 | 0 | 0 | 0 | 0 | -4 | 0 | -4 |
| A0DC | 16 | 0 | 0 | 0 | 0 | 0 | -4 | 0 | -4 |
| D223 | 16 | 0 | 0 | 0 | 0 | 0 | -4 | 0 | -4 |
| 0A76 | 16 | 0 | 0 | 0 | 0 | 0 | -4 | 0 | -4 |
| 425C | 16 | 0 | 0 | 0 | 0 | 4 | 0 | -4 | 0 |
| 23A4 | 16 | 0 | 0 | 0 | 0 | 4 | 0 | -4 | 0 |
| 245C | 16 | 0 | 0 | 0 | 0 | -4 | 0 | 4 | 0 |
| A243 | 16 | 0 | 0 | 0 | 0 | -4 | 0 | 4 | 0 |

These codes have excellent correlation properties for signal detection and selection of the strongest multipath component, having 4 or 5 zero values for autocorrelation side lobes nearest to the main lobe.  When it is required to select codes for independent operation of BSAs, it is important to consider the peak and average cross-correlation values between the different codes (dB relative to main lobe) as shown below.

|  | 44BC |  | A0DC |  | D223 |  | 0A76 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Peak | rms | peak | rms | Peak | rms | Peak | rms |
| 44BC | 0 | -19.3 | -2.5 | -11.3 | -6.0 | -11.3 | -6.0 | -11.3 |
| A0DC | -2.5 | -11.3 | 0 | -19.3 | -6.0 | -11.3 | -6.0 | -11.3 |
| D223 | -6.0 | -11.3 | -6.0 | -11.3 | 0 | -19.3 | -2.5 | -11.3 |
| 0A76 | -6.0 | -11.3 | -6.0 | -11.3 | -2.5 | -11.3 | 0 | -19.3 |
| 425C | -4.1 | -12.0 | -8.5 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 |
| 23A4 | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -8.5 | -12.0 |
| 245C | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -8.5 | -12.0 |
| A243 | -4.1 | -12.0 | -8.5 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 |

|  | 425C |  | 23A4 |  | 245C |  | A243 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Peak | rms | peak | rms | Peak | rms | Peak | rms |
| 44BC | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 |
| A0DC | -8.5 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -8.5 | -12.0 |
| D223 | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 |
| 0A76 | -4.1 | -12.0 | -8.5 | -12.0 | -8.5 | -12.0 | -4.1 | -12.0 |
| 425C | 0 | -18.1 | -6.0 | -11.1 | -6.0 | -13.3 | -6.0 | -13.3 |
| 23A4 | -6.0 | -11.1 | 0 | -18.1 | -6.0 | -13.3 | -6.0 | -13.3 |
| 245C | -6.0 | -13.3 | -6.0 | -13.3 | 0 | -18.1 | -6.0 | -11.1 |
| A243 | -6.0 | -13.3 | -6.0 | -13.3 | -6.0 | -11.1 | 0 | -18.1 |

See Appendix on MSK Approximation.

Although the correlator reference will be properly aligned to the signal, it is necessary to provide for mismatch of the reference and signal to later support 16-ary Orthogonal demodulation.

Because of the linearity of filtering, the chip pulse is approximately symmetric.

This is the Kronecker delta x0 with half-integer values of x allowed.

This is convenient analytically, and it also means that the results can be used directly for PSK waveforms as well as for MSK.

0 still carries an unknown propagation phase.

If a true matched filter were used, then this would be the familiar E/N0.

We shall evaluate the probability of symbol error based only upon the orthogonal-signaling component; in the absence of such errors, the error probability for the DBPSK component is completely negligible.

We use ~ (tilde) to distinguish cyclic correlations from acyclic.

Note that the normalization is preserved, i.e., summing k over the corresponding range of k.

0 still carries an unknown propagation phase.

If a true matched filter were used, then this would be the familiar E/N0.

DiFranco and Rubin, Radar Detection.

Op Cit, DiFranco and Rubin.

We have ignored here the pR1 terms as second-order.

This ignores the receive filtering reduction on the noise, but this limit is simply for comparison.

The implementation loss is assumed the same for all cases.

The filter’s lack of causality is of no concern for present purposes.