IEEE P802.11
Wireless LANs

Additional comments on IEEE Std 802.11-1997

Date:	January 19, 1998

Author:	Henri Moelard
	Lucent Technologies
	Zadelstede 1-10, Nieuwegein, The Netherlands
	Phone: +31 30 6097 551
	Fax: +31 30 6097 556
	e-Mail: moelard@lucent.com

Abstract

This document contains some comments on the 802.11-1997 standard, outside the scope of the maintenance letter ballot 12.

| Seq. # | Clause number | your voter’s ID code |  | Cmnt type E, e, T, t |  | Part of NO vote |  | Comment/Rationale | Recommended change |  | Disposition/Rebuttal |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7.3.1.6 | hmo | E |  | n.a. |  | Reference to MIB attribute aListenInterval. This attribute is no longer in the MIB. |  |  | Refer to MLME-ASSOCIATE.request parameter ListenInterval (see 10.3.6.1) |  |  |  |
| 2 | 11.2.1.1 | hmo | e |  | n.a. |  | In Table 23 a reference is made to MIB attribute aListenInterval. This attribute is no longer in the MIB. |  |  | Refer to “Listen Interval”. |  |  |  |
| 3 | 11.2.1.9 | hmo | e |  | n.a. |  | A reference is made to MIB attribute aListenInterval. This attribute is no longer in the MIB, and the AP should not use the STA’s MIB attributes for normal operation. |  |  | Refer to “Listen Interval”. |  |  |  |
| 4 | 7.3.1.8 | hmo | t |  | n.a. |  | AID value 0 is not (any longer) used to announce broadcast and multicast frames. The TIM Bitmap Control field is used for that. |  |  | Remove the last paragraph of clause 7.3.1.8. |  |  |  |
| 5 | 7.3.2.4 | hmo | e |  | n.a. |  | The Current Channel Number is not clearly defined. |  |  | Refer to CHNL_ID defined in clause 15.4.6.2. |  |  |  |
| 6 | 8.2.3 | hmo | E |  | n.a. |  | The last paragraph before Figure 45, states: “The IV is followed by the MPDU”. This is incorrect; an MPDU is inclusive MAC Header and trailer. |  |  | Change to: “The IV is followed by the Frame Body”. |  |  |  |
| 7 | 8.2.5 | hmo | E |  | n.a. |  | Figure 46, and the note below it, use: “Data (PDU)” and “Data field”. This is incorrect (undefined). |  |  | Change to: “Frame Body”. |  |  |  |
| 8 | 8.2.5 | hmo | E |  | n.a. |  | The paragraph following Figure 46, states: “... IV field immediately preceding the MPDU”. This is incorrect; an MPDU is inclusive MAC Header and trailer. |  |  | Change to: “... IV field immediately preceding the Frame Body”. |  |  |  |
| 9 | 8.2.5 | hmo | E |  | n.a. |  | The paragraph following Figure 46, defines the pad field, but does not state anything about its value. |  |  | Define that the pad field can be any value. |  |  |  |
| 10 | 4, 9.2.5.2, 9.3.3 | hmo | T |  | n.a. |  | STA Short Retry Count and STA Long Retry Count are defined and incremented and reset, but their purpose is not specified; there are no limits defined, and no actions upon exceeding these. |  |  | Remove these definitions and usages. |  |  |  |
| 11 | 6.2.1.3.2 | hmo | E |  | n.a. |  | Uses MIB attributes aShortRetryMax and aLongRetryMax. These have been renamed. |  |  | Use terms dot11ShortRetryLimit and dot11LongRetryLimit. |  |  |  |
| 12 | 9.3.2.2 | hmo | e |  | n.a. |  | CFDurRemaining |  |  | CFPDurRemaining. |  |  |  |
| 13 | 15.3.2 | hmo | E |  | n.a. |  | Table 58 contains attributes no longer available in the MIB, and/or with wrong names. |  |  | Bring in-line with new MIB. |  |  |  |
| 14 | A.4.4.1 | hm0 | e |  | n.a. |  | PC15 refers to Annex C, whereas the MIB is in Annex D. |  |  | Change to Annex D |  |  |  |
| 15 | A.4.4.1 | hmo | e |  | n.a. |  | PC15.3 defined two times. |  |  | Change second one to PC15.5. |  |  |  |