IEEE P802.11
Wireless LANs

Sharing Performance Evaluation for TGa PHY Submission

Date:	Match 1998

Author:	Richard van Nee+, Masahiro Morikura and Hitoshi Takanashi

Lucent Technologies+      NTT Wireless Systems Labs.

Previous proposals from Lucent Technologies and NTT are merged into one proposal that is described by a draft of full document uploaded to the ftp site. Evaluations of the merged proposal are performed in Lucent and NTT. Evaluations of 16-QAM are reported by Lucent, that of DQPSK and DPSK are reported by NTT.

Submission List

1) Sharing Performance Evaluation for TGa PHY Submission (this document)

2) Computer simulation results on view graphs

3) A draft of a merged full document

General Description

(Lucent: 16-QAM (30 Mbit/s and 20 Mbit/s) with coherent detection / NTT: DQPSK (15 Mbit/s, 10 Mbit/s and DBPSK (5 Mbit/s) with differential detection)

| Parameter | Value(s) |
| --- | --- |
| Data Rates Supported | 30, 20, 15, 10, 5 Mbit/s |
| Channel Spacing | 15 MHz |
| Center Frequencies | Lower band ;5170,5185, 5200, 5215, 5230 Middle band ; 5270,5285, 5300, 5315, 5330 Upper band ; 5745,5760, 5775, 5790, 5805 |
| Power Levels | 5.15-5.25 GHz: 30 mW 5.25-5.35 GHz: 150 mW 5.725-5.825 GHz: 600 mW |
| Sensitivities | 30 Mbit/s : -77 dBm (coherent) 20 Mbit/s : -81 dBm (coherent) 15 Mbit/s : -83.5 dBm (differential) 10 Mbit/s : - 86.3dBm (differential) 5 Mbit/s : -89.6 dBm (differential) |
| CCA threshold | -78 dBm |
| Clock Rate accuracy | 40 ppm maximum |
| Carrier Frequency accuracy | 40 ppm maximum |
| Waveform implementation accuracy specification method | insensitive |
| Power Backoff in RF PA | 5 dB for the all rate (Output Back off) |
| Implementation Complexity | Hardware size : 173 k gates (Base band) Power Consumption : (Full duty cycle) 30 Mbit/s : 186 mW 20 Mbit/s : 124 mW 15 Mbit/s : 93 mW 10 Mbit/s : 62 mW 5 Mbit/s : 31 mW |

Per-Rate Feature Summary

| Parameter | 30 Mbit/s | 20 Mbit/s | 15 Mbit/s | 10 Mbit/s | 5 Mbit/s |
| --- | --- | --- | --- | --- | --- |
| Data rate | 30 Mbit/s | 20 Mbit/s | 15 Mbit/s | 10 Mbit/s | 5 Mbit/s |
| ECC method | r=3/4 Convolutional | r=1/2 Convolutional | r=3/4 Convolutional | r=1/2 Convolutional | r=1/2 Convolutional |
| Interleaving method | block of 192 bits | block of 192 bits | 12 x 8 | 12 x 8 | 6 x 8 |
| Suggested minimal sensitivity NF = 10 dB / 1000 Byte packet | -75 dBm (coherent) | -72 dBm (coherent) | -78 dBm (differential) | -81 dBm (differential) | -84 dBm (differential) |
| Suggested Co-Channel rejection required CIR of 1000 Byte packet + | -16 dB (coherent) | -12 dB (coherent) | -10 dB (differential) | -7 dB (differential) | -4 dB (differential) |
| Suggested Adjacent Channel rejection required CIR of 1000 Byte packet + | 11 dB (coherent) | 15 dB (coherent) | 17 dB (differential) | 21 dB (differential) | 23 dB (differential) |
| Suggested Alternate Channel rejection required CIR of 1000 Byte packet + | 50 dB (coherent) | 50 dB (coherent) | 46 dB (differential) | 49 dB (differential) | 50 dB (differential) |
| Implementation Accuracy | Not sensitive by skipping the center sub carrier |  |  |  |  |

Performance

If the receiver implementation complexity can be traded for performance, bring data for typical (simpler?) implementation and for extended (possible, but higher end) implementation.

Performance in Noise and Multipath

Attach graphs of PER vs. Eb/N0, for

1) AWGN channel

2) Exponential Profile Rayleigh Fading channel for TRMS = 25 nsec

3) Exponential Profile Rayleigh Fading channel for TRMS = 50 nsec

4) Exponential Profile Rayleigh Fading channel for TRMS = 100 nsec

5) Exponential Profile Rayleigh Fading channel for TRMS = 150 nsec

6) Exponential Profile Rayleigh Fading channel for TRMS = 250 nsec

7) Attach graph of PER vs. TRMS without additive noise, covering a range of 10 nsec to 500 nsec

Bring the graphs for each data rate supported by the proposed PHY, for packet lengths of 64 and 1000 bytes.

Per-Rate Performance Summary

If the receiver implementation complexity can be traded for performance, bring data for typical (simpler?) implementation and for extended (possible, but higher end) implementation.

Differential Detection

| Parameter | 30 Mbit/s | 20 Mbit/s | 15 Mbit/s | 10 Mbit/s | 5 Mbit/s |
| --- | --- | --- | --- | --- | --- |
| Eb/No at PER=10%, AWGN, 64b | 10 dB | 8 dB | 7.6 dB | 6.5 dB | 6.4 dB |
| Trms at PER=10%, noise free, 64b | 175 ns | 300 ns | 320 ns | > 500 ns | > 500 ns |
| Eb/No @ 20%, with Trms @ 10%, 64b | 20 dB | 16 dB | 20.6 dB | 15.8 dB | 10.9 dB*1 |
| Eb/No at PER=10%, AWGN, 1000b | 12 dB | 10 dB | 8.7 dB | 7.7 dB | 7.4 dB |
| Trms at PER=10%, noise free, 1000b | 150 ns | 225 ns | 240 ns | 460 ns | > 500 ns |
| Eb/No @ 20%, with Trms @ 10%, 1000b | 23 dB | 20 dB | 23.7dB | 19.9 dB | 13.9 dB*1 |
| CCI immunity [dB] | -14 dB | -10.5 dB | -8.3 dB (64B) -9.4 dB (1000B) | -5.5 dB (64B) -6.6 dB (1000B) | -2.3 dB (64B) -3.1 dB (1000B) |
| ACI immunity [dB] | 14 dB | 18 dB | 19.3 dB (64B) 17.8 dB (1000B) | 22.8 dB (64B) 21.3 dB (1000B) | 25.7 dB (64B) 23.8 dB (1000B) |
| CW jammer immunity [dB] | -18 dB | -13 dB | -13.5 dB (64B) -15.2 dB (1000B) | -2.2 dB (64B) -7.7 dB (1000B) | 3.5 dB (64B) 0.9 dB (1000B) |
| Narrowband Gaussian noise immunity [dB] | -18 dB | -13 dB | -11.9 dB (64B) -13.5 dB (1000B) | -7.7 dB (64B) -9.6 dB (1000B) | -3.7 dB (64B) -5.5 dB (1000B) |
| Phase noise tolerance, (BW=50 kHz), rad2 [dBc] at which PER becomes 10% | -14 dBc | -13 dBc | -11.7 dBc (64B) -14.1 dBc (1000B) | -10.1 dBc (64B) -12.5 dBc (1000B) | -6.0dBc (64B) -8.0 dBc (1000B) |

*1 : PER never exceed 10 % with Trms of 25 - 500 ns. PER of 20% is assumed instead at Trms of 500 ns.

Timing and Overhead related parameters

Attach verbal explanation of the assumptions taken for each parameter

| Attribute | Suggested Value |
| --- | --- |
| aSlotTime | 6 s |
| aCCATime | <4 s |
| aRxTxTurnaroundTime | implementation dependent |
| aTxPLCPDelay | implementation dependent |
| aRxTxSwitchTime | implementation dependent |
| aTxRampOnTime | Not applicable |
| aTxRFDelay | implementation dependent |
| aSIFSTime | 13s |
| aRxRFDelay | 4 s |
| aRxPLCPDelay | 7 s |
| aMACProcessingDelay | implementation dependent |
| aTxRampOffTime | Not applicable |
| aPreambleLength | 19s |
| aPLCPHdrLength | 4 µs (for 30Mbit/s) 5 µs (for 20Mbit/s) 6 µs (for 15Mbit/s) 7 µs (for 10Mbit/s) 12 µs (for 5Mbit/s) |
| aMPDUDurationFactor | (coding rate) -1 |
| aAirPropagationTime | <1 µs |
| aCWmin | 15 |
| aCWmax | 1023 |

Level definitions

The suggested minimal sensitivity depends on a system design.