IEEE P802.11
Wireless LANs

TGa Comparison Matrix per 98/156r2

Date:	April 22, 1998

Author:	Naftali Chayat
	BreezeCom

Abstract

This document concentrates the results as submitted by the three proposers on April 12, 1998 with updates done until April 22. The submissions were per tentative template 98/156 draft 2 as distributed among the proposers on April 8, 1998, while the updated data reflects an updated template 98/156r2. The data was compiled into single document by Naftali Chayat (NC) and undergone some editing. The main changes were bringing data to a common format (for example when different proposers used I/C or C/I), and reducing the amount of data in the tables, whenever multiple options were covered. Revision 1 contains corrections of backoff data in BreezeCom+NEC data,

The full data, including graphs, is in documents 98/166 (Lucent+NTT), 98/167r1 (BreezeCom+NEC) and 98/168 (RadioLAN). Please refer to the template document 98/156r2 for explanation of the conditions and methods of computation.

Follows a short summary of the three proposals being considered by the committee.

| # Company | # Lucent Tech. + NTT | # BreezeCom + NEC | # RadioLAN |
| --- | --- | --- | --- |
| Modulation method | OFDM DBPSK, DQPSK or 16-QAM in each subcarrier | Offset Quadrature Modulation (OQPSK/OQAM) | Differential Pulse Position Modulation (16-DPPM, 4-DPPM) |
| Pulse shaping features | 48 subcarriers out of 64 | 50% Square-Root Raised Cosine | 50% Square-Root Raised Cosine |
| Error Correction Coding | Convolutional K=7, R=1/2 or R=3/4 Inter-carrier interleaving | Hamming (31,26) with interleaving, uncoded option | Uncoded, option for Reed Solomon (15,13) coded with interleaving |
| Rates supported | 5 Mbit/s (DBPSK, R=1/2) 10 Mbit/s (DQPSK, R=1/2) 15 Mbit/s (DQPSK, R=3/4) 20 Mbit/s (16-QAM, R=1/2) 30 Mbit/s (16-QAM, R=3/4) | 21 Mbit/s (OQPSK, coded) 25 Mbit/s (OQPSK, uncoded) 42 Mbit/s (OQAM, coded) 50 Mbit/s (OQAM, uncoded) | 10 Mbit/s (4-DPPM) 20 Mbit/s (16-DPPM) 20 Mbit/s (16-DPPM, coded) |
| Number of channels in U-NII band | 5 in 100 MHz, 10 in 200 MHz 15 MHz channel spacing | 4 in 100 MHz, 9 in 200 MHz 20 MHz channel spacing | 3 in 100 MHz, 7 in 200 MHz 30 MHz channel spacing |
| Applicable documents | 97/92, 97/123, 97/137, 98/02, 98/03, 98/12, 98/71r1, 98/72, 98/73, 98/74, | 98/76r1, 98/109, 98/144 | 97/145r1, 98/38, 98/75, 98/132, 98/133 |

TGA Performance Template

General Description, Parameters Common for all Rates

| Parameter | BreezeCom + NEC | Lucent Tech. + NTT | RadioLAN |
| --- | --- | --- | --- |
| Data Rates Supported | 20.9677 Mbit/s (mand), 25.0000 Mbit/s (mand), 41.9355 Mbit/s (opt), 50.0000 Mbit/s (opt), Next rates are covered by the definition but are virtually impractical: 62.9032 Mbit/s, 75.0000 Mbit/s, 83.8710 Mbit/s, 100.0000 Mbit/s | 30, 20 (mandatory), 15 (mandatory), 10, 5 Mbit/s | 10 Mbps uncoded 20 Mbps, uncoded 20 Mbps, coded (optional) |
| Channel Spacing | 20 MHz | 15 MHz | 30 MHz |
| Center Frequencies | lower band: 5170, 5190, 5210, 5230, 5250 MHz middle band: 5270, 5290, 5310, 5330 MHz upper band: 5745, 5765, 5795, 5815 GHz | Lower band : 5170, 5185, 5200, 5215, 5230 MHz Middle band : 5270, 5285, 5300, 5315, 5330 MHz Upper band : 5745, 5760, 5775, 5790, 5805 MHz | 9-Channels Plan: Lower: 5170 MHz, 5200 MHz, 5230 MHz Middle: 5270 MHz, 5300 MHz, 5330 MHz Upper: 5745 MHz, 5775 MHz, 5805 MHz |
| Power Levels | Lower band: 30 mW Middle band: 150 mW Upper band: 600 mW (derived from 12.5 MHz) | 5150 - 5250 MHz : 30mW 5250 - 5350 MHz : 150mW 5725 - 5825 MHz : 600mW | 9-Channels Plan: Lower: 50 mW Middle: 250 mW Upper: 1000 mW |
| CCA threshold | -82 dBm (suggested) | -78 dBm | -70 dBm |
| Clock Rate accuracy | 10ppm | 40 ppm maximum | 100 ppm |
| Carrier Frequency accuracy | 10 ppm (60 kHz) | 40 ppm maximum | 100 ppm |
| Waveform implementation accuracy specification method | RMS residual ISI when optimizing with respect to slack parameters – frequency, phase and timing offset, and a short equalizer | Insensitive | Per Waveform Mask |
| Implementation Complexity | Breeze estimate: 100-200 Kgates, depending on equalizer length. The power consumption for the implementation with 8 tap forward filter is expected to be 300 mW with equalizer adaptation and 200 mW without adaptation along the packet. (based on 0.25 micron process) NEC estimate: 40kgate for simple receiver 100kgate for complex receiver | Hardware size : 173 k gates (Baseband) Power consumption @ 0.35 mm technology 30 Mbit/s : 157 mW 20 Mbit/s : 124 mW 15 Mbit/s : 115mW 10 Mbit/s : 96mW 5 Mbit/s : 83 mW (Full duty cycle) | The complexity of the RF transmitter, the RF receiver, and the baseband processor for L-PPM is rather low. The receiver is non-coherent and therefore quite simple. The baseband processor will be a single-chip Silicon integrated circuit implemented in either high-speed CMOS, or BiCMOS. The DC power consumption is quite efficient due to the pulsed nature of this system, with a low duty cycle. We believe this to be the simplest and most readily achievable approach as compared to the other five proposals. |

Per-Rate Feature Summary

| Proposal and Rate | ECC method | Interleaving method | Suggested minimal sensitivity | Suggested Adjacent Channel rejection | Suggested Alternate Channel rejection | Implementation Accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| LT+NTT 5 Mb | R=1/2, Convolutional | 6x8 | -84dBm (differential) | 23dB (differential) | 50dB (differential) | Not sensitive by skipping the center subcarrier |
| LT+NTT 10 Mb | R=1/2, Convolutional | 12x8 | -81dBm (differential) | 21dB (differential) | 49dB (differential) | as above |
| LT+NTT 15 Mb | R=3/4, Convolutional | 12x8 | -78dBm (differential) | 17dB (differential) | 46dB (differential) | as above |
| LT+NTT 20 Mb | R=1/2, Convolutional | 24x8 | -75dBm (coherent) | 15dB (coherent) | 50dB (coherent) | as above |
| LT+NTT 30 Mb | R=3/4, Convolutional | 24x8 | -72dBm (coherent) | 11dB (coherent) | 50dB (coherent) | Not sensitive by skipping the center subcarrier |
| Br+NEC 21 Mb | Hamming (31,26,3) | 8 bit depth row-col | -77 dBm | 18 dB | 30 dB | -20 dB residual ISI |
| Br+NEC 25 Mb | none | none | -75 dBm | 18 dB | 30 dB | -20 dB residual ISI |
| Br+NEC 42 Mb | Hamming (31,26,3) | 16 bit depth row-col | -67 dBm | 8 dB | 20 dB | -30 dB residual ISI |
| Br+NEC 50 Mb | none | none | -65 dBm | 8 dB | 20 dB | -30 dB residual ISI |
| RadioLAN 10 Mb | None | None | -77 | 17 | 35 | Low |
| RadioLAN 20 Mb | None | None | -75 | 15 | 30 | Medium |
| RadioLAN 20+RS | Reed-Solomon (15, 13) | Depth 4 | -72 | 13 | 30 | Medium |

Per-Rate Performance Summary

Performance in Noise and Multipath

The Received Power is defined as  -174 dBm/Hz + (NF=10 dB) + 10log(Bit_Rate) + Eb/No. For example, at 20 Mbit/s, at Eb/No=12 dB, Pr= -174 dBm/Hz +10dB +73dBHz +12 dB = -79 dBm.

| Proposal and Rate | Pr [dBm] at PER=10%, AWGN, 64b | Pr [dBm] at PER=10%, AWGN, 1000b | Trms at PER=10%, noise free, 64b | Trms at PER=10%, noise free, 1000b | Pr [dBm] @ 20%, with Trms @ 10%, 64b | Pr [dBm] @ 20%, with Trms @ 10%, 1000b |
| --- | --- | --- | --- | --- | --- | --- |
| LT+NTT 5 Mb | -90.6 dBm | -89.6 dBm | > 500 ns | > 500 ns | -86.1 dBm*1 | -83.1 dBm*1 |
| LT+NTT 10 Mb | -87.5 dBm | -86.3 dBm | > 500 ns | 460 ns | -78.2 dBm | -74.1 dBm |
| LT+NTT 15 Mb | -84.6 dBm | -83.5 dBm | 320 ns | 240 ns | -71.6 dBm | -68.5 dBm |
| LT+NTT 20 Mb | -83.0 dBm | -81.0 dBm | 300 ns | 225 ns | -75.0 dBm | -71.0 dBm |
| LT+NTT 30 Mb | -79.2 dBm | -77.2 dBm | 175 ns | 150 ns | -69.2 dBm | -66.2 dBm |
| Br+NEC 21 Mb | -83.5 dBm | -82.5 dBm | 182 nsec (Br) 175 nsec (N8) 120 nsec (N4) | 174 nsec (Br) 175 nsec (N8) 120 nsec (N4) | -72 dBm | -71 dBm |
| Br+NEC 25 Mb | -81.5 dBm | -80 dBm | 167 nsec (Br) 175 nsec (N8) 120 nsec (N4) | 164 nsec (Br) 175 nsec (N8) 120 nsec (N4) | -72 dBm | -71 dBm |
| Br+NEC 42 Mb | -76 dBm | -75.5 dBm | 83 nsec | 75 nsec | -66 dBm | -66 dBm |
| Br+NEC 50 Mb | -74 dBm | -73 dBm | 77 nsec | 73 nsec | -66 dBm | -65 dBm |
| RadioLAN 10 Mb | -90 dBm | -89 dBm | 200 nsec | 180 nsec | -73 dBm | -70 dBm |
| RadioLAN 20 Mb | -85 dBm | -84 dBm | 150 nsec | 140 nsec | -66 dBm | -63 dBm |
| RadioLAN 20+RS | -83 dBm | -82 dBm | 140 nsec | 130 nsec | -65 dBm | -64 dBm |

Notes:

- (by L+N) PER never exceed 10% with Trms of 25 - 500 ns. PER of 20% is assumed instead at Trms of 500 ns

- (by NC) In Breeze+NEC the (Br) denotes performance with (8+23 DFE), the (N8) denotes performance with (8+16 PMA-MLSCE), and (N4) with (4+16 PMA-MLSCE). This data is brought in full because it represents different implementation approaches.

Performance in Interference

| Proposal and Rate | AWGN Sensitivity @NF=10 dB, no degr. [dBm] | CCI immunity [dB] | ACI immunity [dB] | CW jammer immunity [dB] | Narrowband Gaussian noise immunity [dB] | Phase noise tolerance, [dBc] |
| --- | --- | --- | --- | --- | --- | --- |
| LT+NTT 5 Mb | -89.6 dBm | -3.1dB | 23.8dB | 0.9 dB | -5.5 dB | -8.0 dBc |
| LT+NTT 10 Mb | -86.3 dBm | -6.6dB | 21.3dB | -7.7dB | -9.6dB | -12.5dBc |
| LT+NTT 15 Mb | -83.5 dBm | -9.4dB | 17.8dB | -15.2dB | -13.5dB | -14.1dBc |
| LT+NTT 20 Mb | -81.0 dBm | -12.3 dB | 16.5 dB | -15.7 dB | -14.3 dB | -11 dB |
| LT+NTT 30 Mb | -77.2 dBm | -16.5 dB | 12.5 dB | -20.9 dB | -19.4 dB | -12.5 dB |
| Br+NEC 21 Mb | -82.5 dBm | -9 dB | 22.5 dB (2 dB OBO) | -9 dB | -12 dB | -12.5 dBc |
| Br+NEC 25 Mb | -80 dBm | -10 dB | 20.5 dB (2 dB OBO) | -11 dB | -14.5 dB | -14 dBc |
| Br+NEC 42 Mb | -75.5 dBm | -16 dB | 14 dB (4 dB OBO) | -17 dB | -19 dB | -17.5 dBc |
| Br+NEC 50 Mb | -73 dBm | -17 dB | 11.5 dB (4 dB OBO) | -19 dB | -21 dB | -19.5 dBc |
| RadioLAN 10 Mb | -77 dBm | -8 dB | 17 dB | -9 dB | -6 dB | 5 dBc |
| RadioLAN 20 Mb | -75 dBm | -10 dB | 15 dB | -10 dB | -9 dB | 7 dBc |
| RadioLAN 20+RS | -72 dBm | -10 dB | 13 dB | -11 dB | -10 dB | 8 dBc |

Note: (by NC) Original L+N data contains both 64 and 1000 byte data; here only 1000 byte data is presented. In all submissions, I inverted the sign of immunity values whenever required to bring them to common notation.

PA Backoff and Link Budget (see Appendix D for explanation)

| Proposal and Rate | Backoff [dB] @LB Pmax (LB U-NII regulations) | Backoff [dB] @MB Pmax (MB U-NII regulations) | Backoff [dB] @LB Pmax (restricted regulations) | Backoff [dB] @MB Pmax (restricted regulations) | Backoff [dB] @Psat=250 mW, (restricted regulations) |
| --- | --- | --- | --- | --- | --- |
| LT+NTT 5 Mb | 4 dB ? | 5 dB | 8 dB | 10dB | 8 dB |
| LT+NTT 10 Mb | 4 dB ? | 5 dB | 8 dB | 10dB | 8 dB |
| LT+NTT 15 Mb | 4 dB ? | 5 dB | 8 dB | 10dB | 8 dB |
| LT+NTT 20 Mb | 4 dB ? | 5 dB | 8 dB | 10dB | 8 dB |
| LT+NTT 30 Mb | 4 dB ? | 5 dB | 8 dB | 10dB | 8 dB |
| Br+NEC 21 Mb | 2 dB | 2 dB | 5.5 dB | 7.5 dB | 6 dB |
| Br+NEC 25 Mb | 2 dB | 2 dB | 5.5 dB | 7.5 dB | 6 dB |
| Br+NEC 42 Mb | 4 dB *1 | 4 dB *1 | 6.5 dB | 8.5 dB | 7 dB |
| Br+NEC 50 Mb | 4 dB *1 | 4 dB *1 | 6.5 dB | 8.5 dB | 7 dB |
| RadioLAN 10 Mb | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| RadioLAN 20 Mb | 1 | 1 | 1 | 1 | 1 |
| RadioLAN 20+RS | 1 | 1 | 1 | 1 | 1 |

Note: (by Br+NEC) The backoff at 42/50 Mb/s with U-NII regulations is dominated by performance degradation rather than by the regulatory restrictions.

| Proposal and Rate | AWGN Sensitivity @NF=10 dB, no degr. [dBm] | Loss [dB] @LB Pmax | Loss [dB] @MB Pmax | Loss [dB] at @Psat=250 mW, (MB U-NII regulations) | Loss [dB] at @Psat=250 mW, (restricted regulations) |
| --- | --- | --- | --- | --- | --- |
| LT+NTT 5 Mb | -89.6 dBm | 104.4 dB (30 mW) | 111.4 dB (150 mW) | 108.6 dB | 105.6 dB |
| LT+NTT 10 Mb | -86.3 dBm | 101.1 dB | 108.1 dB | 105.3 dB | 102.3 dB |
| LT+NTT 15 Mb | -83.5 dBm | 98.3 dB | 105.3 dB | 102.5 dB | 99.5 dB |
| LT+NTT 20 Mb | -81.0 dBm | 95.8 dB | 102.8 dB | 100.0 dB | 97.0 dB |
| LT+NTT 30 Mb | -77.2 dBm | 92.0 dB | 99.0 dB | 96.2 dB | 93.2 dB |
| Br+NEC 21 Mb | -82.5 dBm | 97 dB (30 mW) | 104 dB (150 mW) | 104 dB | 100.5 dB |
| Br+NEC 25 Mb | -80 dBm | 94.5 dB | 101.5 dB | 101.5 dB | 98 dB |
| Br+NEC 42 Mb | -75.5 dBm | 90 dB | 97 dB | 95.5 dB | 92.5 dB |
| Br+NEC 50 Mb | -73 dBm | 87.5 dB | 94.5 dB | 93 dB | 90 dB |
| RadioLAN 10 Mb | -77 dBm | 94 dB (50 mW) | 101 dB (250 mW) | 100.5 | 100.5 |
| RadioLAN 20 Mb | -75 dBm | 92 dB | 99 dB | 98 | 98 |
| RadioLAN 20+RS | -73 dBm | 90 dB | 97 dB | 96 | 96 |

Note: (by NC) Original L+N data contains both 64 and 1000 byte data; here only 1000 byte data is presented. In addition, the original L+N data contains similar table for link budget with 150 nsec multipath. This was not part of the template 98/156 draft 2 and there is no comparison data from other proposers, therefore it is not brought here.

Interference Limited Aggregate Rate (see Appendix E for explanation)

Indoor (35 log(distance ratio)) propagation model:

| Proposal and Rate | CCI immunity [dB] | D2/D1, Interferer to Transmitter dist. ratio, indoor | Fraction of Area covered, indoor | Aggregate rate per AP, single rate, indoor | Aggregate rate per AP, multirate, indoor | Aggregate rate per AP, multirate, multichannelindoor |
| --- | --- | --- | --- | --- | --- | --- |
| LT+NTT 5 Mb | 3.1 | 1.23 | 0.807 | 4.04 | 4.04 | 40.4 |
| LT+NTT 10 Mb | 6.6 | 1.54 | 0.618 | 6.18 | 7.13 | 71.3 |
| LT+NTT 15 Mb | 9.4 | 1.86 | 0.490 | 7.36 | 9.58 | 95.8 |
| LT+NTT 20 Mb | 12.3 | 2.25 | 0.380 | 7.59 | 11.48 | 114.8 |
| LT+NTT 30 Mb | 16.5 | 2.96 | 0.255 | 7.65 | 14.03 | 140.3 |
| Br+NEC 21 Mb | 9 dB | 1.807 | 0.507 | 10.6 Mbit/s | 10.6 (1 rate) | 95.4 Mbit/s |
| Br+NEC 25 Mb | 10 dB | 1.931 | 0.465 | 11.6 Mbit/s | 12.5 (2 rates) | 112.5 Mbit/s |
| Br+NEC 42 Mb | 16 dB | 2.865 | 0.268 | 11.2 Mbit/s | 17.0 (3 rates) | 153 Mbit/s |
| Br+NEC 50 Mb | 17 dB | 3.06 | 0.243 | 12.1 Mbit/s | 18.9 (4 rates) | 170 Mbit/s |
| RadioLAN 10 Mb | 8 | 1.692667 | 0.55169 | 5.516896 | 5.516896 | 33.1 |
| RadioLAN 20 Mb | 10 | 1.930698 | 0.465713 | 9.314251 | 10.17402 | 61.05 |
| RadioLAN 20+RS | 11 | 2.061986 | 0.426632 | 8.532643 | 10.17402 | 60.05 |

Free Space (20 log(distance ratio)) propagation model:

| Proposal and Rate | CCI immunity [dB] | D2/D1, Interferer to Transmitter dist. ratio, free space | Fraction of Area covered, free space | Aggregate rate per AP, single rate, free space | Aggregate rate per AP, multirate, free space | Aggregate rate per AP, multirate, multichannel free space |
| --- | --- | --- | --- | --- | --- | --- |
| LT+NTT 5 Mb | 3.1 | 1.43 | 0.678 | 3.39 | 3.39 | 33.9 |
| LT+NTT 10 Mb | 6.6 | 2.14 | 0.406 | 4.06 | 5.42 | 54.2 |
| LT+NTT 15 Mb | 9.4 | 2.95 | 0.256 | 3.84 | 6.70 | 67.0 |
| LT+NTT 20 Mb | 12.3 | 4.12 | 0.153 | 3.05 | 7.46 | 74.6 |
| LT+NTT 30 Mb | 16.5 | 6.68 | 0.068 | 2.03 | 8.14 | 81.4 |
| Br+NEC 21 Mb | 9 dB | 2.818 | 0.274 | 5.76 Mbit/s | 5.76 (1 rate) | 51.8 Mbit/s |
| Br+NEC 25 Mb | 10 dB | 3.162 | 0.231 | 5.77 Mbit/s | 6.68 (2 rates) | 60.1 Mbit/s |
| Br+NEC 42 Mb | 16 dB | 6.31 | 0.075 | 3.15 Mbit/s | 7.95 (3 rates) | 71.5 Mbit/s |
| Br+NEC 50 Mb | 17 dB | 7.08 | 0.061 | 3.06 Mbit/s | 8.44 (4 rates) | 76.0 Mbit/s |
| RadioLAN 10 Mb | 8 | 2.511886 | 0.324324 | 3.24324 | 3.24324 | 19.46 |
| RadioLAN 20 Mb | 10 | 3.162278 | 0.230886 | 4.617723 | 5.552101 | 33.3 |
| RadioLAN 20+RS | 11 | 3.548134 | 0.193372 | 3.867439 | 5.552101 | 33.3 |

Timing and Overhead related parameters

| Parameter | BreezeCom + NEC | Lucent Tech. + NTT | RadioLAN |
| --- | --- | --- | --- |
| aSlotTime | 7.4 ms | 6 s | 3 |
| aCCATime | 3.0 ms | < 4s | 1.2 |
| aRxTxTurnaroundTime | 1.4 ms | 8.8 s | 1 |
| aTxPLCPDelay | 0.4 ms | << 1 s | 0.2 |
| aRxTxSwitchTime | 0.4 ms. | << 1 s | 0.1 |
| aTxRampOnTime | 0.4 ms. | Not applicable | 0.5 |
| aTxRFDelay | 0.4 ms. | < 8.8s | 0.2 |
| aSIFSTime | 13.4 ms | 13s | 5 |
| aRxRFDelay | 1.0 ms. | 4s | 0.2 |
| aRxPLCPDelay | 7.0 ms. | 7 s | 0.2 |
| aMACProcessingDelay | 2.0 ms | < 2s | 0.5 |
| aTxRampOffTime | 0.4 ms. | Not applicable | 1.5 |
| aPreambleLength | 12.8 ms | 19 s | 12.8 |
| aPLCPHdrLength | 3.2 ms | s (for 30 Mbit/s) 5s (for 20 Mbit/s) 6s (for 15 Mbit/s) 7s (for 10 Mbit/s) 12s (for 5 Mbit/s) | 4.8 |
| aMPDUDurationFactor | 1.0000 (25, 50 Mbit/s) 1.1923 (21, 42 Mbit/s) | (coding rate)-1 | 1 |
| aAirPropagationTime | 0.8 ms | < 1 s | 0.3 |
| aCWmin | 15 | 15 | 15 |
| aCWmax | 1023 | 1023 | 512 |

Throughput penalty (ratio of MPDU duration to transaction duration)

| Proposal and Rate | 1500B MPDU duration (sec) | DIFS + backoff (sec) | 1500B packet duration | SIFS | ACK packet duration, same rate | ACK packet duration, basic rate | Efficiency, ACK at same rate | Efficiency, ACK at basic rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LT+NTT 5 Mb | 2400 | 70 | 2478.2 | 13 | 54.2 | 30.2 | 0.91764 | 0.92614 |
| LT+NTT 10 Mb | 1200 | 70 | 1249.4 | 13 | 39.8 | 30.2 | 0.87451 | 0.88067 |
| LT+NTT 15 Mb | 800 | 70 | 841.4 | 13 | 35.0 | 30.2 | 0.83385 | 0.83805 |
| LT+NTT 20 Mb | 600 | 70 | 635.0 | 13 | 30.2 | 30.2 | 0.80192 | 0.80192 |
| LT+NTT 30 Mb | 400 | 70 | 433.4 | 13 | 30.2 | 30.2 | 0.73180 | 0.73180 |
| Br+NEC 21 Mb | 572.8 | 83.7 | 601.28 | 13.4 | 22.1 | 22.1 | 0.795 | 0.795 |
| Br+NEC 25 Mb | 480 | 83.7 | 506.88 | 13.4 | 20.5 | 22.1 | 0.769 | 0.767 |
| Br+NEC 42 Mb | 286.4 | 83.7 | 308.64 | 13.4 | 19.9 | 22.1 | 0.673 | 0.669 |
| Br+NEC 50 Mb | 240 | 83.7 | 261.44 | 13.4 | 18.3 | 22.1 | 0.639 | 0.630 |
| RadioLAN 10 Mb | 1200 | 30.5 | 1221.6 | 5 | 32.8 | 32.8 | 0.9303046 | 0.9303046 |
| RadioLAN 20 Mb | 600 | 30.5 | 620 | 5 | 25.6 | 32.8 | 0.8809279 | 0.8717129 |
| RadioLAN 20+RS | 600 | 30.5 | 620 | 5 | 25.6 | 32.8 | 0.8809279 | 0.8717129 |