IEEE P802.11
Wireless LANs

Ballot result and summary of major changes to IEEE 802.11/D4 Rev.

Date:	May 21, 1998

Author:	Vic Hayes
	Lucent technologies
	Zadelstede 1-10
	3431 JZ Nieuwegein, the Netherlands
	Phone: +31 30 609 7528
	Fax: +31 30 609 7556
	e-Mail: vichayes@lucent.com

Ballot result

The LMSC ballot on IEEE P802.11 closed on May 13, 1998 with the following result:

90	Number of eligible people in Ballot Group

68	Affirmative votes

0	Negative votes

4	Abstention votes

72 	Votes =	80 % Returned (75 % required)

5 % Abstebtion

68	Affirmative votes

0	Negative votes

68	Votes =	100 % Affirmative

5 voters submitted comment

These comments were addressed and the disposition of the comment is contained in doc.: 98/213.

The resolution resulted in changes, some of which can be considered to be technical. The full file is available in doc.: 802.11rev/D5.1.

The major changes are summarised in the remainder of this document.

# List of major changes made in IEEE P802.11rev

## Addition of new notification and other objects related to disassociation and authentication:

dot11Deauthenticate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11DeauthenticateReason,

dot11DeauthenticateStation }

STATUS current

DESCRIPTION

"The deauthenticate notification shall be sent when the STA

sends a Deauthentication frame. The value of the notification

shall include the MAC address of the MAC to which the Deauthentication

frame was sent and the reason for the deauthentication.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 2 }

dot11AuthenticateFail NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AuthenticateFailStatus,

dot11AuthenticateFailStation }

STATUS current

DESCRIPTION

"The authenticate failure notification shall be sent when the STA

sends an Authentication frame with a status code other than

“successful”. The value of the notification

shall include the MAC address of the MAC to which the Authentication

frame was sent and the reason for the authentication failure.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 3 }

dot11DisassociateReason OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently transmitted Reason                  Code in a Disassociation frame.  If no Disassociation frame has been transmitted, the value of this attribute shall be 0.”

REFERENCE “IEEE Std 802.11-1997, 7.3.1.7"

::= { dot11StationConfigEntry 15 }

dot11DisassociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the Address 1 field of the most recently transmitted Disassociation frame.  If no Disassociation frame has been transmitted, the value of this attribute shall be 0."

::= { dot11StationConfigEntry 16 }

dot11DeauthenticateReason OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently transmitted Reason                  Code in a Deauthentication frame.  If no Deauthentication frame has been transmitted, the value of this attribute shall be 0."

REFERENCE “IEEE Std 802.11-1997, 7.3.1.7"

::= { dot11StationConfigEntry 17 }

dot11DeauthenticateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the Address 1 field of the most recently transmitted Deauthentication frame.  If no Deauthentication frame has been transmitted, the value of this attribute shall be 0."

::= { dot11StationConfigEntry 18 }

dot11AuthenticateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently transmitted Status                  Code in a failed Authentication frame.  If no failed Authentication frame has been transmitted, the value of this attribute shall be 0."

REFERENCE “IEEE Std 802.11-1997, 7.3.1.9"

::= { dot11StationConfigEntry 19 }

dot11AuthenticateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the Address 1 field of the most recently transmitted failed Authentication frame.  If no failed Authentication frame has been transmitted, the value of this attribute shall be 0."

::= { dot11StationConfigEntry 20 }

## Clarification of the WEP secret key and IV

102. In the ninth paragraph of 8.2.3, replace the sentence “The 64-bit PRNG seed is formed using the secret key as the most significant 40 bits and the initialization vector (IV) as the least significant 24 bits.” with “The PRNG seed is 64 bits.  Bits 0 through 23 of the IV correspond to bits 0 through 23 of the PRNG seed, respectively. Bits 0 through 39 of the secret key correspond to bits 24 through 63 of the PRNG seed, respectively.  The bit and octet numbering conventions in 7.1.1 apply to the PRNG seed, secret key, and the IV. The numbering of the octets of the PRNG seed corresponds to that of the RC4 key.”

## Deprecation of dot11StationID

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID                         MacAddress,

dot11MediumOccupancyLimit              INTEGER,

dot11CFPollable                        TruthValue,

dot11CFPPeriod                         INTEGER,

dot11CFPMaxDuration                    INTEGER,

dot11AuthenticationResponseTimeOut     INTEGER,

dot11PrivacyOptionImplemented      INTEGER,

dot11PowerManagementMode			INTEGER,

dot11DesiredSSID				OCTET STRING,

dot11DesiredBSSType			INTEGER,

dot11OperationalRateSet		OCTET STRING,

dot11BeaconPeriod			INTEGER,

dot11DTIMPeriod				INTEGER,

dot11AssociationResponseTimeOut	INTEGER,

dot11DisassociateReason            INTEGER,

dot11DisassociateStation           MacAddress,

dot11DeauthenticateReason          INTEGER,

dot11DeauthenticateStation         MacAddress,

dot11AuthenticateFailStatus        INTEGER,

dot11AuthenticateFailStation       MacAddress }

dot11StationID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS deprecated

DESCRIPTION

"The purpose of dot11StationID is to allow a manager to identify

a station for its own purposes.  This attribute provides

for that eventuality while keeping the true MAC address

independent.  Its syntax is MAC address and default value

is the station's assigned, unique MAC address."

::= { dot11StationConfigEntry 1 }

## Setting ranges in the MIB on many attributes to minimize storage requirements

## Change the Registration of notification objects in the MIB to comply with SMIv2

The changes on the latter 2 items, and editorial changes, are provided on the next pages.

19. In 11.2.1.1 and 11.2.1.9, replace the first occurrence of “aListenInterval” with “the ListenInterval parameter of the MLME-Associate.request primitive”. Replace the second occurrence of “aListenInterval” in 11.2.1.9 with “ListenInterval”.

26. In  14.6.8, replace “72” in set 3 for North America/most of Europe with “71” and replace “fx(I)” with “fx(i)”.

34. In  15.3.2, Table 58, change:

“dot11CCAModeSupport” into “dot11CCAModeSupported”

“dot11SupportTxAntennas” into “dot11SupportedTxAntenna”

“dot11SupportRxAntennas” into “dot11SupportedRxAntenna”

“dot11DiversitySelectRx” into “dot11DiversitySelectionRx”

“agPhyOperationGroup” into “dot11PhyOperationComplianceGroup”

“agPhyRateGroup” into “dot11PhyRateGroup”

“agPhyAntennaGroup” into “dot11PhyAntennaComplianceGroup”

“agPhyTxPowerGroup” into “dot11PhyTxPowerComplianceGroup”

“agPhyDSSSGroup” into “dot11PhyDSSSComplianceGroup”

“agAntennasListGroup” into “dot11AntennasListGroup”

36. In  16.4, Table 74, delete the rows of the following attributes:

and add a row to Table 74

| dot11PhyTempType | X’01’ | Static | Identical for all conformant PHY |
| --- | --- | --- | --- |

-- **********************************************************************

-- * IEEE 802.11 Management Information Base

-- **********************************************************************

IEEE802dot11-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE,

NOTIFICATION-TYPE,Integer32, Counter32  FROM SNMPv2-SMI

DisplayString , MacAddress, RowStatus   FROM SNMPv2-TC

TruthValue                              FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP         FROM SNMPv2-CONF

ifIndex                                 FROM RFC1213-MIB;

-- **********************************************************************

-- *  MODULE IDENTITY

-- **********************************************************************

ieee802dot11 MODULE-IDENTITY

LAST-UPDATED "9805280000Z"

ORGANIZATION "IEEE 802.11"

CONTACT-INFO

"WG E-mail: stds-802-11@ieee.org

Chair: Vic Hayes

Postal: Lucent Technologies, Inc.

Zadelstede 1-10

Nieuwegein, Netherlands

3431 JZ

Tel: +31 30 609 7528

Fax: +31 30 231 6233

E-mail: vichayes@lucent.com

Editor: Bob O'Hara

Postal: Informed Technology, Inc.

151A Charles Street

New York, NY 10014 USA

Tel: +1 212 463 7937

Fax: +1 212 645 6719

E-mail: bob@informed-technology.com"

DESCRIPTION

"The MIB module for IEEE 802.11 entities.

iso(1).member-body(2).us(840).ieee802dot11(10036)"

::= { 1 2 840 10036 }

-- **********************************************************************

-- *  MIB attribute OBJECT-TYPE definitions follow

-- **********************************************************************

-- **********************************************************************

-- *  SMT Station Config  Table

-- **********************************************************************

dot11StationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11StationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Station Configuration attributes.  In tablular form to

allow for multiple instances on an agent."

::= {  dot11smt 1 }

dot11StationConfigEntry OBJECT-TYPE

SYNTAX Dot11StationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11StationConfigTable.  It is

possible for there to be multiple IEEE 802.11 interfaces

on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an

interface in the context of the Internet-standard MIB is

one-to-one.  As such, the value of an ifIndex object

instance can be directly used to identify corresponding

instances of the objects defined herein.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX {ifIndex}

::= {  dot11StationConfigTable 1 }

dot11MediumOccupancyLimit OBJECT-TYPE

SYNTAX INTEGER (0..1000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum amount of time,

in TU, that a point coordinator may control the usage of

the wireless medium without relinquishing control for long

enough to allow at least one instance of DCF access to the

medium.  The default value of this attribute shall be 100,

and the maximum value shall be 1000."

::= { dot11StationConfigEntry 2 }

dot11CFPollable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"When this attribute is true, it shall indicate that the STA

is able to respond to a CF-Poll with a data frame within a

SIFS time. This attribute shall be false if the STA is not

able to respond to a CF-Poll with a data frame within a SIFS

time."

::= { dot11StationConfigEntry 3 }

dot11CFPPeriod OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute shall describe the number of DTIM intervals

between the start of CFPs.  It is modified by

MLME-START.request primitive."

::= {  dot11StationConfigEntry 4 }

dot11CFPMaxDuration OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute shall describe the maximum duration of the CFP

in TU that may be generated by the PCF. It is modified by

MLME-START.request primitive."

::= {  dot11StationConfigEntry 5 }

dot11AuthenticationResponseTimeOut OBJECT-TYPE

SYNTAX INTEGER (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TU that a

responding STA should wait for the next frame in the

authentication sequence."

::= {  dot11StationConfigEntry 6 }

dot11PrivacyOptionImplemented OBJECT-TYPE

SYNTAX INTEGER {   true (1),   false (2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the IEEE

802.11 WEP option is implemented.  The default value of

this attribute shall be false."

::= {  dot11StationConfigEntry 7 }

dot11PowerManagementMode OBJECT-TYPE

SYNTAX INTEGER { active(1), powersave(2) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the power management

mode of the STA. When set to active, it shall indicate

that the station is not in power-save mode. When set

to powersave, it shall indicate that the station is

in power-save mode. The power management mode is

transmitted in all frames according to the rules

in  7.1.3.1.7."

::= { dot11StationConfigEntry 8 }

dot11DesiredSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute reflects the Service Set ID used

in the DesiredSSID parameter of the most recent

MLME_Scan.request.  This value may be modified

by an external management entity and used by the

local SME to make decisions about the Scanning process."

::= { dot11StationConfigEntry 9 }

dot11BeaconPeriod OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TU that a

station shall use for scheduling Beacon transmissions.

This value is transmitted in Beacon and Probe Response

frames."

::= { dot11StationConfigEntry 12 }

dot11DTIMPeriod OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of beacon

intervals that shall elapse between transmission of

Beacons frames containing a TIM element whose DTIM

Count field is 0. This value is transmitted in

the DTIM Period field of Beacon frames."

::= { dot11StationConfigEntry 13 }

dot11AssociationResponseTimeOut OBJECT-TYPE

SYNTAX INTEGER(1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TU that a

requesting STA should wait for a response to a

transmitted association-request MMPDU."

::= { dot11StationConfigEntry 14 }

Dot11AuthenticationAlgorithmsEntry ::= SEQUENCE {

dot11AuthenticationAlgorithmsIndex      Integer32,

dot11AuthenticationAlgorithm      INTEGER,

dot11AuthenticationAlgorithmsEnable		TruthValue

}

dot11AuthenticationAlgorithmsEnable  OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute, when true at a station, shall enable the acceptance of the authentication algorithm described in the corresponding table entry in authentication frames received by the station that have odd authentication sequence numbers.  The default value of this attribute

shall be 1 for the Open System table entry and 2 for all other table

entries."

::= {  dot11AuthenticationAlgorithmsEntry 3 }

Dot11WEPDefaultKeysEntry ::= SEQUENCE {

dot11WEPDefaultKeyIndex     INTEGER,

dot11WEPDefaultKeyValue     WEPKeytype}

dot11WEPDefaultKeyIndex OBJECT-TYPE

SYNTAX INTEGER (1..4)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the WEP Default Keys Table.

The value of this variable is equal to the WEPDefaultKeyID + 1"

::= {  dot11WEPDefaultKeysEntry 1 }

dot11WEPDefaultKeyValue OBJECT-TYPE

SYNTAX WEPKeytype

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A WEP default secret key value."

::= {  dot11WEPDefaultKeysEntry 2 }

Dot11WEPKeyMappingsEntry ::= SEQUENCE {

dot11WEPKeyMappingIndex     Integer32,

dot11WEPKeyMappingAddress       MacAddress,

dot11WEPKeyMappingWEPOn TruthValue,

dot11WEPKeyMappingValue    WEPKeytype,

dot11WEPKeyMappingStatus	RowStatus }

dot11WEPKeyMappingWEPOn OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Boolean as to whether WEP is to be used when communicating

with the dot11WEPKeyMappingAddress STA."

::= {  dot11WEPKeyMappingsEntry 3 }

dot11WEPKeyMappingValue OBJECT-TYPE

SYNTAX WEPKeytype

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"A WEP secret key value."

::= {  dot11WEPKeyMappingsEntry 4 }

-- **********************************************************************

-- *    dot11PrivacyTable  TABLE

-- **********************************************************************

dot11PrivacyTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PrivacyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes concerned with IEEE 802.11

Privacy.  Created as a table to allow multiple

instantiations on an agent."

::= {  dot11smt 5 }

dot11PrivacyEntry OBJECT-TYPE

SYNTAX Dot11PrivacyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PrivacyTable Table.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX {ifIndex}

::= {  dot11PrivacyTable 1 }

Dot11PrivacyEntry ::= SEQUENCE {

dot11PrivacyInvoked                INTEGER,

dot11WEPDefaultKeyID               INTEGER,

dot11WEPKeyMappingLength           Integer32,

dot11ExcludeUnencrypted            INTEGER,

dot11WEPICVErrorCount              Integer32,

dot11WEPExcludedCount              Integer32}

dot11WEPDefaultKeyID  OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the use of the first,

second, third, or fourth element of the WEPDefaultKeys

array when set to values of zero, one, two, or three.  The

default value of this attribute shall be 0."

REFERENCE "IEEE Std 802.11-1997, 8.3.2"

::= {  dot11PrivacyEntry 2 }

dot11Disassociate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11DisassociateReason,

dot11DisassociateStation }

STATUS current

DESCRIPTION

"The disassociate notification shall be sent when the STA

sends a Disassociation frame. The value of the notification

shall include the MAC address of the MAC to which the Disassociation

frame was sent and the reason for the disassociation.

ifIndex - Each 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 1 }

-- **********************************************************************

-- *    dot11OperationTable  TABLE

-- **********************************************************************

Dot11OperationEntry ::= SEQUENCE {

dot11MACAddress                 MacAddress,

dot11RTSThreshold               INTEGER,

dot11ShortRetryLimit            INTEGER,

dot11LongRetryLimit             INTEGER,

dot11FragmentationThreshold     INTEGER,

dot11MaxTransmitMSDULifetime    INTEGER,

dot11MaxReceiveLifetime         INTEGER,

dot11ManufacturerID             DisplayString,

dot11ProductID                  DisplayString}

dot11RTSThreshold OBJECT-TYPE

SYNTAX INTEGER (0..2347)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

dot11ShortRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

dot11LongRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

dot11FragmentationThreshold OBJECT-TYPE

SYNTAX INTEGER (256..2346)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the current maximum size, in

octets, of the MPDU that may be delivered to the PHY. An MSDU

shall be broken into fragments if its size exceeds the value

of this attribute after adding MAC headers and trailers. An MSDU or MMPDU shall be fragmented when the resulting frame has an individual address in the Address1 field, and the length of the frame is larger than this threshold. The default value for this attribute shall be the lesser of 2346 or the aMPDUMaxLength of the attached PHY and shall never exceed the lesser of 2346 or the aMPDUMaxLength of the attached PHY. The value of this attribute shall never be less than 256.

::= {  dot11OperationEntry 5 }

dot11MaxReceiveLifetime OBJECT-TYPE

SYNTAX INTEGER (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

dot11GroupAddressesIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify

dot11manufacturerOUI OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Takes the value of an organizationally unique identifier."

::= {  dot11ResourceInfoEntry 1 }

dot11manufacturerName OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A printable string used to identify the manufacturer of the

resource.  Maximum string length is 128 octets."

::= {  dot11ResourceInfoEntry 2 }

dot11manufacturerProductName OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A printable string used to identify the manufacturer's product

name of the resource.  Maximum string length is 128 octets."

::= {  dot11ResourceInfoEntry 3 }

dot11manufacturerProductVersion OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

dot11PhyOperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"PHY level attributes concerned with

operation.  Implemented as a table indexed on

ifIndex to allow for multiple instantiations on an

Agent."

::= {  dot11phy 1 }

dot11TempType OBJECT-TYPE

SYNTAX INTEGER {TempType1(1), TempType2(2), TempType3(3)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"There are different operating temperature requirements

dependent on the anticipated environmental conditions. This

attribute describes the current PHY's operating temperature

range capability. Currently defined values and their

corresponding temperature ranges are:

Type 1 = X'01'-Commercial range of 0 to 40 degrees C,

Type 2 = X'02'-Industrial range of

-30 to 70 degrees C."

::= {   dot11PhyOperationEntry 3 }

dot11PhyAntennaTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyAntennaEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for PhyAntenna.  Implemented as a

table indexed on ifIndex to allow for multiple instances on

an agent."

::= {  dot11phy 2}

dot11DiversitySupport OBJECT-TYPE

SYNTAX INTEGER {fixedlist(1), notsupported(2), dynamic(3)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This implementation's support for diversity, encoded as:

X'01'-diversity is available and is performed over the fixed

list of antennas defined in dot11DiversitySelectionRx.

X'02'-diversity is not supported.

X'03'-diversity is supported and control of diversity is also

available, in which case the attribute

dot11DiversitySelectionRx can be dynamically modified by the

LME."

::= {  dot11PhyAntennaEntry 2 }

Dot11PhyTxPowerEntry ::= SEQUENCE {

dot11NumberSupportedPowerLevels  INTEGER,

dot11TxPowerLevel1      INTEGER,

dot11TxPowerLevel2      INTEGER,

dot11TxPowerLevel3      INTEGER,

dot11TxPowerLevel4      INTEGER,

dot11TxPowerLevel5      INTEGER,

dot11TxPowerLevel6      INTEGER,

dot11TxPowerLevel7      INTEGER,

dot11TxPowerLevel8      INTEGER,

dot11CurrentTxPowerLevel    INTEGER}

dot11NumberSupportedPowerLevels OBJECT-TYPE

SYNTAX INTEGER (1..8)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of power levels supported by the PMD.

This attribute can have a value of 1 to 8."

::= {  dot11PhyTxPowerEntry 1 }

dot11TxPowerLevel1 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL1 in mW.

This is also the default power level."

::= {   dot11PhyTxPowerEntry 2 }

dot11TxPowerLevel2 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL2 in mW."

::= {   dot11PhyTxPowerEntry 3 }

dot11TxPowerLevel3 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL3 in mW."

::= {   dot11PhyTxPowerEntry 4 }

dot11TxPowerLevel4 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL4 in mW."

::= {   dot11PhyTxPowerEntry 5 }

dot11TxPowerLevel5 OBJECT-TYPE

SYNTAX INTEGER (0.10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL5 in mW."

::= {   dot11PhyTxPowerEntry 6 }

dot11TxPowerLevel6 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL6 in mW."

::= {   dot11PhyTxPowerEntry 7 }

dot11TxPowerLevel7 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL7 in mW."

::= {   dot11PhyTxPowerEntry 8 }

dot11TxPowerLevel8 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL8 in mW."

::= {   dot11PhyTxPowerEntry 9 }

dot11CurrentTxPowerLevel OBJECT-TYPE

SYNTAX INTEGER (1..8)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

Dot11PhyFHSSEntry ::= SEQUENCE {

dot11HopTime   INTEGER,

dot11CurrentChannelNumber  INTEGER,

dot11MaxDwellTime  INTEGER,

dot11CurrentDwellTime  INTEGER,

dot11CurrentSet    INTEGER,

dot11CurrentPattern    INTEGER,

dot11CurrentIndex  INTEGER}

dot11HopTime OBJECT-TYPE

SYNTAX INTEGER (224)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time in microseconds for the PMD to change from

channel 2 to channel 80"

::= {   dot11PhyFHSSEntry 1 }

dot11CurrentChannelNumber OBJECT-TYPE

SYNTAX INTEGER (0..99)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current channel number of the frequency output by the RF

synthesizer"

::= {   dot11PhyFHSSEntry 2 }

dot11MaxDwellTime OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum time in TU that the transmitter

is permitted to operate on a single channel."

::= {   dot11PhyFHSSEntry 3 }

dot11CurrentDwellTime OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current time in TU that the transmitter shall operate

on a single channel, as set by the MAC.  Default is 19 TU."

::= {   dot11PhyFHSSEntry 4 }

dot11CurrentSet OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current set of patterns the PHY

LME is using to determine the hopping sequence. "

::= {   dot11PhyFHSSEntry 5 }

dot11CurrentPattern OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current pattern the PHY LME is

using to determine the hop sequence."

::= {   dot11PhyFHSSEntry 6 }

dot11CurrentIndex OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

dot11PhyDSSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyDSSSEntry.  Implemented as a

table indexed on ifIndex allow for multiple instances on

an Agent."

::= {  dot11phy 5 }

Dot11PhyDSSSEntry ::= SEQUENCE {

dot11CurrentChannel    INTEGER,

dot11CCAModeSupported  INTEGER,

dot11CurrentCCAMode    INTEGER,

dot11EDThreshold   Integer32}

dot11CurrentChannel OBJECT-TYPE

SYNTAX INTEGER (1..14)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current operating frequency channel of the DSSS

PHY. Valid channel numbers are as defined in 15.4.6.2"

::= {   dot11PhyDSSSEntry 1 }

dot11CCAModeSupported OBJECT-TYPE

SYNTAX INTEGER (1..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11CCAModeSupported is a bit-significant value, representing all of the CCA modes supported by the PHY.  Valid values are:

energy detect only (ED_ONLY) = 01,

carrier sense only (CS_ONLY) = 02,

carrier sense and energy detect (ED_and_CS)= 04

or the logical sum of any of these values."

::= {   dot11PhyDSSSEntry 2 }

dot11CurrentCCAMode OBJECT-TYPE

SYNTAX INTEGER {ED_ONLY(1), CS_ONLY(2), ED_and_CS(4)}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

dot11WEPKeyMappingWEPOn,

dot11WEPKeyMappingValue , dot11WEPKeyMappingAddress,

dot11WEPKeyMappingStatus }

STATUS current

DESCRIPTION

"The SMTPrivacy package is a set of attributes that shall be

present if WEP is implemented in the STA."

::= {dot11Groups 2 }

dot11PhyIRComplianceGroup OBJECT-GROUP

OBJECTS {dot11CCAWatchdogTimerMax, dot11CCAWatchdogCountMax,

dot11CCAWatchdogTimerMin, dot11CCAWatchdogCountMin}

STATUS current

DESCRIPTION

"Attributes that configure the baseband IR for IEEE 802.11."

::= { dot11Groups 12 }

103. In 9.3.3.3, add the following text before the last sentence of the third paragraph : “MaxMPDUTime is the time to transmit the maximum-sized MAC frame, expanded by WEP, plus the time to transmit any PHY preamble, header, trailer, and expansion bits, if any.”

104. In 10.3 and all of its subclauses, replace every occurrence of “1 through 127” with “2 through 127”.